DRAFT TECHNICAL MEMO

An Analysis of Biomass Burning Impacts on Texas

Task 2: Development of STILT-ASP Model into a wildfire analysis tool

Deliverable 2.1: Draft Technical Memo describing and documenting the initial version of the wildfire analysis tool

TCEQ Contract No. 582-15-50414

Work Order No. 582-16-62311-03

Revision 1.0

Prepared by:

Atmospheric and Environmental Research, Inc. (AER)

131 Hartwell Ave.

Lexington, MA 02466

Correspondence to: malvarad@aer.com

Prepared for:

Erik Gribbin

Texas Commission on Environmental Quality

Air Quality Division

Building E, Room 342S

Austin, Texas 78711-3087

May 13, 2016

Document Change Record

| Revision | Revision Date | Remarks |
| --- | --- | --- |
| 0.1 | 12 May 2016 | Internal Version for Review |
| 1.0 | 13 May 2016 | Draft Version Submitted to TCEQ |

Table of Contents

1	Introduction	7

2	Conceptual Model	7

3	STILT-ASP Model Description	8

3.1	STILT Back-Trajectory Calculations	9

3.2	ASP Gas and Aerosol Chemistry Calculations	10

3.2.1	Gas-Phase Chemistry Updates	11

3.2.2	Organic Aerosol Updates	11

3.3	STILT-Chem Initialization, Emission, and Deposition Routines	13

3.3.1	Initialization	13

3.3.2	Emissions	13

3.3.3	Deposition Routines	14

3.3.4	Mixing	15

3.4	Coupling of STILT-Chem and ASP	16

3.4.1	Mapping to ASP Species	16

3.4.2	Tracking Fire-Influenced Parcels	16

3.5	Emission Preprocessor	17

3.5.1	FINN v1.5 Fire Emissions	17

3.5.2	HTAP v2 Anthropogenic Emissions	17

3.5.3	MEGAN v2.1 Biogenic Emissions	18

4	Software Descriptions	18

4.1	Back-trajectory calculation	18

4.1.1	Back-trajectory Software	18

4.1.2	Back-trajectory Inputs	19

4.1.3	Back-trajectory Processing	19

4.1.4	Back-trajectory Output	21

4.2	Forward Chemistry Calculations	23

4.2.1	Forward Chemistry Calculations Software	23

4.2.2	Forward Chemistry Calculations Inputs	24

4.2.3	Forward Chemistry Calculations Processing	24

4.2.4	Forward Chemistry Calculators Output	26

4.3	Emission Preprocessor	26

4.3.1	Emission Preprocessor Software	26

4.3.2	Emission Preprocessor Inputs	27

4.3.3	Emission Preprocessor Processing	28

4.3.4	Emission Preprocessor Output	28

5	Model Calibration, Validation, and Evaluation	29

5.1	Previous Evaluation of STILT and ASP	29

5.2	Testing Back-Trajectory Calculations of STILT-ASP versus STILT	30

5.3	Preliminary O3 runs for Houston and Austin Biomass Burning Events	34

6	Summary	35

7	References	37

List of Figures

Figure 1. Flow chart of the STILT-ASP forward chemistry calculations.	25

Figure 2. Flow chart of the emission preprocessor.	28

Figure 3. O3 (left) and PAN (right) mixing ratios within the Williams fire as observed by Akagi et al. (2012) and simulated by ASP. Asterisks are the measured mixing ratios, with the horizontal error bars showing the uncertainty in the estimated age and the vertical error bars showing the uncertainty in the measurement. Red, black, and green are ASP results for the slow, best-fit (medium), and fast plume dilution rates. Dashed lines are for above-plume photolysis rates, while solid lines are for the middle of the plume, and dotted lines are for the bottom of the plume.	30

Figure 4. Enhancement ratio (g/g) of organic aerosol (OA) to CO2 versus smoke age for the Williams fire. All model results assume the best-fit dilution rate and the photolysis rates corresponding to the middle of the plume. The red circles are the observed values from Akagi et al. (2012).	30

Figure 5. Latitude (top), longitude (middle), and altitude (m) of the 500 particle backtrajectories for the STILT-ASP Windows code (left) and the STILT Linux code (right). Both models were run with identical random number seeds, but with different compilers on different machines.	31

Figure 6. Mean (red), median (blue), and standard deviation (magenta) of the 500 particle back-trajectories for the STILT-ASP Windows code (left) and the STILT Linux code (right). Both models were run with identical random number seeds, but with different compilers on different machines.	32

Figure 7. Mean (red), median (blue), and standard deviation (magenta) of the 500 particle back-trajectories for the STILT-ASP Windows code with a fixed random seed (left) and the random seed based on the system clock (right).	33

Figure 8. STILT-ASP O3 calculated for 11:00 CST on Sept. 7th, 2011 at EPA Site Number 482090614 in Austin, TX using the default fire, anthropogenic, and biogenic emissions from the STILT-ASP emission preprocessor and the NAM12 meteorology.	34

Figure 9. As in Figure 6, but with only non-fire (anthropogenic and biogenic) emissions included.	35

List of Tables

Table 1. Output parameters in PARTICLE.nc file	21

# Introduction

This draft technical memo, along with the draft User’s Guide, completes Deliverable 2.1 of Work Order No. 582-15-54118-01. As stated in the Work Plan, this deliverable is:

Deliverable 2.1:  	A Draft Technical Memo describing and documenting the initial version of the wildfire analysis tool and a Draft User’s Guide for using the tool.

Deliverable Due Date:	May 13, 2016

With the recent establishment of the 70 parts per billion ozone National Ambient Air Quality Standard (NAAQS) the State of Texas has become more susceptible to NAAQS exceedances caused by the transport of ozone, PM2.5, and their precursors from wildfires outside of Texas. The development of an analysis tool that can utilize meteorological and photochemical methods to assess wildfire impacts without a SIP-level modeling effort would be a significant benefit to TCEQ analysts documenting exceptional events.

The purpose of Task 2 of this project is thus to further the development of a trajectory-based modeling tool that will allow the Air Modeling and Data Analysis Section to assess the impact of certain wildfire events on ozone (O3) or fine particulate matter (PM2.5) measurements in the metropolitan areas of Texas. This modeling tool is comprised of the Stochastic Time Inverted Lagrangian Transport (STILT) model (e.g., Lin et al., 2003; Wen et al., 2012, 2013, 2014) with an integrated Aerosol Simulation Program (ASP, Alvarado, 2008; Alvarado and Prinn, 2009; Alvarado et al., 2015). The objective is to update and modify the existing STILT-ASP tool to (a) make it easier to use by TCEQ personnel to analyze the impacts of biomass burning on O3 and PM2.5 in urban areas in Texas and (b) make the tool easier to use on TCEQ computers by modifying the code to run on Windows 7 64-bit hardware with a Graphical User Interface (GUI).

Section 2 of this memo describes the conceptual model for the system, while Section 3 briefly outlines the model equations, assumptions, and limiting conditions on applications, with a focus on those that have not been previously documented in the literature. Section 4 briefly describes the model software routines inputs, processing, and outputs while Section 5 describes the efforts in model calibration, validations, and evaluation to date.

In addition to this technical memo, five other documents are included in this deliverable that help to describe the algorithms, use, and limitations of STILT-ASP. First, the companion STILT-ASP User’s Guide describes how to run the model through the GUI. Second, the thesis of Alvarado (2008) includes detailed information on the algorithms of ASP. Third, Lin et al. (2003) describes the basic equations of STILT. Fourth, Draxler and Hess (1997) describe several of the algorithms of HYSPLIT that are also included in STILT-ASP, including the advection and deposition algorithms. Fifth, the HYSPLIT User’s Guide describes the format of some of the input files that are common to HYSPLIT and STILT-ASP.

# Conceptual Model

The conceptual model for this system is that the O3 and PM2.5 concentrations measured at a given monitor at a specific time (the “receptor”) are due to the sum of the contributions of many Lagrangian air parcels that arrive at the receptor at the same time. The concentrations of O3 and PM2.5 in these parcels is, in turn, determined by an initial concentration value at a starting “boundary” location that is then modified by emissions, deposition, mixing, and chemistry as the parcel is transported from the starting location.

The goal of the STILT-ASP wildfire analysis tool developed in this project is to quantitatively estimate the contributions of fires to the O3 and PM2.5 concentrations measured at a receptor. This is done by identifying parcels that were influenced by fire emissions during their transport to the receptor, determining the contribution of the fires to the primary emission (for PM2.5) and secondary chemical formation (for both O3 and PM2.5) of the pollutants in the parcel, and then summing these fire contributions across all parcels at the receptor.

The secondary chemical formation of O3 in parcels influenced by fire emissions is generally a function of the ratio of the fire emissions of NOx to the fire emissions of non-methane organic compounds (NMOCs), which is in turn determined by the combustion efficiency and the amount of nitrogen in the fuel (e.g., Jaffe and Widger, 2012; Jaffe et al., 2013; Baylon et al., 2014; Alvarado et al., 2015). Other environmental parameters, such as temperature and shortwave (solar) actinic flux, also alter the amount of O3 that can be formed as the parcel is transported. This chemistry can be adequately simulated with small adjustments to current atmospheric chemical mechanisms (e.g., Alvarado et al., 2015).

Primary emissions of PM2.5 from fires are dominated by organic aerosol (OA), with smaller contributions from black carbon (BC) and inorganic salts (e.g., Akagi et al., 2011). The secondary production of PM2.5 in parcels influenced by fire emissions is generally determined by the oxidation chemistry of the semi-volatile and intermediate volatility organic compounds (S/IVOCs) present in the fire emissions. This chemistry can be adequately simulated using a set of “average” reaction rates with OH and product yields for a small set of S/IVOC compounds (e.g., Alvarado et al., 2015).

# STILT-ASP Model Description

A STILT-ASP air quality simulation begins with a stochastic back-trajectory simulation (Section 3.1), followed by a forward chemical simulation that determines tracer concentrations along the generated back trajectories (e.g., Wen et al., 2012). After the back trajectories are calculated, the model solves the Lagrangian chemical continuity equations:

dCqdt=dCqdtemis-βd,q+βw,qCq+dCqdtcond+dCqdtchem+dCqdtmix 		(1)

dnidt=dnidtemis-βd,q+βw,qni+dnidtcond+dnidtcoag+dnidtmix 		(2)

dcq,idt=dcq,idtemis-βd,q+βw,qcq,i+dcq,idtcond+dcq,idtcoag+dcq,idtchem+dcq,idtmix (3)

where Cq is the concentration of gas-phase species q (molecules/cm3 air), βd,q and βw,q are the wet and dry deposition velocities, respectively, for species q, ni is the number concentration of particles in size bin i (particles/cm3 air), cq,i = cq,ini is the concentration of aerosol species q in size bin i (mol/cm3 air) and cq,i is the concentration in each aerosol particle of size bin i (mol/particle). STILT-ASP calculates changes of Cq, ni, and cq,i prognostically, while changes in cq,I are calculated diagnostically.

The concentrations of modeled species are initialized at the endpoint of each back-trajectory using values outputted from a global chemical transport model (Section 3.3.1). Then the concentrations are evolved forward in time along each trajectory to take into consideration the influences of chemical transformation and aerosol dynamics (Section 3.2), emission (Section 3.3.2), deposition (Section 3.3.3), and mixing (Section 3.3.4). Advection, diffusion, emission and deposition in the model are computed in a Lagrangian framework while chemistry is calculated on a fixed grid using a particle-in-grid method (Chock and Winkler, 1994; Stein et al., 2000). The calculation of each of these processes is described in more detail below.

Several changes had to be made to the STILT-Chem framework of Wen et al. (2012) to allow the model to use ASP for chemistry and aerosol calculations. STILT-Chem is described in Section 3.3 and these changes are described in Section 3.4. In addition, we added the ability for STILT-ASP to differentiate between emissions from biomass burning and other sources, and to use “default” values for anthropogenic and biogenic emissions when a full bottom-up emission calculation is not available. This required the development of an emissions preprocessor, which is described in Section 3.5.

## STILT Back-Trajectory Calculations

The STILT model (http://www.stiltmodel.org; Lin et al., 2003) is a Lagrangian particle dispersion model derived from the Hybrid Single-Particle Lagrangian Integrated Trajectory (HYSPLIT) model but that includes a more sophisticated turbulence scheme and modifications that improve the mass-conservation of the simulations and allow the use of customized Weather Research and Forecasting model (WRF) meteorological fields (Nehrkorn et al., 2010), which have been shown to improve the model performance when compared with tracer-release studies (e.g., Hegarty et al., 2013). STILT has been extensively used in inverse modeling to improve emission estimates for greenhouse gases (e.g., McKain et al., 2012, 2015; Miller et al., 2013; Henderson et al., 2015).

The algorithms and equations of STILT are documented in Lin et al. (2003). The STILT model is built upon source code from HYSPLIT (Draxler and Hess, 1997; 1998), using the mean advection scheme from HYSPLIT but employing a different turbulent module. Briefly, STILT assumes that turbulent diffusion of Lagrangian air parcels of equal mass can be modeled as a Markov chain where the particle velocity vector u can be decomposed into a mean component u and a turbulent component u' with the turbulent component following the relation:

u't+∆t=R∆tu't+u''t		(4)

where u″ is a random vector, R is an autocorrelation coefficient, and Δt is the time step. The autocorrelation coefficient R in Equation 1 follows an exponential form R(Δt) = exp(−Δt/TLi), where TLi is the Lagrangian timescale in the i direction (i=u, horizontal; i=w, vertical). TLi is a decorrelation timescale that determines the degree to which the particle's movement behaves like a random walk (TLi = 0) or like advection by mean wind (large TLi). The random velocity u″ in Equation 1 is defined by the relation:

u''=λ1-R2∆t12			(5)

where λ is a random variable usually drawn from a Gaussian distribution with a mean of 0 and a standard deviation, σi, which characterizes the spread in the random velocity. This means that STILT runs are inherently stochastic, as different machine architectures and different random seeds will lead to different selections of λ.

Integrating Equation 1 for an ensemble of particle trajectories (usually of 500 particles) backward in time characterizes the turbulent flow of Lagrangian air parcels toward a given receptor (location and time point). To minimize errors resulting from horizontal interpolation of model variables, the STILT model is designed to perform all computations on the horizontal grid of the meteorological input dataset (Nehrkorn et al., 2010). Particles that reach the edge of the model grid are terminated; in applications using nested grids, computations are restarted using meteorological fields from outer nests. The mean winds from the meteorological model used to drive STILT are used to directly calculate the mean (non-stochastic) velocities. The horizontal Lagrangian timescale TLu is assumed to be 3 hours (Draxler and Hess, 1998) while the parameters TLw and σw characterizing vertical turbulence are represented, following Hanna (1982), as functions of the planetary boundary layer height, roughness length, Monin-Obukhov length, convective velocity scale, and vertical velocity (Lin et al., 2003). STILT also adopted the reflection/transmission scheme for Gaussian turbulence from Thompson et al. (1997), which preserves well-mixed distributions of particles (i.e., no unphysical concentration of particles) across interfaces with step-changes in turbulence parameters (Lin et al., 2003).

A variety of meteorological models can be used to prepare inputs for STILT back-trajectory calculations. However, for STILT-ASP we recommend that users use the National Centers for Environmental Prediction (NCEP) National Center for Atmospheric Research Reanalysis (NARR) 32 km and North American Mesoscale (NAM) 12 km wind fields supplied on the NOAA Air Resources Laboratory (ARL) website (e.g., http://ready.arl.noaa.gov/archives.php), as these fields are publically available and have been tested for use in STILT-ASP as well as in many previous STILT and HYSPLT studies.

## ASP Gas and Aerosol Chemistry Calculations

ASP v2.1 (Alvarado et al., 2015) simulates the gas-phase, aerosol-phase, and heterogeneous chemistry of young biomass-burning smoke plumes, including the formation of O3 and secondary inorganic and organic aerosol. The algorithms and equations of ASP v1.0 are fully documented in Alvarado (2008). Modifications to the current version of ASP v2.1 are documented in Alvarado et al. (2015). In general, Alvarado et al. (2015) made improvements to the chemical mechanism and other input files used in ASP, rather than changes to the fundamental model equations and algorithms described in Alvarado (2008).

The terms in Equations 1-3 that are calculated by ASP represent the change in gas and particle-phase concentrations due to mass transfer between the gas and aerosol phases (cond), coagulation of particles (coag), and irreversible and reversible chemistry (chem). The integration of the different terms of the continuity equations is operator split for computational efficiency. The calculation of these terms are performed by ASP as described in detail in Chapter 2 of Alvarado (2008), and are described briefly below.

### Overview of ASP Algorithms

Aerosols are represented in ASP by a single moving-center sectional size distribution, where the aerosol concentrations are distributed over increments in radius space (Jacobson 1997, 2002, 2005). ASP includes modules to calculate aerosol thermodynamics, gas-to-aerosol mass transfer (condensation/evaporation), and coagulation of aerosols. The aerosol thermodynamics module in ASP uses the Mass Flux Iteration (MFI) method of Jacobson (2005) to calculate the equilibrium concentration of gas and aerosol species. Equilibrium constants for the inorganic electrolyte reactions match those of Fountoukis and Nenes (2007). Binary activity coefficients of inorganic electrolytes are calculated using the Kusik-Meissner method (Kusik and Meissner, 1978; Resch, 1995), as are the mean activity coefficients. The water content of inorganic aerosols is calculated with an iterative routine that calculates water activities for aqueous solutions of a single electrolyte using a formula based on the Gibbs-Duhem equation (Steele, 2004). Steele (2004) and Alvarado (2008) found this approach compares well with other inorganic aerosol thermodynamics models such as ISORROPIA (Nenes et al., 1998; Fountoukis and Nenes, 2007).

Mass transfer between the gas and aerosol phases is calculated in ASP using a hybrid scheme where the flux-limited kinetic equations governing the condensation/evaporation of H2SO4 and organic species are integrated, whereas NH3, HNO3, and HCl are assumed to be in equilibrium (Alvarado, 2008). Aerosol coagulation is calculated using the semi-implicit scheme of Jacobson (2005) with a Brownian coagulation kernel.

### Gas-Phase Chemistry Updates

The gas-phase chemistry within the ASP model for Version 2.1 has been completely revised from ASP v1.0, which used the CalTech Atmospheric Chemistry Mechanism (CACM, Griffin et al., 2005). The revised ASP v2.1 gas phase chemical mechanism includes 1608 reactions between 621 species. All inorganic gas-phase chemistry within ASP v2.1 was updated to follow the IUPAC recommendations (Atkinson et al., 2004; updated data downloaded from http://iupac.pole-ether.fr/, accessed June 2012).

All gas-phase chemistry for organic compounds containing 4 carbons or less has been “unlumped,” i.e. the chemistry for each individual organic compound is explicitly resolved. This was done by following the reactions of the Leeds Master Chemical Mechanism (MCM) v3.2 (http://mcm.leeds.ac.uk/MCM/, accessed June 2012; Jenkin et al., 1997, 2003; Saunders et al., 2003; Bloss et al., 2005) for these species.

The chemical mechanism of isoprene within ASP v2.1 has been updated to follow the Paulot et al. (2009a,b) isoprene scheme, as implemented in GEOS-Chem and including corrections based on more recent studies (e.g., Crounse et al., 2011, 2012). The (lumped) chemistry for all other organic compounds in ASP has been updated to follow the Regional Atmospheric Chemistry Mechanism (RACM) v2 (Goliff et al., 2013).

Photolysis rates are calculated offline using the Tropospheric Ultraviolet and Visible (TUV) radiation model version 5.0 (Madronich and Flocke, 1998) for a cloud-free sky and an altitude of 1 km and solar zenith angles between 0o and 180o, which are then linearly interpolated in ASP to match the input SZA.

### Organic Aerosol Updates

The SOA formation module in ASP v2.1 was updated to follow the semi-empirical Volatility Basis Set (VBS) model of Robinson et al. (2007). Our implementation of this scheme followed the approach used by Ahmadov et al. (2012) to link the VBS scheme with the RACM chemical mechanism within WRF-Chem. We use 9 surrogates or “bins” for semi-volatile, intermediate volatility, low volatility, and extremely low volatility organic compounds (hereafter collectively referred to as “SVOCs” for simplicity) as in Dzepina et al., 2009, rather than only 4 as in Ahmadov et al. (2012). The saturation mass concentration at 300 K (C*, see Robinson et al., 2007) of each SVOC bin differs by a factor of 10, and covers the range from 0.01 to 1.0×106 μg/m3. Note that “SVOC” as defined in this paper includes both semivolatile organic compounds (C* between 10-2 and 103 µg m-3) and intermediate volatility organic compounds (species with C* between 104 and 106 µg m-3) as defined in Dzepina et al. (2009), but we refer to both of these species classes collectively as “SVOCs” rather than as “S/IVOCs” as in Dzepina et al. (2009) for simplicity. Following the Model to Predict the Multiphase Partitioning of Organics (MPMPO) of Griffin et al. (2003, 2005) and Pun et al. (2002), we assumed that an aqueous phase and a mixed hydrophobic organic phase are always present in the aerosol. Partitioning of organics between the gas and hydrophobic phase is governed by Raoult’s law (assuming that all hydrophobic-phase OM is quasi-liquid and can dissolve organics as in Pankow, 1994a,b), while partitioning of organics into the aqueous phase is governed by Henry’s law. Following Pun et al. (2002), we assumed that (1) there is no interaction between the aqueous phase inorganic ions and the aqueous phase organics, and thus no organic salt formation, and (2) the activity coefficients of the organic ions (formed by the dissociation of organic acids) are equivalent to those of the corresponding molecular solute. We further assumed that the pH of the aqueous phase is dominated by the strong inorganic acids and bases, and that the pH effects of the dissociating organic acids are negligible.

Like most organic compounds, SVOCs will react with OH. Most mechanisms for this chemistry (e.g., Robinson et al., 2007; Dzepina et al., 2009; Grieshop et al., 2009a,b; Ahmadov et al., 2012) parameterize this chemistry by assuming that the SVOCs react with OH to form a lower volatility SVOC, as in the reaction:

(R1)

where μ is the relative mass gain due to oxidation (e.g. via O addition), kOH is the reaction rate with OH, and n is the “volatility shift”, or by how many factors of 10 to lower the C* of the product with each OH reaction. This simplified chemistry can be extended to account for the fact that the SVOCs could fragment during oxidation, leading to higher volatility products:

(R2)

where α is the fraction of SVOCi that fragments into SVOCi+1 and VOCj. Shrivastava et al. (2013) used a similar approach to show that adding SVOC fragmentation to WRF-Chem simulations of the Mexico City Plateau improved the model’s ability to simulate the observed concentrations of SOA. However, the highly simplified chemistry of Reactions R1 or R2 is not appropriate for situations where reactions with the SVOC compounds are a potentially significant sink of OH, such as in a concentrated smoke plume. Thus in ASP v2.1, the average, lumped chemistry of the SVOCs is instead parameterized in a more realistic manner for a generic organic species, following the idea of “mechanistic reactivity” (e.g., Carter, 1994; Bowman and Seinfeld, 1994a,b; Seinfeld and Pandis, 1998). After reaction with OH SVOCs produce peroxy radicals (RO2), which can react with NO to form NO2 and HO2, thereby regenerating OH and forming O3. Reactions R3 and R4 show this more general chemical mechanism for the SVOCs:

(R3)

(R4)

where  is assumed to be 4.010-12 cm3 molecule-1 s-1 based on the reaction rate for the peroxy radicals from long-chain alkanes and alkenes with NO in RACM2 (Goliff et al., 2013). We can see that χ-β is the number of NOx lost (implicitly via the addition of a nitrate group to the product SVOCs), 1-δ is the number of HOx lost, and β+δ is the number of O3 made per reaction (by subsequent reactions of NO2 and HO2 to generate O3). For example, the values for long-chain alkanes (HC8) in the RACM2 mechanism (Goliff et al., 2013) would be χ = 1, δ = 0.63, and β = 0.74, such that 0.26 NOx and 0.37 HOx are lost and 1.37 O3 are formed per reaction. Note that the mechanism of Reactions R3 and R4 is still highly simplified: we assume that reaction of SVOC with OH always produces a RO2 radical, and that the RO2 produced does not react with HO2 or another RO2. Also note that Reactions R3 and R4 represent the average chemistry of the unknown species collectively, and may not apply to any individual species in that mixture.

We also adjusted the calculation of aerosol water content to use the “kappa” (κ) parameterization of organic hygroscopicity (Petters and Kreidenweis, 2007) for the lumped SVOCs. In this parameterization, the hygroscopicity parameter κ for the organic species is defined as:

(6)

where aw is the activity of water in the solution (equal to the relative humidity at equilibrium), Vs,i is the volume of the dry organic solute i and Vw,i is the volume of water in the solution. The water content calculated for each organic species, along with that calculated for the inorganic solution (Vw,inorg) are then combined using the Zdanovskii, Stokes, and Robinson (ZSR) approximation (Zdanovskii, 1948; Stokes and Robinson, 1966):

(7)

## STILT-Chem Initialization, Emission, and Deposition Routines

STILT-ASP uses the initialization, emission, and deposition routines from STILT-Chem which are described in detail in Wen et al (2012, 2013, 2014), and so these routines are only described briefly here. The major differences between STILT-Chem and STILT-ASP are the use of ASP as the chemistry and aerosol dynamics solver and various modifications made to STILT-Chem to make it easier to use on stand-alone Windows machines.

### Initialization

After back trajectories are calculated, the concentrations of modeled species are initialized at the endpoint of each back-trajectory using values outputted from a global chemical transport model. In STILT-ASP, we use output from the MOZART-4/GEOS-5 output from the NCAR Chemical Forecasts (http://www.acom.ucar.edu/acresp/forecast/) to initialize the concentration in each Lagrangian parcel, with the species concentrations mapped from the MOZART-4 speciation to the ASP speciation as described in Section 3.4.

### Emissions

When 4D (x, y, z, t) data on emissions is available, as from the Sparse Matrix Operator Kernel Emissions (SMOKE) emissions processing model, changes in particle concentrations are calculated as in Section 2.3 of Wen et al. (2012). However, the “default” emissions calculated with the emission preprocessor (Section 3.5) are only available as 3D (x, y, t) fields, as we don’t have data on the vertical distribution of these emissions. Thus for these emissions we assume that the emitted species are well-mixed within the boundary layer. Thus for gas-phase species q, the rate of change in the molar mixing ratio due to emissions is (Xq=Cqρ, ppmv):

dXqdtemis=FqρHPBL         z≤HPBL		(8)

where Fq is the molar flux of species q (μmol/m2/hr), ρ=PRT is the average molar air density in the PBL (mol/m3), and HPBL is the PBL height (m).

Similar equations are used for emissions of aerosol species:

dnidtemis=QniHPBL         z≤HPBL		(9)

dcq,idtemis=Qcq,iHPBL         z≤HPBL		(10)

where Qni is the number flux of aerosol particles in size bin i (particles/cm2/hr) and Qcq,i is the molar flux of species q in size bin i (mol/cm2/hr) and HPBL is in cm.

### Deposition Routines

Deposition in STILT-ASP is identical to its treatment in STILT-Chem (e.g., Wen et al., 2012). Accordingly, the concentration change of gas-phase species q in a Lagrangian parcel due to dry and wet deposition is expressed in terms of time constants:

dCqdtdep=-βd,q+βw,qCq		(11)

where βd,q and βw,q are time constants for dry and wet deposition for gas-phase species q respectively.

The time constant for dry deposition can be expressed as:

βd,q=Vdry,qZs		(12)

where Vdry,q (cm/s) is the dry deposition velocity for species q. The dry deposition velocities for gas-phase species can be either calculated using a resistance-in-series scheme (Section 5.5 of Draxler and Hess, 1997) within STILT-ASP, or provided explicitly. Dry deposition is only estimated when a Lagrangian parcel moves into the lowest model level, the depth of which is Zs and which is assumed to be the top of the surface layer.

Wet deposition of gases is represented via loss rates computed based on the large-scale and convective precipitation rates. The wet deposition of gases depends upon their solubility. The influences of aqueous phase reactions on the wet deposition are assumed negligible and are not considered in STILT-ASP. For non-reactive gases the wet deposition is a function of the effective Henry’s Law constant. The gaseous wet deposition velocity for species q can be defined as (Draxler and Hess, 1997):

Vwet,q=HqRTPrain		(13)

where R is the universal gas constant and T and Prain are, respectively, air temperature and precipitation rate in a Lagrangian parcel. Hq is the effective Henry’s Law constant of species q.

Gaseous wet removal only occurs for the fraction of the pollutant below the cloud top. The gaseous wet removal time constant is given by:

βw,q=FtVwet,qZp		(14)

where Zp is the depth of the meteorological layer in which the particle is found. Ft is the fraction of the layer that is below the cloud top.

For aerosol species, dry and wet depositon rates are the same for all species within a given size bin. Thus the deposition equations are:

dnidtdep=-βd,i+βinc+βbelni		(15)

dcq,idtdep=-βd,i+βinc+βbelcq,i		(16)

where βd,i, βinc and βbel are time constants for dry deposition, in cloud wet deposition, and below-cloud wet deposition for aerosols in size bin i, respectively.

The dry deposition velocity is the gravitational settling velocity (Vt,i) for the particle calculated within ASP (Eq. 2.121 in Alvarado, 2008). The dry deposition time constant for aerosols is thus:

βd,i=Vt,iZs		(17)

For wet deposition of particulate pollutants, the simplifying assumption of a scavenging ratio is assumed for pollutants located within a cloud layer and the scavenging coefficient is used for pollutant removal in rain below a cloud layer. At the grid points where it is raining, the cloud bottom is defined at the level when the RH first reaches 80% and the cloud top is reached when the RH drops below 60%. All removal amounts are adjusted by the fraction of the pollutant mass that is within the cloud layer by defining the fraction of the pollutant layer that is below the cloud top (Ft) and the fraction of the pollutant layer that is above the cloud bottom (Fb).

For the wet removal of particles by within-cloud processes a scavenging ratio (S = 5.0x105 by volume), which is the ratio of the pollutant's concentration in water to its concentration in air, is expressed as a wet deposition velocity (Draxler and Hess, 1997):

Vinc=SPrain		(18)

The time constant for within-cloud removal is then:

βinc=FtFbVincZp		(19)

Below-cloud removal is then defined directly as a rate constant independent of the precipitation rate.

βbel=8×10-51.0-Fb		(20)

### Mixing

Like STILT-Chem (Wen et al., 2012), STILT-ASP uses the particle-in-grid approach to simulate chemistry, and so uniform mixing of particles in each grid cell is assumed and conducted before the chemical transformations are performed. The meteorological grid is used as the default grid for particle mixing and chemistry. That means particles are well mixed in each cell of the meteorological grid, and each cell is treated as a reactor.

After the chemical transformations have been calculated, the resulting concentrations are then used to update the concentration of each chemical compound in each particle following the method from Stein et al. (2000).

## Coupling of STILT-Chem and ASP

The coupling of STILT-Chem and ASP generally follows the techniques used in Alvarado et al. (2009). In the forward chemistry calculations of STILT-ASP a subroutine is called to initialize the ASP chemical mechanism, aerosol thermodynamics, and photolysis rate input files for ASP, which are stored in ASCII text files. This allows the chemistry of ASP to be changed without recompiling the source code, but changes to these files should be made with caution.

The main interface between the models is called for each Lagrangian parcel at each time step. The temperature, pressure, air density, solar zenith angle, and gas and aerosol concentrations for the parcel are passed from STILT-Chem. The units are converted to match the ASP routines and the state of the ASP chemical “box” (e.g., temperature, pressure, gas concentration, aerosol number and mass concentrations) is updated to match that of the Lagrangian parcel. ASP then calculates the changes in gas and aerosol concentrations due to gas-phase chemistry, aerosol thermodynamics, gas-aerosol mass transfer, and coagulation, and passes the updated concentrations back to STILT-Chem. The concentrations in the Lagrangian parcel are then updated to match the ASP output.

### Mapping to ASP Species

As noted in Section 3.2.2, ASP v2.1 contains 621 individual chemical species. However, in many cases STILT-ASP has to use inputs for emissions or boundary conditions that were derived for different lumped chemical mechanisms. For example, the speciation of non-methane organic compounds (NMOCs) in the NCAR boundary conditions (Section 3.3.1) and the FINN fire emissions (Section 3.5.1) follow the lumped mechanism used within the MOZART-4 model Thus we have developed a set of tools that are used within STILT-ASP to map species from different lumped chemical mechanisms (e.g., MOZART-4, CB4, SAPRC07) to the ASP species. These tools are flexible enough to map a single MOZART species to multiple ASP species and to map multiple MOZART species to a single ASP species depending on an ASCII text file defining the mapping.

### Tracking Fire-Influenced Parcels

In order to be able to determine the impact of fire emissions on the calculated species concentrations, we added a logical flag for each Lagrangian parcel in STILT-ASP that is “TRUE” if the parcel has been influenced by fire emissions and “FALSE” if it has not. We define a parcel as having been influenced by fire emissions if (a) the parcel is in a horizontal location and time (x, y, t) with non-zero fire emissions, and (b) the parcel altitude z is below the boundary layer height. All parcels start with a value of FALSE for the fire parcel flag, but once it is switched to TRUE it remains TRUE for the entire simulation.

At the end of the calculation, the influence of fire emission on the concentration of species i at the receptor is estimated with the following approach. First the average concentration of species q over all parcels at the receptor is calculated (Cq,t). Second, the average initial concentration of species q is calculated (Cq,o)., i.e., the average of the initial parcel concentrations read from the MOZART initial/boundary condition files (Section 3.3.1). For the non-fire particles, we then calculate the difference between the final and initial values for each parcel (∆Cq,k=Cq,k,t-Cq,k,o). This is the net formation of species q in each non-fire parcel k. We can then sum ∆Cq,k over all non-fire parcels and divide by the TOTAL number of parcels (Nparcel) to estimate the impact of the non-fire emissions on species q at the receptor:

∆Cq,non-fire=non-fire∆Cq,kNparcel		(21)

The impact of the fire emissions is then estimated as:

∆Cq,fire=Cq,t-Cq,o-∆Cq,non-fire		(22)

Of course, for species produced by non-linear chemistry (e.g., O3), Equation 22 is just an estimate of the fire impact on the final receptor concentration. We compare the results calculated using this method with results calculated by turning off fire emissions in Section 5.3 below.

## Emission Preprocessor

We have developed an emission preprocessor for STILT-ASP to provide “default” emissions for a screening level STILT-ASP run. The code, which is written in Python, takes publically available sources of emission data, regrids them to match a specified output grid from a WRF GRIB file (usually for the NAM12 or NARR domains), and produces a fire and nonfire (i.e., anthropogenic and biogenic) emission file for use in STILT-ASP. While STILT-ASP will run with standard Sparse Matrix Operator Kernel Emissions (SMOKE) Modeling System emission files as well, this emission preprocessor will allow users to investigate the impacts of fires on O3 and PM2.5 without having to invest the time in preparing gridded, speciated, hourly emissions from a full SMOKE Modeling System run.

### FINN v1.5 Fire Emissions

For the input fire emissions file, the preprocessor takes annual FINN emission files in CSV format as input. Specifically, the code uses the FINN emission with MOZART4 speciation, which are currently available for the years 2002 to 2015 from http://bai.acom.ucar.edu/Data/fire/. These files are then reduced to only include fires over North America for the requested time period. The emissions of semi-volatile and intermediate volatility organic compounds (S/IVOCs, needed to model secondary organic aerosol formation) are estimated as 6.5 times the emissions of primary organic aerosol (by mass) as recommended by Shrivastava et al. (2015). The daily fire emissions are then added to the appropriate output grid boxes and a representative diurnal cycle for fire emissions is applied based on work from the Western Regional Air Partnership (WRAP) as implemented in the FINN preprocessor for WRF-Chem. The emissions are then written to a netCDF4 file that is used as input to STILT-ASP. STILT-ASP then remaps the MOZART4 mechanism species into the ASP species needed for the run.

### HTAP v2 Anthropogenic Emissions

For the nonfire emissions file, anthropogenic emissions are taken from the HTAP v2 inventory. This inventory has global monthly emissions for 2008 and 2010 for several anthropogenic source sectors (i.e., Air, Ships, Energy, Industry, Transport, Residential, and Agriculture) at a 0.1o x 0.1o latitude-longitude horizontal resolution. As only two years are available, currently 2010 is used for all years after 2009, while 2008 is used for previous years, but future work could apply appropriate annual growth factors to the emissions. The preprocessor takes these monthly emission files, applies an appropriate speciation to the non-methane volatile organic compound (NMVOC) emissions for North America based on EPA recommendations with the species remapped to match the ASP speciation. The fraction of NOx released as NO2 is estimated to be 9.5% by mole, based on the average fraction from the 2008 and 2011 National Emission Inventories (NEI) from EPA. The emissions of semi-volatile and intermediate volatility organic compounds (S/IVOCs, needed to model secondary organic aerosol formation) are estimated as 6.5 times the emissions of primary organic aerosol (by mass) as recommended by Shrivastava et al. (2015). An assumed diurnal cycle is then applied to the emissions, based on the average diurnal cycle of anthropogenic CO during the 2010 NOAA CalNex campaign. The emissions are then regridded to match the output WRF grid (usually for the NAM12 or NARR domains) and mapped to the ASP species. All the needed input netCDF files from the Hemispheric Transport of Air Pollutants (HTAP) v2 inventory (downloaded from http://edgar.jrc.ec.europa.eu/htap_v2/) will be supplied to TCEQ along with the emission preprocessor code when the final report is delivered.

### MEGAN v2.1 Biogenic Emissions

Biogenic emissions for the nonfire emissions file are taken from monthly emissions estimated using the Model of Emissions of Gases and Aerosols from Nature (MEGAN) v2 (Guenther et al., 2006) model for the years 2000 to 2010 at a horizontal resolution of 0.5o x 0.5o. These species are then mapped into the ASP speciation, and an assumed diurnal cycle based on isoprene emission during the 2010 NOAA CalNex campaign is applied. The emissions are then regridded to match the output WRF grid. All the needed input netCDF files from the MEGAN v2 inventory (downloaded from http://lar.wsu.edu/megan/docs/05degree_MEGAN/) will be supplied to TCEQ along with the emission preprocessor code. Finally, the anthropogenic and biogenic emission are combined into a single file for use in STILT-ASP.

# Software Descriptions

The STILT-ASP User’s Guide provides the information necessary to run STILT-ASP on a Windows machine through the GUI, and this is the recommended way to run STILT-ASP. Here we provide more detail on the source code, inputs and outputs of the STILT-ASP model. Our discussion is divided between the back-trajectory calculation (Section 4.1), the forward chemistry calculation (Section 4.2), and the emission preprocessor for default emissions (Section 4.3).

## Back-trajectory calculation

### Back-trajectory Software

- Main Program: hymodelc_backward.F90

- Libraries:

  - STILT/ - All of the STILT source files needed to do the back-trajectory calculation. The files all contain descriptive headers. See www.stilt-model.org for more detail.

### Back-trajectory Inputs

A meteorological input file is needed to run the back-trajectory calculation. STILT-ASP supports only ARL-formatted meteorology data; other formats such as GRIB are not compatible, but NOAA ARL supplies a set of tools to convert other meteorological files to ARL format. STILT-ASP currently supports daily files using the NAM12 grid (12-km resolution) and monthly files using the NARR grid (32 km resolution) downloaded from https://ready.arl.noaa.gov/archives.php.

Two other configuration files are used: the SETUP.CFG and CONTROL files. The SETUP.CFG namelist file sets several model parameters and flags, and is described on p. 181 of the attached HYSPLIT User’s Guide. The SETUP.CFG file in STILT-ASP should generally not be adjusted directly, as only certain settings are compatible with the STILT-ASP calculations. The CONTROL file sets parameters such as the current date, length of backtrajectories, etc., and it’s format is described on p. 76 of the attached HYSPLIT Users Guide. In STILT-ASP, these parameters are generally adjusted by the GUI (see Section 3 of the STILT-ASP User’s Guide) and the CONTROL file should not be adjusted directly.

### Back-trajectory Processing

The steps in the STILT back-trajectory calculation are given in Figure 1.

Figure 1. Flow chart for the STILT back-trajectory calculations.

### Back-trajectory Output

The back-trajectory runs produce an output NetCDF file (PARTICLE.nc) that is used as input to the forward chemistry calculations (Section 4.2). The contents of this output file are given in Table 1.

Table 1. Output parameters in PARTICLE.nc file

| Variable Name | Dimensions | Units | Description |
| --- | --- | --- | --- |
| Jjet | (times) | minutes | Time elapsed since January 1 1900, in minutes |
| Ddt | (times) | Minutes | Timestep (constant value in STILT-ASP) |
| Particle_indices | (particles) | N/A | 1..#particles |
| Ipos | (times, particles) | N/A | Particle lon index on met grid |
| Jpos | (times, particles) | N/A | Particle lat index on met grid |
| Kpos | (times, particles) | N/A | Particle alt index on met grid |
| Xlon | (times, particles) | Degrees | Particle longitude (-180 to 180) |
| Ylat | (times, particles) | Degrees | Particle latitude (-90 to 90) |
| Wagl | (times, particles) | m | Particle altitude above ground level |
| Psfc | (times, particles) | mbar | Particle surface air pressure |
| Fcld | (times, particles) | N/A | Particle cloud cover fraction |
| Ppar | (times, particles) | mbar | Particle air pressure |
| Qpar | (times, particles) | 0-1 | Relative humidity fraction |
| Pgc | (times, particles) | mbar | Internal grid cell air pressure |
| Tgc | (times, particles) | K | Internal grid cell air temperature |
| Tpar | (times, particles) | K | Particle air temperature |
| Pu0 | (times, particles) | m/s | U-velocity at 10m |
| Pv0 | (times, particles) | m/s | V-velocity at 10m |
| Pt0 | (times, particles) | K | Surface temperature |
| Dgc | (times, particles) | kg/m3 | Internal grid cell air density |
| Dns | (times, particles) | kg/m3 | Particle air density |
| Dsgc | (times, particles) | W/m2 | Particle downward shortwave radiation flux |
| Qgc | (times, particles) |  | Internal grid cell humidity |
| Qgcv | (times, particles, levels) |  | Internal grid cell air humidity profile |
| Smpt | (times, particles) |  | Particle sampling time |
| Fp | (times, particles) |  | Particle footprint |
| Pzpos | (times, particles) |  | Particle height |
| Psigw | (times, particles) |  | Particle sigma level |
| Pptyp | (times, particles) |  | Pollutant type |
| Pland | (times, particles) | N/A | Land use category |
| Paero | (times, particles) | m | Aerodynamic roughness length |
| Psfcl | (times, particles) |  | Height of the surface layer |
| Pustr | (times, particles) | m/s | Friction velocity |
| Ppsi | (times, particles) |  | Integrated stability profile |
| Phdwpx | (times, particles) |  | Horizontal distribution pollutant |
| Prain | (times, particles) | m/min | Precipitation rate |
| Vdens | (times, particles, levels) | kg/m3 | Particle density profile |
| Vtemp | (times, particles, levels) | K | Particle temperature profile |
| Vwind | (times, particles) | m/s | Magnitude of wind speed at surface |
| Vpres | (times, particles, levels) | mbar | Particle pressure profile |
| Vrhfr | (times, particles, levels) | N/A | Particle relative humidity profile |
| Pbl_height | (times, particles) | m | PBL/mixing layer height |
| Pzndx | (times, particles) | N/A | Vertical fractional index position |
| Hsfc | (times, particles) | m | Height of the surface layer |
| Kkg | (times) | N/A | Meteorological grid index number |
| kkt | (times) | N/A | Current active Meteorological grid number |

## Forward Chemistry Calculations

### Forward Chemistry Calculations Software

- Main Program: hymodelc_forward.F90

- Libraries:

  - STILT-Chem/ - similar to the STILT library, but with added routines for handling chemistry.

  - ASP/

    - InorganicInputDecks/ - These ASP input files (*.in) define the chemistry and other parameters of the ASP model. They have descriptive names (e.g., GasPhaseChems.in, PhotoRates.in) and headers that describe the file contents and formats.

    - src/ - The ASP source code. All files have descriptive headings with dependencies, linked header files (if any) and subroutines and functions listed.

      - asp_wrapper.f – file with routines to connect ASP to STILT-Chem and perform various unit coversions.

      - StepASP.f90 – file with the routines to initialize ASP within other models and couple the ASP calculations with the host model.

      - ModelParameters.f90 - File with specified physical and other constant for ASP as well as utility routines.

      - DMiLay.f, MIEV0.f, MieLay.f - Files for calculating aerosol optical properties using core-shell Mie theory.

      - ReadInputDeck.f90 – file that reads the main input deck for ASP (./ASP/InorganicInputDecks/ASPInputDeck.in) that controls which parts of the model are calculated (e.g., gas chemistry, coagulation) and other settings.

      - Chemistry/ - Routines for initializing and calculating gas phase chemistry.

      - Coagulation/ - Routines for calculating aerosol coagulation.

      - ComputationalSubroutines/ - Mathematical utility routines (e.g., lsodes.f to solve the ordinary differential equations of gas-phase chemistry and condensation)

      - Condensation/ - Routines to calculate gas-aerosol mass transfer

      - EulerianEnvironment/ - Routines that define the state of the ASP chemical “box”

      - InfrastructuralCode/ - Routines for handling strings and counters

      - LagrangianParticles/ - Routines that define the aerosol data structure and calculate various properties of the aerosol.

      - OutputRoutines/ - Routines that produce various output files for ASP

      - UNIFAC/ - Routines for calculating activity coefficients for organic species by group-contribution methods

### Forward Chemistry Calculations Inputs

The inputs for the forward chemistry calculation include the PARTICLE.nc file produced by the back-trajectory run (Section 4.1.4) and the SETUP.CFG and CONTROL files from that back-trajectory run (Section 4.1.2).

In addition, a chemical boundary condition file is required. The only option currently supported is for NetCDF files generated by the MOZART-4/GEOS-5 runs of the NCAR Chemical Forecasts (Section 3.3.1).

Emission files are also required. When using the default emissions, these files can be the fire and non-fire NetCDF emission files produced by the STILT-ASP emission preprocessor (Section 3.5 and 4.3). 4D emission files from the SMOKE model can also be used – see Section 3.6 of the STILT-ASP User’s Guide.

A set of chemical species for output must also be selected, and the user can select two output modes – one in CSV format and one in NetCDF format.

Other inputs for ASP (chemical mechanism, photolysis rates) are included in the ./ASP/InorganicInputDecks/ directory, as stated in Section 4.2.1. Users should generally not have to adjust these files.

### Forward Chemistry Calculations Processing

The steps in the forward model calculation are shown in Figure 2.

Figure 2. Flow chart of the STILT-ASP forward chemistry calculations.

### Forward Chemistry Calculators Output

The output consists of concentrations of the requested species, produced either as a simple comma-separated-value text file or as a comprehensive NetCDF file containing much more information on the evolution of chemical compounds in each Lagrangian parcel suitable for advanced users.

## Emission Preprocessor

### Emission Preprocessor Software

The emission preprocessor is written in Python 2.7. The source code includes four Python library files:

- finn_lib.py: This is the library file for processing FINN v1.5 emissions, which includes two functions:

  - process_finn, which takes an input annual FINN v1.5 CSV file using the MOZART-4 NMOC  speciation and reduces it to only the fires that fall on a specific day in the specified latitude-longitude range. This reduced list is then written to a new CSV file.

  - regrid_finn, which takes the FINN v1.5 fire emissions from the reduced CSV file produced by process_finn, (i) regrids them to a given 2D latitude-longitude grid (usually from the NAM12 or NARR GRIB files), (ii) applies an input diurnal cycle to the emissions, and (iii) converts the units of the emissions to μmol/m2/hr for gas-phase species and μg/m2/hr for aerosol species.

- htap_lib.py: This library contains the functions that convert HTAP v2 emission files to NetCDF files matching a given lat-lon gird from a WRF GRIB file. It includes three functions:

  - get_emis, which extracts the emission array for a given species from a given HTAP v2 netCDF file.

  - get_htap_grid, which extracts the latitude-longitude array from the HTAP v2 netCDF file.

  - regrid_htap, which takes the HTAP v2 emissions from the netCDF files and (i) assigns them to ASP species, (ii) regrids them to a given 2D latitude-longitude grid (usually from the NAM12 or NARR GRIB files), (iii) applies an input diurnal cycle to the emissions, and (iv) converts the units of the emissions to μmol/m2/hr for gas-phase species and μg/m2/hr for aerosol species.

- megan_lib.py: This is the library file for processing the MEGAN v2.1 emissions that only includes one function, regrid_megan, which extracts the global MEGAN v2.1 emissions for a single month from the netCDF files and (i) assigns them to ASP species, (ii) regrids them to a given 2D latitude-longitude grid (usually from the NAM12 or NARR GRIB files), (iii) applies an input diurnal cycle to the emissions, and (iv) converts the units of the emissions to μmol/m2/hr for gas-phase species and μg/m2/hr for aerosol species.

- stilt_asp_emissions_preprocessor.py: This library includes the main program for the emission preprocessor, which calls the above library functions to regrid the FINN, HTAP, and MEGAN emission data to a common 2D latitude-longitude grid from a GRIB file and apply fixed diurnal cycles to the emissions. It then combines the intermediate monthly (for HTAP and MEGAN) or daily (for FINN) files to make two hourly emissions files (one for fire emissions and one non-fire emissions) for all of the input dates. The library also includes the following functions:

  - make_logfile, which sets up the log file for each run.

  - set_grid, which reads in a WRF GRIB file and uses it to define the output emission grid.

  - make_time_arrays, which takes the starting and ending time strings in 'YYYY-MM-DD' format, does some checks, and then returns four arrays of the length of the number of days in the start and end range.

  - combine_finn_days, which uses the finn_lib.py library to regrid FINN data for a set of days and apply the diurnal cycle. It then produces a combined fire emission file for the entire day range.

  - combine_htap_megan_months, which takes the regridded HTAP and MEGAN data for a given month and produces a combined nonfire emission file for the month.

  - combine_nonfire_days, which calls the functions to regrid HTAP and MEGAN data for a set of months and apply the diurnal cycle. It then produces a combined non-fire emission file for the entire day range.

### Emission Preprocessor Inputs

The main program can be called from the command line (python stilt_asp_emissions_preprocessor.py). The required command line flags are:

- '-sd', a string with the start date for the output emissions files in YYYY-MM-DD format. Default is '2011-04-01'.

- '-ed', a string with the end date for the output emissions files in YYYY-MM-DD format. Default is '2011-04-02'. Note that currently the start date and end date must be in the same year.

- '-m', a string with the name of the model used to define the output latitude-longitude grid, like NAM12 or NARR. Default is ‘NAM12’.

- '-gp', a string with the full file path of the GRIB file defining the output latitude-longitude grid. Default is cwd+'/grib_files/namanl_218_20110801_0000_000.grb' where cwd is the current working directory.

- '-fp', a string with the full path of the daily annual FINN v1.5 emission files. Default is cwd+'/finn_v1.5_files'. Note that the FINN v1.5 CSV file has to be in this directory in the format of '/GLOBAL_FINNv15_'+YYYY+'_MOZ4_7112014.txt' – if this changes, the “prefix’ input to the function combine_finn_days must be changed to match.

- '-mp', a string with the full path of the MEGAN v2.1 global emission files for 2000-2010. Default is cwd+'/megan_v2.1_files'. These input files are supplied with the emission preprocessor.

- '-hp', a string with the full path of the HTAP v2 global emission files for 2008 and 2010. Default is cwd+'/htap_v2_files’. These files are provided with the emission preprocessor, as they must follow a specific subdirectory structure.

- '-ip', a string with the full path of the intermediate output directory. Default is cwd+'/intermed'.

- '-op', a string with the full path of the final output directory. Default is cwd+'/output'.

There are also two optional command-line arguments related to the log file:

- '-l', the full name and path of the log file used throughout the mapping process.  Not required, defaults to stdout.

- '-p', the level of verbosity specified for logging information. "INFO" is the default setting.  Possible values are "DEBUG", "INFO", "WARNING", "ERROR" and "CRITICAL", in decreasing order of verbosity.

### Emission Preprocessor Processing

Figure 3 outlines the steps in the emission preprocessor. The subroutines in combine_nonfire_days are looped over for all requested months, while the subroutines in combine_finn_days are looped over for every requested day. The intermediate files produced in these subroutines are not listed in the chart.

Figure 3. Flow chart of the emission preprocessor.

### Emission Preprocessor Output

As the HTAP and MEGAN files are provided with the preprocessor, along with GRIB files describing the NAM12 and NARR output grids (namanl_218_20110801_0000_000.grb and   narr-a_221_20110801_0000_000.grb in the grib_files sub-directory, respectively), the user only needs to supply the FINN CSV file for their requested year downloaded from http://bai.acom.ucar.edu/Data/fire/. Intermediate netCDF files are produced for monthly emissions from MEGAN and HTAP and daily emissions from FINN and are placed in the ‘-ip’ directory. The final merged non-fire and fire emission files are placed in the ‘-op’ directory.

The final fire and non-fire emission NetCDF files contain 3D grids (x, y, and hour) for the emissions of each species in units of µmol/m2/hr for gas-phase species and µg/m2/hr for aerosol species. Emissions start at midnight local solar time and have 24 hours for each requested day. Each file also contains the 2D (x,y) arrays of the grid box latitudes and longitudes. The fire files currently follow the MOZART-4 NMOC speciation, which is remapped to ASP’s speciation within STILT-ASP (Section 3.4.1). The non-fire emissions have already been remapped to ASP species within the preprocessor.

# Model Calibration, Validation, and Evaluation

As of the time of this draft technical memo, we are still performing the calibration, validation, and evaluation of STILT-ASP. Thus, this section should change substantially in the final draft. Below we report our progress so far.

## Previous Evaluation of STILT and ASP

The individual components of STILT-ASP have been calibrated, validated, and evaluated in previous studies. STILT has been extensively used in inverse modeling to improve emission estimates for greenhouse gases (e.g., McKain et al., 2012, 2015; Miller et al., 2013; Henderson et al., 2015). Hegarty et al. (2013) performed an intercomparison of three widely used LPDMs (STILT, HYSPLIT, and FLEXPART), all driven by identical meteorological output from WRF. The validation data consisted of measurements from controlled tracer releases provided by the NOAA Air Resources Laboratory. While other tracers have been used for validation of transport models, the controlled releases, where the emissions and resulting concentrations are accurately known, constitute the “gold standard” for tracer model validation. For this location and quiescent synoptic situation (primarily governed by an anticyclone in the eastern U.S.), all three LPDMs were in overall agreement with measurements. However, under more challenging synoptic conditions, all three models failed to reproduce observations due to errors in the driving meteorological fields produced by WRF. These results provide new insights into the underlying limitations of the meteorological models that degrade the overall accuracy of the LPDMs. A paper describing these results has been published. The field data and the original reports are available through the Data Archive of Tracer Experiments and Meteorology (DATEM - http://www.arl.noaa.gov/DATEM.php), statistical and graphical outputs from the LPDM simulations are available at ftp://arlftp.arlhq.noaa.gov/117/results and the WRF data generated under this grant are available at  http://www.arl.noaa.gov/DATEM_WRF-ARW.php.

ASP has been extensively used to study the chemical and physical transformations of aerosols within BB smoke plumes and the optical properties of aerosols. Alvarado and Prinn (2009) used ASP combined with a Lagrangian parcel model to simulate the gas- and aerosol-phase chemistry in several smoke plumes, including an African savannah fire (the Timbavati fire). They found that uncharacterized organic species in the smoke plume were likely responsible for the observed rapid formation of SOA, while heterogeneous chemistry of NO2 and SO2 was a possible explanation for the rapid formation of aerosol nitrate and sulfate within the smoke plume. Alvarado et al. (2009) expanded on this study by incorporating ASP into a 3-D Eulerian smoke plume model to simulate the fluid dynamics, radiative transfer, gas-phase chemistry, and aerosol-phase chemistry in the Timbavati smoke plume. This was the first time that all of these processes had been simulated simultaneously for a young BB smoke plume.

Recently Alvarado et al. (2015) evaluated ASP simulations for a fire in California (Williams fire, Akagi et al., 2012), as shown in Figures 1 and 2. This study showed that ASP (all lines in Figure 1, blue line in Figure 2) could simulate most of the observations (e.g., OA, O3, NOx, OH) using reasonable assumptions about the chemistry of the unidentified organic compounds. This method provides a chemically realistic way for determining the average chemistry of unidentified organic compounds in the smoke plume.

Figure 4. O3 (left) and PAN (right) mixing ratios within the Williams fire as observed by Akagi et al. (2012) and simulated by ASP. Asterisks are the measured mixing ratios, with the horizontal error bars showing the uncertainty in the estimated age and the vertical error bars showing the uncertainty in the measurement. Red, black, and green are ASP results for the slow, best-fit (medium), and fast plume dilution rates. Dashed lines are for above-plume photolysis rates, while solid lines are for the middle of the plume, and dotted lines are for the bottom of the plume.

Figure 5. Enhancement ratio (g/g) of organic aerosol (OA) to CO2 versus smoke age for the Williams fire. All model results assume the best-fit dilution rate and the photolysis rates corresponding to the middle of the plume. The red circles are the observed values from Akagi et al. (2012).

## Testing Back-Trajectory Calculations of STILT-ASP versus STILT

The version of STILT built on Windows using Cygwin for TCEQ was compared to an operational version of STILT at AER built on Linux.  It was found that the two versions did not match on a particle-particle basis, but that both versions produce very similar distributions of trajectories, as shown in Figure 6.

Figure 6. Latitude (top), longitude (middle), and altitude (m) of the 500 particle backtrajectories for the STILT-ASP Windows code (left) and the STILT Linux code (right). Both models were run with identical random number seeds, but with different compilers on different machines.

Plotting the mean, median, and standard distributions of the particle trajectories, as in Figure 7, leads to similar conclusions – the runs are very similar, but are not identical.

Figure 7. Mean (red), median (blue), and standard deviation (magenta) of the 500 particle back-trajectories for the STILT-ASP Windows code (left) and the STILT Linux code (right). Both models were run with identical random number seeds, but with different compilers on different machines.

Since major errors such as file read discrepancies would likely have been encountered earlier in the build process, it is thought that these differences are due to different treatment of the random number generators used to account for turbulent fluctuations in the STILT model. To test this, an additional Windows/Cygwin run was performed.  This run was identical to the previous Windows run, except a run parameter was added that incorporates a random seed value based on the computer clock.

Figure 8. Mean (red), median (blue), and standard deviation (magenta) of the 500 particle back-trajectories for the STILT-ASP Windows code with a fixed random seed (left) and the random seed based on the system clock (right).

Figure 8 shows that the differences between the STILT-ASP model run on the same machine with different seeds show differences of about the same magnitude as those seen between the Windows STILT-ASP trajectories and the Linux STILT back-trajectories.

We thus conclude that, given the stochastic nature of STILT, the observed differences between the Windows and Linux back trajectories can be attributed to the different architecture, build, and run environments, and not to an erroneous Windows build. The remaining minor stochastic differences should not have a significant impact on the average results of the code.

Figure 9. STILT-ASP O3 calculated for 11:00 CST on Sept. 7th, 2011 at EPA Site Number 482090614 in Austin, TX using the default fire, anthropogenic, and biogenic emissions from the STILT-ASP emission preprocessor and the NAM12 meteorology. The top panel shows the O3 concentration in the 500 simulated Lagrangian parcels. The middle panel shows the SZA for each particle. The bottom panel shows the average O3 concentration for all of the particles (black), the initial average ozone value (blue), and an estimate of the fire impact on O3 (red shading) calculated using Equation 22.

## Preliminary O3 runs for Houston and Austin Biomass Burning Events

We have performed preliminary STILT-ASP runs of the August 26-29, 2011 biomass burning smoke event in Houston, TX and the September 7-8, 2011 burning event in Austin, TX. We have tested runs both with and without fire emissions using both the NAM-12 and NARR meteorological inputs. Figure 9 gives an example of a STILT-ASP run for a 2-day back-trajectory from Austin. In this configuration, the model underestimates the average O3 observed 76 ppb, modeled 61 ppb) and overestimates the NOx (not shown, observed 2.0 ppb, modeled 6.5 ppb). These results with the default emissions are consistent with recent studies suggesting that NOx emissions may be overestimated in the EPA NEI 2011 inventory by a factor of 2 (e.g., Anderson et al., 2014). Further work will reduce the anthropogenic NOx emissions and increase the biogenic VOC emissions and determine if that improves the model performance.

Figure 10. As in Figure 9, but with only non-fire (anthropogenic and biogenic) emissions included.

Figure 9 shows that using the method of Equation 22, we would estimate that ~6 ppb of the O3 measured at the Austin monitor was due to fire emissions. However, the STILT-ASP run shown in Figure 10, which is the same as the Figure 9 run except that fire emissions are removed, suggests that a much smaller amount (~1 ppb) of the O3 was from fire emissions. This suggests the need for further refinement of the method described in Equation 22 to estimate fire impacts on O3.

# Summary

This technical memo has described the algorithms of the STILT-ASP model and our efforts to date on model calibration, validation, and evaluation. Our evaluation to date suggests more work is needed before STILT-ASP is ready to be used to assess the impact of fire emission on O3 and PM2.5, but we expect the final code delivered at the end of the project to be suitable for this use.

# References

Ahmadov, R., McKeen, S. A., Robinson, A. L., Bahreini, R., Middlebrook, A. M., de Gouw, J. A., Meagher, J., Hsie, E.-Y., Edgerton, E., Shaw, S., and Trainer, M.: A volatility basis set model for summertime secondary organic aerosols over the eastern United States in 2006, J. Geophys. Res., 117, 2156–2202, 2012.

Akagi, S. K., Yokelson, R. J., Wiedinmyer, C., Alvarado, M. J., Reid, J. S., Karl, T., Crounse, J. D., and Wennberg, P. O.: Emission factors for open and domestic biomass burning for use in atmospheric models, Atmos. Chem. Phys., 11, 4039-4072, doi:10.5194/acp-11-4039-2011, 2011.

Akagi, S. K., Craven, J. S., Taylor, J. W., McMeeking, G. R., Yokelson, R. J., Burling, I. R., Urbanski, S. P., Wold, C. E., Seinfeld, J. H., Coe, H., Alvarado, M. J., and Weise, D. R.: Evolution of trace gases and particles emitted by a chaparral fire in California, Atmos. Chem. Phys., 12, 1397–1421, 2012.

Alvarado, M. J., and Prinn, R. G.: Formation of Ozone and Growth of Aerosols in Young Smoke Plumes from Biomass Burning, Part 1: Lagrangian Parcel Studies, J. Geophys. Res., 114, D09306, 2009.

Alvarado, M. J., Wang, C., and Prinn, R. G.: Formation of Ozone and Growth of Aerosols in Young Smoke Plumes from Biomass Burning, Part 2: 3D Eulerian Studies, J. Geophys. Res., 114, D09307, doi:10.1029/2008JD011186, 2009.

Alvarado, M. J., Lonsdale, C. R., Yokelson, R. J., Akagi, S. K., Coe, H., Craven, J. S., Fischer, E. V., McMeeking, G. R., Seinfeld, J. H., Soni, T., Taylor, J. W., Weise, D. R., and Wold, C. E.: Investigating the links between ozone and organic aerosol chemistry in a biomass burning plume from a prescribed fire in California chaparral, Atmos. Chem. Phys., 15, 6667-6688, doi:10.5194/acp-15-6667-2015, 2015.

Atkinson, R., Baulch, D. L., Cox, R. A., Crowley, J. N., Hampson, R. F., Hynes, R. G., Jenkin, M. E., Rossi, M. J., and Troe, J.: Evaluated kinetic and photochemical data for atmospheric chemistry: Volume I - gas phase reactions of Ox, HOx, NOx and SOx species, Atmos. Chem. Phys., 4, 1461–1738, doi:10.5194/acp-4-1461-2004, 2004.

Baylon, P., D. A. Jaffe, N. L. Wigder, H. Gao, and J. Hee: Ozone enhancement in Western US wildfire plumes at the Mt. Bachelor Observatory: The Role of NOx, Atmos. Environ., 109, 297-304, doi:10.1016/j.atmosenv.2014.09.013, 2014.

Bloss, C., Wagner, V., Jenkin, M. E., Volkamer, R., Bloss, W. J., Lee, J. D., Heard, D. E., Wirtz, K., Martin-Reviejo, M., Rea, G., Wenger, J. C., and Pilling, M. J.: Development of a detailed chemical mechanism (MCMv3.1) for the atmospheric oxidation of aromatic hydrocarbons, Atmos. Chem. Phys., 5, 641–664, doi:10.5194/acp-5-641-2005, 2005.

Bowman, F. M., and Seinfeld, J. H.: Fundamental basis of incremental reactivities of organics in ozone formation in VOC/NOx mixtures, Atmos. Environ., 28(20), 3359–3368, 1994a.

Bowman, F. M., and Seinfeld, J. H. Ozone productivity of atmospheric organics, J. Geophys. Res., 99(D3), 5309–5324, 1994b.

Carter, W. P.: Development of ozone reactivity scales for volatile organic compounds, Air and Waste, 44(7), 881–899, 1994.

Chock, D.P., Winkler, S.L., 1994. A particle grid air quality modeling approach, 2. Coupling with chemistry. Journal of Geophysical Research 99 (D1), 1033e1041.

Crounse, J. D., Paulot, F., Kjaergaard, H. G., and Wennberg, P.O.: Peroxy radical isomerization in the oxidation of isoprene, Phys. Chem. Chem. Phys., 13, 13607–13613, 2011.

Crounse, J. D., Knap, H. C., Ørnsø, K. B., Jørgensen, S., Paulot, F., Kjaergaard, H. G., and Wennberg, P. O.: Atmospheric Fate of Methacrolein. 1. Peroxy Radical Isomerization Following Addition of OH and O2, J. Phys. Chem. A, 116, 5756–5762, 2012.

Draxler, R.R., Hess, G.D.: Description of the HYSPLIT 4 Modeling System. NOAA Technical Memorandum ERL ARL-224, 24, 1998.

Draxler, R. R., & Hess, G. D.: An overview of the HYSPLIT_4 modelling system for trajectories. Australian meteorological magazine, 47(4), 295-308, 1998.

Dzepina, K., Volkamer, R. M., Madronich, S., Tulet, P., Ulbrich, I. M., Zhang, Q., Cappa, C. D., Ziemann, P. J., and Jimenez, J. L.: Evaluation of recently-proposed secondary organic aerosol models for a case study in Mexico City, Atmos. Chem. Phys., 9, 5681–5709, doi:10.5194/acp-9-5681-2009, 2009.

Fountoukis, C. and Nenes, A.:  ISORROPIA II: a computationally efficient thermodynamic equilibrium model for K+–Ca2+–Mg2+–NH4+–Na+–SO42−–NO3−–Cl−–H2O aerosols, Atmos. Chem. Phys., 7, 4639–4659, doi:10.5194/acp-7-4639-2007, 2007.

Grieshop, A. P., Logue, J. M., Donahue, N.M., and Robinson A. L.: Laboratory investigation of photochemical oxidation of organic aerosol from wood fires 1: measurement and simulation of organic aerosol evolution, Atmos. Chem. Phys., 9, 1263–1277, doi:10.5194/acp-9-1263-2009, 2009a.

Grieshop, A. P., Donahue, N. M., and Robinson, A. L.: Laboratory investigation of photochemical oxidation of organic aerosol from wood fires 2: analysis of aerosol mass spectrometer data, Atmos. Chem. Phys., 9, 2227–2240, doi:10.5194/acp-9-2227-2009, 2009b.

Griffin, R. J., Nguyen, K., Dabdub, K. D., and Seinfeld, J. H.: A coupled hydrophobic-hydrophilic model for predicting secondary organic aerosol formation, J. Atmos. Chem., 44, 171–190, 2003.

Griffin, R. J., Dabdub, D., and Seinfeld, J. H.: Development and initial evaluation of a dynamic species-resolved model for the gas phase chemistry and size-resolved gas/particle partitioning associated with secondary organic aerosol formation, J. Geophys. Res., 110, D05304, doi:10.1029/2004JD005219, 2005.

Goliff, W.S., Stockwell, W. R., and Lawson, C. V.: The regional atmospheric chemistry mechanism, version 2, Atmos. Environ., 68, 174–185, 2013.

Guenther, A., T. Karl, P. Harley, C. Wiedinmyer, P. I. Palmer, and C. Geron: Estimates of global terrestrial isoprene emissions using MEGAN (Model of Emissions of Gases and Aerosols from Nature), Atmos. Chem. Phys., 3181-3210, doi:10.5194/acp-6-3181-2006, 2006.

Hanna, S. R., Applications in air pollution modeling, in Atmospheric Turbulence and Air Pollution Modeling, edited by F. T. M. Nieuwstadt and H. van Dop, 358 pp., D. Reidel, Norwell, Mass., 1982.

Hegarty, J., R. R. Draxler, A. F. Stein, J. Brioude, M. Mountain, J. Eluszkiewicz, T. Nehrkorn, F. Ngan, and A. Andrews: Evaluation of Lagrangian Particle Dispersion Models with Measurements from Controlled Tracer Releases. J. Appl. Meteor. Climatol., 52, 2623–2637, doi: 10.1175/JAMC-D-13-0125.1, 2013.

Henderson, J. M., Eluszkiewicz, J., Mountain, M. E., Nehrkorn, T., Chang, R. Y.-W., Karion, A., Miller, J. B., Sweeney, C., Steiner, N., Wofsy, S. C., and Miller, C. E.: Atmospheric transport simulations in support of the Carbon in Arctic Reservoirs Vulnerability Experiment (CARVE), Atmos. Chem. Phys., 15, 4093-4116, doi:10.5194/acp-15-4093-2015, 2015.

Jacobson, M. Z.: Development and application of a new air pollution modeling system - II. Aerosol module structure and design, Atmos. Environ., 31, 131–144, 1997.

Jacobson, M. Z.: Analysis of aerosol interactions with numerical techniques for solving coagulation, nucleation, condensation, dissolution, and reversible chemistry among multiple size distributions, J. Geophys. Res., 107, 4366-4388, doi:10.1029/2001JD002044, 2002.

Jacobson, M. Z.: Fundamentals of Atmospheric Modeling, 2nd ed., Cambridge University Press, 2005.

Jaffe, D. A., and N. L. Wigder : Ozone production from wildfires: A critical review, Atmospheric Environment, 51, 1–10, 2012.

Jaffe, D. A., N. Wigder, N. Downey, G. Pfister, A. Boynard, and S. B. Reid: Impact of wildfires on ozone exceptional events in the western US, Environ. Sci. Technol., 47(19), 11065–11072, 2013.

Jenkin, M. E., Saunders, S. M., Wagner, V., and Pilling, M. J.: Protocol for the development of the Master Chemical Mechanism, MCM v3 (Part B): tropospheric degradation of aromatic volatile organic compounds, Atmos. Chem. Phys., 3, 181–193, doi:10.5194/acp-3-181-2003, 2003.

Jenkin, M. E., Saunders, S. M., and Pilling, M. J.: The tropospheric degradation of volatile organic compounds: a protocol for mechanism development, Atmos. Environ., 31(1), 81–104, ISSN 1352-2310, DOI: 10.1016/S1352-2310(96)00105-7, 1997.

Kusik, C., and Meissner, H.: Electrolyte activity coefficients in inorganic processing, in Fundamental Aspects of Hydrometallurgical Processes, AIChE Symposium Series, edited by T. Chapman, L. Taularides, G. Hubred, and R. Wellek, pp. 14–20, AIChE, New York, 1978.

Lin, J. C., Gerbig, C., Wofsy, S. C., Andrews, A. E., Daube, B. C., Davis, K. J., and Grainger, C. A.: A near-field tool for simulating the upstream influence of atmospheric observations: the Stochastic Time-inverted Lagrangian Transport (STILT) model, J. Geophys. Res., 108, 4493, doi:10.1029/2002JD003161, 2003.

Madronich, S., and Flocke, S.: The role of solar radiation in atmospheric chemistry. Handbook of Environmental Chemistry, P. Boule, Ed., Springer, 1-26, 1998.

McKain, K., S. C. Wofsy, T. Nehrkorn, J. Eluszkiewicz, J. R. Ehleringer, and B. B. Stephens: Assessment of ground-based atmospheric observations for verification of greenhouse gas emissions from an urban region, PNAS, 109, 8423-8428, 2012.

McKain, K., A. Down, S. M. Raciti, J. Budney, L. R. Hutyr, C. Floerchinger, S. C. Herndon, T. Nehrkorn, M. S. Zahniserg, R. B. Jackson, N. Phillips, and S. C. Wofsy: Methane emissions from natural gas infrastructure and use in the urban region of Boston, Massachusetts, PNAS, 112, 1941-1946, 2015.

Miller, S. M., S. C. Wofsy, A. M. Michalak, E. A. Kort, A. E. Andrews, S. C. Biraud, E. J. Dlugokencky, J. Eluszkiewicz, M. L. Fischerg, G. Janssens-Maenhouth, B. R. Milleri, J. B. Miller, S. A. Montzka, T. Nehrkorn, and C. Sweeney: Anthropogenic emissions of methane in the United States, PNAS 110, 20018-20022, doi: 10.1073/pnas.1314392110 , 2013.

Nehrkorn, T., Eluszkiewicz, J., Wofsy, S. C., Lin, J. C., Gerbig, C., Longo, M., & Freitas, S.: Coupled weather research and forecasting–stochastic time-inverted lagrangian transport (WRF–STILT) model, Meteorol. Atmos. Phys., 107(1-2), 51-64, doi: 10.1007/s00703-010-0068-x, 2010.

Nenes, A., Pandis, S. N., and Pilinis, C.: ISORROPIA: A new thermodynamic equilibrium model for multiphase multicomponent inorganic aerosols, Aquatic Geochemistry, 4, 123–152, 1998.

Paulot, F., Crounse, J. D., Kjaergaard, H. G., Kroll, J. H., Seinfeld, J. H., and Wennberg, P. O.: Isoprene photooxidation: new insights into the production of acids and organic nitrates, Atmos. Chem. Phys., 9, 1479–1501, 2009a.

Paulot, F., Kurten, A., St Clair, J. M., Seinfeld, J. H., and Wennberg, P.O.: Unexpected Epoxide Formation in the Gas-Phase Photooxidation of Isoprene, Science, 325, 730–733, 2009b.

Petters, M. D., and Kreidenweis, S. M.: A single parameter representation of hygroscopic growth and cloud condensation nucleus activity, Atmos. Chem. Phys., 7, 1961–1971, doi:10.5194/acp-7-1961-2007, 2007.

Pun, B. K., Griffin, R. J., Seigneur, C., and Seinfeld, J. H.: Secondary organic aerosol, 2, Thermodynamic model for Gas/Particle partitioning of molecular constituents, J. Geophys. Res., 107, 4333-4347, doi:10.1029/2001JD000542, 2002.

Resch, T.: A framework for modeling suspended multicomponent particulate systems with applications to atmospheric aerosols., Ph.D. thesis, Massachusetts Institute of Technology, 1995.

Robinson, A. L., Donahue, N. M., Shrivastava, M. K., Weitkamp, E. A., Sage, A. M., Grieshop, A. P., Lane, T. E., Pierce, J. R., and Pandis, S. N.: Rethinking Organic Aerosols: Semivolatile Emissions and Photochemical Aging, Science, 315, 1259–1262, 2007.

Saunders, S. M., Jenkin, M. E., Derwent, R. G., and Pilling, M. J.: Protocol for the development of the Master Chemical Mechanism, MCM v3 (Part A): tropospheric degradation of non-aromatic volatile organic compound, Atmos. Chem. Phys., 3, 161–180, 2003.

Seinfeld, J. H., and Pandis, S. N.: Atmospheric Chemistry and Physics, John Wiley and Sons, New York, NY, 1998.

Shrivastava, M.K.B, Zelenyuk, A., Imre, D., Easter, R. C., Jr., Beranek, J., Zaveri, R. A., and Fast, J. D.: Implications of Low Volatility SOA and Gas-Phase Fragmentation Reactions on SOA Loadings and their Spatial and Temporal Evolution in the Atmosphere. J. Geophys. Res., 118, 3328-3342.  doi:10.1002/jgrd.50160, 2013.

Shrivastava, M., R. C. Easter, X. Liu, A. Zelenyuk, B. Singh, K. Zhang, P.-L. Ma, D. Chand, S. Ghan, J. L. Jimenez, Q. Zhang, J. Fast, P. J. Rasch, and P. Tiitta: Global transformation and fate of SOA: Implications of low-volatility SOA and gas-phase fragmentation reactions. J. Geophys. Res. Atmos., 120, 4169–4195. doi: 10.1002/2014JD022563, 2015.

Steele, H. D.: Investigations of cloud altering effects of atmospheric aerosols using a new mixed eulerian-lagrangian aerosol model, Ph.D. thesis, Massachusetts Institute of Technology, (http://web.mit.edu/cgcs/www/MIT_CGCS_Rpt74.pdf), 2004.

Stein, A.F., Lamb, D., Draxler, R.R.: Incorporation of detailed chemistry into a three-dimensional Lagrangian-Eulerian hybrid model: application to regional tropospheric ozone. Atmospheric Environment 34, 4361e4372, 2000.

Stokes, R. H., and Robinson, R. A.: Interactions in aqueous nonelectrolyte solutions. I. Solute-Solvent Equilibria, J. Phys. Chem., 70, 2126–2130, 1966.

Thomson, D. J., W. L. Physick, and R. H. Maryon, Treatment of interfaces in random walk dispersion models, J. Appl. Meteorol., 36, 1284–1295, 1997.

van der Werf, G. R., Randerson, J. T., Giglio, L., Collatz, G. J., Mu, M., Kasibhatla, P. S., Morton, D. C., DeFries, R. S., Jin, Y., and van Leeuwen, T. T.: Global fire emissions and the contribution of deforestation, savanna, forest, agricultural, and peat fires (1997–2009), Atmos. Chem. Phys., 10, 11707-11735, doi:10.5194/acp-10-11707-2010, 2010.

Wen, D., Lin, J. C., Millet, D. B., Stein, A. F., & Draxler, R. R.: A backward-time stochastic Lagrangian air quality model. Atmospheric Environment, 54, 373-386, 2012.

Wen, D., Lin, J. C., Zhang, L., Vet, R., and Moran, M. D.: Modeling atmospheric ammonia and ammonium using a stochastic Lagrangian air quality model (STILT-Chem v0.7), Geosci. Model Dev., 6, 327-344, doi:10.5194/gmd-6-327-2013, 2013.

Wen, D., Zhang, L., Lin, J. C., Vet, R., and Moran, M. D.: An evaluation of ambient ammonia concentrations over southern Ontario simulated with different dry deposition schemes within STILT-Chem v0.8, Geosci. Model Dev., 7, 1037-1050, doi:10.5194/gmd-7-1037-2014, 2014.

Wiedinmyer, C., S. K. Akagi, R. J. Yokelson, L. K. Emmons, J. A. Al-Saadi, J. J. Orlando, and A. J. Soja.: The Fire Inventory from NCAR (FINN): A High Resolution Global Model to Estimate the Emissions from Open Burning. Geosci. Model Dev., 4, 625-641, 2011.

Zdanovskii, A. B.: New methods for calculating solubilities of electrolytes in multi-component systems, Zhur. Fiz. Kim., 22, 1475–1485, 1948.

See http://iek8wikis.iek.fz-juelich.de/HTAPWiki/WP1.1?action=AttachFile&do=view&target=EPA_SOURCE+SPECIATION+FOR+NORTH+AMERICA.xlsx

https://ready.arl.noaa.gov/HYSPLIT_data2arl.php#INFO