STILT-ASP USER’S GUIDE

Task 2 – Development of STILT-ASP into a wildfire analysis tool

Deliverable 2.1: Draft User’s Guide for Using the Tool

TCEQ Contract No. 582-15-50414

Work Order No. 582-16-62311-03

Revision 1.0

Prepared by:

Atmospheric and Environmental Research, Inc. (AER)

131 Hartwell Ave.

Lexington, MA 02466

Correspondence to: malvarad@aer.com

Prepared for:

Erik Gribbin

Texas Commission on Environmental Quality

Air Quality Division

Building E, Room 342S

Austin, Texas 78711-3087

May 13, 2016

Table of Contents

1	Background	6

2	Software and Hardware Requirements	6

2.1	Operating System	6

2.2	Required Libraries	6

2.2.1	Cygwin (64-bit)	6

2.2.2	Java (64-bit, Version 8)	6

2.2.3	Python 2.7	6

2.3	Hardware Specifications	7

3	How to Run STILT-ASP	8

3.1	Run Mode	9

3.2	Input Parameters	10

3.3	Output Species	11

3.4	Optional Parameters	11

3.5	Boundary Conditions	12

3.6	Emissions	13

3.6.1	SMOKE/ZF	13

3.6.2	HTAP	14

3.6.3	MEGAN	14

3.6.4	FINN	15

3.6.5	Nonfire	15

3.7	Meteorology	15

3.8	ASP	16

4	Processing	17

5	Outputs and Archival	19

List of Figures

Figure 1: User Interface for STILT-ASP v0.1.	8

Figure 2: Flowchart for STILT-ASP toolkit.	18

List of Tables

Table 1: STILT-ASP Modules Requiring Python.	7

Table 2: Input Parameters in STILT-ASP	10

Table 3: Common Gas Species in ASP.	11

Table 4: Common Aerosol Species in ASP.	11

Table 5: Optional Parameters in STILT-ASP.	12

Table 6: Emission File Types in STILT-ASP.	13

Table 7: FTP Addresses for NARR and NAM12 Gridfiles.	16

Table 8: ASP Aerosol Scheme Parameters.	16

Table 9: Archive Modes in STILT-ASP.	19

Table 10: File Nametags Used in STILT-ASP.	19

Document Change Record

| Revision | Revision Date | Remarks |
| --- | --- | --- |
| 0.1 | 12 May 2016 | Internal Version for Review |
| 1.0 | 13 May 2016 | Rough Draft Supplied to TCEQ for Review |

# Background

The STILT-ASP toolkit provides a mechanism to predict the relative contribution of wildfires to atmospheric pollution at a specific receptor (space-time location).  The tool has two main programs: a Lagrangian particle back-trajectory model and a chemistry solver that computes the concentration of various chemical species for each particle along the back-trajectories.

The user starts a STILT-ASP run by executing the STILT-ASP.exe program.  This will load a Graphical User Interface (GUI) to the model.  The user then will select a series of input, output and configuration parameters, and run the model.  Output data are then archived to a user-specified location on the file system.

Due to the requirements of the Cygwin license, both executable binaries and source code for all programs in the STILT-ASP toolkit are provided. As such, an advanced user will have the opportunity to modify and build the model from source code.  However, this is neither necessary nor recommended for the standard STILT-ASP operator.

# Software and Hardware Requirements

## Operating System

STILT-ASP is currently configured to run on a 64-bit Windows 7 workstation only.  No other operating systems are supported at this time.

## Required Libraries

STILT-ASP currently requires the following programs and inputs in order to run properly:

### Cygwin (64-bit)

All executable code for the back trajectory calculation and chemistry solver algorithms was built using the 64-bit version of Cygwin, a Linux-to-Windows emulation mechanism. The 32-bit version of Cygwin was not used as certain dependencies such as NetCDF and IOAPI are not perfectly cross-compatible between 32- and 64-bit architectures.

The required Cygwin libraries are provided with the STILT-ASP toolkit, and it is anticipated that only the advanced user will need to worry about installation of Cygwin.  For more information, see the official Cygwin webpage at http://cygwin.com.

### Java (64-bit, Version 8)

The Graphical User Interface (GUI) used to run STILT-ASP was built with 64-bit Java version 8.  Versions 7 and below have security flaws and are not supported.

### Python 2.7

Two components of STILT-ASP require Python. These are listed in Table 1 .

Table 1: STILT-ASP Modules Requiring Python.

| Process | Executable Name | Used By? | Description |
| --- | --- | --- | --- |
| Emissions Preprocessor | emissionsPreprocessor.exe | All Users | If unprocessed HTAP, MEGAN and FINN files are used during a run, the emissions preprocessor will produce the needed non-fire and fire emission files in NetCDF format. |
| Pollutant file specifications in STILT-Chem format | AspToDirt.exe | Expert Users | If the ASP configuration files (./ASP/InorganicInputDecks/) are changed (which should not be done except by an advanced user), this utility must be run to regenerate the pollutant specifications in STILT-Chem format. |

The following modules not in the Python 2.7 standard library must be present in the Python build:

- Numpy (version TBD)

- NetCDF4

- Pygrib

- Basemap (mpl_toolkits.basemap)

All the required Python modules are included into the executables in Table 1 using the py2exe utility.

## Hardware Specifications

STILT-ASP v0.1 should only be run on a Windows 7 Professional 64-bit workstation with at least 16 GB of RAM.  Exact RAM requirements depend on user inputs.  The three primary factors affecting the RAM usage and run time of STILT-ASP are the following:

- Length of back trajectory: The STILT portion of the program outputs the entire history of the back trajectory for each Lagrangian parcel, and the STILT-ASP portion will eventually load this parcel history into RAM for the chemistry solver.  Assuming the timestep stays constant, the RAM usage for a single time series of data is thus linearly proportional to the length of the back trajectory calculation.  STILT-ASP has not been tested for back trajectories longer than 1 week and the memory usage for such a trajectory may become prohibitive.

- Desired output timestep: STILT-ASP requires a fixed integer timestep (in minutes) over which to calculate the chemistry step for each particle.  Assuming the size of the back trajectory period stays constant, the amount of RAM consumed is inversely proportional to the timestep size.  The default timestep for STILT-ASP runs is 15 minutes when using the 32-km NARR meteorology grids and 5 minutes when using the 12-km NAM12 meteorology grids.  It is not recommended that these values be modified except by an expert user.

- Number of Lagrangian parcels in the simulation: STILT-ASP can calculate back-trajectories for an aribitrary number of Lagrangian parcels per timestep.  It is not expected that the operator will modify the default (500 particles), but an increase in this number will result in additional RAM usage and slower run time for both the back-trajectory and chemistry solver calculations.

- Number of output species:  STILT-ASP is programmed to record the entire time and particle history of a select number of chemical species specified by the user.  This variable affects only the chemistry solver algorithm.

The back-trajectory calculation (STILT) generally consumes 4 to 6GB of RAM during a run and requires anywhere from 10-50 minutes per runStiltBackTrajectories.exe call on a Windows 7 Professional 64-bit workstation with a 4-core AMD Opterontm 6176SE processor running at 2.30 GHz and 16 GB of RAM.

The RAM requirements and run time for the chemistry solver step is more variable.  A 2-day back trajectory using the NARR grids and only a single desired species uses under 8 GB of RAM and requires 1-2 hours to complete on the above configuration, while a 1-week back-trajectory at 2-minute resolution with all possible ASP chemical species desired for output can use ≥ 20 GB of RAM and require in excess of 18 hours to finish a single runStiltAsp.exe execution.

# How to Run STILT-ASP

Figure 1: User Interface for STILT-ASP v0.1.

To start STILT-ASP, double-click on the STILT-ASP.exe executable icon.  The user interface shown in Figure 1 will appear.  Inputs are added into ASP by accessing the menu bar at the top of the screen and the buttons along the left side. After all inputs are set, the model is run by clicking on the red “RUN” button in the upper-right corner of the screen.  Status information will be output to the CONSOLE space shown in Figure 1.

Instructions for inserting the proper inputs into STILT-ASP are provided below.

## Run Mode

This pull-down menu located on the menu bar along the top of the screen allows the user to choose one of three operation modes:

- Back-Trajectory Only: This mode generates the propagation history of each Lagrangian parcel according to the configuration specified in the Input Parameters and Meteorology menus.  This is referred to as the “STILT” portion of processing and is executed by the runStiltBackTrajectories.exe file.  No boundary conditions, emissions or ASP inputs of any kind are required for this mode.

Each execution of runStiltBackTrajectories.exe outputs a single NetCDF file (PARTICLE.nc) containing the full parcel history, after which the file is given a unique identification tag and moved to a fixed subfolder of the archive directory specified in the Optional Parameters menu.  It should be noted that these files are 2-3 GB each, and it is the responsibility of the user to ensure there is sufficient space in the archive directory to support these files.

The back-trajectory-only mode is useful for end users that want to calculate the full history for each Lagrangian parcel ahead of time and then run or rerun the chemistry solver at a later time.  This may be useful due to the long processing time required by the chemistry solver in comparison to the back-trajectory calculation.

- Chemistry Solver Only: This mode takes the output from the back trajectory calculation and evaluates the chemical concentrations at each time step.  This is the “STILT-ASP” portion of processing and is executed by the runStiltAsp.exe file.

This processing step is much more computationally demanding than the back trajectory calculation, and also requires explicit specification of nearly all configuration information in order to generate an accurate result.  The primary input in this step is the single NetCDF file (per executable call) containing the Lagrangian parcel history that was generated in the back trajectory calculation; if STILT-ASP cannot find this file it will be unable to proceed further.  If available, runStiltAsp.exe will read this file, along with the chemical configuration and boundary condition/emission inputs, and provide chemical concentrations (ppbv for gases, µg/m3 for particulate matter) for the desired chemical species supplied by the Output Species menu.  This output data can be produced either as a simple comma-separated-value text file or as a comprehensive NetCDF file containing much more information on the evolution of chemical compounds in each Lagrangian parcel suitable for advanced users.

- Full Run: This mode will run the back-trajectory (STILT) calculation and chemistry solver (STILT-ASP) steps in immediate succession.

## Input Parameters

This menu along the menu bar at the top of the screen allows the user to enter the simulation time range and receptor parameters (only a single receptor per run is currently permitted in STILT-ASP).  Clicking the prompt will bring up a pull-down menu containing the inputs listed in Table 2, all of which are required for all run modes described in Section 3.1. Values for each field must be entered in the corresponding prompt.

Table 2: Input Parameters in STILT-ASP

| Field Name | Field Type | Value Range | Units | Description |
| --- | --- | --- | --- | --- |
| Year | Integer | 2002-2016 | N/A | The 4-digit starting year of the simulation |
| Month | Integer | 1-12 | N/A | The starting month of the simulation |
| Day | Integer | 1-31 | N/A | The starting day of the simulation |
| Hour | Integer | 0-23 | N/A | The starting hour (UTC) of the simulation |
| Receptor Latitude | Decimal | -90.0 to 90.0 | Degrees North | Latitude of the desired output receptor |
| Receptor Longitude | Decimal | -180.0 to 180.0 | Degrees East | Longitude of the desired output receptor |
| Receptor Altitude | Decimal | >0 | Meters | Desired altitude above MSL at which to produce output |
| Back Trajectory Length | Integer | 1-168 | Hours | The number of hours for which to extend the back trajectory calculation (ignored if forward-only run is specified). Currently limited to a 1 week maximum. |
| Forecast Period | Integer | ≥ 0 | Hours | The number of receptor hours over which to iterate (enter 0 for a single run) |

It should be noted that STILT-ASP has never been tested for multiple receptors simultaneously, and the GUI does not provide a mechanism for inputting multiple receptors at a time.

## Output Species

STILT-ASP allows the operator to choose which chemical species are selected for output.  The user may choose from a set of commonly used species or an expanded list containing all chemical species in ASP by clicking on the Output Species menu at the top of the screen.

Species are divided according to phase and chemical family.  The operator may click on the particular pull-down menu until a list of species for that phase/family type are displayed, and each species is selected by clicking on the corresponding box.

Table 3: Common Gas Species in ASP.

| O3 | NO | NO2 | OH | Ethane | HO2 | CH4 |
| --- | --- | --- | --- | --- | --- | --- |
| NO3 | N2O5 | HONO | CO | CO2 | HCHO | NH3 |

Table 4: Common Aerosol Species in ASP.

| PM2.5 | Fine Black Carbon | Fine Organic Aerosol |
| --- | --- | --- |

## Optional Parameters

This pull-down menu allows the end user to further customize the STILT-ASP run by allowing for additional configuration information.  However, none of the following fields are required in order for STILT-ASP to work properly, and it is recommended that these fields only be modified by an expert user.

Table 5: Optional Parameters in STILT-ASP.

| Field Name | Field Type | Value Range | Units | Description | Default Value |
| --- | --- | --- | --- | --- | --- |
| Number of Particles | Integer | ≥1 | N/A | Number of puffs/parcels to carry in the simulation | 500 |
| Forecast Interval | Integer | [TBD] | Hours | The number of hours over which to iterate when proceeding forward in time from run to run | 1 |
| Fire Emission Threshold | Decimal | >= 0.0 | ppmv (NOT ppbv) | The minimum change in CO and/or CO2 concentration required to indicate the presence of a fire | 0.0 |
| Log (L) or Console (C)? | Character | L or C | N/A | If L, STILT-ASP logging information is written to a file with an identification tag. Otherwise, output printed to console. | C |
| STILT timestep | Integer | TBD | Minutes | Integration/ Iteration timestep used by STILT and STILT-ASP | 15 for NARR, 5 for NAM12 |
| Output File Type | Integer | 0 | N/A | This flag controls the resulting output of the STILT-ASP step. | 0 = CSV 1 = NetCDF |
| Archive Directory | String | N/A | N/A | Directory where all output products are placed and retrieved | ${STILT_ASP_ROOTDIR}/ TestArchives |

It should be noted that the back-trajectory executable (STILT) can be theoretically configured to output the chemical concentrations on grids different from the input meteorology.  However, this option is not compatible with STILT-ASP, as the chemistry solver did not function properly when confronted with a timestep of 0 seconds during testing.  For this reason, STILT-ASP requires both a fixed time/integration step (with output values present at each time) and a chemical output grid of the same resolution as the input meteorology grids.

## Boundary Conditions

Clicking this button will bring up a pull-down menu allowing the user to provide chemical boundary conditions to STILT-ASP.  The only option currently supported is for NetCDF files generated by the Model for Ozone and Related Chemical Tracers (MOZART) model, and only one file is allowed per STILT-ASP run.

MOZART files are not currently available directly via FTP.  Prospective users must instead request these files from NCAR externally via a web form (currently found at www.acom.ucar.edu/wrf-chem/mozart.shtml).  As a result, STILT-ASP will only permit the operator to add a single MOZART file local to the file system.  This is done by clicking the “Local” button under the MOZART prompt.  A file browser interface will pop up, allowing the user to find the desired MOZART file on the file system.

A MOZART file is not required for the back trajectory (STILT) portion of STILT-ASP, and is ignored if provided.  If the chemistry solver is invoked, the program will notify the user if the MOZART file is unable to be found.

## Emissions

The emissions button provides access to chemical emissions data.  Clicking on the emissions button shows a pull-down menu containing options for each of the files mentioned in Table 6.   Local files are found by clicking the “Local” button and browsing the file system.  For data types that allow remote file access, remote files are retrieved by clicking the “Remote” button.  The operator will select the local destination folder for the content to be downloaded using the file browser window under the “Destination” button, followed by entering the full remote path to the file (FTP/SFTP/HTTP/HTTPS) in the box beside the “Remote Path” window.  Similarly to the boundary conditions files, no emissions are required for the STILT portion of STILT-ASP, and any files provided are unused.

Table 6 lists the types of emissions data are currently allowed by STILT-ASP. Each file type is discussed below.

Table 6: Emission File Types in STILT-ASP.

| File Type | Description | Max Number of Files Allowed | Data Format |
| --- | --- | --- | --- |
| SMOKE (4D) | 4D emission grid file (Lambert projection) from the SMOKE model | 3 | IOAPI-formatted NetCDF |
| ZF | Vertical level heights in 4D SMOKE emission grid | 3 | NetCDF |
| HTAP | Anthropogenic emissions from the EDGAR-HTAP project | TBD | NetCDF |
| MEGAN | Biogenic emissions as generated by the Model of Emissions of Gases and Aerosols from Nature (MEGAN) | TBD | NetCDF |
| FINN | Fire Inventory files from NCAR | TBD | ASCII (raw) or NetCDF (preprocessed) |
| Nonfire | HTAP+MEGAN preprocessed files | 1 | NetCDF |

### SMOKE/ZF

The Space Matrix Operator Kernel Emissions (SMOKE) model is capable of generating 4D pollutant emission data in a Lambert projection and outputting these data to IOAPI-compliant NetCDF data files.  These files can be in various resolutions, such as 4-, 12- and 36-km grids.

The operator may retrieve up to 3 SMOKE emission files from the local filesystem by first clicking on the “Emissions” menu on the left side of the screen, then clicking on the “SMOKE” button that appears.  This loads a menu allowing the user to choose the actual emissions data (SMOKE) or the vertical level heights on a given emission grid (ZF).

Clicking on the “SMOKE” menu will produce buttons for each SMOKE file, and clicking a button will load a file browser menu allowing the user to locate the desired SMOKE emission grid.  If more than one SMOKE file is desired, the operator should keep in mind that STILT-ASP expects SMOKE files #1, #2, #3 to be ordered in a decreasing grid resolution sequence (i.e., the finest grid should be #1, the second-finest grid should be #2 and so on).

The 4D emission grid files from the SMOKE model do not contain the definition of the vertical level heights in the emission grid.  These files must be loaded separately, and this is done by clicking on the aforementioned “ZF” button located beneath the “SMOKE” menu under the main “Emissions” button on the left side of the screen.  Each SMOKE emissions file requires a single ZF file specifying the vertical level heights for the corresponding emissions grid.

STILT-ASP has not been tested with both SMOKE and HTAP/MEGAN inputs simultaneously. Using a preprocessed FINN file only (without HTAP/MEGAN input) in addition to SMOKE data in STILT-ASP may or may not result in a double-counting of emissions due to wildfires, depending on whether or not the SMOKE run that generated the emission grids took wildfires into account.  There is no way for STILT-ASP to determine the status of fire emissions in a SMOKE file during a run, so the operator may use SMOKE files at their own risk.

These files are exceptionally large; the 4 km resolution 4D SMOKE output files can be in excess of 100 GB per file.  It is the responsibility of the user to ensure there is sufficient space in the file system to hold these files.

### HTAP

The EDGAR (Emissions Database for Global Atmospheric Research) program has compiled HTAP (Hemispheric Transport of Air Pollution) anthropogenic emission data for the years 2008 and 2010 for general use.

The HTAP files are not currently available via FTP (this is due to a JRC security issue). The needed HTAP files are provided as part of the STILT-ASP toolkit, but can also be retrieved via a web form outside of STILT-ASP and downloaded to a single directory if not already on the local filesystem.  Once this is done, the operator may click the “HTAP” prompt under the Emissions menu, which loads a file browser window.  The user should then select the directory containing the HTAP emissions files, which are later processed by the STILT-ASP emissions preprocessor. Years before 2009 use the 2008 emissions, while years 2009 and later use the 2010 emissions.

### MEGAN

The Model of Emission of Gases and Aerosols from Nature (MEGAN) model produces biogenic emission files for use in STILT-ASP.  MEGAN data are available in several formats but only the NetCDF format files for the years 2000-2010 are compatible with STILT-ASP.

The needed MEGAN files are provided as part of the STILT-ASP toolkit, but can also be retrieved via a web form external to STILT-ASP and downloaded to a single directory.  After the files are locally available, the user may click on the “MEGAN” prompt under the “Emissions” menu on the left side of the screen.  This will bring up a file browser interface.  The operator should select the directory containing the MEGAN input files, which will be loaded into the emissions preprocessor later on. Years later than 2010 use the 2010 emissions.

### FINN

Fire INventory file from NCAR (FINN) files contain daily fire emission products for inclusion into models such as MOZART and GEOS-Chem in ASCII comma-separated (CSV) form.   STILT-ASP currently expects all FINN emission files to have MOZART speciation and will not work properly otherwise.  Modifications to allow for SAPRC and CB speciation are proposed for inclusion in a future version.

The FINN button currently provides for retrieval of local files only, as remote files are not available via FTP and must be externally downloaded using a web form.   Once the files have been downloaded, the operator should click on the “Local” button to load a file system browser interface.

STILT-ASP will accept two types of FINN input, a single NetCDF file (corresponding to the output of the STILT-ASP emissions preprocessor) or a single directory.  If the operator enters a directory, STILT-ASP will retrieve all CSV files in that directory and trigger a flag forcing the emissions preprocessor to convert the files into a form compatible with the runStiltAsp.exe program.

If a NetCDF file is added, STILT-ASP will assume that the operator will not be running the emissions preprocessor and that a merged/regridded HTAP/MEGAN NetCDF emission file from non-fire sources will be added to the input set using the Nonfire button.

### Nonfire

As discussed in Sections 3.6.2 and 3.6.3, STILT-ASP may be configured to run the emissions preprocessor by loading HTAP, MEGAN and FINN files into the tool in their unprocessed state.  STILT-ASP also allows the operator the option of skipping the preprocessor by loading a single HTAP/MEGAN merged NetCDF file into STILT-ASP.  This is done by clicking on the “Nonfire” button and using the file browser interface that appears.

No other file or directory types are compatible with the Nonfire option, and if the operator chooses to use this option it is required that a single preprocessed FINN file be added using the “FINN” option detailed in Section 3.6.4. If this condition is not satisfied, STILT-ASP will throw an error in the scripting stage and the run will terminate.

## Meteorology

STILT-ASP supports only ARL-formatted meteorology data; other formats such as GRIB are not compatible.  The Air Resources Laboratory (ARL) provides documentation and utilities as to how to convert GRIB files to ARL format; see Meteorology Data Conversion Utilities for more information.

STILT-ASP currently supports daily files using the NAM12 grid (12-km resolution) and monthly files using the NARR grid (32 km resolution) downloaded from the FTP sites documented in Table 7.  The operator should click on the Meteorology pull-down menu and select the grid type.  Clicking on that menu will give the user the option to select daily (only NAM12) or monthly (only NARR) files upon clicking on the next pull-down menu.

Clicking on the “Local” menu will bring up a file browser window, allowing a user to select either a single monthly file or a single directory containing a set of daily meteorology grid files.  It should be noted that if the selected path is a file, STILT-ASP will assume that it is a monthly file, and that if the selected path is a directory, STILT-ASP will assume that the given directory contains the desired daily files.

If desired, the operator may instead retrieve the meteorology data from a remote (FTP) source by clicking the “Remote” button.  FTP retrieval is currently performed using a Windows version of the Linux utility “wget”.  The user has the option to let STILT-ASP automatically retrieve the desired files (recommended) by clicking on the “Default” menu and using the file browser window under the “Destination” button to select the destination folder on the local system for the data files.  This option will automatically retrieve all the meteorology files of the chosen type for the needed year and month and deposit them to the folder selected by the “Destination” button.

Advanced users may instead directly enter a custom wget command to be executed by STILT-ASP by clicking on the “Custom” button.  The operator will then enter the full command in the “Command” box and choosing the destination folder on the local system by using the file browser contained in the “Destination” button.  As previously mentioned, it is the responsibility of the STILT-ASP operator to ensure there is sufficient space on the filesystem to store the remote files.

Table 7: FTP Addresses for NARR and NAM12 Gridfiles.

| Grid Type | Interval | FTP Directory Address | File Names |
| --- | --- | --- | --- |
| NARR | Monthly | ftp://arlftp.arlhq.noaa.gov/narr/ | NARR<4-digit year><2-digit month> |
| NAM12 | Daily | ftp://arlftp.arlhq.noaa.gov/archives/nam12/ | <4-digit year><2-digit month><2-digit day>_nam12 |

## ASP

Clicking on the ASP button will allow the operator to choose between several simple ASP configuration options, each accessible by clicking on their corresponding entry in the ASP menu.  Current options are listed below:

Table 8: ASP Aerosol Scheme Parameters.

| Name | Type | Units | Explanation | Default Value |
| --- | --- | --- | --- | --- |
| ASP Aerosol Scheme | (Radio Button) | N/A | This option allows the operator to choose between the following aerosol schemes: - No aerosols - Standard ASP volatility basis set (VBS) scheme - AERO6 (CMAQ) scheme - Simple Empirical Model | No aerosols |

As previously mentioned, the model execution begins after clicking the red “RUN” button at the upper-right corner of the screen.  Do not press this button until everything needed to run the model has been input via the GUI.

# Processing

The processing flow for the STILT-ASP toolkit is shown below in Figure 2:

Figure 2: Flowchart for STILT-ASP toolkit.

# Outputs and Archival

After STILT-ASP has completed, the various output files produced by both STILT (back trajectory) and STILT-ASP (chemistry solver) are deposited into the specified archive directory.  There are two archive profiles available; the archive profile is automatically chosen by STILT-ASP based on the value of the “Output Type” field in the Output Parameters menu.

Table 9: Archive Modes in STILT-ASP.

| Archive Profile | Files Archived (STILT) | Files Archived (STILT-ASP) | When/How to Use? |
| --- | --- | --- | --- |
| Normal | PARTICLE.nc | ConcOut.csv | Output Type = 0 |
| Verbose | PARTICLE.nc; CONTROL; MESSAGE; SETUP.CFG; CONC.CFG; hymodelc.out; cdump; JCLmessage | ConcOut.nc; CONTROL; MESSAGE; SETUP.CFG; CONC.CFG; hymodelc.out; cdump; JCLmessage | Output Type = 1 |

In order for the chemistry solver to function correctly, there must be a NetCDF back trajectory file in the specified archive directory that contains the exact same filename identification tag as the one generated by the STILT-ASP GUI.  Therefore, if the user wishes to run the back trajectory calculation and return to the chemistry solver at a later time, STILT-ASP will only work properly if all of the time, receptor and directory information input into the chemistry solver run is identical to those used in the previous back trajectory run.

A combination of the following parameters forms the file nametag, which is then appended to the end of the generic file name in question.

Table 10: File Nametags Used in STILT-ASP.

| Model Type | Name Tag |
| --- | --- |
| STILT (back-trajectory) | [YYYY]-[MM]-[DD]-[HH]-[R]-[NHRS]back-[MET]-[NPART]par-STILT |
| STILT-ASP (chemistry solver) | [YYYY]-[MM]-[DD]-[HH]-[R]-[NHRS]back-[MET]-[NPART]par-[EMISTYP]-STILTASP |

Explanation:

- YYYY = 4-digit year

- MM = 2-digit month

- DD = 2-digit day

- HH = 2-digit hour

- R = Receptor name

- NHRS = Number of hours over which the back trajectory is/was calculated

- MET = Name of the meteorology grid (“NARR” or “NAM12”)

- NPART = Number of puffs/parcels used in the simulation

- EMISTYP = An identification string telling the user which emission sources, if any, have been used in the simulation (form TBD)