DRAFT FINAL REPORT

Investigating the Impact of Meteorology on O3 and PM2.5 Trends, Background Levels, and NAAQS Exceedances

TCEQ Contract No. 582-15-50415

Work Order No. 582-15-54118-01

Revision 1.0

Prepared by:

Atmospheric and Environmental Research, Inc. (AER)

131 Hartwell Ave.

Lexington, MA 02466

Prepared for:

Erik Gribbin

Texas Commission on Environmental Quality

Air Quality Division

Building E, Room 342S

Austin, Texas 78711-3087

August 15, 2015

Document Change Record

| Revision | Revision Date | Remarks |
| --- | --- | --- |
| 1.0 | 15 August 2015 | Draft version submitted to TCEQ |

TABLE OF CONTENTS

Abstract	9

1	Introduction	10

1.1	Project Objectives	10

1.2	Purpose and Background	10

1.2.1	Trends in O3 and PM2.5	10

1.2.2	Regional Background Concentrations of O3 and PM2.5	10

1.2.3	Synoptic- and Urban-scale Meteorological Controls on O3 and PM2.5	11

1.3	Report Outline	13

2	Task 2: Effects of Meteorology on O3 and PM2.5 Trends	13

2.1	Input Data and Processing	13

2.2	Generalized Additive Model	13

2.2.1	Baseline GAMs (gam01_baseline)	14

2.2.2	Extended GAMs (gam02_extended and gam03_extended)	16

2.2.3	Cross Validation Analysis	17

2.3	GAMs for Background PM2.5 and O3	17

2.4	Meteorologically Adjusted Trends of O3 and PM2.5	17

2.5	Conclusions	26

3	Task 3: Background O3 and PM2.5	27

3.1	Daily Estimates of Regional Background O3 and PM2.5 (TCEQ Method)	27

3.2	Spatial and temporal trends of background O3 and PM2.5	27

3.3	Alternative Methods To Determine Regional Background O3 and PM2.5	32

3.3.1	Determining Background O3 with PCA	32

3.3.2	Determining Background PM2.5 with PCA	36

3.3.3	Determining Background O3 with Satellites	39

3.3.4	Determining Background PM2.5 with Satellites	40

4	Task 4: Importance of Synoptic/Mesoscale Meteorological Conditions in Explaining/Forecasting Background and Maximum O3 and PM2.5	41

4.1	Synoptic Map Type Analysis	41

4.1.1	Technical Method and Results	41

4.1.2	Discussion	51

4.2	Urban-scale meteorological predictors of O3 and PM2.5	52

4.2.1	Logistic Regression Approach	52

4.2.2	Results and Discussion	59

5	Quality Assurance Steps and Reconciliation with User Requirements	63

5.1	Task 2: Development of GAMs	63

5.2	Task 3: Background O3 and PM2.5	64

5.3	Task 4: Synoptic and Mesoscale Controls of O3 and PM2.5	65

6	Conclusions	67

7	Recommendations for Future Study	71

8	References	72

Appendix A. Effects of Meteorology on O3 and PM2.5 Trends	73

A.1	Technical Approach	73

A.1.1	Input Data and Processing	74

A.1.2	HYSPLIT Back Trajectories	76

A.1.3	Generalized Additive Model (GAM) Fitting Procedure	78

A.1.4	Baseline GAMs (gam01_baseline)	79

A.1.5	Extended GAMs (gam02_extended and gam03_extended)	89

A.1.6	Cross-Validation Analysis	102

A.2	File Descriptions	106

A.2.1	Input data (./data/)	108

A.2.2	Data Processing Scripts (./scripts/)	108

A.2.3	HYSPLIT	110

A.2.4	Processed Input Data Files in CSV Format (./csv_files/)	111

A.2.5	GAM scripts (./full_gam_fits/)	112

A.2.6	GAM Output Files (./full_gam_fits/o3_model/ and ./full_gam_fits/pm2.5_model/)	112

A.3	Quality Assurance Steps	114

A.3.1	Model Evaluation	114

A.3.2	Model Documentation	115

B	Appendix B. Technical Memo: Estimating Background O3 and PM2.5	118

B.1	Introduction	118

B.2	Technical Approach	118

B.2.1	Selection of Background Sites	118

B.2.2	Calculation of MDA8 O3 and daily average PM2.5 Values for Each Site	120

B.2.3	Estimating Preliminary Background Values	120

B.2.4	Linear Regressions Test and Outlier Analysis	120

B.3	File Descriptions	123

B.3.1	Urban Area Ozone Files (file name = *_flagged_O3_v3.csv, six files in total)	123

B.3.2	Urban Area PM2.5 Files (file name = *_flagged_PM_v2.csv, six files in total)	124

B.3.3	Texas O3 Background (file name = TX_State_bkgrd_O3_calc.csv)	125

B.3.4	Texas PM2.5 Background (file name = TX_State_PM_calc.csv)	126

B.4	Quality Assurance Steps	126

C	Appendix C: File Descriptions in Final Deliverable Package	127

C.1	./P1952_trend_plots.xlsx	127

C.2	Subdirectory ./MAPTYPE/	127

C.2.1	Map Type Files	127

C.2.2	R Scripts	127

C.2.3	Updated GAM data files (./MAPTYPE/*_merged_GLM_all_type_exceed.csv)	128

C.2.4	R output files for logistic regression (./MAPTYPE/*/RData_logistic_gam*)	128

C.3	Subdirectory ./PCA/SCRIPTS	129

Appendix D: Logistic Regression Probability Plots for DFW, SA, and ARR	130

List of Figures

Figure 1. Original (dashed lines) and meteorologically adjusted (solid lines) annual averages for total and background O3 (top) and PM2.5 (bottom) for the Houston/Galveston/Brazoria urban area. Equations for the OLS linear regressions are shown on the plot as well.	20

Figure 2. As in Figure 1 but for the Dallas/Fort Worth urban area.	21

Figure 3. As in Figure 1 but for the San Antonio urban area.	22

Figure 4. As in Figure 1 but for the Austin/Round Rock urban area.	23

Figure 5. Original (dashed lines) and meteorologically adjusted (solid lines) annual averages for total O3 (top) and PM2.5 (bottom) for the Beaumont/Port Arthur urban area. Equations for the OLS linear regressions are shown on the plot as well.	24

Figure 6. As in Figure 5 but for the Tyler/Longview/Marshall urban area.	25

Figure 7. Box-and-whisker plots for the background MDA8 O3 for Austin/Round Rock, Beaumont/Port Arthur and Tyler-Longview-Marshall as estimated using the TCEQ method. The red line is the median, the dashed black line is the mean. Box edges show the 25th and 75th percentiles, the whiskers and crosses show the range of the data.	28

Figure 8. Box-and-whisker plots for the background MDA8 O3 for Dallas/Fort Worth, Houston/Galveston/Brazoria, and San Antonio as estimated using the TCEQ method.	29

Figure 9. Box-and-whisker plots for the background daily average PM2.5 for Austin/Round Rock, Beaumont/Port Arthur and Tyler-Longview-Marshall as estimated using the TCEQ method.	30

Figure 10. Box-and-whisker plots for the background daily average PM2.5 for Dallas/Fort Worth, Houston/Galveston/Brazoria, and San Antonio as estimated using the TCEQ method.	31

Figure 11. PCA-derived background ozone in Houston applied over the entire ozone season dataset (x-axis, 10 years and all sites) compared to our original TCEQ method of determining background ozone (y-axis).	33

Figure 12. PCA-derived background ozone in Houston compared to the original TCEQ method.  This approach applied the PCA over time spans during the ozone season; May to July (red) and August to October (blue) with the overall slope and r2 value printed in black.	34

Figure 13. Monthly (left) and yearly (right) background ozone (ppbv) as derived using the PCA method.  Box plots during the ozone season for Austin, Dallas, Houston and San Antonio are shown.	35

Figure 14. PCA-derived background PM2.5 in Houston compared to the original TCEQ method. The PCA was applied to the entire 10-year time span for all sites.	37

Figure 15. PCA-derived background PM2.5 in Houston in just July months compared to the original TCEQ method.	38

Figure 16. PCA-derived background PM2.5 in Houston compared to the original TCEQ method.  The PCA was applied to 3 different time spans: August to October (blue), April to July (red) and November-March (green).	39

Figure 17. From van Donkelaar et al., (2015).  Spatial plot (Top) and scatter plot (bottom) over the US of optimal estimation (OE) approach (far right) for simulating near-surface PM2.5 concentrations compared to GEOS-Chem (center) and in situ measurements from the AERONET sites (far left).  Presented at the 7th annual GEOS-Chem meeting at Harvard University, 2015.	40

Figure 18. Synoptic maps types determined from 850 mbar geopotential height fields from the 32 km North American Regional Reanalysis using the method of Hegarty et al (2007).	42

Figure 19. Box and whisker plots of the distributions of total MDA8 O3 (ppbv, top left), background MDA8 O3 (ppbv, top right), total daily average PM2.5 (µg m-3, bottom left), and background daily average PM2.5 (µg m-3, bottom right) for the Houston/Galveston/Brazoria urban area. The thick black line is the median of the distribution, the boundaries of the boxes are the 25th and 75th percentiles, and the whiskers cover the range of the data or all values within 1.5 times of the interquartile range (IQR) of the box, whichever is smaller. The circles denote outliers beyond 1.5 × IQR of the box. The horizontal lines show the criteria denoting “high” values of each metric, as discussed in the text.	43

Figure 20. As in Figure 19, but for the Dallas/Fort Worth urban area.	45

Figure 21. As in Figure 19, but for the San Antonio urban area.	47

Figure 22. As in Figure 19, but for the Austin/Round Rock urban area.	49

Figure 23. Probability of the total MDA8 O3 exceeding 70 ppbv for the Houston/Galveston/Brazoria urban area as a function of afternoon mean temperature (oC), daily wind speed (m/s), and synoptic type (as defined in Section 4.1).	55

Figure 24. As in Figure 23, but for the probability that background MDA8 O3 will exceed 55 ppbv.	56

Figure 25. As in Figure 23, but for the probability that total daily average PM2.5 will exceed 17 g/m3.	57

Figure 26. As in Figure 23, but for the probability that background daily average PM2.5 will exceed 13 g/m3.	58

Figure A.1. Ensemble back-trajectory run for the Houston/Galveston/Brazoria area on August 25, 2013.	78

Figure A.2. Smooth functions for the baseline GAM (gam01_baseline) fit to HGB MDA8 O3 data. The y-axis scale is the scale of the “linear predictor”, i.e. the deviation of the natural logarithm of the MDA8 O3 in ppbv from its mean value.	81

Figure A.3. Year-to-year deviations from 2005 for the baseline GAM (gam01_baseline) fit to HGB MDA8 O3 data. The y-axis scale is the scale of the “linear predictor”, i.e. the deviation of the natural logarithm of the MDA8 O3 in ppbv from its mean value. The black center bar is the mean value while the error bars are the 95% confidence intervals. The red and blue circles are the mean values from the two-fold cross-validation analysis of Section A.1.6.	82

Figure A.4. GAM evaluation plots for the baseline GAM (gam01_baseline) fit to HGB MDA8 O3 data.	83

Figure A.5. Smooth functions fit for the baseline GAM (gam01_baseline) fit to HGB daily average PM2.5 data. The y-axis scale is the scale of the “linear predictor”, i.e. the deviation of the natural logarithm of the daily average PM2.5 in µg m-3 from its mean value.	85

Figure A.6. Year-to-year deviations from 2005 for the baseline GAM (gam01_baseline) fit to HGB daily average PM2.5 data. The y-axis scale is the scale of the “linear predictor”, i.e. the deviation of the daily average PM2.5 in µg m-3 from its mean value. The black center bar is the mean value while the error bars are the 95% confidence intervals. The red and blue circles are the mean values from the two-fold cross-validation analysis of Section A.1.6.	86

Figure A.7. GAM evaluation plots for baseline GAM (gam01_baseline) fit to HGB MDA8 O3 data.	87

Figure A.8. Smooth functions for the small extended GAM (gam03_extended) fit to HGB MDA8 O3 data.	97

Figure A.9. Year-to-year deviations from 2005 for the small extended GAM (gam03_extended) fit to HGB MDA8 O3 data.	98

Figure A.10. GAM evaluation plots for the small extended GAM (gam03_extended) fit to HGB MDA8 O3 data.	99

Figure A.11. Smooth functions for the small extended GAM (gam03_extended) fit to HGB daily average PM2.5 data.	100

Figure A.12. Year-to-year deviations from 2005 for the small extended GAM (gam03_extended) fit to HGB daily average PM2.5 data.	101

Figure A.13. GAM evaluation plots for the small extended GAM (gam03_extended) fit to HGB daily average PM2.5 data.	102

Figure A.14. Scatterplots for the GAM-predicted (x-axis) versus the measured (y-axis) values of maximum daily average PM2.5 for the Houston/Galveston/Brazoria area using gam03_extended. The top row uses mtot to predict the first (left) and second (right) of the randomly distributed halves of the dataset. The bottom row uses m2, which was trained on data set 2, to predict the “test” data set 1 (left) and uses m1 to predict data set 2 (right). The black line is a linear fit of the predicted to actual values, while the red dashed line is the 1:1 line.	104

Figure A.15. Houston/Galveston/Brazoria fits for maximum daily average PM2.5 versus HYSPLIT back trajectory bearing for mtot (black with error bars), m1 (red) and m2 (blue) for gam03_extended. Predicted values for 200 randomly selected datapoints are plotted.	105

Figure A.16. Flow chart showing the processing from the original data sources (green boxes) to the final CSV file (red box) that is used as input for the GAM fitting scripts.	107

Figure A.17. Flow chart showing the processing from the input CSV file generated at the end of Figure A.16 (red box) to the GAM output files (light green box).	107

Figure B.1. Maximum versus background MDA8 O3 values for the HGB area.	122

Figure D.1. Probability of the total MDA8 O3 exceeding 70 ppbv for the Dallas/Fort Worth urban area as a function of afternoon mean temperature (oC), daily wind speed (m/s), and synoptic type (as defined in Section 4.1).	130

Figure D.2. As in Figure D.1, but for the probability of background MDA8 O3 exceeding 55 ppbv.	131

Figure D.3. As in Figure D.1, but for the probability of total daily average PM2.5 exceeding 17 g/m3.	132

Figure D.4. As in Figure D.1, but for the probability of background daily average PM2.5 exceeding 13 g/m3.	133

Figure D.5. As in Figure D.1 but for the San Antonio urban area.	134

Figure D.6. As in Figure D.2 but for the San Antonio urban area.	135

Figure D.7. As in Figure D.3 but for the San Antonio urban area.	136

Figure D.8. As in Figure D.4 but for the San Antonio urban area.	137

Figure D.9. As in Figure D.1 but for the Austin/Round Rock urban area.	138

Figure D.10. As in Figure D.2 but for the Austin/Round Rock urban area.	139

Figure D.11. As in Figure D.3 but for the Austin/Round Rock urban area.	140

Figure D.12. As in Figure D.4 but for the Austin/Round Rock urban area.	141

List of Tables

Table 1. Urban areas of interest to this study.	10

Table 2. Projected Schedule for TCEQ Work Order No. 582-15-54118-01	12

Table 3. Original and meteorologically adjusted linear trends (±95% confidence intervals) of total and background (BG) MDA O3 and daily average PM2.5 from 2005-2015 using the gam03 models. Trends significantly different from zero with 95% confidence are in bold.	18

Table 4. 90th percentile of the total and background (BG) MDA8 O3 and daily average PM2.5 values for each Group 1 urban area for 2005-2010. Only values during the O3 season (May-Oct.) are considered for O3.	42

Table 5. Percentage of observations below the criteria chosen to represent “high” values of total and background (BG) MDA8 O3 and daily average PM2.5 values for each Group 1 urban area for 2005-2010. The chosen criteria are in parentheses in the first row. Only values during the O3 season (May-Oct.) are considered for O3.	42

Table 6. Percentage of observations above the criteria chosen to represent “high” values of total and background (BG) MDA8 O3 and daily average PM2.5 values for the Houston/Galveston/Brazoria urban area. The chosen criteria are in parentheses in the first column. Only values during the O3 season (May-Oct.) are considered for O3.	44

Table 7. As in Table 6 but for the Dallas/Fort Worth urban area.	46

Table 8. As in Table 6 but for the San Antonio urban area.	48

Table 9. As in Table 6 but for the Austin/Round Rock urban area.	50

Table 10. Deviance explained (%, bold) and URBE score (unitless, italics) for the logistic models for total and background O3 and PM2.5	59

Table A.1. Surface meteorological sites selected for GAM fitting.	74

Table A.2. IGRA sites used for each urban area.	75

Table A.3. NCDC surface sites used for each urban area.	76

Table A.4. Meteorological parameters used in the "baseline" GAMs. The column name is given in italics.	79

Table A.5. Deviance explained by the baseline GAMs (gam01_baseline) for each urban area and pollutant and the corresponding GCV and AIC values.	89

Table A.6. Meteorological predictors that were not significant at the =0.001 level for the baseline GAMs (gam01_baseline).	89

Table A.7. Meteorological parameters used in the extended MDA8 O3 GAMs	91

Table A.8. Meteorological parameters used in the extended daily average PM2.5 GAMs	92

Table A.9. Deviance explained by the large extended GAMs (gam02_extended) for each urban area and pollutant and corresponding GCV and AIC values.	93

Table A.10. Deviance explained by small extended GAMs (gam03_extended) for each urban area and pollutant and corresponding GCV and AIC values.	93

Table A.11. Meteorological predictors that were not significant at the =0.001 level for the small extended GAMs (gam03_extended).	96

Table A.12. Cross-validation root-mean-square (RMS) results for gam03_extended.	106

Table A.13. “Suspicious” fits that show significantly different functional forms between mtot, m1, and m2 for gam03_extended.	106

Table A.14. AQS site numbers for the selected background sites for each urban area.	110

Table B.1. AQS site numbers for the selected background sites for each urban area.	118

Table B.2. Sites used to calculate background O3 and PM2.5 for the State of Texas as a whole.	119

Table B.3. Number of background points quality flagged for each urban area and pollutant.	122

Table B.4. Background sites that were replaced if final_flag = TRUE	123

# Abstract

In this project, AER investigated the impact of meteorology on O3 and PM2.5 in six urban areas in Texas (Houston/Galveston/Brazoria, Dallas/Fort Worth, San Antonio, Austin/Round Rock, Beaumont/Port Arthur, and Tyler/Longview/Marshal). The purpose of this project was to:

1) Investigate the spatial and temporal trends of regional background O3 and PM2.5 and

2) Determine what synoptic- and urban-scale meteorological conditions are important in explaining and forecasting high concentrations of O3 and PM2.5.

To accomplish this, we first estimated daily regional background concentrations of O3 and PM2.5 for a ten-year period (2005-2014) for six urban areas and for the State of Texas as a whole using both the TCEQ method (lowest value at a set of background sites) and a principal component analysis (PCA) based technique. We then derived updated generalized additive models (GAMs) relating urban total and background O3 and PM2.5 to urban-scale meteorological predictors. We find that urban-scale meteorological predictors can explain 65-80% of the variability in urban O3, but only 30-40% of the variability in urban PM2.5. After using these relationships to correct the observed trends in total and background O3 and PM2.5, we find significant (95% confidence) negative trends in all four pollution metrics for Houston/Galveston/Brazoria and Dallas/Fort Worth, as well as significant negative trends in some of the pollution metrics for the other urban areas. Our seasonal trends analysis shows that background O3 is fairly constant through the O3 season (May-October) in Dallas/Fort Worth and Tyler/Longview/Marshall, but has a minimum in July for the other urban areas. In contrast, background PM2.5 peaks in June and July in all urban areas.

We then determined a set of five synoptic “types” covering 70% of all days (and 58% of the days in the O3 season) for our ten-year study period. We found that the relative frequency of high O3 and PM2.5 events in the urban areas varied significantly with these synoptic types. We used logistic regression to develop a model that predicted the probability of high O3 and PM2.5 events as a function of three variables: the derived synoptic map types, afternoon mean temperature, and daily mean wind speed. We used these functions to determine necessary and sufficient conditions for high O3 and PM2.5 events, and determined that these conditions vary significantly between the urban areas considered here.

We recommend that future work focus on:

- Developing additional methods to determine regional background concentrations, including further research into the differences between the TCEQ and PCA-based background estimates for O3 and the use of satellite observations, combined with chemical transport models, to determine background PM2.5,

- Further investigations into the synoptic types controlling high O3 and PM2.5 in Texas, and

- Further developing the GAMs and logistic models developed in this project to forecast air quality in Texas urban areas and evaluate the ability of 3D Eulerian air quality models to correctly simulate the impact of meteorology on O3 and PM2.5.

# Introduction

## Project Objectives

AER performed a research project titled “Investigating the Impact of Meteorology on O3 and PM2.5 Trends, Background Levels, and NAAQS Exceedances” for the Texas Commission on Environmental Quality (TCEQ). The objectives of this project were to:

- Determine the effects of meteorology on trends in O3 and PM2.5 by developing new generalized linear models (GLM) for O3 and PM2.5 concentrations to selected meteorological variables for the urban areas in Table 1.

- Estimate the regional background concentrations of O3 and PM2.5 for the urban areas in Table 1.

- Investigate the synoptic and urban-scale meteorological conditions that are associated with (i.e., are necessary and/or sufficient for) high concentrations of background and total O3 and PM2.5 in the “Group 1” urban areas in Table 1.

Table 1. Urban areas of interest to this study.

| Group 1 Urban Areas | Group 2 Urban Areas |
| --- | --- |
| Dallas/Fort Worth (DFW) | Beaumont/Port Arthur (BPA) |
| Houston/Galveston/Brazoria (HGB) | Tyler-Longview-Marshall (TLM) |
| San Antonio (SA) |  |
| Austin/Round Rock (ARR) |  |

The schedule of deliverables for this project is given in Table 2, while the purpose and background of each of the three tasks is summarized below.

## Purpose and Background

### Trends in O3 and PM2.5

As the formation and loss of pollutants such as O3 and PM2.5 are strongly influenced by meteorology, inter-annual trends in these pollutants represent a combination of changes due to inter-annual variability in meteorology and changes due to air quality policy actions and other economic and societal trends. Statistical techniques are thus used to account for the effect that meteorological variations have on the trends of O3 and PM2.5 so that the adjusted trends can be used to assess the effectiveness of air quality policy. A common approach to performing this “meteorological adjustment” is to use a generalized additive model (GAM, Wood, 2006) to describe the potentially non-linear relationship between measured O3 (maximum daily 8-hour average, or MDA8) or PM2.5 (daily average) concentrations and selected meteorological variables (e.g., Camalier et al., 2007). In this project, AER derived updated GAMs for urban O3 and PM2.5 for the urban areas in Table 1. AER used these models to account for the effect that meteorological variations have on the trends of O3 and PM2.5.

### Regional Background Concentrations of O3 and PM2.5

Daily surface concentrations of O3 and PM2.5 in urban areas can be considered as the sum of O3 and PM2.5 produced within the urban area (either through primary emissions of PM2.5 or through secondary chemical production of O3 and PM2.5) and a “regional background” that is transported into the urban area. Accurate estimates of this regional background are critical to determining the potential for further reductions in O3 and PM2.5 concentrations in urban areas through control of local emissions of primary PM2.5 and the precursors of O3 and PM2.5.

In this project, AER determined daily regional background estimates of O3 and PM2.5 for a ten-year period (2005-2014) for the urban areas in Table 3 and for the State of Texas as a whole using the TCEQ method (i.e., the lowest value observed at defined “background” sites near the border of the area of interest, Berlin et al., 2013). AER then used these background estimates to investigate the spatial and temporal trends of regional background O3 and PM2.5.

AER also explored other data-based ways of determining regional background concentrations of O3 and PM2.5 (e.g., the principle component analysis method of Langford et al., 2012, or the use of satellite observations of O3 and aerosols) using data from the “Group 1” urban areas.

### Synoptic- and Urban-scale Meteorological Controls on O3 and PM2.5

In this project AER investigated what synoptic- and urban-scale meteorological conditions are important in explaining and forecasting high concentrations of O3 and PM2.5 in the “Group 1” urban areas listed in Table 1. We identified necessary and/or sufficient meteorological conditions that lead to high concentration events (e.g., above 90th percentile) for these pollutants. Meteorological conditions leading to both high regional background levels and high total levels of O3 and PM2.5 were identified.

Table 2. Projected Schedule for TCEQ Work Order No. 582-15-54118-01

| Milestones | Planned Date |
| --- | --- |
| Task 1 - Work Plan |  |
| 1.1: TCEQ-approved Work Plan | April 3, 2015 |
| 1.2: TCEQ-approved QAPP | April 3, 2015 |
| Task 2 - Effects of Meteorology on O3 and PM2.5 Trends |  |
| 2.1: Monthly teleconferences or meetings to deliver project status to the TCEQ Project Manager | Monthly |
| 2.2: Deliver a technical memo describing GLMs relating meteorological variables to measured MDA8 O3 and PM2.5 for urban areas in Table 1 based on data for the O3 season (May through October) from 2005-2014 and PM2.5 from 2005-2014. AER shall also attach R scripts and other computer codes used to generate and/or analyze the GLMs. | June 30, 2015 |
| Task 3 – Estimating Background O3 and PM2.5 |  |
| 3.1: Daily estimates of regional background O3 (May through October) and PM2.5 (all year) by the TCEQ’s method for 2005-2014 for the Group 1 and Group 2 metropolitan areas, as well as the state of Texas. | May 29, 2015 |
| 3.2: Deliver, as part of the draft and final reports, an analysis of the spatial and temporal trends in the estimates of regional background O3 and PM2.5. | Draft: August 15, 2015 Final: August 31, 2015 |
| 3.3: Deliver, as part of the draft and final reports, an analysis of alternative data-based methods to determine regional background O3 and PM2.5. | Draft: August 15, 2015 Final: August 31, 2015 |
| Task 4 – Importance of Synoptic/Mesoscale Meteorological Conditions in Explaining/Forecasting Maximum O3 and PM2.5 |  |
| 4.1: Deliver, as part of the draft and final reports, a description of synoptic map “types” associated with high levels of background and total O3 and PM2.5 for the Group 1 metropolitan areas. | Draft: August 15, 2015 Final: August 31, 2015 |
| 4.2: Deliver, as part of the draft and final reports, a description of urban-scale meteorological predictors of O3 and PM2.5 exceedances for the Group 1 metropolitan areas. | Draft: August 15, 2015 Final: August 31, 2015 |
| Task 5 – Draft and Final Reports |  |
| 5.1: Draft Report for TCEQ review and approval, delivered electronically via file transfer protocol or e-mail in Microsoft Word format and PDF format | August 15, 2015 |
| 5.2: Final Report delivered electronically via file transfer protocol or e-mail in Microsoft Word format and PDF format | August 31, 2015 |

## Report Outline

This Draft Final Report documents the methods and pertinent accomplishments of this project, including comprehensive overviews of each task, a summary of the data collected and analyzed during this work, key findings, shortfalls, limitations and recommended future tasks. It satisfies Deliverable 5.1 of the Work Plan for Work Order No. 582-15-54118-01

Deliverable 5.1: Draft Report for TCEQ review and approval, delivered electronically via file transfer protocol or e-mail in Microsoft Word format and PDF format.

Deliverable Due Date: August 15, 2015

This report contains three sections that describe the methods and major findings for Task 2 (Effects of Meteorology on O3 and PM2.5 Trends, Section 2), Task 3 (Estimating Background O3 and PM2.5, Section 3) and Task 4 (Importance of Synoptic/Mesoscale Meteorological Conditions in Explaining/Forecasting Maximum O3 and PM2.5, Section 4). Technical memos previously delivered to TCEQ relating to Tasks 2 and 3 are included as Appendices A and B, respectively, while Appendix D includes additional plots relating to Task 4.

Section 5 discusses the Quality Assurance performed for the project, including answers to the assessment questions from the Quality Assurance Project Plan (QAPP). Section 6 summarizes the conclusions of our study, and Section 7 lists our recommendations for further research.

In addition, Appendix C describes the files that are included in the final deliverable package (Deliverable 5.2).

# Task 2: Effects of Meteorology on O3 and PM2.5 Trends

The technical memo delivered on June 30, 2015 described the generalized linear model (GLM) that related meteorological variables to measured MDA8 O3 and PM2.5 for urban areas in Table 1 on data for the O3 season (May through October) from 2005-2014 and PM2.5 from 2005-2014. It is discussed and summarized below.

## Input Data and Processing

The procedure in which we derived the daily minimum and maximum MDA8 ozone, and PM2.5, meteorological parameters is described in detail in Appendix A. Effects of Meteorology on O3 and PM2.5 Trends and Appendix B. Technical Memo: Estimating Background O3 and PM2.5.  Figure A.16 and Figure A.17 displays the order of functions and scripts that take the ozone, PM2.5 and meteorological measurement data (Section A.2) to the CSV-ready files used in the GAM modeling function and background analysis.  The raw input data sets are described in Section A.1.1, the HYSPLIT Back Trajectories for each urban region is described in Section A.2.3, the calculation of MDA8 ozone and hourly PM2.5 is described in Section B.2.2, and estimates of the TCEQ method background is described in Section B.2.3.

## Generalized Additive Model

As the formation and loss of O3 and PM2.5 are strongly influenced by meteorology, inter-annual trends in these pollutants represent a combination of changes due to inter-annual variability in meteorology and changes due to air quality policy actions and other economic and societal trends. Statistical techniques are thus used to account for the effect that meteorological variations have on the trends of O3 and PM2.5 so that the adjusted trends can be used to assess the effectiveness of air quality policies. A common approach to performing this “meteorological adjustment” is to use a generalized linear model (GLM, Zheng et al., 2006) to describe the potentially non-linear relationship between measured urban O3 MDA8 or PM2.5 (daily average) concentrations and selected meteorological variables taken from an array of candidate meteorological variables (e.g., Camalier et al., 2007). TCEQ has developed such models in the past, but these models have not been updated since 2008.

In our procedure, we fit the maximum MDA8 O3 value and the maximum 24-hour average PM2.5 value for each urban area using the generalized additive model (GAM) modeling function in the mgcv package in R (Wood, 2006). The GAM can be written as follows:

gμi=βo+f1xi,1+f2xi,2+⋯fnxi,n+fpDi+Wd+Yk

where i is the ith day’s observation, gμi is the “link” function (here, a log link is used), xi,j are the n meteorological predictors fit, with the corresponding fjxi,j being a (initially unknown) smooth function of xi,j made from a cubic-spline basis set. Following Camalier et al. (2007), three non-meteorological predictors are also included: a smooth function fpDi of the Julian day of the year (Di); a factor for the day of the week Wd and a factor for the year Yk. As we are only fitting O3 data during the O3 season (May-October), fpDi is built with a non-periodic cubic spline basis for O3, but for PM2.5, a periodic cubic spline basis is used. To reduce the possibility of over-fitting the data, we set the “gamma” parameter to 1.4 for these fits, as recommended by Wood (2006).

We also added an automated process to determine if a predictor that is not significant at the  = 0.001 level could be eliminated from the fit without significantly degrading the performance of the model. In this process, the meteorological predictor with the highest p value is removed and a second GAM is fit. This is then compared to the original model using the ANOVA procedure in R. If the second model with the variable removed is not different from the original model at the   = 0.01 level, the variable is “dropped” from the fit and the variable with the next highest p value is tested. If the second model is significantly worse than the original model, the variable is kept and no other variables are tested or dropped. Because of this, although the GAMs for a given pollutant may start with the same predictors for all urban areas, the final GAM selected may have different predictors depending on which variables were dropped for each urban area.

### Baseline GAMs (gam01_baseline)

We developed “baseline” GAMs for the maximum MDA8 O3 and daily average PM2.5 in each area, where we use the eight meteorological parameters identified as significant by Camalier et al. (2007) in their study of O3 in eastern US cities. These parameters are listed in Table A.4 with the results discussed in Section A.1.4.

#### MDA8 O3 Results Summary

- The daily maximum temperature functions all show increasing O3 with increasing temperature, but the fits for DFW, SA, and ARR become flat for temperatures > 30 °C, while the other areas show no such flattening off.

- The mid-day RH functions all show decreasing O3 with increasing RH, and have a similar shape for all urban areas (relatively flat until 60% RH, then increasing at higher RH).

- O3 decreases with morning wind speed for all urban areas except ARR (where it is fairly flat).

- O3 either decreases with afternoon wind speed or the predictor is not significant.

- All urban areas except DFW show increasing O3 with increasing stability (T_diff_925mb). The predictor is fairly flat for DFW with maxima at either end that may  not be significantly different from zero.

- The deviation of the 850 mbar temperature from the monthly average (T_dev_850mb) is  insignificant for ARR and SA, and may just be fitting noise for the other urban areas as  there is little consistency in the functional forms.

- O3 decreases with HYSPLIT back-trajectory distance up to ~1000 km, at which point it  becomes highly uncertain due to the low number of points, but may begin to increase.

- All the urban areas show a drop in O3 at a HYSPLIT bask-trajectory bearing of ~150°° (southeast), likely due to reduced background O3 from flows from the Gulf of Mexico.

- The day-of-year function shows a minimum at ~200 Julian days (July) in each urban area.

#### Maximum Daily Average PM2.5 Results Summary

- All urban areas generally show PM2.5 increasing with daily maximum temperature, but the effect is fairly weak for ARR, and SA, DFW, and HGB suggest that the trend flattens out or reverses at temperatures > ~30 °C.

- The fits for mid-day RH are very uncertain at low (< 40%) and high (> 80%) values, and the functional shape changes significantly between urban areas, with SA and ARR generally showing decreasing PM2.5 with increasing RH, HGB and BPA showing an opposite trend, and TLM showing a maximum around 70% RH.

- PM2.5 either trends down with increasing morning wind speed or the effect is insignificant.

- PM2.5 generally trends down with increasing afternoon wind speed, but HGB, DFA, and BPA show a highly uncertain upward trend for wind speeds greater than 6 m/s.

- All urban areas show increasing PM2.5 with increasing stability (T_diff_925mb), but the effect is fairly weak for TLM.

- PM2.5 generally trends upward with increasing deviation of the 850 mbar temperature from the monthly average (T_dev_850mb).

- PM2.5 decreases with HYSPLIT back-trajectory distance up to ~500 km, at which point it becomes flatter and highly uncertain due to the low number of points.

- All urban areas show a maximum for PM2.5 around a HYSPLIT back-trajectory bearing of ~60° (northeast) and a minimum around 320° (northwest), possibly due to the relative difference in the PM2.5 concentrations in the western and eastern US. Most urban areas also show a secondary minimum around ~150° (southeast), likely due to flows from the Gulf of Mexico.

The day-of-year functions for all urban areas are lower in the summer, likely reflecting the higher mixing heights in this season. The maximum is generally between 50-100 Julian days (around March), and ARR, SA, and HGB show a secondary maximum at ~200 Julian days (July).

### Extended GAMs (gam02_extended and gam03_extended)

We also explored whether a different set of meteorological predictors than those used by Camalier et al. (2007) and used in the baseline GAMs of A.1.4 could provide a better fit to the maximum MDA8 O3 and maximum daily average PM2.5 for each urban area. The procedure we used is described in detail in Section A.1.5.

#### MDA8 O3 Results Summary

- The afternoon mean temperature functions all show increasing O3 with increasing temperature, but the fits for DFW, SA, and ARR flatten out for temperatures > 30 °C, while the other areas show no such flattening off.

- O3 generally increases with increasing diurnal temperature change, but the effect is weak.

- The daily average RH functions all show decreasing O3 with increasing RH, but the  effect is relatively weak in HGB.

- O3 generally increases with dew point temperature up until 10-15 °C, after which point O3 decreases. This is consistent with the competing effects of temperature and humidity on O3 production.

- O3 decreases with daily average wind speed for all urban areas, but the effect is strongest  in HGB and SA.

- All urban areas except BPA show increasing O3 with increasing stability (T_diff_850mb).  However, O3 decreases at the highest values of T_diff_850mb for SA (-5 to 0 °C)

- Daily wind direction generally has little impact on the O3, and is likely just fitting noise.

- O3 decreases with HYSPLIT back-trajectory distance up to ~500 km, at which point it  becomes highly uncertain due to the low number of points, but may begin to increase.

- All the urban areas show a drop in O3 at a HYSPLIT bask-trajectory bearing of ~150° (southeast), likely due to reduced background O3 from flows from the Gulf of Mexico.

The day-of-year function shows a slight decrease over the length of the O3 season for all  urban areas, with an area of nearly flat slope at ~200-225 Julian days (July-August).

#### Maximum Daily Average PM2.5 Results Summary

- All urban areas generally show PM2.5 increasing with afternoon mean temperature, but the effect is fairly weak for ARR, and SA, DFW, and HGB suggest that the trend flattens out or reverses at temperatures > ~30 °C.

- The fits for average RH generally peak at 60-70% and fall off at lower and higher RH values, although SA and ARR show a second peak at the lowest extreme values (~20%).

- PM2.5 generally increases with increasing temperature at 925 mbar, but HGB shows a significant increase at the lower extreme.

- PM2.5 generally trends down with increasing daily average wind speed, but HGB and BPA show an upward trend for wind speeds greater than 6 m/s, possibly related to marine aerosol production.

- All urban areas show increasing PM2.5 with increasing stability (T_diff_850mb).

- PM2.5 decreases with HYSPLIT back-trajectory distance up to ~500 km, at which point it becomes flatter and highly uncertain due to the low number of points. The DFW fit is  fairly flat, showing little dependence on back-trajectory distance.

- All urban areas show a maximum for PM2.5 around a HYSPLIT bask-trajectory bearing  of ~60° (northeast). DFW, SA, ARR, and TLM show a minimum around 320° (northwest), possibly due to the relative difference in the PM2.5 concentrations in the western and eastern US. However, the urban areas near the Gulf of Mexico (HGB and BPA) have a minimum around ~150° (southeast), likely due to flows from the Gulf of Mexico.

- PM2.5 generally decreases with increasing solar radiation, possibly due to increased cloudiness leading to more rapid oxidation of SO2 into aerosol sulfate.

- The day-of-year functions for all urban areas are lower in the summer, likely reflecting the higher mixing heights in this season. The maximum is generally between 50-100 Julian days (around March), and ARR and SA show a secondary maximum at ~200 Julian days (July).

### Cross Validation Analysis

In order to test for over-fitting in our GAMs, as well as to test the robustness of our results for the functional relationships between the meteorological predictors and O3 and PM2.5, we performed a two-fold cross-validation experiment for each GAM.  Details of this cross-validation are described in Section A.1.6.

## GAMs for Background PM2.5 and O3

We also used the same approach used to derive the gam03 models described in Section 2.2.2 to fit GAMs for the background MDA8 O3 and daily average PM2.5 for the four Group 1 urban areas. These models, called back_gam03 here for convenience, will be provided to TCEQ as part of the final deliverable package described in Appendix C and are used to calculate meteorologically adjusted trends in background O3 and PM2.5 in Section 2.4. One thing to note is that, while the model intercept (o) and year-to-year variability terms (Yk) differ between the models fit to total and background pollutant values, the shape and magnitude of the smooth functions for the meteorological predictors is remarkably consistent between the gam03 and back_gam03 models. The only noticeable difference is for O3 in the Houston/Galveston/Brazoria area, where the GAM for total O3 shows a stronger dependence on mean afternoon temperature and daily average wind speed than the background O3 GAM, suggesting that those predictors have a strong influence on local chemical production of O3 in HGB. Further work should attempt to fit measures of locally produced O3 and PM2.5 (i.e., total minus background) to meteorological predictors to see if these dependences differ significantly from those for regional background and total O3 and PM2.5.

## Meteorologically Adjusted Trends of O3 and PM2.5

We used the “extended” gam03 models described in Sections 2.2.2 and 2.3 to determine the meteorologically adjusted trends in total and background MDA8 O3 and daily average PM2.5. In this procedure, we use the Yk terms from the GAM equation in Section 2.2 to determine the relative difference between the annual averages after meteorology has been taken into account. Our equation for the annual averages is thus

gμk=βo+Yk+co

where k is the kth year’s average and co is a constant. The constant co is needed because of how R treats factor variables. In order to have an identifiable model, one of the factor levels, in this case the year 2005, must be set to have a value of Yk = 0. However, the year 2005 if frequently the year with the largest annual average O3 and PM2.5 values in the original data set. This results in Yk values that are predominantly less than 0, leading to meteorologically adjusted annual averages that do not have the same 10-year average as the original data set. To avoid this issue, we add a constant co to the meteorologically adjusted annual averages so that the 10-year averages in the original and meteorologically-adjusted trend data are identical. The value of the meteorologically-adjusted linear trends over 2005-2015 is relatively insensitive to the value of co.

The original and meteorologically-adjusted annual averages are shown in Figure 1 through Figure 6 below. The trend estimates, determined by ordinary least squares (OLS) linear regression of the annual averages, are summarized in Table 3 below. As expected, the meteorologically adjusted annual averages show less year-to-year variation and generally show trends closer to zero than the original data. The largest impact of the meteorological adjustment is on the trends for O3 in Houston/Galveston/Brazoria and Beaumont/Port Arthur. In general, the meteorological adjustment affects estimates of O3 trends more than PM2.5 trends – this is to be expected as the GAMs account for a larger fraction of the observed variability in O3 than for PM2.5.

No positive trends with time are observed for any of the six urban areas examined here for 2005-2014 either before or after meteorological adjustment. The meteorologically-adjusted negative trends are significant at an  = 0.05 level for all pollutant metrics at HGB and DFW. The adjusted trends are also significant for total MDA8 O3 at TLM, for total PM2.5 at BPA and TLM, and for background PM2.5 at SA and ARR. However, the meteorologically adjusted trends in the background O3 and PM2.5 for the four Group 1 urban areas are similar to the meteorologically adjusted trends in the total, suggesting most of the observed trend in total O3 and PM2.5 is due to trends in the regional background rather than changes in local production. The major exception is O3 in San Antonio, which shows a trend near zero in total O3 but a significant negative trend of -1.01±0.87 ppbv/year in background O3, suggesting that local O3 production may have increased in San Antonio between 2005-2014. Houston/Galveston/Brazoria and Dallas/Fort Worth show slightly larger decreases in total O3 than in background O3, suggesting that local production has decreased in these urban areas.

Table 3. Original and meteorologically adjusted linear trends (±95% confidence intervals) of total and background (BG) MDA O3 and daily average PM2.5 from 2005-2015 using the gam03 models. Trends significantly different from zero with 95% confidence are in bold.

| Urban Area | Total MDA8 O3 (ppbv/year) |  | BG MDA8 O3 (ppbv/year) |  | Total Daily PM2.5 (g/m3/year) |  | BG Daily PM2.5 (g/m3/year) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Orig. | Met. Adj. | Orig. | Met. Adj. | Orig. | Met. Adj. | Orig. | Met. Adj. |
| HGB | -1.48±0.73 | -0.46±0.40 | -0.91±0.57 | -0.41±0.31 | -0.55±0.13 | -0.39±0.14 | -0.48±0.11 | -0.37±0.11 |
| DFW | -0.81±0.90 | -0.68±0.39 | -0.61±0.84 | -0.57±0.49 | -0.17±0.10 | -0.15±0.09 | -0.24±0.10 | -0.23±0.08 |
| SA | -0.02±0.85 | 0.00±0.56 | -1.00±1.03 | -1.01±0.87 | -0.08±0.09 | -0.08±0.14 | -0.10±0.08 | -0.12±0.07 |
| ARR | -0.44±0.75 | -0.35±0.42 | -0.33±0.98 | -0.36±0.56 | -0.10±0.09 | -0.10±0.12 | -0.21±0.08 | -0.21±0.10 |
| BPA | -1.03±0.65 | -0.15±0.55 | NA | NA | -0.48±0.11 | -0.34±0.11 | NA | NA |
| TLM | -0.78±1.05 | -0.66±0.41 | NA | NA | -0.34±0.15 | -0.34±0.08 | NA | NA |

Figure 1. Original (dashed lines) and meteorologically adjusted (solid lines) annual averages for total and background O3 (top) and PM2.5 (bottom) for the Houston/Galveston/Brazoria urban area. Equations for the OLS linear regressions are shown on the plot as well.

Figure 2. As in Figure 1 but for the Dallas/Fort Worth urban area.

Figure 3. As in Figure 1 but for the San Antonio urban area.

Figure 4. As in Figure 1 but for the Austin/Round Rock urban area.

Figure 5. Original (dashed lines) and meteorologically adjusted (solid lines) annual averages for total O3 (top) and PM2.5 (bottom) for the Beaumont/Port Arthur urban area. Equations for the OLS linear regressions are shown on the plot as well.

Figure 6. As in Figure 5 but for the Tyler/Longview/Marshall urban area.

## Conclusions

- The Generalized Additive Model (GAMs) relating meteorological variables to the maximum MDA8 O3 for each urban area generally explain 65-80% of the deviance (i.e. variability), consistent with the results of Camalier et al. (2007). The GAMs also generally show good fits with normally-distributed residuals and little dependence of the residual variance on the predicted value.

- In contrast, the GAMs relating meteorological variables to the maximum daily average PM2.5 for each urban area only explain 30-40% of the deviance, and generally show much poorer fits with long, positive residual tails and a strong dependence of the variance of the residuals on the predicted value.

- Using meteorological predictors different from those listed in Camalier et al. (2007) can result in an improved GAM for MDA8 O3 and daily average PM2.5, but the improvement is less significant for PM2.5.

- Two-fold cross validation analysis shows that the GAM fitting procedure results in GAMs only perform slightly worse for the “test” data set as they do for the “training” data set, and thus the GAMs show little evidence of over-fitting.

- However, the cross validation analysis also shows that the smooth function fit for each meteorological predictor can vary substantially depending on which half of the data is used to train the GAM. Thus the individual smooth functions from each GAM should be used with caution.

- We find that the general trends of the relationships rarely change significantly between the urban areas. For O3, the major differences are that DFW, SA, and ARR show the O3 trend with afternoon temperature flattening out above 30 °C and that the impact of relative humidity is fairly weak in HGB. For PM2.5 , the major differences are between the cities near the Gulf of Mexico (HGB and BPA) and the others, with the cities near the Gulf showing increasing PM2.5 at wind speed above 5 m/s and a minimum in PM2.5 at a HYSPLIT bearing of 120° instead of at 320°.

- Similarly, we find that the meteorological relationships fit to background O3 and PM2.5 are substantially identical to those fit to total O3 and PM2.5, with the possible exception of HGB O3.

- No positive trends with time are observed for any of the six urban areas examined here for 2005-2014 either before or after meteorological adjustment. The meteorologically-adjusted negative trends are significant at an  = 0.05 level for all pollutant metrics at HGB and DFW. The adjusted trends are also significant for total MDA8 O3 at TLM, for background O3 at SA, for total PM2.5 at BPA and TLM, and for background PM2.5 at SA and ARR. The results suggest most of the observed trends in total O3 and PM2.5 are due to trends in the regional background rather than changes in local production, with the exception of O3 in SA (where local production appears to be increasing) and HGB and DFW (where local production appears to be decreasing).

# Task 3: Background O3 and PM2.5

## Daily Estimates of Regional Background O3 and PM2.5 (TCEQ Method)

The detailed description of our application of the TCEQ method to derive background O3 and PM2.5 concentrations are discussed in Appendix B. This method selects the lowest valid measured value at a set of “background” sites around the urban area as the background estimate.

We performed a linear regression quality check of these results as discussed in detail in Section B.2. Figure B.1 shows a scatterplot of the background MDA8 O3 value versus the maximum MDA8 O3 value for the HGB area. The solid black line is the linear fit, and the dotted and dashed black lines are the upper and lower 95% (or 2σ) confidence intervals, respectively. In this example, 89 of the 1834 valid data points (4.9%) have maximum MDA8 O3 values that fall above the upper confidence internal of the linear fit, suggesting that these background estimates are lower than would be expected given the maximum values seen in the urban area. Table B.3 gives the number of such points for each urban area and pollutant.

Similar to Berlin et al. (2013), we performed further analysis of the points that were above the 95% confidence interval of the fit (e.g., where high_flag = TRUE). First, we identified the subset of those points where (a) high_flag = TRUE AND (b) at least one other background site in the urban area had a valid MDA8 O3 or daily average PM2.5 value for that day AND (c) the valid values at the other background sites were all more than 10% larger than the preliminary background estimate. Note that the later two criteria have to be true for replacing the preliminary background estimate with a value from a different background site to make a significant impact on any subsequent analysis. Data points that met all three criteria are flagged in the csv files in a column called “final_flag”, with a value of TRUE meaning that the above criteria were satisfied. The number of points with final_flag = TRUE for each urban area is shown in Table B.3. For these points, we have included the AQS site number and the MDA8 O3 or daily average PM2.5 value for the background site with the second largest value in the csv files as an alternate background estimate.

However, we only replaced the preliminary background value if:

- The final_flag = TRUE

- The estimate was for the HGB or BPA areas, as these areas near the Gulf of Mexico could plausibly have times when the gulf/lake breeze front affects some of the outlying background sites, but does not affect the urban area as a whole.

- The preliminary background site was between the city and the Gulf of Mexico (or the city and Sabine Lake). These sites are given in Table B.4.

These final estimates were delivered to TCEQ as part of Deliverable 3.1. The files in this deliverable are discussed in Section B.2.3.

## Spatial and temporal trends of background O3 and PM2.5

Figure 7 and Figure 8 show the seasonal (left) and annual (right) trends in the background MDA8 O3 for the six urban areas of interest, while Figure 9 and Figure 10 show the same for background daily average PM2.5.

Figure 7. Box-and-whisker plots for the background MDA8 O3 for Austin/Round Rock, Beaumont/Port Arthur and Tyler-Longview-Marshall as estimated using the TCEQ method. The red line is the median, the dashed black line is the mean. Box edges show the 25th and 75th percentiles, the whiskers and crosses show the range of the data.

Figure 8. Box-and-whisker plots for the background MDA8 O3 for Dallas/Fort Worth, Houston/Galveston/Brazoria, and San Antonio as estimated using the TCEQ method.

Figure 9. Box-and-whisker plots for the background daily average PM2.5 for Austin/Round Rock, Beaumont/Port Arthur and Tyler-Longview-Marshall as estimated using the TCEQ method.

Figure 10. Box-and-whisker plots for the background daily average PM2.5 for Dallas/Fort Worth, Houston/Galveston/Brazoria, and San Antonio as estimated using the TCEQ method.

Background MDA8 O3 is fairly constant with month during the O3 season for Tyler- Longview-Marshall and Dallas/Fort Worth, but has a July minimum for the other urban areas. In contrast, median background PM2.5 peaks in June and July.  Houston/Galveston/Brazoria and Beaumont/Port Arthur show a clear decreasing trend in background O3 and PM2.5 – the analysis of Section 2.4 suggests that these trends are reduced but still present when the effect of varying meteorology is accounted for. The range of values for a given month or year is large for all cities, with Houston having the largest O3 spread and most outliers, possibly due to the fact that it has the largest dataset.  The month of June for O3 in particular has a large spread of values, including very high (> 70 ppbv) background values for Houston and San Antonio.

## Alternative Methods To Determine Regional Background O3 and PM2.5

### Determining Background O3 with PCA

In Langford et al. (2009), principal component analysis (PCA) was applied to the large dataset of MDA8 values at 30 sites across Houston for a 2.5-month timespan (August to October 2006). The PCA approach attempts to isolate the large day-to-day regional changes in the MDA8 ozone, and Langford et al. (2009) were able to associate regional meteorological patterns with the patterns of covariance as determined by the PCA using associated meteorological data provided by the National Centers for Environmental Prediction (NCEP) reanalysis. They found that nearly 84% of the variance in the MDA8 ozone near Houston was described by the first Principal Component (PC1) and could be attributed to the regional background ozone concentration. PC2 and PC3 described 6% and 3.5% of the variance in MDA8 ozone, and were attributed to local photochemistry and transport, respectively. After determining that PC1 described that large majority of variance and represented regional background ozone, the following equation was applied to calculate the hourly background ozone for Houston, O3RMt.

O3RMt= O3+o(O3)f1α1(t)

where O3 is the mean of all MDA8 ozone values for the entire time period, oO3 is the standard deviation of that mean, f1 is the variance contribution of PC1 (0.84 in Langford et al., 2009) and α1(t)  is the score (or amplitude) of PC1 at each hour.

Performing a PCA of our MDA8 O3 data for the Group 1 metropolitan areas required that we first create a full, interpolated dataset without any missing values. We calculated MDA8 values following the steps described in Appendix B. We then filtered out any sites where less than 75% of data points were valid for the 10-year period during the ozone season (May to October, 2005-2014). Next, we spatially interpolated the dataset to replace any missing MDA8 values. If the data point for that day was located outside of the cluster of sites with valid data points, we applied a nearest-neighbor interpolation, and if that point was located within the cluster of sites with valid data points, we applied a cubic interpolation in latitude and longitude.

Once this complete dataset was established, we applied the PCA using the eigenvector- eigenvalue calculation in R to the entire 10-year time span for Houston, which resulted in a similar variance contribution to that of Langford et al. (2009), where PC1, PC2 and PC3 had variance contributions of 83%, 6%, and 2%, respectively. However, when assuming PC1 represented the regional background contribution and applying the above equation, the values were well correlated to our original background estimates from the TCEQ method (see Appendix B), but produced a much larger and unrealistic range of background concentrations (-100 to 250 ppbv), as seen in Figure 11.

Figure 11. PCA-derived background ozone in Houston applied over the entire ozone season dataset (x-axis, 10 years and all sites) compared to our original TCEQ method of determining background ozone (y-axis).

After further discussion via email with Langford, and examining the day-of-year functional fits from our meteorological analysis of MDA8 O3 described in Appendix A. Effects of Meteorology on O3 and PM2.5 Trends, we found that the mid-ozone-season wind shift in July may be contributing to the unrealistic PC1 scores and thus giving unrealistic background concentrations. In the month of July there is a shift in prevailing Gulf flow winds, causing there to be a potential of two or more sites on opposite ends of the city contributing to background values. We thus performed the PCA analysis separately for two periods, May-July and August-October, as the meteorological patterns in each period are expected to be similar. The results are presented in Figure 12, where the correlation with the background from the TCEQ method remains high, and the range of concentrations is more realistic.

Figure 12. PCA-derived background ozone in Houston compared to the original TCEQ method.  This approach applied the PCA over time spans during the ozone season; May to July (red) and August to October (blue) with the overall slope and r2 value printed in black.

Similar correlation results were seen for the remaining Group 1 urban areas, Austin, Dallas and San Antonio where the r2 values were 0.91, 0.89, 0.88, respectively. However the slopes indicate differences between the background ozone estimated by the TCEQ method and those estimated by PCA, and the relationship between the two background estimates varies significantly between cities, which differ between the four urban areas examined (i.e., 0.71, 1.5, 0.75, and 1.27 for Houston, Austin, Dallas, and San Antonio, respectively). Further work is needed to determine the reason of the differences in these two estimates of background O3, and why the differences vary with urban area.

Figure 13. Monthly (left) and yearly (right) background ozone (ppbv) as derived using the PCA method.  Box plots during the ozone season for Austin, Dallas, Houston and San Antonio are shown.

#### PCA-derived Background O3 Temporal and Spatial Analysis

Figure 13 shows the monthly and yearly background MDA8 box plots for the Group 1 urban areas as determined by the PCA analysis described above.  These results compare well with the temporal analysis discussed above in Section 3.1 of MDA8 background ozone as determined using the TCEQ method.  The seasonal background ozone (left) peaks in September and has a minimum concentration in July.  This is consistent with the GAM analysis (see Section A.1.5.2) and the TCEQ method (see Section 3.2) for all of the urban areas except for Dallas, where there is a less pronounced trough, and the minimum average background occurs in October.  The yearly background MDA8 ozone also appears to be similar for all urban areas and to the July progress report results (however less pronounced), where there is a slight overall average decrease but with year-to-year variation.

### Determining Background PM2.5 with PCA

The same PCA approach was applied to the PM2.5 dataset provided by TCEQ. First, a complete dataset was established, with the same interpolation method described above. Then the PCA was applied to the entire 10-year timespan, where the entire year was analyzed, not just the ozone season. The initial variance contributions in Houston for PC1, PC2 and PC3 were 87%, 4.4% and 3.0%, respectively, suggesting that PC1, if assumed to be associated with the regional background, plays an even more significant role to the overall variance than it did for O3. However, similar to our background ozone PCA method, when we applied the equation for Langford et al. (2009) to calculate the regional background PM2.5 (Figure 11) the correlation was reasonable, but the range of concentrations proved unrealistic (-10 to 60 μg m3, see Figure 14).

Figure 14. PCA-derived background PM2.5 in Houston compared to the original TCEQ method. The PCA was applied to the entire 10-year time span for all sites.

After further investigation and reference to the GAM meteorological analysis from Appendix A. Effects of Meteorology on O3 and PM2.5 Trends,we recognized that there is more than one potential meteorological shift throughout the year influencing PM2.5.  After attempting to split the years up into different, meteorologically-similar periods, we found that even if the PCA was applied to each month individually, the Langford et al. (2009) approach still gave an unphysical range of estimated background concentrations. For example, Figure 15 shows that even when PCA is only applied to the month of July for 2005-2014, the background estimates from Equation 1 are still unphysical.

Figure 15. PCA-derived background PM2.5 in Houston in just July months compared to the original TCEQ method.

We attempted a variety of subsections in the year, including the example in Figure 16 which is split up into; 1) April-July, 2) August- October and 3) November-March. However, none of these attempts resulted in significant improvements, thus a further analysis is needed to derive a more comprehensive PCA-based method for determining background PM2.5 for the Houston area.  For the other 3 urban areas that we applied the PCA method to, similar unrealistic results were seen. Thus we conclude that PCA is not a reasonable way to derive background estimates for PM2.5 for the 6 urban areas considered here.

Figure 16. PCA-derived background PM2.5 in Houston compared to the original TCEQ method.  The PCA was applied to 3 different time spans: August to October (blue), April to July (red) and November-March (green).

### Determining Background O3 with Satellites

We performed a literature review in order to explore alternative methods to measure surface O3 that could improve the understanding of spatial and temporal ozone trends across Texas.  Such a dataset could improve our understanding of local and background ozone trends and the contributing photochemical and meteorological conditions leading to high ozone events.

Zoogman et al. (2014) discuss a data assimilation system that uses the GEOS-Chem chemical transport model (CTM) and the proposed design for the GEO-CAPE (GEOstationary Coastal and Air Pollution Events) satellite to calculate a better representation of surface ozone.  They argue that the assimilation of satellite ozone data into a CTM can be further improved by using correlated multispectral CO measurements, which have better boundary layer sensitivity than ozone.  The observed model-transport error of CO could assist in identifying model-transport ozone errors, resulting in improved surface ozone representation.  They apply this framework to a regional-scale Observing System Simulation Experiment (OSSE) of GEO-CAPE over North America and conclude that this technique could provide improved constraints on surface ozone.  Additionally, a satellite that is more sensitive to O3 in the boundary layer but less sensitive than CO in the boundary layer would also improve the results. However, GEO-CAPE does not yet have a launch data scheduled, and existing satellite instruments have insufficient coverage and vertical sensitivity to reliably separate boundary-layer O3 from the values in the free troposphere, although in the meantime attempts are being made to combine UV-VIS radiance measurements (e.g., from NASA’s Ozone Monitoring Instrument) with infrared radiance measurements (e.g., from NASA’ Tropospheric Emission Spectrometer) to provide increased sensitivity to boundary-layer O3.

### Determining Background PM2.5 with Satellites

In contrast, a satellite-based technique to retrieve regional PM2.5 concentrations at fine spatial and temporal resolutions is much further developed and shows significant promise.  Van Donkelaar et al. (2013) present an optimal estimation algorithm using aerosol optical depth (AOD) from the NASA Moderate Resolution Imaging Spectroradiometer (MODIS) to determine PM2.5 concentrations near the surface at a 0.01° latitude x 0.01° longitude resolution. GEOS-Chem is used to provide the a priori values and the AOD/PM2.5 relationship, which is a function of the atmospheric vertical structure, aerosol type, and meteorological factors. The LIDORT radiative transfer model is used to simulate the dependence of the top of the atmosphere reflectance on the aerosol type and distribution, and AERONET measurements of AOD are used for surface validation.  Compared to measurements, this optimal estimation technique proves to be better at predicting surface PM2.5 concentrations than the original MODIS and GEOS-Chem outputs, as can been seen in Figure 17.  The results from such an approach in Texas could prove to be extremely useful in understanding the spatial and temporal trends of PM2.5.

Figure 17. From van Donkelaar et al., (2015).  Spatial plot (Top) and scatter plot (bottom) over the US of optimal estimation (OE) approach (far right) for simulating near-surface PM2.5 concentrations compared to GEOS-Chem (center) and in situ measurements from the AERONET sites (far left).  Presented at the 7th annual GEOS-Chem meeting at Harvard University, 2015.

# Task 4: Importance of Synoptic/Mesoscale Meteorological Conditions in Explaining/Forecasting Background and Maximum O3 and PM2.5

## Synoptic Map Type Analysis

### Technical Method and Results

The synoptic map typing was performed for all days during 2005 – 2014 using the 1200 UTC 850 hPa geopotential height fields from the 32 km North American Regional Reanalysis using the method of Hegarty et al. (2007).  The five most common synoptic map types were identified which enabled the classification of 70% of the days during the 10-year study period (and 58% of the days in the ten May-October O3 seasons) as being under the influence of one of those types.  The five types, shown in Figure 18, are described below.

- MT 1 occurred on 599 days (364 during the O3 seasons) and featured an anticyclone over the eastern Gulf of Mexico with a trough in the Central Plains extending into northwest TX. These features produced a general south-southwest flow over much of TX.

- MT 2 occurred on 472 days (127 during the O3 seasons) and featured a cyclonic circulation centered over the Midwest with a ridge extending southeast to northwest over Mexico and extreme southern TX. This pattern likely produces a light NW flow over much of TX.

- MT 3 occurred on 515 days (474 during the O3 seasons) and featured a large anticyclone centered over the eastern Gulf of Mexico states extending in to the Gulf and westward in to eastern TX.  A broad trough is aligned along the eastern Rocky Mountains. This pattern produces moderate to strong southeasterly flow over much of eastern TX.

- MT 4 occurred on 628 days (196 during the O3 seasons) and featured a broad trough in the Central Plains with an anticyclone centered over the western Caribbean of southern Florida and extending westward in to eastern TX. This pattern produces a general southwest flow over TX.

- MT 5 occurred on 344 days (143 during the O3 seasons) and features an anticyclone over the western Gulf of Mexico. This pattern features a south to southwestern flow over much of TX.

The synoptic type classification for each day in the years 2005-2014 are in the file tceq_map_type.csv in the final deliverable (see Appendix C). Days that don’t fit any of the five types are indicated as type “-999”. We then wrote an R script (syn_type_boxplot.R, see Appendix C) to determine the mean, standard deviation, and quartiles of both total and background MDA8 O3 and daily average PM2.5 for each synoptic type. This analysis focused on the Group 1 urban areas from Table 1. Figure 19 through Figure 22 show box plots of the O3 and PM2.5 distributions for each city and synoptic type.

In order to determine if there was a relationship between synoptic type and the likelihood of high total or background MDA8 O3 and daily average PM2.5 values, we first needed a quantitative definition of a “high” value of each metric. We derived these metrics by examining the 90th percentile of the distribution of each of the four metrics or each of the four Group 1 urban areas, which are shown in Table 4.  We then chose criteria that were roughly in line with these 90th percentiles: 70 ppbv for total MDA8 O3, 55 ppbv for background MDA8 O3, 17.0 µg m-3 for total daily average PM2.5, and 13.0 µg m-3 for background daily average PM2.5. Table 5 shows the percentage of days below these criteria for each urban area (i.e., the percentile corresponding to the chosen criteria). These criteria values are also shown as horizontal lines on the box plots in Figure 19 through Figure 22. We then used the script syn_type_boxplot.R to determine the percentage of days over these criteria for each synoptic type. These values are summarized in Table 6 through Table 9.

Figure 18. Synoptic maps types determined from 850 mbar geopotential height fields from the 32 km North American Regional Reanalysis using the method of Hegarty et al (2007).

Table 4. 90th percentile of the total and background (BG) MDA8 O3 and daily average PM2.5 values for each Group 1 urban area for 2005-2010. Only values during the O3 season (May-Oct.) are considered for O3.

| Urban Area | Total MDA8 O3 (ppbv) | BG MDA8 O3 (ppbv) | Total Daily PM2.5 (µg m-3) | BG Daily PM2.5 (µg m-3) |
| --- | --- | --- | --- | --- |
| HGB | 84.3 | 54.9 | 20.7 | 14.6 |
| DFW | 83.5 | 60.6 | 18.0 | 13.9 |
| SA | 70.3 | 58.3 | 17.0 | 15.6 |
| ARR | 67.4 | 60.1 | 16.2 | 13.3 |

Table 5. Percentage of observations below the criteria chosen to represent “high” values of total and background (BG) MDA8 O3 and daily average PM2.5 values for each Group 1 urban area for 2005-2010. The chosen criteria are in parentheses in the first row. Only values during the O3 season (May-Oct.) are considered for O3.

| Urban Area | Total MDA8 O3 (70 ppbv) | BG MDA8 O3 (55 ppbv) | Total Daily PM2.5 (17 µg m-3) | BG Daily PM2.5 (13 µg m-3) |
| --- | --- | --- | --- | --- |
| HGB | 76.6% | 90.1% | 77.4% | 84.7% |
| DFW | 73.1% | 80.2% | 87.4% | 87.3% |
| SA | 89.7% | 86.5% | 90.1% | 82.5% |
| ARR | 92.9% | 82.7% | 91.6% | 89.3% |

Figure 19. Box and whisker plots of the distributions of total MDA8 O3 (ppbv, top left), background MDA8 O3 (ppbv, top right), total daily average PM2.5 (µg m-3, bottom left), and background daily average PM2.5 (µg m-3, bottom right) for the Houston/Galveston/Brazoria urban area. The thick black line is the median of the distribution, the boundaries of the boxes are the 25th and 75th percentiles, and the whiskers cover the range of the data or all values within 1.5 times of the interquartile range (IQR) of the box, whichever is smaller. The circles denote outliers beyond 1.5 × IQR of the box. The horizontal lines show the criteria denoting “high” values of each metric, as discussed in the text.

Table 6. Percentage of observations above the criteria chosen to represent “high” values of total and background (BG) MDA8 O3 and daily average PM2.5 values for the Houston/Galveston/Brazoria urban area. The chosen criteria are in parentheses in the first column. Only values during the O3 season (May-Oct.) are considered for O3.

| Pollutant Metric | Synoptic Type | Percentage Above Criteria (%) |
| --- | --- | --- |
| Total MDA8 O3 (70 ppbv) | -999 | 36.2 |
|  | 1 | 8.3 |
|  | 2 | 34.0 |
|  | 3 | 17.6 |
|  | 4 | 11.9 |
|  | 5 | 29.0 |
| BG MDA8 O3 (55 ppbv) | -999 | 14.2 |
|  | 1 | 5.2 |
|  | 2 | 20.2 |
|  | 3 | 4.2 |
|  | 4 | 4.5 |
|  | 5 | 15.0 |
| Total Daily PM2.5 (17 µg m-3) | -999 | 25.8 |
|  | 1 | 22.7 |
|  | 2 | 13.7 |
|  | 3 | 26.7 |
|  | 4 | 19.0 |
|  | 5 | 24.7 |
| BG Daily PM2.5 (13 µg m-3) | -999 | 19.9 |
|  | 1 | 14.7 |
|  | 2 | 6.3 |
|  | 3 | 20.1 |
|  | 4 | 12.0 |
|  | 5 | 13.3 |

Figure 20. As in Figure 19, but for the Dallas/Fort Worth urban area.

Table 7. As in Table 6 but for the Dallas/Fort Worth urban area.

| Pollutant Metric | Synoptic Type | Percentage Above Criteria |
| --- | --- | --- |
| Total MDA8 O3 (70 ppbv) | -999 | 34.9 |
|  | 1 | 11.7 |
|  | 2 | 21.3 |
|  | 3 | 25.3 |
|  | 4 | 13.4 |
|  | 5 | 40.0 |
| BG MDA8 O3 (55 ppbv) | -999 | 25.2 |
|  | 1 | 10.7 |
|  | 2 | 26.6 |
|  | 3 | 15.6 |
|  | 4 | 10.5 |
|  | 5 | 30.0 |
| Total Daily PM2.5 (17 µg m-3) |  | 15.6 |
|  | 1 | 14.9 |
|  | 2 | 3.0 |
|  | 3 | 23.4 |
|  | 4 | 6.0 |
|  | 5 | 8.2 |
| BG Daily PM2.5 (13 µg m-3) | -999 | 14.5 |
|  | 1 | 17.4 |
|  | 2 | 0.7 |
|  | 3 | 25.7 |
|  | 4 | 5.7 |
|  | 5 | 8.8 |

Figure 21. As in Figure 19, but for the San Antonio urban area.

Table 8. As in Table 6 but for the San Antonio urban area.

| Pollutant Metric | Synoptic Type | Percentage Above Criteria |
| --- | --- | --- |
| Total MDA8 O3 (70 ppbv) | -999 | 16.3 |
|  | 1 | 2.4 |
|  | 2 | 19.2 |
|  | 3 | 5.1 |
|  | 4 | 5.2 |
|  | 5 | 9.0 |
| BG MDA8 O3 (55 ppbv) | -999 | 21.1 |
|  | 1 | 3.5 |
|  | 2 | 22.3 |
|  | 3 | 6.4 |
|  | 4 | 7.5 |
|  | 5 | 15.0 |
| Total Daily PM2.5 (17 µg m-3) | -999 | 10.2 |
|  | 1 | 14.2 |
|  | 2 | 1.6 |
|  | 3 | 13.1 |
|  | 4 | 10.4 |
|  | 5 | 7.1 |
| BG Daily PM2.5 (13 µg m-3) | -999 | 19.3 |
|  | 1 | 24.5 |
|  | 2 | 3.2 |
|  | 3 | 21.4 |
|  | 4 | 16.8 |
|  | 5 | 14.2 |

Figure 22. As in Figure 19, but for the Austin/Round Rock urban area.

Table 9. As in Table 6 but for the Austin/Round Rock urban area.

| Pollutant Metric | Synoptic Type | Percentage Above Criteria |
| --- | --- | --- |
| Total MDA8 O3 (70 ppbv) | -999 | 10.7 |
|  | 1 | 1.0 |
|  | 2 | 9.6 |
|  | 3 | 3.7 |
|  | 4 | 3.7 |
|  | 5 | 14.0 |
| BG MDA8 O3 (55 ppbv) | -999 | 25.6 |
|  | 1 | 5.2 |
|  | 2 | 33.0 |
|  | 3 | 8.6 |
|  | 4 | 13.4 |
|  | 5 | 20.0 |
| Total Daily PM2.5 (17 µg m-3) | -999 | 9.3 |
|  | 1 | 10.6 |
|  | 2 | 1.1 |
|  | 3 | 12.8 |
|  | 4 | 8.1 |
|  | 5 | 5.9 |
| BG Daily PM2.5 (13 µg m-3) | -999 | 12.1 |
|  | 1 | 14.8 |
|  | 2 | 1.1 |
|  | 3 | 16.2 |
|  | 4 | 8.1 |
|  | 5 | 9.1 |

### Discussion

The first thing to note is that with these criteria, there are no synoptic types where high values never happen, and there are no synoptic types where high values always happen. Thus the synoptic type by itself is neither necessary nor sufficient to determine if a given day will have elevated levels of O3 or PM2.5. However, the frequency of elevated levels of O3 and PM2.5 are clearly different between the different synoptic types, and this pattern is occasionally different between the Group 1 urban areas.

The second thing to note is that unclassified days in the O3 season (those that do not match any of the 5 synoptic types, and thus indicated as MT -999) have relatively high total and background O3 values for all four urban areas. This may point to a limitation in our current map-typing scheme, which uses data from the entire year to determine the five most-frequent types. Further work is thus required to redo the classification for just the O3 season and to increase the number of synoptic types.

Below we discuss the results for each of the pollutant metrics in turn.

#### Total MDA8 O3

In Houston/Galveston/Brazoria (Figure 19 and Table 6), the synoptic types can be sorted into two groups for the percentage of high values of total MDA8 O3: a low group (MT 1, MT 3, and MT4) and a high group (MT 2, MT 5, and unclassified or “MT -999”). Overall, 32.3 % of days in the high group have total MDA8 O3 above 70 ppbv, compared to only 13.7% of the days in the low group. MT 2 and MT 5 both feature stagnant high-pressure systems over Southeast Texas, and so it is not surprising that these clear days with low winds are favorable for O3 production. MT 1 and MT 3 have flow coming from the Gulf of Mexico over Houston, and MT 4 has a relatively fast southwesterly flow, both of which would tend to reduce the O3 levels in Houston. The percentage of unclassified days with high levels of total MDA8 O3 is also high (32.6%), and far more days in the O3 season fall into this type than into MT 2 or MT 5, so further work may be needed to classify these days into additional synoptic types for further analysis.

In Dallas/Fort Worth (Figure 20 and Table 7), the synoptic types fall into three groups with high (MT 5 and MT -999, 35-40%), medium (MT 2 and MT 3, 21-25%), and low (MT 1 and MT 4, 12-13%) percentages of days with total MDA8 O3 above 70 ppbv. MT 2 has relatively fewer high values in DFW than it did in HGB, likely because the northwesterly flow in MT 2 is stronger near DFW than near the Gulf of Mexico. In addition, while the flow from the Gulf in MT 3 tends to reduce O3 for HGB, it has less of an effect on DFW, likely because in this map type DFW receives outflow from most of eastern Texas.

In San Antonio (Figure 21 and Table 8), MT 2 and MT -999 have the highest percentage of high values. MT 5 has high levels about half as frequently as MT 2, mainly because MT 5 is less stagnant over San Antonio than MT 2, while both types have similar stagnation over Houston. The relatively fast flow of MT 1 over San Antonio leads to relatively few high O3 days.

The results for Austin/Round Rock (Figure 22 and Table 9) are similar to Houston in that MT 1, MT 3, and MT4 have a low percentage of high O3 days (1-4%) while MT 2, MT 5, and MT -999 have a high percentage (8-18%). This is consistent with the meteorology affecting Austin being similar to that affecting Houston for these synoptic types.

#### Background MDA8 O3

The background O3 results for Houston/Galveston/Brazoria follow a similar pattern to the total O3 results discussed in Section 4.1.2.1, with MT 2, MT 5, and MT -999 having a relatively high percentage of high background O3 days. This same pattern holds for Dallas/Fort Worth, San Antonio, and Austin/Round Rock, and likely reflects the higher stagnation during these synoptic types. The fact that the synoptic types affect background O3 similarly in all three urban areas suggests that the TCEQ method for calculating background O3 (see Section 3 and Appendix B) is capturing regional, synoptic influences on O3 in these urban areas.

#### Total Daily Average PM2.5

In Houston/Galveston/Brazoria (Figure 19 and Table 6), all of the synoptic types have similar percentages of days with total PM2.5 above 17 ug/m3 except for MT 2, which is significantly lower. As noted above, this is a relatively stagnant type over Houston with a slow northwesterly flow. San Antonio (Figure 21 and Table 8) and Austin/Round Rock (Figure 22 and Table 9) are similar, with MT 2 as a clear outlier with a low percentage of high PM2.5 days. The fact that the synoptic types with flow from the Gulf (MT 1 and MT 3) do not have appreciably lower PM2.5 may be due to the transport of marine aerosol into the urban regions, as also suggested by our GAM fits (see Section A.1.5.2). Similarly, the fact that the PM2.5 distributions are less sensitive to synoptic type than O3 is consistent with our GAM results (Section 2.2.2), which showed that less of the variation of PM2.5 could be attributed to meteorology than was the case for O3.

In contrast, in Dallas/Fort Worth (Figure 20 and Table 7), MT 2, MT 4, and MT 5 all have relatively low frequencies of high PM2.5 values. Both MT 4 and MT 5 have flow coming into DFW from the southwest, which gives les frequent high PM2.5 values than the southerly to southeasterly flow in types MT 1 and MT 3.

#### Background Daily Average PM2.5

In Houston/Galveston/Brazoria (Figure 19 and Table 6), there is more difference between the types in terms of the percentage of days with high background PM2.5 than there is for total PM2.5. While MT 2 is again clearly lower than the other types, MT 1, MT 4, and MT 5 are in a group between MT 2 and the high group of MT 3 and MT -999. This again suggests that marine aerosol are a significant part of the background and total PM2.5 in Houston when the flow is from the Gulf, as that flow dominates in MT 3 while the flow in MT 4 and MT 5 is from the southwest.

In Dallas/Fort Worth (Figure 20 and Table 7), there appear to be four groups: MT 2 at the low end (1%), MT 4 and MT 5 (6-9%), MT 1 and MT -999 (15-17%), and MT 3 (26%). This is again consistent with flow from the southwest bringing relatively lower background PM2.5 to DFW than flow from eastern Texas. Austin/Round Rock (Figure 22 and Table 9) shows a similar dependence of the background PM2.5 on the synoptic types to DFW.

San Antonio (Figure 21 and Table 8) only has MT 2 as a clear low outlier, similar to the results for total PM2.5.

## Urban-scale meteorological predictors of O3 and PM2.5

### Logistic Regression Approach

One goal of this project (Deliverable 4.2) is to determine if there are necessary and/or sufficient synoptic or urban-scale meteorological criteria for events of “high” total and background O3 and PM2.5 (here we again define “high” using the criteria in Section 4.1.1). There are likely no conditions where the probability of high O3 and PM2.5 is negligibly close to zero or one. Thus, in order to make our investigation of “necessary and/or sufficient” conditions for high O3 and PM2.5 tractable, we adopt the following probability definitions, recognizing that they are arbitrary choices:

- “Necessary” will refer to conditions that must be true for the probability of high O3 and PM2.5 (as defined in Section 4.1.1) to be greater than 20%.

- “More likely than not” will refer to conditions that, when true, give a greater than 50% chance of high O3 and PM2.5.

- “Sufficient” will refer to conditions that, when true, give a greater than 80% chance of high O3 and PM2.5.

Two ways to determine necessary and/or sufficient meteorological conditions have already been presented. First, the gam03 and back_gam03 models described in Sections 2.2.2 and 2.3 can be used to predict the actual values of total and background O3 and PM2.5 given the set of urban-scale meteorological predictors listed in Table A.7 and Table A.8. These predicted values and their confidence intervals can be used to estimate the probability that there will be high O3 and PM2.5 given a set of meteorological conditions. Second, in Section 4.1.2 we have shown that the probability of high O3 and PM2.5 events does vary between synoptic types.

Here we use the technique of logistic regression to create GAM models relating smooth functions of urban-scale and synoptic-scale meteorological variables to the probability that a high O3 or PM2.5 event will occur. Similar to the GAM equation in Section 2.2, the logistic regression equation is given by

gμi=βo+f1xi,1+f2xi,2+⋯fnxi,n+Sm

where i is the ith day’s observation of whether or not a high O3 or PM2.5 event occurred (coded as 1 for true and 0 for false), gμi is the “link” function (here, a logit link is used with a binomial probability distribution, unlike the log link and Gaussian distribution used for the GAMs of Section 2.2), and xi,j are the n urban scale meteorological predictors fit, with the corresponding fjxi,j being a (initially unknown) smooth function of xi,j made from a cubic-spline basis set. We do not include the day of week, year, and day of year variables in our logistic regression. Instead, we include a factor (Sm) describing the synoptic types described in Section 4.1. To reduce the possibility of over-fitting the data, we set the “gamma” parameter to 1.4 for these fits, as recommended by Wood (2006).

In order to simplify our analysis, we focused on just two urban-scale meteorological predictors, afternoon mean temperature and daily average wind speed. These variables were chosen as they seemed to have the biggest impact on the predicted values of both total and background O3 and PM2.5 in our GAM fits from Sections 2.2.2 and 2.3. We then plot the probability of a high O3 or PM2.5 event estimated by the logistic regression equation as a function of afternoon mean temperature and daily average wind speed, with a separate plot for each combination of pollutant metric, Group 1 urban area, and synoptic type. This results in 16 figures with six panels in each figure. The plots of Houston are included as Figure 23 through Figure 26  in this section, while the plots for the other three Group 1 urban areas are included in Appendix D: Logistic Regression Probability Plots for DFW, SA, and ARR below.

One thing to note is that these logistic models and the associated figures can be used to forecast the probability of a high O3 or PM2.5 events based on a corresponding meteorological forecast. The forecast for geopotential height at 850 mbar can be used to determine the synoptic type, and then the forecast of 10 m winds and 2 m temperatures at the locations listed in Table A.1 can be used to estimate the probability of an event. Further work should explore this approach further to determine the performance of such forecasts (e.g., do events occur 50% of the time a probability of 50% is forecast).

Figure 23. Probability of the total MDA8 O3 exceeding 70 ppbv for the Houston/Galveston/Brazoria urban area as a function of afternoon mean temperature (oC), daily wind speed (m/s), and synoptic type (as defined in Section 4.1).

Figure 24. As in Figure 23, but for the probability that background MDA8 O3 will exceed 55 ppbv.

Figure 25. As in Figure 23, but for the probability that total daily average PM2.5 will exceed 17 g/m3.

Figure 26. As in Figure 23, but for the probability that background daily average PM2.5 will exceed 13 g/m3.

Table 10. Deviance explained (%, bold) and URBE score (unitless, italics) for the logistic models for total and background O3 and PM2.5

| Urban Area | Total MDA8 O3 | BG MDA8 O3 | Total Daily PM2.5 | BG Daily PM2.5 |
| --- | --- | --- | --- | --- |
| HGB | 25.7 -0.187 | 14.4 -0.450 | 9.7 -0.035 | 10.4 -0.239 |
| DFW | 35.0 -0.243 | 18.3 -0.186 | 14.7 -0.341 | 18.0 -0.363 |
| SA | 22.4 -0.480 | 13.7 -0.310 | 15.0 -0.432 | 12.8 -0.175 |
| ARR | 17.3 -0.565 | 14.2 -0.193 | 13.3 -0.487 | 13.8 -0.397 |

### Results and Discussion

The percent deviance explained by the logistic model and the Un-Biased Risk Estimator (URBE) score for each logistic model is given in Table 10. The models explain the largest percentage of deviance for total MDA8 O3, but even here only 17-35% of the deviance is explained, suggesting mot of the variability is due to other parameters not included in the model. Additional meteorological predictors from Table A.7 and Table A.8 could be added to increase the amount of deviance explained, but adding predictors would make interpreting the results in terms of necessary and sufficient conditions more difficult.

The daily average wind speed is always a significant predictor at the  = 0.001 level, and the afternoon temperature is always significant at the  = 0.005 and significant at the  = 0.001 level expect for the background O3 fits for HGB and SA. The differences between the factors for the synoptic types are occasionally significant, but many types are found to be similar to each other, as expected from our discussion in Section 4.1.2.

Below we discuss the results for each of the pollutant metrics in turn.

#### Total MDA8 O3

As expected, the general trend for total MDA8 O3 in all four urban areas is to increase with increasing afternoon mean temperature and decreasing with daily average wind speed. We focus first on the HGB (Figure 23) and DFW (Figure D.1) urban areas, as these are the only urban areas that have meteorological conditions where the probability of total MDA8 O3 being above 70 ppbv is greater than 80%, and is thus “sufficient” under our definition. However, we must also note that these are the best-monitored urban areas as well, which may influence the high probabilities.

For HGB and DFW, once you correct for the relative distribution of mean afternoon temperature and average wind speed in each synoptic type, the synoptic types only account for a minor change in the probabilities. This suggests that the patterns seen for MDA8 O3 with synoptic type in Section 4.1.2.1 are mainly due to differences in the temperature and wind speed distributions within each type.

In HGB, the “necessary” conditions (probability > 20%) for high total O3 are afternoon temperatures above 17 oC and average wind speeds below 3.5 m/s. In contrast, DFW requires temperatures above 20 oC for the odds of a high total O3 event to get above 20%, but the wind speed can be as high as 4 m/s. The “sufficient” conditions (probability > 80%) in HGB are afternoon temperatures above ~29 oC and average wind speeds less than ~1.5 m/s. For DFW, the conditions are also temperatures above ~29 oC and the wind speed below 1-2 m/s, with the exact value depending on synoptic type. The gradients of the probabilities with respect to afternoon temperature and average wind speed look very similar between HGB and DFW.

For SA (Figure D.5) and ARR (Figure D.9), we should first note that the minimum and maximum measured average wind speeds in these areas were ~3.5 and 6 m/s, respectively, so data at these higher wind speeds is an extrapolation and should not be trusted.

In SA, the probabilities of high total O3 events are a much stronger function of wind speed than of temperature, unlike in HGB and DFW where the two predictors had more equal influence. The “necessary” conditions are average wind speed below ~1.5 m/s and afternoon temperatures above ~21 oC. The “more likely than not” conditions (probability > 50%) are that you must be in synoptic types MT -999, MT 2, MT 4, or MT 5 (i.e., the synoptic flow is not from the Gulf of Mexico as in MT 1 and MT 3), average wind speeds below ~0.5 m/s, and afternoon temperatures above 25-29 oC, with the critical temperature varying with synoptic type.

In ARR, the probability of a high total O3 event never gets above 20% for MT 1 (southerly flow from the Gulf) and never gets above 50% for MT 3 (southwesterly flow from the Gulf). The “necessary” conditions are thus a synoptic type other than MT 1, afternoon temperatures above 24-27 oC and wind speeds below 1-2 m/s, depending on synoptic type. The “more likely than not” conditions are near zero wind speeds and afternoon temperatures above 33 oC for MT 4 (fast southeasterly flow) and wind speed below 1 m/s and afternoon temperatures above 27 oC for MT -999, MT 2, or MT 5.

#### Background MDA8 O3

As expected, the patterns in the probabilities of high background O3 are similar to the patterns for total O3 for each urban area. For HGB (Figure 24), the “necessary” conditions are afternoon temperatures above ~17 oC and average wind speeds below ~2.5 m/s for all synoptic types except MT 3 (southeasterly Gulf flow). For MT 3, the temperature must be above 23 oC and the wind speeds below 1 m/s. A high background O3 event is “more likely than not” for temperatures above 23-25 oC and wind speeds below 1 m/s for all synoptic types except MT 3, where calm conditions and temperatures above 32 oC are required. “Sufficient” conditions for high background O3 are rare, only occurring for temperatures above 36 oC for MT 2 and MT 5.

For DFW, the “necessary” conditions are wind speeds below ~3 m/s and afternoon temperatures above 18 oC (MT 2 only) or above 20 oC (all other types). The “sufficient” conditions are wind speeds near 0 m/s and afternoon temperatures above 28 oC.

For SA, probabilities of high background O3 events over 50% are extremely rare, and the probabilities are mainly a function of wind speed and synoptic type. Wind speeds below 1 m/s are “necessary” for an event to occur.

For ARR, the apparent second maxima at high wind speeds is likely an artifact of the model extrapolating to wind speeds not included in the actual observations. The probabilities for a high background O3 event are a strong function of synoptic type in this urban area, and are highest for MT -999 and 2 (unclassified and stagnant conditions) and lowest for MT 1 and 3 (flow from the Gulf).

- For MT 1 and MT3, the “necessary” conditions are temperatures above 25 oC and wind speeds below 2 m/s, with no conditions reaching 50% probability.

- For MT 4 and MT 5 (southeasterly flow), the “necessary” conditions are  temperatures above 22 oC and wind speeds below 3 m/s, while a high event is “more likely than not” for wind speeds below 1.5 m/s and temperatures above 27 oC.

- For MT -999 and MT 2 (unclassified and stagnant conditions), the “necessary conditions are temperatures above 19-21 oC with a weak dependence on wind speed. A high background O3 event is “more likely than not” for temperatures above 23-25 oC and wind speeds below 2 m/s.

#### Total Daily Average PM2.5

The dependence of the probability of a high total PM2.5 event on daily average wind speed is dramatically different than that for high total O3 events for all four urban areas. HGB and DFW (Figure 25 and Figure D.3) both show the probabilities increasing with decreasing wind speed for wind speeds below 2-3 m/s, but the probabilities become independent of wind speed at higher wind speeds. In SA (Figure D.7), the probabilities mainly depend on wind speed as was the case for O3, but the plots show a secondary maxima for afternoon temperatures near 34 oC and average wind speeds near 2.5 m/s. In ARR (Figure D.11), the probabilities are rarely above 20% and the patterns are likely due to numerical errors, and so are not discussed any further.

For HGB, when the wind speeds are above ~ 3 m/s, the “necessary” conditions are afternoon temperatures above 30 oC for MT -999, MT 2, and MT 3, above 27 oC  for MT 1 and MT4, and above 25 oC for MT5. For wind speeds below 3 m/s, the necessary conditions are wind speeds below 2 m/s . The “sufficient” conditions are temperatures above 20-22 oC (MT 5 and MT1), 25 oC (MT -999, 2, and 3) or 29 oC (MT 4) and wind speeds below 1 m/s.

For DFW, when the wind speeds are above ~ 3 m/s, the “necessary” conditions are afternoon temperatures above 30 oC for MT 1, above 32 oC for MT 3, and above 35 oC (MT -999, 54, 5). There are no “sufficient” conditions for a high PM2.5 event for MT 2 (stagnant conditions). For MT 1, temperatures above 30 oC and wind speeds below 1 m/s are “sufficient”, and temperatures above 35 oC with near 0 wind speeds are “sufficient” for the other synoptic types.

There are no “sufficient” conditions for a high PM2.5 event in SA. For MT 2, the “necessary” conditions are temperatures above 26 oC and wind speeds near 0 m/s. For the other synoptic types, the “necessary” conditions are wind speeds below 1 m/s, with a high PM2.5 event being “more likely than not” for temperatures above 27 oC and wind speeds near 0.. In addition, the “necessary” conditions for a high PM2.5 event exist for wind speeds around 2 m/s and temperatures around 35 oC, with the size of this secondary maximum depending on the synoptic type.

#### Background Daily Average PM2.5

For HGB (Figure 26), we see a parabolic dependence of the probability of a high background PM2.5 on the daily average wind speed with a minimum around 3.5 m/s. The increased probability of a high background PM2.5 event at high wind speeds likely reflects an increase in marine or dust aerosol emission at higher wind speeds. The “necessary” conditions for an event are afternoon temperatures above 15-19 oC during calm conditions, temperatures above 30-34 oC at the wind speed minimum at 3.5 m/s, and temperatures above 25-27 oC at high wind speeds (> 6 m/s). There are few “sufficient” conditions for an event, but a high background PM2.5 event is “more likely than not” for temperatures above 27-29 oC for calm conditions and above 35 oC for fast wind speeds.

For DFW (Figure D.4), the probability of an event is below 20% or MT 2. For the other types, when wind speeds are greater than 2 m/s, the probability of a high PM2.5 event is a function of afternoon temperature and synoptic type only, with the threshold temperatures at 27 oC (MT 1), 31 oC (MT -999 and MT 3), 33 oC (MT 5) and 35 oC (MT 4). At wind speeds below 2 m/s, the probabilities are strong functions of both temp and wind speed. No conditions are “sufficient”, but an event is “more likely than not” for temperatures above 28-33 oC and wind speeds below 1-2 m/s, with the critical values depending on the synoptic type.

For SA (Figure D.8), the data for temperatures above 35 oC are extrapolations as the temperature only rarely gets this high. As in total PM2.5, we see a secondary probability maximum around 3 m/s and 35 oC. “More likely than not” conditions rarely occur, and the “necessary” conditions are largely independent of wind speed. There are no “necessary” conditions for MT 2, and the critical temperature for the other types varies between 26-29 oC.

As for total PM2.5, the probabilities for a high background PM2.5 event in ARR are rarely above 20%, and so are not discussed further.

# Quality Assurance Steps and Reconciliation with User Requirements

All work on the project was done in accordance with the Quality Assurance Project Plan (QAPP). All scripts and data files used in this project were inspected by team members different from the original author to ensure they were correct, and any errors noted in early versions were fixed. Other required evaluations are contained within the report (for example, see Section A.1.6) In addition, if further analysis or feedback from TCEQ uncovers any errors in the provided files, we will correct those and provide TCEQ with corrected files.

In addition, the QAPP listed several questions that needed to be addressed for each project task. These questions are addressed below.

## Task 2: Development of GAMs

  - Do the relationships between meteorological variables and O3 and PM2.5 described in the developed GAMs make physical sense given our conceptual models of O3 and PM2.5 emissions, chemistry, and transport?

As noted in Sections A.1.4.2 and A.1.5.2, the functional dependencies in the GAMs between the predictors related to temperature, RH, wind speed, vertical stability, and HYSPLIT bearing are all qualitatively consistent with our conceptual understanding of O3 and PM2.5 emissions, chemistry, and transport.

  - Are these relationships consistent with the scientific literature?

As noted in Section A.1.4.2, our GAMs for MDA8 O3 are consistent with those found for eastern US cities by Camalier et al. (2007).

  - Does the change in the relationships between urban areas make physical sense given our conceptual models of O3 and PM2.5 emissions, chemistry, and transport?

We find that the general trends of the relationships rarely change significantly between the urban areas. For O3, the major differences are that DFW, SA, and ARR show the O3 trend with afternoon temperature flattening out above 30 oC and that the impact of relative humidity is fairly weak in HGB. For PM2.5, the major differences are between the cities near the Gulf of Mexico (HGB and BPA) and the others, with the cities near the Gulf showing increasing PM2.5 at wind speed above 5 m/s and a minimum in PM2.5 at a HYSPLIT bearing of 120o instead of at 320o.

  - Are the HYSPLIT back-trajectories used in the model development reasonable? How sensitive are these trajectories to the initial location?

As noted in Section A.1.2, the HYSPLIT back-trajectories used in the model development appear reasonable and generally consistent with the surface wind speed and direction measured near the center of each urban area. The ensemble back-trajectory results suggest that our results are representative of the air masses entering each urban area, but that differences in distance of less than ~100 km and differences in bearing of less than ~20o are unlikely to be significant.

  - How well does the GAM reproduce the testing sets in the cross-validation evaluation?

As noted in Section A.1.6, the two-fold cross-validation showed that the GAMs fit to half of the data fit the other half of the data nearly as well as the GAMs fit to all of the data.

  - Does the cross-validation evaluation of the models show evidence of over-fitting?

As noted in Section A.1.6, there is no evidence of over-fitting in the overall MDA8 O3 and daily average PM2.5 predictions. However, the functional relationships between the meteorological predictors and O3 and PM2.5 are occasionally sensitive to which half of the dataset is used for the fit, and so caution must be used in interpreting these relationships.

  - Under what conditions are the GAMs expected to be valid? What conditions give exceptionally large residuals?

Strictly speaking, the GAMs are only expected to be valid during the periods for which they were fit, and when the data is taken from the sources and sites noted in this memo. Extrapolations to other times and monitoring locations may be problematic, and the GAMs ability in this regard has not been assessed in this project. We have not identified any set of necessary or sufficient conditions that lead to large residuals in the GAMs.

## Task 3: Background O3 and PM2.5

  - Are the derived background estimates, and their spatial and temporal variation, consistent with our conceptual models of O3 and PM2.5 emissions, chemistry, and transport?

The overall trends of background O3 and PM2.5 are decreasing, consistent with our understanding of reduction of pollutant emissions (primarily NOx and SO2) over this time period. Background O3 has a minimum in July and a maximum in September for urban areas near the Gulf of Mexico, consistent with the seasonal shifts in synoptic conditions.

  - Are these estimates consistent with the scientific literature?

The main literature comparison in the study of Houston background O3 from 2000-2012 by Berlin et al. (2013). They found that unadjusted total MDA8 O3 in HGB decreased at a rate of -0.89±0.66 ppbv/year from 2000 to 2012. Our calculated annual averages for the MDA8 O3 for all Houston sites from 2005-2014 (see Figure 1 and Figure 8) show the same year-to-year variation as seen in that study during the overlapping period, but our average values are lower, probably due to different sets of sites being included in the two studies, but further study should be performed to resolve the discrepancy. Our unadjusted trend for 2005-2014 (-1.48±0.73 ppbv/year) is consistent with their estimate, especially as 2013 and 2014 had lower average MDA8 O3 than any of the years from 2005-2012.

Similarly, the year to year variability of our TCEQ background O3 estimates for HGB are consistent with those from Berlin et al. (2013), with no evidence of an “offset” as seen in the total MDA8 O3 values.  Our trend estimate for 2005-2014 (-0.91±0.57 ppb/year) and the Berlin et al. (2013) estimate for 2000-2012 (-0.21±0.39 ppbv/year) are consistent to within the 95% confidence intervals, especially considering the low values seen in 2013 and 2014.

The trends in our PCA-based background O3 estimates (see Figure 13) are similar to the 6-station PCA results from Berlin et al. (2013) in that both show a slight decreasing trend. However, differences exist in both the year-to-year variations and the overall magnitude of this background, likely due to our consideration of more sites for the entire May-October O3 season. As noted below, further research is needed to understand the differences between the TCEQ and PCA estimates of background O3.

  - What are the uncertainties in the background estimates, and under what conditions are they valid?

The major uncertainties in the background estimates calculated using the TCEQ method are, first, that they assume that the regional background can be estimated as the lowest value observed at a selected number of sites around the urban area. This neglects the fact that the urban areas in Texas likely influence each other’s “background”, and so our background estimates cannot be interpreted as estimates of what the concentrations would be with all Texas sources removed. A second uncertainty is that the different urban areas have very different numbers of monitoring sites, and so the regional background is likely undersampled for some urban areas, especially the Group 2 areas. So long as these caveats are kept in min, the values should be valid throughout 2005-2014, except for PM2.5 in the TLM urban area, as noted in Section B.2.4.

The PCA-derived background estimates from Section 3.3 should be considered as experimental, but the estimates for O3 show good correlation with the TCEQ-based estimates. Further research is needed to understand the differences between the two different estimates of background O3.

  - Are the derived background estimates, and their spatial and temporal variation, consistent with the other data-based methods explored in this task? If not, is there a reasonable explanation for the differences?

As noted in Section 3.3.1, the TCEQ and PCA-based estimates of background O3 are well-correlated with each other (r2 > 0.79), and the year-to-year variability in the background O3 distributions also appear consistent between the two methods. However, the slopes of the linear relationships between the two estimates differ between urban areas for reasons that are still unclear.

## Task 4: Synoptic and Mesoscale Controls of O3 and PM2.5

  - Are the derived synoptic types and identified mesoscale meteorological controls on extreme and background concentrations of O3 and PM2.5 consistent with our conceptual understanding of O3 and PM2.5 emissions, chemistry, and transport?

The synoptic types identified in Section 4.1 appear to be reasonable, including a mixture of stagnant conditions, flow from the Gulf of Mexico, flow from the Western US, and flow from the southeast. The dependence of O3 and PM2.5 on urban-scale meteorological predictors and on synoptic types is consistent with our understanding of O3 and PM2.5, as discussed in Sections 2.2, 4.1.2, 4.2.2, and A.1.5.2.

  - Are these estimates consistent with the scientific literature?

The GAM results are consistent with Camalier et al. (2007) as noted above, and the synoptic type analysis is similar to that performed in Hegarty et al. (2007). The results of the logistic regressions of Section 4.2.2 are reasonable, except for some extrapolated conditions noted in the text.

# Conclusions

Here we summarize the conclusions of our analysis, with reference to the corresponding report section and project deliverable.

- The Generalized Additive Models (GAMs) relating meteorological variables to the maximum MDA8 O3 for each urban area generally explain 65-80% of the deviance (i.e. variability), consistent with the results of Camalier et al. (2007). The GAMs also generally show good fits with normally-distributed residuals and little dependence of the residual variance on the predicted value (Sections 2.2.2.1 and A.1.5.2, Deliverable 2.2).

- In contrast, the GAMs relating meteorological variables to the maximum daily average PM2.5 for each urban area only explain 30-40% of the deviance, and generally show much poorer fits with long, positive residual tails and a strong dependence of the variance of the residuals on the predicted value (Sections 2.2.2.2 and A.1.5.2, Deliverable 2.2).

- Using meteorological predictors different from those listed in Camalier et al. (2007) can result in an improved GAM for MDA8 O3 and daily average PM2.5, but the improvement is less significant for PM2.5 (Sections 2.2.2.1, 2.2.2.2, and A.1.5.2, Deliverable 2.2).

- We find that the general trends of the relationships rarely change significantly between the urban areas. For O3, the major differences are that DFW, SA, and ARR show the O3 trend with afternoon temperature flattening out above 30 °C and that the impact of relative humidity is fairly weak in HGB. For PM2.5, the major differences are between the cities near the Gulf of Mexico (HGB and BPA) and the others, with the cities near the Gulf showing increasing PM2.5 at wind speeds above 5 m/s and a minimum in PM2.5 at a HYSPLIT bearing of 120° instead of at 320° (Sections 2.2.2.1, 2.2.2.2, and A.1.5.2, Deliverable 2.2).

- We calculated estimates of total and background MDA8 O3 and daily average PM2.5 for the period 2005-2015, with the background estimated calculated using the TCEQ method described in Berlin et al. (2013) (Sections 3.1 and B.2, Deliverable 3.1).

- We find that the meteorological relationships determined by fitting GAMs to background O3 and PM2.5 are substantially identical to those fit to total O3 and PM2.5, with the possible exception of HGB O3 (Section 2.3, Deliverables 3.2 and 4.2).

- After meteorological adjustment via the GAMs fit to total and background O3 and PM2.5 for each urban area, several negative trends in pollutant metrics between 2005-2014 were observed to be significant at a 95% confidence level and no positive trends were observed (Section 2.4, Deliverables 2.2, 3.2, and 4.2).

  - For HGB, total and background MDA8 O3 decreased at -0.46±0.40 ppbv/year and -0.41±0.31 ppbv/year, respectively, while total and background daily average PM2.5 decreased by -0.39±0.14 μg/m3/year and -0.37±0.11 μg/m3/year, respectively.

  - For DFW, total and background MDA8 O3 decreased at -0.68±0.39 ppbv/year and -0.57±0.49 ppbv/year, respectively, while total and background daily average PM2.5 decreased by -0.15±0.09 μg/m3/year and -0.23±0.08 μg/m3/year, respectively.

  - In SA, background O3 and PM2.5 decreased by -1.01±0.87 ppbv/year and -0.12 ±0.07 μg/m3/year, respectively.

  - In ARR, background PM2.5 decreased by -0.21±0.07 μg/m3/year.

  - In BPA, total PM2.5 decreased by -0.34±0.11 μg/m3/year.

  - In TLM, total O3 and PM2.5 decreased by -0.66±0.44 ppbv/year and -0.34 ±0.08 μg/m3/year, respectively.

- Background MDA8 O3 is fairly constant with month during the O3 season for Tyler- Longview-Marshall and Dallas/Fort Worth, but has a July minimum for the other urban areas. In contrast, median background PM2.5 peaks in June and July. The range of values for a given month or year is large for all cities, with Houston having the largest O3 spread and the most outliers, possibly due to the fact that it has the largest dataset (Section 3.2, Deliverable 3.2)

- We find that the principal component analysis (PCA) based method of Langford et al. (2009) can give reasonable values for background O3 in the four Group 1 urban areas as long as the PCA analysis is performed separately for the May-July and August-October halves of the period. These PCA-based background estimates are well-correlated with the values derived using the TCEQ method, and show similar seasonal and inter-annual variability. However, the slope of the linear relationship between the two background O3 estimates varies substantially between urban areas for reasons that are currently unclear and require further analysis (Section 3.3.1, Deliverable 3.3).

- In contrast, the PCA-based background estimates for PM2.5 give unphysical values regardless of what time period is used for the analysis (Section 3.3.2, Deliverable 3.3).

- Currently existing methods, such as those of van Donkelaar et al. (2013), exist for using satellite observations to determine surface PM2.5 concentrations at a high (0.01o latitude by 0.01o longitude) horizontal resolution. These methods should be further explored as an additional way of estimating regional background PM2.5 (Section 3.3.4, Deliverable 3.3).

- In contrast, current satellite techniques have difficulty separating the boundary-layer O3 mixing ratios from the free tropospheric values, but techniques that combining different satellite instruments or use future instruments may make this possible (Section 3.3.3, Deliverable 3.3).

- We identified 5 synoptic types based on the NARR 850 mbar geopotential height fields that allow for the classification of 70% of all day in the 10-year study period and 58% of the days in the May-October O3 season. However, the remaining unclassified days account for a significant proportion of high O3 and PM2.5 events, which suggests that future work should adjust the method of Hegarty et al. (2007) should be adjusted to classify more days (Section 4.1.1, Deliverable 4.1).

- We defined criteria for “high” levels of total and background MDA8 O3 and daily average PM2.5 based on the observed 90th percentile values of the Group 1 urban areas. These criteria are 70 ppbv for total MDA8 O3, 55 ppbv for background MDA8 O3, 17.0 µg m-3 for total daily average PM2.5, and 13.0 µg m-3 for background daily average PM2.5 (Section 4.1.1, Deliverable 4.1).

- We found that the relative percentage of “high” O3 and PM2.5 events varied between the synoptic types and urban areas (Section 4.1.2, Deliverable 4.2).

  - For HGB, the stagnant types MT 2 and MT 5, and the unclassified “MT -999” days, had high percentages (> 30%) of days with events of high total and background O3, but type MT 2 had significantly fewer high PM2.5 events (14% for total PM2.5) than the other synoptic types (19-27%).

  - For DFW, types MT 5 and MT -999 have a very high percentage (> 35%) of days with high total O3 and the percentage of background O3 events is high for MT2, MT 5, and MT -999 (> 25%). MT 1 and MT 3,which have southerly and southeasterly flow, as well as the unclassified days in MT -999, have a relatively high percentage of high PM2.5 events (15-25%).

  - For SA, types MT 2 and MT -999 are associated with high O3 events (16-19% for total, 21-22% for background) and type MT 2 has an unusually low number of high PM2.5 events (2-3% versus 7-25% for the other types).

  - ARR has similar O3 results to HGB, with MT 2, MT 5, and MT -999 relatively high (8-18% for total, 20-33% for background), with MT 2 having a relatively low percentage of high PM2.5 events (1% versus 6-13%).

- We performed logistic regressions to determine how the probability of high O3 and PM2.5 event in each urban area changed with afternoon mean temperature, daily average wind speed, and synoptic type. These predictors were chosen as they had been shown to be important in our GAM models and our previous analysis of the synoptic types. We used these probability models to investigate “necessary” (defined as giving a probability of a high event > 20%) and “sufficient” (defined as giving a probability of a high event > 80%) criteria for high O3 and PM2.5 events (Section 4.2, Deliverable 4.2).

  - For HGB, “sufficient” conditions for high total O3 are afternoon temperatures above 29 oC and wind speed below 1-2 m/s depending on synoptic type. High PM2.5 events can occur at both low and high wind speeds, but “sufficient” conditions include wind speeds below 1 m/s and afternoon temperatures above a critical value that varies between 20-29 oC with synoptic type.

  - For DFW, “sufficient” conditions for high total and background O3 are wind speeds below 1-2 m/s and afternoon mean temperatures above ~29 oC, with the exact values depending on synoptic type. “Sufficient” conditions for high total PM2.5 are temperatures above 30-35 oC depending on synoptic type.

  - For SA, there are no “sufficient” conditions for high O3 or PM2.5 events. High total O3 events are ”more likely than not” when the synoptic flow is not from the Gulf and the afternoon temperatures are above 25-29 oC. High total PM2.5 events are “more likely than not” when temperatures are above 27 oC and wind speeds are near zero.

  - For ARR, high total O3 events are “more likely than not” for near zero wind speeds and afternoon temperatures above 33 oC for MT 4 (fast southeasterly flow) and wind speed below 1 m/s and afternoon temperatures above 27 oC for the relatively stagnant conditions of MT -999, MT 2, or MT 5. The probability of high PM2.5 events is generally less than 20% under all conditions.

# Recommendations for Future Study

As already noted in the report, there are several questions raised by the results of our current study that would benefit from further investigation. For example, not all of the urban areas in our study were equally well-sampled for either air quality or meteorological parameters. While this is unavoidable for a historical data study like this, future work could quantify the impact of the relative sparsity of observations for some urban areas on the robustness of our conclusions, especially those about differences between urban areas. For example, further work is needed to determine if the slope of the linear relationship between the TCEQ method and PCA-based method background O3 estimates varies substantially between urban areas because of actual differences between the urban areas or differences in their monitoring networks.

In addition, the synoptic typing method used in this study has provided valuable information on the dependence of high O3 and PM2.5 events on synoptic conditions, but the fact that 30% of all days and 42% of days in the O3 season are not covered by the five current types, and the fact that these unclassified days have a relatively high percentage of high O3 events, suggests the need for further refinements to the synoptic typing technique to classify more of the remaining days.

We also discussed how satellite observations could be used to derive high-resolution estimates of surface PM2.5 concentration, which could be used to refine regional background estimates. Future work should pursue this possibility and compare the derived background estimates with in situ measured values in Texas.

The GAMs developed in this study to relate meteorological predictors to the concentrations of total O3 and PM2.5, as well as the logistic GAMs used to determine necessary and sufficient conditions for high O3 and PM2.5 events, should be further developed and refined to provide accurate forecasts of air quality for the urban areas studied in this project.

Finally, these GAMs derived from monitor network data should be compared with similar GAMs fit to meteorological and chemical data from 3D Eulerian air quality models like CAMx and CMAQ to determine if these models accurately represent the dependence of O3 and PM2.5 concentrations, and the probability of high O3 and PM2.5 events, on meteorology. Differences discovered between the two sets of GAMs could point towards missing physics or incorrect parameterizations in the current Eulerian air quality models.

# References

Berlin, S. R., A. O. Langford, M. Estes, M. Dong, and D. D. Parrish (2013) Magnitude, decadal changes, and impact of regional background ozone transported into the greater Houston, Texas, area, Environ. Sci. & Technol., 47, 13985-92.

Camalier, L., Cox, W., and Dolwick, P. (2007), The effects of meteorology on ozone in urban areas and their use in assessing ozone trends, Atmos. Environ., 41, 7127-7137.

Draxler, R. R. and G. D. Hess (1997), Description of the HYSPLIT_4 modeling system. NOAA Tech. Memo. ERL ARL-224, 24 pp.

Draxler, R. R. and G. D. Hess (1998), An overview of the HYSPLIT_4 modeling system for trajectories, dispersion, and deposition, Aust. Meteorol. Mag., 47, 295-308.

Hegarty, H. R. R. Draxler, A. F. Stein, J. Brioude, M. Mountain, J. Eluszkiewicz, T. Nehrkorn, F. Ngan, and A. Andrews (2013), Evaluation of Lagrangian Particle Dispersion Models with Measurements from Controlled Tracer Releases. J. Appl. Meteor. Climatol., 52, 2623–2637, doi: http://dx.doi.org/10.1175/JAMC-D-13-0125.1

Hegarty, J., H. Mao, and R. Talbot (2007), Synoptic controls on summertime surface ozone in the northeastern United States, J. Geophys. Res., 112, D14306, doi:10.1029/2006JD008170.

Langford, A. O., J. Brioude, O. R. Cooper, C. J. Senff, R. J. Alvarez II, R. M. Hardesty, B. J. Johnson, and S. J. Oltmans (2012), Stratospheric influence on surface ozone in the Los Angeles area during late spring and early summer of 2010, J. Geophys. Res., 117, D00V06, doi:10.1029/2011JD016766.

Starkweather, J. (2011), Cross Validation techniques in R: A brief overview of some methods, packages, and functions for assessing prediction models, available at https://www.unt.edu/rss/class/Jon/Benchmarks/CrossValidation1_JDS_May2011.pdf

van Donkelaar, A., R. V. Martin, R. J. D. Spurr, R. Burnett (2015), Combining GEOS-Chem, satellite, and ground monitors for improved PM2.5 population exposure estimates, 7th Annual GEOS-Chem Meeting, Harvard University, May 4th-7th, 2015

van Donkelaar, A., R. V. Martin, R. J. D. Spurr, E. Drury, L. A. Remer, R. C. Levy, and J. Wang (2013), Optimal estimation for global ground-level fine particulate matter concentrations, J. Geophys. Res. Atmos., 118, 5621–5636, doi:10.1002/jgrd.50479.

Wood, S. N. (2006), Generalized Additive Models: An Introduction with R, part of the “Texts in Statistical Science” series, Chapman & Hall/CRC, New York.

Zoogman, P., D.J. Jacob, K. Chance, H.M. Worden, D.P. Edwards, L. Zhang, (2014) Improved monitoring of surface ozone by joint assimilation of geostationary satellite observations of ozone and CO, Atmos. Environ. 84, 254-261.

# Appendix A. Effects of Meteorology on O3 and PM2.5 Trends

This appendix documents the files provided to TCEQ to complete Deliverable 2.2 of Work Order No. 582-15-54118-01. The GAMs and all associated data and scripts are in the gzipped tar file for the deliverable, which can be downloaded from the AER ftp server at:

ftp://ftp.aer.com/anonymous/pub/malvarad/p1952_deliverable_2_2_R1_0.tar.gz

Our major findings are (see Sections A.1.4, A.1.5, and A.1.6 for more details):

- The GAMs relating meteorological variables to the maximum MDA8 O3 for each urban area generally explain 65-80% of the deviance (i.e. variability), consistent with the results of Camalier et al. (2007). The O3 GAMs also generally show good fits with normally-distributed residuals and little dependence of the residual variance on the predicted value.

- In contrast, the GAMs relating meteorological variables to the maximum daily average PM2.5 for each urban area only explain 30-40% of the deviance, and generally show much poorer fits with long, positive residual tails and a strong dependence of the variance of the residuals on the predicted value.

- Using meteorological predictors different from those listed in Camalier et al. (2007) can result in an improved GAM for MDA8 O3 and daily average PM2.5, but the improvement is less significant for PM2.5.

- Two-fold cross validation analysis shows that the GAM fitting procedure results in GAMs that only perform slightly worse for the “test” data set as they do for the “training” data set, and thus the GAMs show little evidence of overfitting.

- However, the cross validation analysis also shows that the smooth function fit for some meteorological predictors can vary substantially depending on which half of the data is used to train the GAM. Thus the individual smooth functions from each GAM should be used with caution.

Section A.1 of this memo briefly outlines the technical approach used to prepare the generalized additive models (GAMs) in the deliverable and Section A.2 describes the files in the deliverable. Section A.3 briefly outlines the quality assurance steps that have been performed.

## Technical Approach

As described in the Work Plan, AER derived updated GAMs for O3 and PM2.5 for selected monitoring sites within the urban areas in Table A.1. Surface meteorological sites selected for GAM fitting.. For O3, only data during the O3 season (May to October) was analyzed, but PM2.5 data for the entire year was analyzed.

AER first fit the data to the 8 meteorological parameters that were determined to give the best fit for urban O3 by Camalier et al. (2007). As in that paper, a daily transport distance and transport direction were determined by 24-hour back-trajectories calculated with the HYSPLIT model (Draxler and Hess, 1997, 1998) driven with meteorology from the 32 km horizontal resolution North American Regional Reanalysis (NARR).

In addition to these “baseline” GAMs (referred to as “gam01_baseline” below and in the deliverable files), AER explored whether the addition or substitution of other meteorological variables significantly increased the amount of variability explained by the model. This resulted in two additional GAMs (“gam02_extended” and “gam03_extended”) that are also included in the deliverable.

One of the dangers of using GAMs to perform the meteorological adjustment of pollutant trends is the possibility of “over-fitting,” where some of the variability that is actually due to changes in air quality policy is accounted for in the GAM by the meteorological variables. AER explored the potential errors from over-fitting via cross validation. In cross validation, some of the data (the testing set) is removed before building the GLM. The remaining data (the training set) is used to derive the GAM parameters. The testing set can then be used to test the performance of the GAM in predicting “unseen” data (e.g., Starkweather et al., 2011).

Section A.1.1 below describes the input data used to generate the GAMs, including a discussion of the processing we performed on the raw data to make it suitable for generating the GAMs. Section A.1.2 describes the generation and evaluation of the HYSPLIT back trajectories. Section A.1.3 gives an overview of our GAM fitting procedure, followed by an overview of the GAM results for both the baseline (Section A.1.4) and extended (Section A.1.5) GAMs. Section A.1.6 then presents the results of the cross-validation analysis of the “gam03_extended” GAMs from Section A.1.5.

### Input Data and Processing

#### TCEQ Monitor Data

The TCEQ provided AER with air quality and meteorological monitoring data from the air quality monitoring network operated by the TCEQ, its grantees, or local agencies whose data is stored in the Texas Air Monitoring Information System (TAMIS) in and near the urban areas listed in Table 1 covering a ten-year period (2005-2014). AER then built Python scripts that processed the TCEQ air quality and meteorological data and calculate the average (daily, morning, afternoon, etc.) and derived quantities (e.g., deviations from 10-year monthly averages) needed for the GAM fitting. Following Camalier et al. (2007), these average and derived quantities for each urban area were calculated using a single surface site in the center of the urban area combined with the nearest radiosonde location available. The selected surface sites for each urban region are given in Table A.1 - they were selected to maximize the amount of data available at each site.

Table A.1. Surface meteorological sites selected for GAM fitting.

| Urban Area | Site # | Latitude (°) | Longitude (°) |
| --- | --- | --- | --- |
| Houston/Galveston/Brazoria | 482011035 | 29.7337263 | -95.2575931 |
| Dallas/Fort Worth | 484391002 | 32.8058183 | -97.3565675 |
| San Antonio | 480290055 | 29.4072945 | -98.431251 |
| Austin/Round Rock | 484530014 | 30.3544356 | -97.7602554 |
| Beaumont/Port Arthur | 482450009 | 30.0364221 | -94.0710606 |
| Tyler-Longview-Marshall | 481830001 | 32.3786823 | -94.7118107 |

As noted in the Appendix to the original deliverable, we developed a python script (calc_bkgrd_ozone.py, see Section A.2.2) that calculated the MDA8 O3 (ppbv) for all of the monitoring sites in the six urban areas. The MDA8 for a site was calculated as follows:

- A running 8-hour average was calculated for each hour, averaged over that hour and the following seven hours. At least 6 hours in this 8-hour range had to have valid O3 measurements for the 8-hour average to be considered valid.

- The largest of each of the calculated 8-hour averages in a day was selected as the MDA8 for that day.

- The maximum and minimum of the valid MDA8 O3 values for all sites in the urban area were determined.

- The minimum of the valid MDA8 O3 values for the selected background sites were determined as the daily background concentration for that area.

A similar script (calc_pm25.py) was used to calculate daily average PM2.5 values from the available hourly data. This average was calculated as follows:

- If more than one PM2.5 instrument was active for a site, the reported hourly values were averaged.

- A daily average PM2.5 value was then calculated for each site. At least 18 hours of that day had to have valid PM2.5 measurements for the daily average to be considered valid.

- The maximum and minimum of the valid PM2.5 values for all sites in the urban area were determined.

- The minimum of the valid PM2.5 values for the selected background sites were determined as the daily background concentration for that area.

Two additional python scripts (calc_GLM_all.py and calc_GLM_NCDC.py) were used to calculate the potential meteorological predictors. The TCEQ monitor data, Integrated Global Radiosonde Archive data (IGRA, Section A.1.1.2) and the integrated surface hourly (ISH) database of the National Climatic Data Center (NCDC, Section A.1.1.3), along with the previously calculated MDA8 and PM2.5 maximum and minimum concentrations and parameter from the HYSPLIT back trajectories (Section A.1.2), were merged by a final script (merge_param_all_Camalier.py).  This script then outputs the final CSV file used in fitting the GAM model.  These scripts are all described further in Section 3.

#### IGRA Radiosonde Data

The Integrated Global Radiosonde Archive (IGRA) provided upper atmosphere data used to derive the meteorological predictors for the GAMs. These data can be downloaded at ftp://ftp.ncdc.noaa.gov/pub/data/igra. Table A.2 describes the sites selected for each urban area, which were selected because they were the closest sites to the center of each urban area that had continuous data for the 2005-2014 period. Section A.2.1.1 describes these files in further detail.

Table A.2. IGRA sites used for each urban area.

| Urban Area | ID | Station Name | Lat. (o) | Lon. (o) |
| --- | --- | --- | --- | --- |
| Houston/Galveston/Brazoria | 72249 | FORT WORTH | 32.8 | -97.3 |
| Dallas/Fort Worth | 72240 | LAKE CHARLES | 30.12 | -93.22 |
| San Antonio | 72261 | DEL RIO | 29.37 | -100.92 |
| Austin/Round Rock | 72261 | DEL RIO | 29.37 | -100.92 |
| Beaumont/Port Arthur | 72240 | LAKE CHARLES | 30.12 | -93.22 |
| Tyler-Longview-Marshall | 72248 | SHREVEPORT | 32.45 | -93.83 |

#### NCDC Integrated Surface Hourly Data

We have also added data from the integrated surface hourly (ISH) database of the National Climatic Data Center (NCDC) to our dataset. We used the NCDC data to get estimates of surface pressure and relative humidity, as this data was not generally available in the TCEQ dataset. The NCDC sites used for each urban area are described in Table A.3 below. These sites were selected because they were the closest sites to the center of each urban area that had continuous data for the 2005-2014 period. The dataset is described further in Section A.2.1.2.

Table A.3. NCDC surface sites used for each urban area.

| Urban Area | USAF-WBAN_ID | Station Name | Lat. (o) | Lon. (o) |
| --- | --- | --- | --- | --- |
| DFW | 722590 03927 | DALLAS/FT WORTH INTERNATIONAL | 32.898 | -97.019 |
| HGB | 722430 12960 | G BUSH INTERCONTINENTAL AP/HOU | 29.98 | -95.36 |
| SA | 722530 12921 | SAN ANTONIO INTERNATIONAL AIRP | 29.544 | -98.484 |
| ARR | 722544 13958 | AUSTIN-CAMP MABRY ARMY NATIONA | 30.321 | -97.76 |
| BPA | 722410 12917 | SOUTHEAST TEXAS REGIONAL AIRPO | 29.951 | -94.021 |
| TLM | 722470 03901 | EAST TEXAS REGIONAL ARPT | 32.385 | -94.712 |

#### NARR Data

The North American Regional Reanalysis (NARR) meteorological data are available from 1979 to 2014 on a 3 hourly, 32 km grid.  The NARR is an extension of the NCEP Global Reanalysis but only for North America. Combining the higher resolution NCEP Eta Model (32km/45 layer) with a data assimilation system optimized for regional reanalysis results in better accuracy of the meteorological variables compared to the NCEP Global Reanalysis.  The NARR data can be downloaded from the NOAA Air Resources Library (ARL) ftp server at ftp://arlftp.arlhq.noaa.gov/narr.

### HYSPLIT Back Trajectories

We ran 24-hour HYSPLIT back-trajectories for each urban region for the 2005-2014 period. These back-trajectories were calculated using the 32 km horizontal resolution NARR, as these data were available in a form suitable to drive HYSPLIT for our entire study period (2005-2014), as opposed to the 12 km North American Mesoscale (NAM-12) data called for in the Work Plan, which were only available for 2008-2014. As in Camalier et al. (2007), these back-trajectories are calculated assuming an initial height of 300 m above ground level (AGL) and are started at noon local solar time. The starting points for the back-trajectories are the selected surface meteorological sites given in Table A.1 above. The HYSPLIT model (Draxler and Hess, 1997, 1998) is available for download from the HYSPLIT website (http://ready.arl.noaa.gov/HYSPLIT.php). The performance of HYSPLIT driven with NARR meteorological fields was evaluated with tracer release studies by Hegarty et al. (2013).

The endpoints of the back-trajectories were used to calculate the 24-hour transport direction and distance for each urban area for the 2005-2014 period. This was done using the R functions bearing and distMeeus from the geosphere package (see the script ./hysplit_trajec/ calc_trajec.src, described in Section A.2.3.3). The function bearing gets the initial bearing (direction; azimuth) to go from point 1 to point 2 following the shortest path (a Great Circle). The function distMeeus calculates the shortest distance between two points (i.e., the ’great-circle-distance’ or ’as the crow flies’) using the WGS84 ellipsoid.

The HYSPLIT back-trajectories used in the model development appear reasonable and are generally consistent with the surface wind speed and direction measured near the center of each urban area. The HYSPLIT back-trajectory distance is generally correlated with the urban area average surface wind speed with a linear correlation coefficient (R) of 0.4-0.6. The frequency of both the daily average wind direction and the HYSPLIT back-trajectory bearings peak around 150o (southeast, from the Gulf of Mexico) for all urban areas. However, the HYSPLIT back-trajectory bearings also show a secondary maximum at 0o (north) not seen in the daily average wind directions.

We also examined a few ensemble back-trajectories, initialized from slightly different locations, to determine the potential uncertainty of the back-trajectory calculations. Figure A.1 shows an example ensemble back-trajectory calculation for August 25, 2013 in Houston/Galveston/Brazoria, a day of high MDA8 O3. We can see that the back-trajectories all follow a consistent qualitative shape, although the exact locations of the end points can differ. These results give us confidence that our HYSPLIT results are representative of the air masses entering the urban areas, but that differences in distance of less than ~100 km and differences in bearing of less than ~20o are unlikely to be significant.

Figure A.1. Ensemble back-trajectory run for the Houston/Galveston/Brazoria area on August 25, 2013.

### Generalized Additive Model (GAM) Fitting Procedure

In our procedure, we fit the maximum MDA8 O3 value and the maximum 24-hour average PM2.5 value for each urban area using the GAM modeling function in the mgcv package in R (Wood, 2006). The GAM can be written as follows:

gμi=βo+f1xi,1+f2xi,2+⋯fnxi,n+fpDi+Wd+Yk

where i is the ith day’s observation, gμi is the “link” function (here, a log link is used), xi,j are the n meteorological predictors fit, with the corresponding fjxi,j being a (initially unknown) smooth function of xi,j made from a cubic-spline basis set. Following Camalier et al. (2007), three non-meteorological predictors are also included: a smooth function fpDi of the Julian day of the year (Di); a factor for the day of the week Wd and a factor for the year Yk. As we are only fitting O3 data during the O3 season (May-October), fpDi is built with a non-periodic cubic spline basis for O3, but for PM2.5, a periodic cubic spline basis is used. To reduce the possibility of over-fitting the data, we set the “gamma” parameter to 1.4 for these fits, as recommended by Wood (2006).

We also added an automated process to determine if a predictor that is not significant at the  = 0.001 level could be eliminated from the fit without significantly degrading the performance of the model. In this process, the meteorological predictor with the highest p value is removed and a second GAM is fit. This is then compared to the original model using the ANOVA procedure in R. If the second model with the variable removed is not different from the original model at the   = 0.01 level, the variable is “dropped” from the fit and the variable with the next highest p value is tested. If the second model is significantly worse than the original model, the variable is kept and no other variables are tested or dropped. Because of this, although the GAMs for a given pollutant may start with the same predictors for all urban areas, the final GAM selected may have different predictors depending on which variables were dropped for each urban area.

### Baseline GAMs (gam01_baseline)

#### Description

We have developed “baseline” GAMs for the maximum MDA8 O3 and daily average PM2.5 in each area, where we use the eight meteorological parameters identified as significant by Camalier et al. (2007) in their study of O3 in eastern US cities. These parameters are listed in Table A.4 below. The automated process to remove insignificant predictors was not used for these fits.

Table A.4. Meteorological parameters used in the "baseline" GAMs. The column name is given in italics.

Daily maximum temperature (oC, daily_max_T)

Mid-day average (10 am–4 pm average) relative humidity (%, NCDC.Mid.day.RH)

Morning (7–10 am) average wind speed (m s-1, morning_ws)

Afternoon (1–4 pm) average wind speed (m s-1, afternoon_ws)

Morning surface temperature difference (1200 UTC) (temperature at 925 mb–temperature at surface at 1200 UTC) (oC, T_diff_925mb)

Deviation in 1200 UTC temperature of 850 mb surface from 10-year monthly average (oC, T_dev_850mb)

Transport direction (degrees clockwise from North, HYSPLIT_DIST..m.)

Transport distance (m, HYSPLIT_DIST..m.)

#### Results

To illustrate the results, we discuss the baseline GAM fits for Houston in detail. Similar plots for all urban areas are contained in the deliverable as described in Section A.2.6. Figure A.2 shows the smooth functions from the baseline GAM fit of the natural logarithm of the HGB maximum MDA8 O3 values to the meteorological predictors in Table A.4. 95% confidence intervals are shown in red. The periodic day of year function is also shown. This model explains 74% of the deviance of the MDA8 O3 values. This is consistent with the Camalier et al. (2007) results, which showed the predictive power of their models (measured by the R2 statistic) to be between 0.56 and 0.80 for the cities in that study. In this case, all eight meteorological predictors and the day-of-year function are statistically significant at the α= 0.001 level. As expected, the model fit shows O3 generally increasing with daily maximum temperature, decreasing with RH, decreasing with wind speed, and increasing with vertical stability (positive values of T_diff_925mb). In addition, the predicted O3 mixing ratio drops when the wind is from the southeast, as expected for air blowing from the Gulf of Mexico to Houston. The day-of-year function may reflect the fact that the mean mixing height increases in the summer, leading to a decrease in MDA8 O3 in the middle of the ozone season. For the weekday factor variables, the largest average MDA8 values are Wednesday-Friday, with Sunday having the lowest average MDA8 values, as expected. The differences between Sunday and the Wednesday-Friday period are significant at the α= 0.001 level.

The year-to-year differences in the meteorologically-adjusted natural logarithm of MDA8 O3 are shown in Figure A.3. All of the differences from the base year of 2005 are statistically significant at the α= 0.001 level except for 2006. However, the two-fold cross-validation tests (described in Section A.1.6 below) show that the year-to-year changes in MDA8 O3 determined with different randomly-distributed halves of the dataset can give very different results (the red and blue circles in Figure A.3), although these are generally within the 95% confidence interval of the original fit. It is also unclear why there would be a sudden increase between 2010 and 2011 that is not accounted for by the meteorological predictors. Thus, while we can be reasonably confident that the meteorologically-adjusted MDA8 O3 for Houston in 2014 was significantly lower than in 2005, the magnitude and shape of the trend over the years is less certain.

The standard GAM evaluation plots (made with the gam.check function in R) for this case are shown in Figure A.4. These plots indicate a good fit, as the model residuals are roughly normally distributed and show no trend versus predicted value. The variance of the residuals is lower for low values of the predictor, but this reflects the fact that the measured MDA8 O3 values cannot go below 0.

Figure A.2. Smooth functions for the baseline GAM (gam01_baseline) fit to HGB MDA8 O3 data. The y-axis scale is the scale of the “linear predictor”, i.e. the deviation of the natural logarithm of the MDA8 O3 in ppbv from its mean value.

Figure A.3. Year-to-year deviations from 2005 for the baseline GAM (gam01_baseline) fit to HGB MDA8 O3 data. The y-axis scale is the scale of the “linear predictor”, i.e. the deviation of the natural logarithm of the MDA8 O3 in ppbv from its mean value. The black center bar is the mean value while the error bars are the 95% confidence intervals. The red and blue circles are the mean values from the two-fold cross-validation analysis of Section A.1.6.

Figure A.4. GAM evaluation plots for the baseline GAM (gam01_baseline) fit to HGB MDA8 O3 data.

Figure A.5 shows the smooth functions from the baseline GAM fit of the natural logarithm of the HGB maximum daily average PM2.5. This model only explains 38% of the deviance in the PM2.5 values, and so the baseline meteorological parameters in Table A.4 give a much poorer prediction than the same parameters do for O3. Again, all eight meteorological predictors and the day-of-year function are statistically significant at the α= 0.001 level. Like O3, increasing maximum temperature generally leads to increasing PM2.5. However, in this case there is an indication that at the highest temperatures this relationship may not hold, possibly because evaporation of semi-volatile organic and ammonium nitrate aerosol begins to compete with the increased chemical production of secondary aerosol with increasing temperature. The impacts of wind speed are much less strong as well, potentially reflecting increased dust and marine aerosol emission at high wind speeds. Similarly, the impact of air blowing from the Gulf is less pronounced for PM2.5, perhaps reflecting the increased transport of marine aerosol to Houston during these periods. All weekdays have larger PM2.5 values than Sunday, and with the exception of Saturday these differences are significant at the α= 0.001 level.

The year-to-year differences in the meteorologically-adjusted natural logarithm of daily average PM2.5 are shown in Figure A.6. The years 2009 to 2014 are all significantly lower than 2005 at the α= 0.001 level. The two-fold cross-validation tests (described in Section A.1.6 below) show little difference in the observed trend, with both randomly-distributed halves of the dataset showing slight increases in 2006 and 2007 followed by dramatic decreases.

The GAM evaluation plots in Figure A.7 indicate a poorer fit for PM2.5 than for O3, as the residuals show a long positive tail and the variance of the residuals is a strong function of the value of the linear predictor.

Table A.5 below summarizes the percentage of the deviance explained by the baseline GAMs for each urban area for MDA8 O3 and daily-average PM2.5. For O3, the values vary between 65.7% (San Antonio) and 73.9% (Houston/Galveston/Brazoria), similar to the range of 56-80% reported by Camalier et al. (2007). The performance for PM2.5 is much poorer for all urban areas, with values between 30.0% (Beaumont/Port Arthur) and 37.8% (Houston/Galveston/Brazoria).

The generalized cross validation (GCV; see p.132 of Wood, 2006) score and Akaike’s Information Criterion (AIC; see p.68 of Wood, 2006) for each GAM is also shown in Table A.5. Both of these criteria attempt to compensate for the fact that adding redundant parameters to a model will always increase the likelihood of the model (and the amount of deviance explained), even if the new parameters are only “modeling the noise” of the data, i.e., over-fitting the data. For a given urban area and pollutant, the model with the lower GCV score and AIC is considered to be a better fit for the data. These scores will be compared to the values from the extended GAMs discussed in Section A.1.6.

Figure A.5. Smooth functions fit for the baseline GAM (gam01_baseline) fit to HGB daily average PM2.5 data. The y-axis scale is the scale of the “linear predictor”, i.e. the deviation of the natural logarithm of the daily average PM2.5 in µg m-3 from its mean value.

Figure A.6. Year-to-year deviations from 2005 for the baseline GAM (gam01_baseline) fit to HGB daily average PM2.5 data. The y-axis scale is the scale of the “linear predictor”, i.e. the deviation of the daily average PM2.5 in µg m-3 from its mean value. The black center bar is the mean value while the error bars are the 95% confidence intervals. The red and blue circles are the mean values from the two-fold cross-validation analysis of Section A.1.6.

Figure A.7. GAM evaluation plots for baseline GAM (gam01_baseline) fit to HGB MDA8 O3 data.

Table A.6 lists the meteorological predictors in each urban area that were not significant at the =0.001 level for maximum MDA8 O3 and maximum daily average PM2.5. In addition, we examined the smooth functions fit for each predictor for similarities and differences between the urban areas. For maximum MDA8 O3:

- The daily maximum temperature functions all show increasing O3 with increasing temperature, but the fits for DFW, SA, and ARR become flat for temperatures > 30 oC, while the other areas show no such flattening off.

- The mid-day RH functions all show decreasing O3 with increasing RH, and have a similar shape for all urban areas (relatively flat until 60% RH, then increasing at higher RH).

- O3 decreases with morning wind speed for all urban areas except ARR (where it is fairly flat).

- O3 either decreases with afternoon wind speed or the predictor is not significant.

- All urban areas except DFW show increasing O3 with increasing stability (T_diff_925mb). The predictor is fairly flat for DFW with maxima at either end that may not be significantly different from zero.

- The deviation of the 850 mbar temperature from the monthly average (T_dev_850mb) is insignificant for ARR and SA, and may just be fitting noise for the other urban areas as there is little consistency in the functional forms.

- O3 decreases with HYSPLIT back-trajectory distance up to ~1000 km, at which point it becomes highly uncertain due to the low number of points, but may begin to increase.

- All the urban areas show a drop in O3 at a HYSPLIT bask-trajectory bearing of ~150o (southeast), likely due to reduced background O3 from flows from the Gulf of Mexico.

- The day-of-year function shows a minimum at ~200 Julian days (July) in each urban area.

For maximum daily average PM2.5:

- All urban areas generally show PM2.5 increasing with daily maximum temperature, but the effect is fairly weak for ARR, and SA, DFW, and HGB suggest that the trend flattens out or reverses at temperatures > ~30 oC.

- The fits for mid-day RH are very uncertain at low (< 40%) and high (> 80%) values, and the functional shape changes significantly between urban areas, with SA and ARR generally showing decreasing PM2.5 with increasing RH, HGB and BPA showing an opposite trend, and TLM showing a maximum around 70% RH.

- PM2.5 either trends down with increasing morning wind speed or the effect is insignificant.

- PM2.5 generally trends down with increasing afternoon wind speed, but HGB, DFA, and BPA show a highly uncertain upward trend for wind speeds greater than 6 m/s.

- All urban areas show increasing PM2.5 with increasing stability (T_diff_925mb), but the effect is fairly weak for TLM.

- PM2.5 generally trends upward with increasing deviation of the 850 mbar temperature from the monthly average (T_dev_850mb).

- PM2.5 decreases with HYSPLIT back-trajectory distance up to ~500 km, at which point it becomes flatter and highly uncertain due to the low number of points.

- All urban areas show a maximum for PM2.5 around a HYSPLIT back-trajectory bearing of ~60o (northeast) and a minimum around 320o (northwest), possibly due to the relative difference in the PM2.5 concentrations in the western and eastern US. Most urban areas also show a secondary minimum around ~150o (southeast), likely due to flows from the Gulf of Mexico.

- The day-of-year functions for all urban areas are lower in the summer, likely reflecting the higher mixing heights in this season. The maximum is generally between 50-100 Julian days (around March), and ARR, SA, and HGB show a secondary maximum at ~200 Julian days (July).

Table A.5. Deviance explained by the baseline GAMs (gam01_baseline) for each urban area and pollutant and the corresponding GCV and AIC values.

| Urban Area | MDA8 O3 |  |  | Daily-Average PM2.5 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Deviance Explained (%) | GCV | AIC | Deviance Explained (%) | GCV | AIC |
| DFW | 72.8 | 87.71 | 13,060 | 35.2 | 17.05 | 19,360 |
| HGB | 73.9 | 118.2 | 12,680 | 37.8 | 18.28 | 19,010 |
| SA | 65.7 | 80.80 | 13,080 | 33.6 | 16.07 | 19,040 |
| ARR | 66.6 | 73.63 | 12,730 | 34.0 | 15.19 | 19,200 |
| BPA | 71.2 | 93.68 | 12,640 | 30.0 | 23.65 | 20,160 |
| TLM | 70.4 | 70.70 | 12,670 | 35.8 | 16.87 | 19,210 |

Table A.6. Meteorological predictors that were not significant at the =0.001 level for the baseline GAMs (gam01_baseline).

| Urban Area | MDA8 O3 | Daily-Average PM2.5 |
| --- | --- | --- |
| DFW | None | NCDC.Mid.day.RH, afternoon_ws, HYSPLIT_DIST..m. |
| HGB | None | None |
| SA | T_dev_850mb | morning_ws |
| ARR | T_dev_850mb | morning_ws |
| BPA | morning_ws | morning_ws |
| TLM | afternoon_ws | T_dev_850mb, afternoon_ws |

### Extended GAMs (gam02_extended and gam03_extended)

#### Description

We explored whether a different set of meteorological predictors than those used by Camalier et al. (2007) and used in the baseline GAMs of Section A.1.4 could provide a better fit to the maximum MDA8 O3 and maximum daily average PM2.5 for each urban area. We used a three-step procedure to select an appropriate subset of meteorological predictors for these extended GAMs.

First, a large set of potential meteorological predictors was assembled from the TCEQ, IGRA, and NCDC ISH data described in Section A.1.1, as well as the HYSPLIT back-trajectory endpoints described in Section A.1.2. The 60 potential predictors in Camalier et al. (2007) were used to guide the assembly of this set. The final files containing these predictors are described in Section A.2.4.2, and predictors in those files are listed in the file ./csv_files/final_files/GAMparam_readme.txt in the deliverable.

Second, the meteorological predictors were screened to remove combinations of variables that were both (a) highly correlated with each other and (b) likely represented the same physical quantity. Highly correlated variables generally represent the same information, and including both of them in the GAM can cause problems, just as including two nearly identical variables in a linear fit can result in arbitrarily large, unconstrained values of the slopes for each variable. In this step, we focused on identifying the true number of reasonably independent (uncorrelated) variables that best correlated with the maximum MDA8 O3 and daily average PM2.5 for each urban area. For example, of the four initial surface temperature variables (maximum, morning average, afternoon average, and diurnal change), it was found that the first three were highly correlated with each other (R > 0.8). This is to be expected, as the maximum temperature will generally happen in the afternoon, and days with hot afternoons generally have hot mornings as well. Thus we conclude that there are only two independent surface temperature variables in that set, one representing an effective maximum temperature and one representing the diurnal temperature change. As the mean afternoon temperature was most correlated with MDA8 O3 and daily average PM2.5, it was selected to represent the effective maximum temperature in the extended GAM fits. Similar analyses were performed for the variable sets representing humidity, combinations of temperature and humidity (e.g., dew point temperature and apparent temperature), surface wind speed and direction, upper air temperature, and pressure/geopotential height.

Third, the variables that passed the correlation screening described above were used to form initial GAMs for each urban area and pollutant. This would occasionally reveal additional variables that appeared to be strongly linked, such that the smooth function fit to each variable would have a very large uncertainty, and the two members of the pair would have opposing (cancelling) effects. In these cases, one member of the pair was removed and the fit run again.

The selected meteorological predictors for maximum MDA8 O3 are listed in Table A.7 while the predictors for maximum daily average PM2.5 are listed in Table A.8. These predictors were used to fit the “large” extended GAMs (gam02_extended). These fits did use the automated selection procedure described in Section A.1.3 to remove insignificant predictors. Analysis of the final GAMs showed that some predictors were either dropped or not significant at the = 0.001 level for 4 or more of the urban areas. Thus, these predictors were removed and an additional “small” extended GAM fit was performed (gam03_extended). The variables removed from these fits are indicated at the bottom of Table A.7 and Table A.8.. Note for Tyler-Longview-Marshall, the large and small extended GAM fits are identical, as the variables removed for the small extended GAM were also removed from the large extended GAM by the automated selection procedure.

Table A.7. Meteorological parameters used in the extended MDA8 O3 GAMs

| Meteorological Variable | Column Name | In gam03? |
| --- | --- | --- |
| Afternoon (1–4 pm) mean temperature (oC) | afternoon_mean_T | Yes |
| Diurnal temperature change (oC) | diurnal_T | Yes |
| Daily average relative humidity (%) | NCDC.Avg.RH | Yes |
| Daily average dew point (oC) | NCDC.Avg.Dew.Point..C. | Yes |
| Daily average wind speed (m s-1) | daily_ws | Yes |
| Daily average wind direction (degrees clockwise from North) | daily_wd | Yes |
| Morning surface temperature difference (1200 UTC) (temperature at 850 mbar –temperature at surface at 1200 UTC) (oC) | T_diff_850mb | Yes |
| Transport direction (degrees clockwise from North) | HYSPLIT.bearing..degrees.from.N. | Yes |
| Transport distance (km) | HYSPLIT_DIST..m. | Yes |
| Deviation in 1200 UTC temperature of 850 mbar surface from 10-year monthly average (oC) | T_dev_850mb | NO |
| Geopotential Height at 850 mbar and 1200 UTC (m) | GH_850.m. | NO |
| Surface solar radiation (Langy/min) | SolarRadiation.Langy.min. | NO |

Table A.8. Meteorological parameters used in the extended daily average PM2.5 GAMs

| Meteorological Variable | Column Name | In gam03? |
| --- | --- | --- |
| Afternoon (1–4 pm) mean temperature (oC) | afternoon_mean_T | Yes |
| Daily average relative humidity (%) | NCDC.Avg.RH | Yes |
| Temprature at 925 mbar and 1200 UTC (oC) | T_925mb | Yes |
| Daily average wind speed (m s-1) | daily_ws | Yes |
| Morning surface temperature difference (1200 UTC) (temperature at 850 mbar –temperature at surface at 1200 UTC) (oC) | T_diff_850mb | Yes |
| Transport direction (degrees clockwise from North) | HYSPLIT.bearing..degrees.from.N. | Yes |
| Transport distance (km) | HYSPLIT_DIST..m. | Yes |
| Surface solar radiation (Langy/min) | SolarRadiation.Langy.min. | Yes |
| Deviation in 1200 UTC temperature of 850 mbar surface from 10-year monthly average (oC) | T_dev_850mb | NO |
| Diurnal temperature change (oC) | diurnal_T | NO |
| Daily average wind direction (degrees clockwise from North) | daily_wd | NO |

#### Results

Table A.9 summarizes the percentage of the deviance explained by the large extended GAMs for each urban area for MDA8 O3 and daily-average PM2.5, while Figure 21 shows the same for the small extended GAMs. The tables show that the large extended GAMs (gam02_extended) give slightly better fits than the small extended GAMs (gam03_extended). However, this difference is fairly small, and an examination of the smooth fits for the variables contained in each GAM show little difference in the functional shape. Despite the lower GCV and AIC scores, it seems likely that the additional predictive power from the large extended GAMS over the small is mainly from having an additional three variables to use to fit the noise.

For maximum MDA8 O3, both extended GAMs are clear improvements over the baseline GAMs described in Section A.1.4, as indicated both by the larger percentage of deviance explained (range of 74-79% versus 65-74%) and the lower GCV and AIC scores. For maximum daily average PM2.5, the improvement is less clear, with only two urban areas (Dallas/Fort Worth and Houston/Galveston/Brazoria) showing both lower GCV and AIC scores in the small extended GAMs than in the baseline GAM.

Based on these results, we recommend using the small extended GAMs (gam03_extended) for most purposes, with the baseline GAMS (gam01_baseline) mainly used for comparison with the results of Camalier et al. (2007). In the rest of Section 2, we focus on the small extended GAMs (gam03_extended). However, all three sets of GAMs are included in the deliverable for completeness.

Table A.9. Deviance explained by the large extended GAMs (gam02_extended) for each urban area and pollutant and corresponding GCV and AIC values.

| Urban Area | MDA8 O3 |  |  | Daily-Average PM2.5 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Deviance Explained (%) | GCV | AIC | Deviance Explained (%) | GCV | AIC |
| DFW | 78.7 | 69.51 | 12,160 | 39.2 | 16.22 | 18,800 |
| HGB | 79.3 | 97.46 | 11,480 | 40.9 | 17.42 | 17,800 |
| SA | 76.1 | 57.31 | 12,390 | 36.1 | 15.55 | 19,030 |
| ARR | 75.0 | 55.81 | 12,210 | 36.4 | 14.69 | 19,320 |
| BPA | 76.1 | 79.19 | 12,380 | 30.9 | 23.49 | 20,120 |
| TLM | 73.7 | 63.21 | 12,600 | 37.5 | 16.37 | 19,390 |

Table A.10. Deviance explained by small extended GAMs (gam03_extended) for each urban area and pollutant and corresponding GCV and AIC values.

| Urban Area | MDA8 O3 |  |  | Daily-Average PM2.5 |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Deviance Explained (%) | GCV | AIC | Deviance Explained (%) | GCV | AIC |
| DFW | 78.2 | 70.47 | 12,350 | 38.2 | 16.27 | 19,100 |
| HGB | 78.6 | 98.52 | 12,520 | 38.8 | 18.04 | 18,120 |
| SA | 75.6 | 58.46 | 12,560 | 34.8 | 15.74 | 19,080 |
| ARR | 74.5 | 56.33 | 12,370 | 34.2 | 15.07 | 19,420 |
| BPA | 75.5 | 79.84 | 12,550 | 30.1 | 23.73 | 20,380 |
| TLM | 73.7 | 63.21 | 12,600 | 37.5 | 16.37 | 19,390 |

Similar to Section A.1.4, we discuss the small extended GAMs for Houston in detail to illustrate the results. Similar plots for all urban areas are contained in the deliverable as described in Section A.2.6. Figure A.8 shows the smooth functions from the small extended GAM fit of the natural logarithm of the HGB maximum MDA8 O3 values to the meteorological predictors. The periodic day of year function is also shown, and the 95% confidence intervals are shown in red. All meteorological predictors used in gam03_extended were significant at the α= 0.001 level except for average relative humidity, but that predictor was not removed by the automated selection procedure. As expected, the model fit shows O3 generally increasing with daily maximum temperature, decreasing with increased humidity (increasing RH and dew point temperature), decreasing with wind speed, and increasing with vertical stability (positive values of T_diff_850mb). In addition, the predicted O3 mixing ratio drops when the wind is from the southeast, as expected for air blowing from the Gulf of Mexico to Houston. The day-of-year function is generally decreasing through the ozone season. For the weekday factor variables, the largest average MDA8 values are Wednesday-Friday, similar to the baseline GAM results. The differences between Sunday and the Wednesday-Friday period are significant at the α= 0.001 level.

The year-to-year differences in the meteorologically-adjusted natural logarithm of MDA8 O3 are shown in Figure A.9. All of the differences from the base year of 2005 are statistically significant at the α= 0.001 level except for 2007. The sudden change between 2010 and 2011 seen in the baseline GAM (Figure A.3) is now gone, so that there is now a sharp decrease from 2007 to 2008 followed by a gradual (but not statistically significant) increase.

The standard GAM evaluation plots for this case are shown in Figure A.10. These plots indicate a good fit, as the model residuals are roughly normally distributed and show no trend versus predicted value.

Figure A.11 shows the smooth functions from the baseline GAM fit of the natural logarithm of the HGB maximum daily average PM2.5. All eight meteorological predictors and the day-of-year function are statistically significant at the α= 0.001 level. Like O3, increasing maximum temperature generally leads to increasing PM2.5. However, as in the baseline GAM there is an indication that at the highest temperatures this relationship may not hold, possibly because of the evaporation of semi-volatile aerosol components. Increasing wind speed tends to decrease PM2.5 at low values (0-4 m/s) but appears to increase PM2.5 at higher values (4-7 m/s), possibly reflecting increased dust and marine aerosol emission with higher wind speeds. Similarly, the impact of air blowing from the Gulf is less pronounced for PM2.5, perhaps reflecting the increased transport of marine aerosol to Houston during these periods. PM2.5 increases with increased vertical stability (positive values of T_diff_850mb) as expected. The negative dependence on solar radiation may reflect that lower values of solar radiation are seen on cloudy days, and SO2 is rapidly oxidized to aerosol sulfate within clouds. The day-of-year dependence is consistent with an increase in the mean mixing layer height in the summer, leading to relatively lower values of PM2.5 on those days. For the weekday factor variables, the largest daily average PM2.5 values are Tuesday-Friday, with Sunday having the lowest values, as expected. The differences between Sunday and the Tuesday-Friday period are significant at the α= 0.001 level.

The year-to-year differences in the meteorologically-adjusted natural logarithm of daily average PM2.5 are shown in Figure A.12, and are very similar to the results for the baseline case shown in Figure A.6. Similar to O3, PM2.5 drops significantly between 2007 and 2008, but unlike O3, PM2.5 continues to drop in the following years. As in the baseline case, the years 2009 to 2014 are all significantly lower than 2005 at the α= 0.001 level. In addition, the two-fold cross-validation tests (described in Section A.1.6 below) show little difference in the observed trend, with both randomly-distributed halves of the dataset showing slight increases in 2006 and 2007 followed by dramatic decreases.

The GAM evaluation plots in Figure A.13 also indicate a poorer fit for PM2.5 than for O3, as the residuals show a long positive tail and the variance of the residuals is a strong function of the value of the linear predictor, as was the case for the baseline GAMs.

Table A.11 lists the meteorological predictors in each urban area that were not significant at the =0.001 level for maximum MDA8 O3 and maximum daily average PM2.5.  Note that solar radiation measurements were not available for San Antonio or Austin/Round Rock.

In addition, we examined the smooth functions fit for each predictor for similarities and differences between the urban areas. For maximum MDA8 O3:

- The afternoon mean temperature functions all show increasing O3 with increasing temperature, but the fits for DFW, SA, and ARR flatten out for temperatures > 30 oC, while the other areas show no such flattening off.

- O3 generally increases with increasing diurnal temperature change, but the effect is weak.

- The daily average RH functions all show decreasing O3 with increasing RH, but the effect is relatively weak in HGB.

- O3 generally increases with dew point temperature up until 10-15 oC, after which point O3 decreases. This is consistent with the competing effects of temperature and humidity on O3 production.

- O3 decreases with daily average wind speed for all urban areas, but the effect is strongest in HGB and SA.

- All urban areas except BPA show increasing O3 with increasing stability (T_diff_850mb). However, O3 decreases at the highest values of T_diff_850mb for SA (-5 to 0 oC)

- Daily wind direction generally has little impact on the O3, and is likely just fitting noise.

- O3 decreases with HYSPLIT back-trajectory distance up to ~500 km, at which point it becomes highly uncertain due to the low number of points, but may begin to increase.

- All the urban areas show a drop in O3 at a HYSPLIT bask-trajectory bearing of ~150o (southeast), likely due to reduced background O3 from flows from the Gulf of Mexico.

- The day-of-year function shows a slight decrease over the length of the O3 season for all urban areas, with an area of nearly flat slope at ~200-225 Julian days (July-August).

For maximum daily average PM2.5:

- All urban areas generally show PM2.5 increasing with afternoon mean temperature, but the effect is fairly weak for ARR, and SA, DFW, and HGB suggest that the trend flattens out or reverses at temperatures > ~30 oC.

- The fits for average RH generally peak at 60-70% and fall off at lower and higher RH values, although SA and ARR show a second peak at the lowest extreme values (~20%).

- PM2.5 generally increases with increasing temperature at 925 mbar, but HGB shows a significant increase at the lower extreme.

- PM2.5 generally trends down with increasing daily average wind speed, but HGB and BPA show an upward trend for wind speeds greater than 6 m/s, possibly related to marine aerosol production.

- All urban areas show increasing PM2.5 with increasing stability (T_diff_850mb).

- PM2.5 decreases with HYSPLIT back-trajectory distance up to ~500 km, at which point it becomes flatter and highly uncertain due to the low number of points. The DFW fit is fairly flat, showing little dependence on back-trajectory distance.

- All urban areas show a maximum for PM2.5 around a HYSPLIT bask-trajectory bearing of ~60o (northeast). DFW, SA, ARR, and TLM show a minimum around 320o (northwest), possibly due to the relative difference in the PM2.5 concentrations in the western and eastern US. However, the urban areas near the Gulf of Mexico (HGB and BPA) have a minimum around ~150o (southeast), likely due to flows from the Gulf of Mexico.

- PM2.5 generally decreases with increasing solar radiation, possibly due to increased cloudiness leading to more rapid oxidation of SO2 into aerosol sulfate.

- The day-of-year functions for all urban areas are lower in the summer, likely reflecting the higher mixing heights in this season. The maximum is generally between 50-100 Julian days (around March), and ARR and SA show a secondary maximum at ~200 Julian days (July).

We also compared the functional forms in the extended GAMs to those of the baseline GAMS described in Section A.1.4. For O3, although the exact predictors used varied between the models, the functional shapes for temperature, RH, stability, and HYSPLIT 24-hour back-trajectory bearing and distance were very similar between the two models. However, the shape of the day-of-year function changed dramatically, and the daily wind speed dependence in the extended GAMS was generally stronger than the afternoon and morning wind speed effects in the baseline GAMs. For PM2.5, the functional shapes for temperature, RH, stability, wind speed, HYSPLIT 24-hour back-trajectory bearing and distance, and day-of-year were all very similar between the baseline and extended models.

Table A.11. Meteorological predictors that were not significant at the =0.001 level for the small extended GAMs (gam03_extended).

| Urban Area | MDA8 O3 | Daily-Average PM2.5 |
| --- | --- | --- |
| DFW | None | HYSPLIT_DIST..m. |
| HGB | NCDC.Avg.RH | None |
| SA | None | SolarRadiation.Langy.min. (not measured) |
| ARR | None | SolarRadiation.Langy.min. (not measured) |
| BPA | T_diff_850mb (dropped) | T_925mb |
| TLM | None | None |

Figure A.8. Smooth functions for the small extended GAM (gam03_extended) fit to HGB MDA8 O3 data.

Figure A.9. Year-to-year deviations from 2005 for the small extended GAM (gam03_extended) fit to HGB MDA8 O3 data.

Figure A.10. GAM evaluation plots for the small extended GAM (gam03_extended) fit to HGB MDA8 O3 data.

Figure A.11. Smooth functions for the small extended GAM (gam03_extended) fit to HGB daily average PM2.5 data.

Figure A.12. Year-to-year deviations from 2005 for the small extended GAM (gam03_extended) fit to HGB daily average PM2.5 data.

Figure A.13. GAM evaluation plots for the small extended GAM (gam03_extended) fit to HGB daily average PM2.5 data.

### Cross-Validation Analysis

In order to test for over-fitting in our GAMs, as well as to test the robustness of our results for the functional relationships between the meteorological predictors and O3 and PM2.5, we performed a two-fold cross-validation experiment for each GAM. To do this, the original dataset was randomly separated into two halves (data sets 1 and 2). We then fit two GAMs (hereafter m1 and m2) using the two halves of the data. The performance of these GAMs on the half of the data they were not trained on was then compared to the performance of the corresponding GAM that was fit on all the data (hereafter mtot).

Figure A.14 shows scatterplots of the GAM-predicted (x-axis) versus the measured (y-axis) values of maximum daily average PM2.5 for the Houston/Galveston/Brazoria area using gam03_extended. We can see that the performance of m1 and m2 on their respective test data sets is similar to the performance of the original GAM mtot. This can also be seen in Table A.12, which shows the root-mean-square (RMS) differences between the GAM-predicted and measured O3 and PM2.5 values for gam03_extended. The change in the RMS between mtot and m1 and m2 is generally small (< 1 ppbv for O3 and < 0.25 µg m-3 for PM2.5). As the training set and testing set RMS errors are thus similar, we conclude there is little evidence of overfitting in our GAMs.

However, the individual functional forms relating the meteorological and date predictors to O3 and PM2.5 can occasionally be significantly different between mtot, m1, and m2, suggesting that these relationships, although statistically significant, may not be robust or scientifically meaningful. For example, Figure A.15 shows the Houston fits for maximum daily average PM2.5 versus HYSPLIT back-trajectory bearing for mtot (black with error bars), m1 (red), and m2 (blue) for gam03_extended. Predicted values for 200 randomly selected data points are plotted. We see m2 significantly differs from mtot between 0o and 100o, suggesting the functional form from mtot may not be robust in this region. Plots similar to Figure A.15 for all GAMs and their terms are contained in the deliverable, as described in Section A.2.6. Other “suspicious” functional forms for PM2.5 and O3 in the gam03_extended fits are listed in Table A.13, but we note that as these are for a single random division of the dataset, these results merely indicate a potential problem, but do not by themselves prove that the functional relationships are incorrect.

Figure A.14. Scatterplots for the GAM-predicted (x-axis) versus the measured (y-axis) values of maximum daily average PM2.5 for the Houston/Galveston/Brazoria area using gam03_extended. The top row uses mtot to predict the first (left) and second (right) of the randomly distributed halves of the dataset. The bottom row uses m2, which was trained on data set 2, to predict the “test” data set 1 (left) and uses m1 to predict data set 2 (right). The black line is a linear fit of the predicted to actual values, while the red dashed line is the 1:1 line.

Figure A.15. Houston/Galveston/Brazoria fits for maximum daily average PM2.5 versus HYSPLIT back trajectory bearing for mtot (black with error bars), m1 (red) and m2 (blue) for gam03_extended. Predicted values for 200 randomly selected datapoints are plotted.

Table A.12. Cross-validation root-mean-square (RMS) results for gam03_extended.

| Urban Area | MDA8 O3 (ppbv) |  |  |  | Daily Average PM2.5 (µg m-3) |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Data Set 1 |  | Data Set 2 |  | Data Set 1 |  | Data Set 2 |  |
|  | mtot | m2 | mtot | m1 | mtot | m2 | mtot | m1 |
| DFW | 7.79 | 8.27 | 8.13 | 8.56 | 3.95 | 4.07 | 3.90 | 4.03 |
| HGB | 9.09 | 10.07 | 9.70 | 10.53 | 4.08 | 4.26 | 4.15 | 4.27 |
| SA | 7.37 | 7.94 | 7.20 | 7.76 | 3.77 | 3.94 | 3.95 | 4.07 |
| ARR | 7.04 | 7.67 | 7.23 | 7.72 | 3.79 | 3.93 | 3.79 | 3.89 |
| BPA | 8.35 | 9.11 | 8.70 | 9.21 | 4.80 | 5.02 | 4.71 | 4.93 |
| TLM | 7.80 | 8.14 | 7.46 | 7.76 | 4.45 | 4.56 | 3.41 | 3.55 |

Table A.13. “Suspicious” fits that show significantly different functional forms between mtot, m1, and m2 for gam03_extended.

| Urban Area | MDA8 O3 | Daily Average PM2.5 |
| --- | --- | --- |
| DFW | HYSPLIT.bearing..degrees.from.N., diurnal_T | NCDC.Avg.RH |
| HGB | T_diff_850mb | HYSPLIT.bearing..degrees.from.N., SolarRadiation.Langy.min |
| SA | None | None |
| ARR | None | None |
| BPA | None | HYSPLIT.bearing..degrees.from.N., NCDC.Avg.RH |
| TLM | None | HYSPLIT_DIST..m., T_diff_850mb |

## File Descriptions

This section describes all of the files included in the deliverable. Figure A.16 is a flow chart showing the processing from the initial data sources to the final CSV file used as input for the GAM fitting. These files are described in Sections A.2.3.1 to A.2.4. Figure A.17 shows the scripts that use the CSV file produced at the end of Figure A.16 to produce and evaluate the GAMs. These scripts and the output files produced are described in Sections A.2.5 and A.2.6, respectively.

Figure A.16. Flow chart showing the processing from the original data sources (green boxes) to the final CSV file (red box) that is used as input for the GAM fitting scripts.

Figure A.17. Flow chart showing the processing from the input CSV file generated at the end of Figure A.16 (red box) to the GAM output files (light green box).

Note that all R scripts below were run using R version 3.1.1 (2014-07-10) and package mgcv v1.8-0 on an x86_64-redhat-linux-gnu (64-bit) platform (CentOS release 5.11 Final) with Dual-Core AMD Opteron™ Processor 2218 and 8 GB RAM per core. All python scripts were run using Python v3.4.3 and Ipython v3.1.0 on a MacBook Pro with a 3.1 Ghz Intel Core I7 processor and 16 GB of RAM running Mac OS X Yosemite Version 10.10.3. The HYSPLIT runs were performed using a K shell (ksh) script on a Linux cluster running SUSE Linux Enterprise Server v11 (x86_64) with 12 Intel® Xeon® CPUs (X5650 @ 2.67GHz) and 4 GB RAM per processor. The Microsoft Excel spreadsheets were made using Microsoft Excel for Mac v14.5.2. All scripts should run on any Linux or Mac OS X system with the correct versions of R, Python, and Microsoft Excel installed.

### Input data (./data/)

This directory contains the raw IGRA and NCDC data used in this project. The raw TEMIS monitor data provided by TCEQ is not included in the deliverable.

#### IGRA Data (./data/IGRA_data/)

The Integrated Global Radiosonde Archive (IGRA) provided upper atmosphere data used to derive some of the meteorological predictors. The sites selected are described in Table A.2, with the data files named #####.dat according to the ID number of the selected sites along with a readme.txt file that describes the data format and measurements.  The relevant measurements include the geopotential height, temperature and dewpoint depression at several altitudes with -8888 values indicating original value has been removed by IGRA and -9999 was never present.

#### NCDC data (./data/NCDC_data/)

This directory contains the National Climatic Data Center (NCDC) Integrated Surface Hourly (ISH) dataset used to get estimates of surface pressure and relative humidity, as this data was not generally available in the TCEQ dataset.  Each urban area has a directory within ./data/NCDC_data/ that contains the raw data (###dat.txt) and two station description files (###inv.txt and ###stn.txt). The data from the station description files is also in Table A.3. The raw data file contains daily data from 2005-2014.  Missing data is indicated by ***.

### Data Processing Scripts (./scripts/)

- ./scripts/calc_bkgrd_ozone.py : This script reads in the ozone monitor data provided by the TCEQ to calculate the maximum daily 8 hour average (MDA8) O3 for each urban area. After filtering out non-data the script derives the maximum and minimum MDA8 for all urban locations, as well as the minimum (background) MDA8 O3 value for all selected background sites according to the technique described in Section A.1.1 and A.2.1. The selected background sites are listed in Table A.14 as well as in the script itself.  The produced CSV files are input to the script ./scripts/merge_param_all_Camalier.py (described below), which will combine O3 daily values with the GLM parameters. The outputs of this script were previously supplied to TCEQ as Deliverable 3.1.

- ./scripts/calc_pm25.py : This script reads in the PM2.5 monitor data provided by the TCEQ to calculate the maximum and minimum daily PM2.5 concentrations for all urban locations, as well as the daily minimum (background) concentrations for the selected background sites according to the technique described in Section A.1.1. These background sites are listed in Table A.14 as well as in the script itself.  The produced CSV files are input to the script ./scripts/merge_param_all_Camalier.py (described below), which will combine PM2.5 daily values with the GLM parameters. The outputs of this script were previously supplied to TCEQ as Deliverable 3.1 (Appendix B. Technical Memo: Estimating Background O3 and PM2.5).

- ./scripts/calc_GLM_all.py :  This script reads in TCEQ monitor site and IGRA (upper atmosphere) measurements to derive daily GLM parameters described in the script itself.  It performs all the necessary conversions (ex. Fahrenheit to Celsius, mph to m s-1) and derivations (ex. wind direction u component, dewpoint to RH based on August-Roche-Magnus approximation), to compile the full list of daily meteorological predictors, except those from the NCDC (described below).  See the script for full details on all conversions and derivations.  It creates the intermediate files for each urban area located in ./csv_files/intermed_files/TCEQ_files/, and these output files are used in  ./scripts/merge_param_all_Camalier.py.

- ./scripts/calc_GLM_NCDC.py : This script reads in the NCDC data to derive daily meteorological predictors indicated as an NCDC parameter.  It performs all the necessary conversions (ex. Fahrenheit to Celsius) and derivations (ex. Apparent Temperature according to the National Digital Forecast Database).  See the script for full details on all conversions and derivations.  It creates the intermediate files for each urban area located in /csv_files/intermed_files/NCDC_files/, and these output files are used in ./scripts/merge_param_all_Camalier.py.

- ./scripts/merge_param_all_Camalier.py : This script reads in all intermediate files described above for each urban location.  This includes the daily maximum and background concentrations for O3 and PM2.5, as well as daily values for all meteorological predictors.  It aligns the date for all files, checks for missing data and replaces with ‘nan’ if there is no data.  It creates the final merged files that are located in /csv_files/final_files and are used in the GAM fitting scripts described in Section A.2.5.

Table A.14. AQS site numbers for the selected background sites for each urban area.

| Urban Area | Total # of Sites | # of Background Sites | AQS Site Numbers of Background Sites |
| --- | --- | --- | --- |
| DFW | 28 | 11 | 481210034, 481211032, 481215008, 481391044, 482210001, 482311006, 482510003, 482570005, 483491051, 483670081, 484390075 |
| HGB | 69 | 31 | 480390618, 480390619, 480391003, 480391004, 480391016, 480710013, 481570696, 481670697, 481671034, 481675005, 482010029, 482010066, 482010552, 482010553, 482010554, 482010555, 482010556, 482010557, 482010558, 482010559, 482010560, 482010561, 482010563, 482010617, 482011042, 482011050, 482910699, 483390078, 483390698, 483395006, 483739991 |
| SA | 15 | 8 | 480290059, 480290501, 480290502, 480910503, 480910505, 481870504, 481870506, 481875004 |
| ARR | 12 | 8 | 482090675, 480210684, 481490001, 482090614, 482091675, 484530020, 484910690, 484916602 |
| BPA | 17 | 5 | 482450022, 482450101, 482450628, 483611001, 483611100 |
| TLM | 4 | 4 | 484230007, 481830001, 482030002, 480370004 |

### HYSPLIT

#### HYSPLIT run script (./HYSPLIT_runs_out/)

- ./HYSPLIT_runs_out/multitraj.sh  : A K shell script that runs the 24-hour HYSPLIT 4 back-trajectories for each urban region for the 2005-2014 period described in Section A.1.2. The script consists of multiple nested loops over inner to outer city, day, month and year. Each time through the loop the city, day, month, and year information is written to the CONTROL text file that is input to HYSPLIT and the HYSPLIT run is executed. Upon run completion the trajectory endpoint is extracted from the trajectory output file, tdump, and appended to the appropriate tdump_city CSV file.

#### HYSPLIT back trajectory endpoints (./HYSPLIT_runs_out/)

- ./HYSPLIT_runs_out/tdump_* : One of six intermediate CVS files generated from the ./HYSPLIT_runs_out/multitraj.sh script, one for each urban area of interest. * is a 3-letter code indicating the urban area. The first line in each file lists the 3-letter city code and the latitude and longitude of the trajectory origin.  The starting back trajectory elevation is always 300 m above ground level (agl) and not included in these files.  The rest of the lines are the endpoint time and location data, one line per endpoint.  The lines include the following:

  - Trajectory run - will always be 1 in this application, ignore

  - Trajectory number – will always be 1 in this applications, ignore

  - YEAR – 2-digit format

  - Month

  - Day

  - Hour – always 18 UTC

  - Minute – always 0

  - Second –always 0

  - Trajectory age – always -24 (indicating a 24 hour back trajectory)

  - Latitude

  - Longitude- west is negative

  - Elevation-  meters AGL

  - Pressure – hPa

#### HYSPLIT distance and bearing calculation script and output (./hysplit_trajec/)

- ./hysplit_trajec/calc_trajec.src : This R script takes the 24 hour back-trajectory endpoint files from the ./HYSPLIT_runs_out/ directory and calculates the distance and bearing from the starting point to the end point of the trajectory using the R functions bearing and distMeeus from the geosphere package as described in Section A.1.2.

- ./hysplit_trajec/trajec-info-*.csv : CSV file produced by ./hysplit_trajec/calc_trajec.src that contains the distance and bearing for the back trajectories. A separate file exists for each urban area. These files are used as inputs by ./scripts/merge_param_all_Camalier.py (Section A.2.2).

### Processed Input Data Files in CSV Format (./csv_files/)

#### Intermediate CSV Files (./csv_files/NCDC_files/ and ./csv_files/TCEQ_files/)

These files include the meteorological predictors derived from the NCDC, TCEQ and IGRA datasets described in Section 3.1 using the scripts described in Section A.2.2 (./scripts/calc_GLM_NCDC.py and ./scripts/calc_GLM_all.py respectively).  They contain daily GLM values for all urban locations from 2005-2014 and are used as input by ./scripts/merge_param_all_Camalier.py.

#### Final CSV Files (./csv_files/final_files/)

These files are created by ./scripts/merge_param_all_Camalier.py (Section A.2.2), which combines all daily meteorological predictors with the O3 and PM2.5 concentrations for each location.  The file includes daily values from 2005-2014, with missing values indicated by ‘nan’. These files are used as inputs by the GAM scripts described in Section A.2.5.

### GAM scripts (./full_gam_fits/)

#### Correlation Screening

- ./full_gam_fits/cor_test_mja.R : A log of R commands that shows how to read in the final CSV data files and assess a set of variables for correlation, as described in Section A.1.5.1. Note that this is NOT a script you can run as-is, it merely is a record of the necessary commands.

- ./full_gam_fits/cor_test_results_ozone.xlsx : A Microsoft Excel spreadsheet showing the families of variables tested in the initial correlation screening and the selected variables for ozone in each city.

- ./full_gam_fits/cor_test_results_pm2.5.xlsx : Same as above but for PM2.5.

#### GAM Fitting

- ./full_gam_fits/gam_fitting_automated_v2.r : The main GAM fitting script. The options are described at the top of the script. It takes a CSV data file and arrays specifying types of modeled variables, fits a GAM model (as specified or finds the best fit by eliminating variables), and produces (see Section A.2.6):

  - A log of final model diagnostics: summary, gam.check, & table summarizing iterations (if find.best.fit is TRUE).  Log may optionally include model summaries for every model iteration (if verbose is TRUE and find.best.fit is TRUE)

  - gam.check plot

  - smooth variable function plots (if create.plots is TRUE)

  - R data object containing final model (mod) and associated variable arrays (factor.vars, linear.vars, cr.vars, and cc.vars).  This can be loaded and reused for plots or other diagnostics later in R.

- ./full_gam_fits/automate_gam_fitting.src : A driver script for ./full_gam_fits/gam_fitting_automated_v2.r that sets the necessary inputs.

#### Cross-Validation

- ./full_gam_fits/crossval_pm.R : An R script that performs a cross-validation check on our PM2.5 GAMs. It randomly divides the original dataset into two halves, then fits a GAM to each half separately. The performance of these GAMs on the half of the data they were not trained on is then compared to the performance of the corresponding GAM fit on all the data. The smooth functional fits for all three GAMs are also plotted to check for differences between the two halves. At the top of the script, change “city” and “model” to test the appropriate GAM.

- ./full_gam_fits/crossval_o3.R : Same as above, but for the O3 GAMS.

### GAM Output Files (./full_gam_fits/o3_model/ and ./full_gam_fits/pm2.5_model/)

The output directories ./full_gam_fits/o3_model/ and ./full_gam_fits/pm2.5_model/ both contain one subdirectory for each urban area (e.g., ./full_gam_fits/o3_model/Houston/). Each of these urban area subdirectories contains a subdirectory for each of the three GAMs contained in the deliverable, such as:

- ./full_gam_fits/o3_model/Houston/o3gam01_baseline/

- ./full_gam_fits/o3_model/Houston/o3gam02_extended/

- ./full_gam_fits/o3_model/Houston/o3gam03_extended/

The files contained in each of these model directories are described below, using the file names from ./full_gam_fits/o3_model/Houston/o3gam03_extended/ as an example :

- .RData_o3gam03_extended_Houston : An R data file containing the GAM as an element in the list ‘mod’ (e.g., for this case it the GAM can be accessed as mod[[‘o3gam03_extended’]]). The script ./full_gam_fits/crossval_pm.R shows an example of how to load the GAM object (L32-35) and rebuild the GAM formula (L37-45) using this data file.

- model_results_Houston_20150626.log : The log file for the GAM fit as produced by the script ./full_gam_fits/gam_fitting_automated_v2.r. The first line shows the input data file from ./csv_files/final_files/. The summary of the final selected GAM (after any automated dropping of variables) is in this file after the phrase “FINAL MODEL DIAGNOSTICS”. A table at the end of the file summarizes the variables that were tested and dropped by the automated selection procedure described in Section A.1.3.

- plot_o3gam03_extended_Houston_smoothfunc-noresid.png : A figure showing the smooth functional fits for the GAM, as in Figure A.2.

- plot_o3gam03_extended_Houston_smoothfunc.png : As above, but with the partial residuals overplotted.

- gam.check_o3gam03_extended_Houston.png : A figure showing the standard diagnostic plots for the GAM, as in Figure A.4.

- cross_val/ : A subdirectory containing the output of the cross-validation scripts ./full_gam_fits/crossval*.R. These files include:

  - .RData_o3gam03_extended_Houston_crossval : An R data file containing the original GAM fit (mtot) and the two fits to the randomly selected halves of the data (m1 and m2). The seed number (seed.num) used in the cross-validation script is also stored, as are the indices of the halves of the data used to fit m1 and m2 (ind1 and ind2) and the indices of the 200 randomly-selected data points used to make the cross-validation figures (ind3).

  - crossval_scatter_Houston_o3gam03_extended.png : Scatter plots of the predicted (x-axis) versus actual (y-axis) MDA8 O3 or daily average PM2.5 values, as in Figure A.14.

  - cross val_m1_Houston_o3gam03_extended.png : A figure showing the smooth functional fits for the GAM m1 fit to the data in ind1, similar to Figure A.2.

  - cross val_m2_Houston_o3gam03_extended.png : Same as above but for the GAM m2 fit to the data in ind2.

  - crossval_terms*.png : Plots of the smooth function predictions for 200 randomly selected data points (ind3), similar to Figure A.15. The files contain the column names of the variables used in the fit. The y-axis scale is the scale of the “linear predictor”, i.e. the deviation of the natural logarithm of the MDA8 O3 or the daily average PM2.5 in µg m-3 from its mean value. The black center bar is the mean value while the error bars are the 95% confidence intervals. The red and blue circles are the mean values from the two-fold cross-validation analysis of Section A.1.6.

## Quality Assurance Steps

In addition to the analyses described in Section A.1.3, other quality assurance checks were made. All scripts used in this project were inspected by team members different from the original author to ensure they were calculating properly, and any errors noted in early versions were fixed. In addition, if further analysis or feedback from TCEQ uncovers any errors in the provided files, we will correct those and provide TCEQ with corrected files as part of our Final Report.

The project Quality Assurance Project Plan (QAPP) listed several questions that needed to be addressed as part of the GAM evaluation, as well as several required pieces of model documentation. These are addressed below in Sections A.3.1 and A.3.2, respectively.

### Model Evaluation

The QAPP stated that the evaluation of the GAMs produced in this project would address the following questions:

  - Do the relationships between meteorological variables and O3 and PM2.5 described in the developed GAMs make physical sense given our conceptual models of O3 and PM2.5 emissions, chemistry, and transport?

As noted in Sections A.1.4.2 and A.1.5.2, the functional dependencies in the GAMs between the predictors related to temperature, RH, wind speed, vertical stability, and HYSPLIT bearing are all qualitatively consistent with our conceptual understanding of O3 and PM2.5 emissions, chemistry, and transport.

  - Are these relationships consistent with the scientific literature?

As noted in Section A.1.4.2, our GAMs for MDA8 O3 are consistent with those found for eastern US cities by Camalier et al. (2007).

  - Does the change in the relationships between urban areas make physical sense given our conceptual models of O3 and PM2.5 emissions, chemistry, and transport?

We find that the general trends of the relationships rarely change significantly between the urban areas. For O3, the major differences are that DFW, SA, and ARR show the O3 trend with afternoon temperature flattening out above 30 oC and that the impact of relative humidity is fairly weak in HGB. For PM2.5, the major differences are between the cities near the Gulf of Mexico (HGB and BPA) and the others, with the cities near the Gulf showing increasing PM2.5 at wind speed above 5 m/s and a minimum in PM2.5 at a HYSPLIT bearing of 120o instead of at 320o.

  - Are the HYSPLIT back-trajectories used in the model development reasonable? How sensitive are these trajectories to the initial location?

As noted in Section A.1.2, the HYSPLIT back-trajectories used in the model development appear reasonable and generally consistent with the surface wind speed and direction measured near the center of each urban area. The ensemble back-trajectory results suggest that our results are representative of the air masses entering each urban area, but that differences in distance of less than ~100 km and differences in bearing of less than ~20o are unlikely to be significant.

  - How well does the GAM reproduce the testing sets in the cross-validation evaluation?

As noted in Section A.1.6, the two-fold cross-validation showed that the GAMs fit to half of the data fit the other half of the data nearly as well as the GAMs fit to all of the data.

  - Does the cross-validation evaluation of the models show evidence of over-fitting?

As noted in Section A.1.6, there is no evidence of over-fitting in the overall MDA8 O3 and daily average PM2.5 predictions. However, the functional relationships between the meteorological predictors and O3 and PM2.5 are occasionally sensitive to which half of the dataset is used for the fit, and so caution must be used in interpreting these relationships.

  - Under what conditions are the GAMs expected to be valid? What conditions give exceptionally large residuals?

Strictly speaking, the GAMs are only expected to be valid during the periods for which they were fit, and when the data is taken from the sources and sites noted in this memo. Extrapolations to other times and monitoring locations may be problematic, and the GAMs ability in this regard has not been assessed in this project.

We have not yet identified any set of necessary or sufficient conditions that lead to large residuals in the GAMs. We will continue investigating this and provided updated results with our final report.

### Model Documentation

The QAPP listed several required parts for the model documentation. These are listed below along with where to find the corresponding documentation in this memo.

- The final model description, hardware and software requirements, including programming language, model portability, memory requirements, required hardware/software for application, and data standards for information storage and retrieval

- The final descriptions of the GAMs are given in Sections A.1.4 and A.1.5. The software versions and computers used to run the scripts supplied in the deliverable are documented in the beginning of Section A.2.

- The equations on which the model is based

- The main GAM equation is given in Section A.1.3. More details on the GAM fitting procedure can be found in Wood (2006).

- The underlying assumptions used in the model development

- The GAM development procedure and any underlying assumptions are discussed in Section A.1. Underlying assumptions of the mgcv R package used to perform the fits are discussed in Wood (2006).

- Flow charts of model inputs, processing, and outputs

- Figure A.16 and Figure A.17 contain flow charts showing the processing of data from the initial data sources through to the GAMs and their evaluation scripts.

- Descriptions of the software routines

- The scripts developed in this project are described in Sections A.2.2, A.2.3, and A.2.5.

- Data base description

- The non-TCEQ initial data and the processed intermediate data used to generate the GAMS is contained in the deliverable, as noted in Sections A.2.4. The sources of this data are described in Section A.1.1.

- A copy of the source code

- Copies of all scripts developed in this project are contained in the deliverable, as described in Sections A.2.2 and A.2.5.

- Explanation of error messages

- Error messages produced using the GAMs in R are described in the documentation of the mcgv package. Error messages in the R and Python scripts supplied in this project are self-explanatory and generally refer to errors in the specified inputs (i.e., missing input files, incorrect parameter settings).

- Parameter values and sources

- Parameter values used in the R and python scripts and the sources of those values are documented in the scripts themselves.

- Restrictions on model application, including assumptions, parameter values and sources, boundary and initial conditions, validation/calibration of the model, output and interpretation of model runs;

As noted above, the functional relationships between the meteorological predictors and O3 and PM2.5 are occasionally sensitive to which half of the dataset is used for the fit, and so caution must be used in interpreting these relationships.

- Limiting conditions on model applications, with details on where the model is or is not suited

- As noted above, the GAMs are only expected to be valid during the periods for which they were fit, and when the data is taken from the sources and sites noted in this memo. Extrapolations to other times and monitoring locations may be problematic, and the GAMs ability in this regard has not been assessed in this project.

- Actual input data (type and format) used

- The non-TCEQ initial data and the processed intermediate data used to generate the GAMS is contained in the deliverable, as noted in Section A.2.4. The sources of this data are described in Section 2.1.

- Overview of the immediate (non-manipulated or post-processed) results of the model runs (model application only)

- The original HYSPLIT back-trajectory model results are contained in the deliverable and described in Section A.2.3.2. The post-processed distance and bearing outputs are contained in the intermediate CSV files described in Section A.2.3.3 and in the final CSV files described in Section A.2.4.2.

- Output of model runs and interpretation

- Section A.2.6 describes the output files from our GAM fits and cross-validation analysis contained in the deliverable. These results are discussed and interpreted in Sections A.1.4, A.1.5, and A.1.6.

- User's guide (electronic or paper)

- This technical memo serves as the user’s guide for all the scripts in the deliverable as well as the GAMs provided therein.

- Instructions for preparing data files (model development only)

Input data files for the GAMs must be prepared in a way that matches the format of the final CSV files described in Section A.2.4.2. The units of the variables much match those given in Table A.4 (gam01_baseline), Table A.7 (gam02_extended), and Table A.8 (gam03_extended). The data processing scripts described in Section A.2.2 and contained in the deliverable can be used to prepare these files, but any comma-separated-value file with the necessary columns will work as well.

- Example problems complete with input and output

- The input and output of the scripts and GAMs developed in this project are contained in the deliverable and described in Section A.2. Section A.2.6 describes the output files from our GAM fits and cross-validation analysis contained in the deliverable, which can also be used as example problems.

- A report of the model calibration, validation, and evaluation (model development only).

The calibration of the GAMs, defined as “adjusting model parameters within physically defensible ranges until the resulting predictions give the best possible or desired degree of fit to the observed data,” was done as part of the GAM fitting procedure described in Section A.1.3. The verification of the GAMs was performed via the two-fold cross-validation described in Section A.1.6.

The evaluation of the HYSPLIT back-trajectories is described in Section A.1.2. The GAMs were evaluated as described in Sections A.1.6, as well as by addressing the quality assurance questions in Section A.3.1.

# Appendix B. Technical Memo: Estimating Background O3 and PM2.5

## Introduction

This appendix documents the files provided to TCEQ to complete Deliverable 3.1 of Work Order No. 582-15-54118-01. Section B.2 briefly outlines the technical approach used to prepare the files in the deliverable (provided via email to Erik Gribbin of TCEQ as a gzipped tar file: p1952_deliverable_3_1_R1_1.tar.gz) and Section B.3 describes the format of the files. Section B.4 briefly outlines the quality assurance steps that have been performed. Further details and analysis of the results will be included in the project Final Report.

## Technical Approach

As described in the Work Plan, our approach follows the TCEQ method described in Berlin et al. (2013). This method requires: the selection of background sites; the calculation of the maximum daily 8-hour average (MDA8) for ozone (O3) and the daily average of fine particulate matter (PM2.5) at each site; estimating a preliminary background value as the lowest of the valid values for the background sites; and then further investigations to ensure the values are appropriate background estimates. These steps are described in detail below.

### Selection of Background Sites

The initial data for our analysis was provided by Erik Gribbin of TCEQ, which consisted of hourly-average measurements of O3 and PM2.5 at several sampling sites surrounding the urban areas of interest. After consultation with TCEQ, we selected “background” monitor sites near the edge of each urban area. These background sites were chosen to be at a significant distance from major pollutant emission sources. The AQS site numbers for the selected background sites for each urban area are given in Table B.1. Note that for the ARR and TLM areas, most or all of the available urban sites are considered potential “background” sites due to the limited number of sampling sites available.

Table B.1. AQS site numbers for the selected background sites for each urban area.

| Urban Area | Total # of Sites | # of Background Sites | AQS Site Numbers of Background Sites |
| --- | --- | --- | --- |
| DFW | 28 | 11 | 481210034, 481211032, 481215008, 481391044, 482210001, 482311006, 482510003, 482570005, 483491051, 483670081, 484390075 |
| HGB | 69 | 31 | 480390618, 480390619, 480391003, 480391004, 480391016, 480710013, 481570696, 481670697, 481671034, 481675005, 482010029, 482010066, 482010552, 482010553, 482010554, 482010555, 482010556, 482010557, 482010558, 482010559, 482010560, 482010561, 482010563, 482010617, 482011042, 482011050, 482910699, 483390078, 483390698, 483395006, 483739991 |
| SA | 15 | 8 | 480290059, 480290501, 480290502, 480910503, 480910505, 481870504, 481870506, 481875004 |
| ARR | 12 | 8 | 482090675, 480210684, 481490001, 482090614, 482091675, 484530020, 484910690, 484916602 |
| BPA | 17 | 5 | 482450022, 482450101, 482450628, 483611001, 483611100 |
| TLM | 4 | 4 | 484230007, 481830001, 482030002, 480370004 |

To calculate the background MDA8 O3 for the State of Texas as a whole, we used two approaches, the first using data from TCEQ sites near the Texas border, and the second using data from sites in the US EPA Clean Air Status and Trends Network (CASTNET). The CASTNet sites used to calculate Texas background O3 are listed in Table 3; a csv file (CASTNet_site_info.csv) with the latitudes, longitudes, and elevations of the CASTNet sites is included in the deliverable.

To calculate the background daily average PM2.5 for the State of Texas, we used data from sites in the Interagency Monitoring of Protected Visual Environments (IMPROVE) network near the Texas border, as TCEQ sites near the Texas border rarely made PM2.5 measurements. The IMPROVE sites used to calculate Texas background O3 are listed in Table B.2; a csv file (IMPROVELocTable.csv) with the latitudes, longitudes, and elevations of the IMPROVE sites is included in the deliverable.

Table B.2. Sites used to calculate background O3 and PM2.5 for the State of Texas as a whole.

| Pollutant (Network) | # of Background Sites | IDs of Background Sites |
| --- | --- | --- |
| O3 (TCEQ) | 23 | 484790017, 484790016, 484790313, 482150043, 480610006, 482730314, 483550025, 482450101, 481675005, 480391003, 482030002, 480370004, 482311006, 483670081, 480650007, 480650004, 480650005, 481351014, 481350003, 481410058, 800060003, 481410029, 481410057 |
| O3 (CASTNet) | 10 | CHA467, PET427, MEV405, CHE185, CAD150, CVL151, SUM156, EVE419, PAL190, BBE401 |
| PM2.5 (IMPROVE) | 12 | BOAP1, SAAN1, WHIT1, GUMO1, SACR1, BIBE1, ELLI1, WIMO1, CACR1, SIKE1, HOUS1, BRET1 |

### Calculation of MDA8 O3 and daily average PM2.5 Values for Each Site

We developed a python script (calc_bkgrd_ozone.py) that calculated the MDA8 O3 (ppbv) for all of the monitoring sites in the six urban areas. The MDA8 for a site was calculated as follows:

- A running 8-hour average was calculated for each hour, averaged over that hour and the following seven hours. At least 6 hours in this 8-hour range had to have valid O3 measurements for the 8-hour average to be considered valid.

- The largest of each of the calculated 8-hour averages in a day was selected as the MDA8 for that day.

A similar script (calc_pm25.py) was used to calculate daily average PM2.5 values from the available hourly data. This average was calculated as follows:

- If more than one PM2.5 instrument was active for a site, the reported hourly values were averaged.

- A daily average PM2.5 value was then calculated for each site. At least 18 hours of that day had to have valid PM2.5 measurements for the daily average to be considered valid.

For the background values for the State of Texas as a whole, the MDA8 values for the TCEQ sites in Table B.2 were calculated as above. The CASTNet and IMPROVE data was already provided as appropriately averaged values. The scripts calc_TX_bkgrd_ozone.py and calc_TX_bkgrd_PM25.py were used to process the data.

### Estimating Preliminary Background Values

The lowest of the daily MDA8 O3 values in the background sites for each urban area were selected as our preliminary background estimates. In addition, the maximum and minimum MDA8 O3 values for all urban sites in the area were also calculated.

Similarly, the lowest of the daily average PM2.5 values in the background sites for each urban area were selected as our preliminary background estimates. In addition, the maximum and minimum daily average PM2.5 values for all urban sites in the area were also calculated.

For the State of Texas as a whole, we calculated separate background values for O3 from the TCEQ sites and the CASTNet sites. In both cases the minimum valid MDA8 value was used. The minimum valid IMPROVE PM2.5 value was used as the PM2.5 background estimate for Texas.

### Linear Regressions Test and Outlier Analysis

We investigated the preliminary background estimates for each urban area by performing a linear regression of the preliminary background values (x) versus the maximum values (y) of MDA8 O3 and daily average PM2.5 using the R software package (using scripts bckgd_fit_o3.R and bckgd_fit_pm.R). For example, Figure B.1 shows a scatterplot of the background MDA8 O3 value versus the maximum MDA8 O3 value for the HGB area (the other fit figures are included in the original appendix attached to this deliverable). The solid black line is the linear fit, and the dotted and dashed black lines are the upper and lower 95% (or 2σ) confidence intervals, respectively. In this example, 89 of the 1834 valid data points (4.9%) have maximum MDA8 O3 values that fall above the upper confidence internal of the linear fit, suggesting that these background estimates are lower than would be expected given the maximum values seen in the urban area. Table B.3 gives the number of such points for each urban area and pollutant. All such data points are identified in the csv files in a column called “high_flag”, with a value of TRUE meaning that day was above the upper 95% confidence interval for that day. Given the skewed distribution of both the background and maximum MDA8 O3 and daily average PM2.5, very few points were identified below the lower confidence interval of the fit (one MDA8 O3 value for Dallas, six PM2.5 values for Austin, and three PM2.5 values for Houston) and so these points are not flagged in the csv files.

In addition, for some days only one monitoring site within the urban area had a valid MDA8 O3 or daily average PM2.5 value, so that the maximum and preliminary background estimates were identical. These data points are identified in the csv files in a column called “eq_flag”, with a value of TRUE meaning that day only had a single site with valid data, and so the maximum and background estimates are equal (see also Section 3). For example, this is true of all background PM2.5 estimates for the Tyler-Longview-Marshall area (TLM), as there was only a single site with valid PM2.5 data (see Table B.3). While we have included these data points in our background estimates for completeness, we strongly recommend that users be careful about including these points in their analyses, as they may bias the results of, for example, the average difference between the maximum and background values.

Figure B.1. Maximum versus background MDA8 O3 values for the HGB area.

Table B.3. Number of background points quality flagged for each urban area and pollutant.

| Urban Area | MDA8 O3 |  |  |  | Daily Average PM2.5 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | # high | # eq | # final | # replaced | # high | # eq | # final | # replaced |
| DFW | 78 | 0 | 46 | 0 | 118 | 21 | 68 | 0 |
| HGB | 89 | 2 | 61 | 43 | 149 | 11 | 101 | 75 |
| SA | 67 | 117 | 37 | 0 | 129 | 490 | 0 | 0 |
| ARR | 72 | 217 | 20 | 0 | 172 | 100 | 0 | 0 |
| BPA | 78 | 14 | 65 | 49 | 112 | 226 | 0 | 0 |
| TLM | 93 | 30 | 0 | 0 | NA | 3509 | NA | NA |

Similar to Berlin et al. (2013), we performed further analysis of the points that were above the 95% confidence interval of the fit (e.g., where high_flag = TRUE). First, we identified the subset of those points where (a) where high_flag = TRUE AND (b) at least one other background site in the urban area had a valid MDA8 O3 or daily average PM2.5 value for that day AND (c) the valid values at the other background sites were all more than 10% larger than the preliminary background estimate. Note that the later two criteria have to be true for replacing the preliminary background estimate with a value from a different background site to make a significant impact on any subsequent analysis. Data points that met all three criteria are flagged in the csv files in a column called “final_flag”, with a value of TRUE meaning that the above criteria were satisfied. The number of points with final_flag = TRUE for each urban area is shown in Table B.3. For these points, we have included the AQS site number and the MDA8 O3 or daily average PM2.5 value for the background site with the second largest value in the csv files as an alternate background estimate.

However, we only replaced the preliminary background value if:

- The final_flag = TRUE

- The estimate was for the HGB or BPA areas, as these areas near the Gulf of Mexico could plausibly have times when the gulf/lake breeze front affects some of the outlying background sites, but does not affect the urban area as a whole.

- The preliminary background site was between the city and the Gulf of Mexico (or the city and Sabine Lake). These sites are given in Table B.4.

Table B.4. Background sites that were replaced if final_flag = TRUE

| Urban Area | AQS Site Numbers of Background Sites that could be replaced |
| --- | --- |
| HGB | 480391016, 480390618, 481671034, 480390619 |
| BPA | 482450628, 482450101 |

The total number of background sites where data was replaced with the second highest value is given in Table B.3. These replacements do not significantly impact the statistics of the background estimates.

For the State of Texas as a whole, we did not perform a similar sort of analysis, as there is difficulty in deciding what is the appropriate maximum value to use for the state as a whole. The preliminary background estimates are thus identical to the final background estimates.

## File Descriptions

The data contained in the csv files included in the deliverable are described below. All files are in comma-separated-value (csv) format unless otherwise stated.

### Urban Area Ozone Files (file name = *_flagged_O3_v3.csv, six files in total)

Column Descriptions:

- Date:  In YYYYMMDD format. Note only dates in the ozone season (May-October) will have valid values.

- AQS_Code_max: AQS site number of the site in the urban area with the maximum MDA8 ozone.

- O3_max..ppbv.: Maximum of the valid MDA8 ozone (ppbv) values for all sites in the urban area.

- AQS_Code_min_max: AQS site number of the site with the minimum valid MDA8 ozone (ppbv) for all sites in an urban area. Note this may not be equal to the background estimate (Columns 6 and 7), as that is the minimum for the background sites only.

- O3_min_max..ppbv.: Minimum of the valid MDA8 ozone (ppbv) values for all sites in an urban area. Note this may not be equal to the background estimate (Columns 6 and 7), as that is the minimum for the background sites only.

- AQS_Code_min:  AQS site number of the preliminary background estimate.

- O3_min_bkgrd..ppbv.: The preliminary background estimate, calculated as the minimum valid MDA8 ozone (ppbv) for the background sites in an urban area.

- high_flag: TRUE if this day was above the 95% confidence interval for a linear fit of the preliminary background MDA8 ozone value (x, Column 7) against the maximum MDA8 ozone value (y, Column 3). See Section B.2.4.

- eq_flag: TRUE if this day only had one valid MDA8 ozone value for the urban area, and so the preliminary background MDA8 ozone value (x, Column 7) and the maximum MDA8 ozone value (y, Column 3) are equal. We strongly recommend that users be careful about including the points flagged as TRUE in their analysis, as they may bias the results of, for example, the average difference between the maximum and background values. See Section B.2.4.

- final_flag: TRUE if (a) high_flag (Column 8) is TRUE, AND (b) at least one other background site in the urban area had a valid MDA8 ozone value for that day, AND (c) the valid values at the other background sites were all more than 10% larger than the preliminary background estimate. See Section B.2.4.

- X2nd.Highest.MDA8.AQS.Code: If final_flag = TRUE, this contains the AQS site number of the background site with the second lowest MDA8 value.

- X2nd.Highest.MDA8..ppbv.: If final_flag = TRUE, this contains the second lowest MDA8 value of the background sites.

- Final.MDA8.Background.AQS.Code: AQS site number of the final MDA8 background estimate (ppbv), with some HGB and BPA values replaced as described in Section 2.4.

- Final.MDA8.Background..ppbv.: final MDA8 background estimate (ppbv), with some HGB and BPA values replaced as described in B.2.4.

### Urban Area PM2.5 Files (file name = *_flagged_PM_v2.csv, six files in total)

Column Descriptions:

- Date:  In YYYYMMDD format.

- AQS_Code_max: AQS site number of the site in the urban area with the maximum daily average PM2.5 value.

- PM2.5_max..ug.m.3.): Maximum of the valid daily average PM2.5 values (µg m-3) for all sites in the urban area.

- AQS_Code_min_max: AQS site number of the site with the minimum valid daily average PM2.5 values (µg m-3) for all sites in an urban area. Note this may not be equal to the background estimate (Columns 6 and 7), as that is the minimum for the background sites only.

- PM2.5_min_max..ug.m.3: Minimum of the valid daily average PM2.5 values (µg m-3) for all sites in an urban area. Note this may not be equal to the background estimate (Columns 6 and 7), as that is the minimum for the background sites only

- AQS_Code_min: AQS site number of the preliminary background estimate.

- PM2.5_min_bkgrd..ug.m.3.: The preliminary background estimate, calculated as the minimum valid daily average PM2.5 values (µg m-3) for the background sites in an urban area.

- high_flag: TRUE if this day was above the 95% confidence interval for a linear fit of the preliminary background MDA8 ozone value (x, Column 7) against the maximum MDA8 ozone value (y, Column 3). See Section 2.4.

- eq_flag: TRUE if this day only had one valid MDA8 O3 value for the urban area, and so the preliminary background daily average PM2.5 values (µg m-3) (x, Column 7) and the maximum daily average PM2.5 values (µg m-3) (y, Column 3) are equal. We strongly recommend that users be careful about including the points flagged as TRUE in their analysis, as they may bias the results of, for example, the average difference between the maximum and background values. See B.2.4.

- final_flag: TRUE if (a) high_flag (Column 8) is TRUE, AND (b) at least one other background site in the urban area had a valid daily average PM2.5 value for that day, AND (c) the valid values at the other background sites were all more than 10% larger than the preliminary background estimate.

- X2nd.Highest.PM2.5.AQS.Code: If final_flag = TRUE, this contains the AQS site number of the background site with the second lowest daily average PM2.5 value.

- X2nd.Highest.PM2.5..ug.m.3.: If final_flag = TRUE, this contains the AQS site number of the background site with the second lowest daily average PM2.5 value.

- Final.PM2.5.Background.AQS.Code: AQS site number of the final daily average PM2.5 background estimate (µg m-3), with some HGB and BPA values replaced as described in B.2.4.

- Final.PM2.5.Background..ug.m.3.: Final daily average PM2.5 background estimate (µg m-3), with some HGB and BPA values replaced as described in B.2.4.

### Texas O3 Background (file name = TX_State_bkgrd_O3_calc.csv)

Column Descriptions:

- Date: In YYYYMMDD format. Note only dates in the ozone season (May-October) will have valid values.

- AQS_Code_min: AQS site number of the preliminary background estimate using TCEQ sites near the Texas border.

- O3_min(bkgrd): The preliminary background estimate, calculated as the minimum valid MDA8 ozone (ppbv) for the TCEQ sites near the Texas border.

- CASTNet ID: CASTNet ID code for the CASTNet site near the Texas border with the lowest MDA8 O3 value for the day.

- CASTNet O3_min (bkgrd): The CASTNet-based background estimate, calculated as the minimum valid MDA8 ozone (ppbv) for the CASTNet sites near the Texas border.

### Texas PM2.5 Background (file name = TX_State_PM_calc.csv)

Column Descriptions:

- Date: In YYYYMMDD format. Note that as these estimates are based on IMPROVE network data, data is only available one out of every 3 days.

- IMPROVE ID: IMPROVE ID code of the IMPROVE site near the Texas border with the lowest daily average PM2.5 value.

- PM_min(bkgrd, ug/m^3): The background estimate for the State of Texas, calculated as the minimum valid daily average PM2.5 values (µg m-3) for the background sites in an urban area.

## Quality Assurance Steps

In addition to the analyses described in B.2.4, other quality assurance checks were made. First, all scripts used in this project were independently inspected to ensure they were calculating properly, and any errors noted in early versions were fixed. Second, the statistics of the background and maximum values for each urban area were investigated to ensure that they were reasonable and did not change unexpectedly between file versions.

# Appendix C: File Descriptions in Final Deliverable Package

All associated data and scripts for this project are contained in the deliverable packages, which can be downloaded from the AER ftp server at:

ftp://ftp.aer.com/pub/malvarad/TCEQ/p1952_deliverable_2_2_R1_0.tar.gz

ftp://ftp.aer.com/pub/malvarad/TCEQ/p1952_deliverable_3_1_R1_1.tar.gz

ftp://ftp.aer.com/pub/malvarad/TCEQ/p1952_deliverable_5_2_R1_0.tar.gz

The files contained in the packages for Deliverables 2.2 and 3.1 are documented in Sections A.2 and B.3, respectively. Here we discuss the additional files included in the final Deliverable 5.2.

## ./P1952_trend_plots.xlsx

This is a Microsoft Excel file (made using Microsoft Excel for Mac 2011 v14.5.3) that was used to produce the meteorologically-adjusted annual averages and linear tends discussed in Section 2.4. The spreadsheet contains the data used to create the plots as well as the plots themselves.

## Subdirectory ./MAPTYPE/

This subdirectory contains the files used to perform the synoptic map type analyses and logistic regressions discussed in Section 4. The individual files are described below.

### Map Type Files

#### ./MAPTYPE/narr_maptype_2005_2014_850_70_5types.dat

This is an ASCII text file that contains two columns, the date (MMDDYYYY, with no zeros as spacers) and the determined synoptic type for that date. The types are described in Section 4.1.1.

#### ./MAPTYPE/tceq_map_type.xlsx

This is a Microsoft Excel file (made using Microsoft Excel for Mac 2011 v14.5.3) that was used to convert the data in ./MAPTYPE/narr_maptype_2005_2014_850_70_5types.dat into a comma-separated-value (CSV) text file with a date column that matches that in the CSV input files for the GAM fitting described in Section A.2.4.2.

#### ./MAPTYPE/tceq_map_type.csv

A CSV file produced from ./MAPTYPE/tceq_map_type.csv that is used by ./MAPTYPE/syn_type_boxplot.R to merge the synoptic type data with the CSV input files for the GAM fitting described in Section A.2.4.2. The columns are Month, Day, Year, Syn.Type (Synoptic Type), and Date (in YYYYMMDD format).

### R Scripts

#### ./MAPTYPE/syn_type boxplot.R

This script reads in ./MAPTYPE/tceq_map_type.csv and the CSV input files for the GAM fitting described in Section A.2.4.2 and then produces box plots of how total and background O3 and PM2.5 vary between the synoptic types. It calculates the mean and standard deviation of O3 and PM2.5 for each type, as well as the percentage of days in each type above a fixed threshold (70 ppb for total MDA8 O3, 55 ppb for background MDA8 O3, 17 ug/m3 for total daily PM2.5 and 13 ug/m3 for background daily PM2.5). It also fits the log of the concentrations to a linear model of the synoptic types.

Text outputs are written to the log file ./MAPTYPE/syn_type_boxplot.log, while the box plots for each city are saved to the files ./MAPTYPE/syn_type_boxplot*.png, where the * is the city name.

Finally, the code produces updated GAM data files (./MAPTYPE/*_merged_GLM_all_type_exceed.csv) with additional columns that identify the synoptic types and the days that had values above the fixed threshold. These files are used as input by the script ./MAPTYPE/logistic_regress.R.

#### ./MAPTYPE/logistic_regress.R

This script reads in the data files produced by ./MAPTYPE/syn_type_boxplot.R (./MAPTYPE/*_merged_GLM_all_type_exceed.csv) and performs a logistic regression to determine how the probability of O3 and PM2.5 exceeding certain thresholds varies with meteorology, as described in Section 4.2.1. The output files are stored in separate subdirectories for each city (e.g., ./MAPTYPE/Houston/). These include:

- A text log file (e.g., ./MAPTYPE/Houston/logistic_regress_Houston.log)

- Probability plots for each pollutant metric (e.g., ./MAPTYPE/Houston/log_regress_plot_Houston_o3.max.exceed.png is the same as Figure 23)

- An R data file (e.g., ./MAPTYPE/Houston/.RData_logistic_gam_Houston) that contains the fitted model objects for each pollutant metric.

### Updated GAM data files (./MAPTYPE/*_merged_GLM_all_type_exceed.csv)

These are CSV files created by ./MAPTYPE/syn_type boxplot.R and used as input by ./MAPTYPE/logistic_regress.R. They are identical to the files described in Section A.2.4.2 except for the addition of the following columns

- Month, Day, Year, and Syn.Type, following the format described in Section C.2.1.3.

- o3.max.exceed, o3.bg.exceed, pm.max.exceed, pm.bg.exceed: These are logical arrays that describe if the given pollutant metric (total MDA8 O3, background MDA8 O3, total daily average PM2.5, background daily average PM2.5, respectively) is equal to or greater than (TRUE) or less than (FALSE) the thresholds described in Section 4.1 (70 ppb for total MDA8 O3, 55 ppb for background MDA8 O3, 17 ug/m3 for total daily PM2.5 and 13 ug/m3 for background daily PM2.5).

### R output files for logistic regression (./MAPTYPE/*/RData_logistic_gam*)

This is an R data file that contains the following R variables

- mod: A list containing the four GAM model objects made by the logistic regression of each of the logical arrays described in Section C.2.3

- modeled.vars: An array of the names of the four logical arrays used for the logistic regression. These are also the names of the four model objects in mod.

- City: The city name.

- Temps: The array of afternoon mean temperatures (oC) used to produce the output probability plots.

- Winds: The array of daily average wind speeds (m/s) used to produce the output probability plots.

- Types: The array of synoptic types (see Section 4.1) used to produce the output probability plots.

## Subdirectory ./PCA/SCRIPTS

This directory contains the following python scripts used to perform the PCA analysis of O3 and PM2.5 in each urban area and use the results to calculate background estimates.

- /calc_PCA_bkgrd_ozone.py: This script takes the raw TCEQ measurement data for all sites.  It then calculates the MDA8 ozone for all sites and filters out those that have less than 75 % of the data for the ozone season during the 10 year time-span. It creates the .csv files ready to be spatially interpolated in interp_PCA_bkgrd_ozone.py

- /interp_PCA_bkgrd_ozone.py: This file takes the MDA8 ozone file created in calc_PCA_bkgrd_ozone.py and spatially interpolates any missing datapoints by lat and lon.  It creates the .csv files that are to be used for the PCA analysis in R  pca_script.R

- /compare_bkgrdO3.py: This script reads in the final PCA-predicted background O3 values and plots the results compared to the original TCEQ-method predicted background O3 and calculates the correlation statistics.

- /calc_PCA_PM2.5_bkgrd.py: This script takes the raw TCEQ measurement data for all sites.  It then calculates the daily average PM2.5 for all sites and filters out those that have less than 75 % of the data for the entire year during the 10 year time-span. It creates the .csv files ready to be spatially interpolated in interp_PCA_bkgrd_PM25.py

- /interp_PCA_bkgrd_PM25.py:  This file takes the PM2.5 file created in calc_PCA_bkgrd_ozone.py and spatially interpolates any missing data points by lat and lon.  It creates the .csv files that are to be used for the PCA analysis in R: pca_script.R

- /compare_bkgrdPM.py: This script reads in the final PCA-predicted background PM2.5 values and plots the results compared to the original TCEQ-method predicted background PM2.5  and calculates the correlation statistics.

- /pca_script.R:  this script follows the Eigenvector calculation to do a PCA analysis on the data sets created in interp_PCA_bkgrd_ozone.py and interp_PCA_bkgrd_PM.py  it then outputs the completed background calculation to the txt files to be used in compare_bkgrdO3.py and compare_bkgrdPM.py.

# Appendix D: Logistic Regression Probability Plots for DFW, SA, and ARR

Figure D.1. Probability of the total MDA8 O3 exceeding 70 ppbv for the Dallas/Fort Worth urban area as a function of afternoon mean temperature (oC), daily wind speed (m/s), and synoptic type (as defined in Section 4.1).

Figure D.2. As in Figure D.1, but for the probability of background MDA8 O3 exceeding 55 ppbv.

Figure D.3. As in Figure D.1, but for the probability of total daily average PM2.5 exceeding 17 g/m3.

Figure D.4. As in Figure D.1, but for the probability of background daily average PM2.5 exceeding 13 g/m3.

Figure D.5. As in Figure D.1 but for the San Antonio urban area.

Figure D.6. As in Figure D.2 but for the San Antonio urban area.

Figure D.7. As in Figure D.3 but for the San Antonio urban area.

Figure D.8. As in Figure D.4 but for the San Antonio urban area.

Figure D.9. As in Figure D.1 but for the Austin/Round Rock urban area.

Figure D.10. As in Figure D.2 but for the Austin/Round Rock urban area.

Figure D.11. As in Figure D.3 but for the Austin/Round Rock urban area.

Figure D.12. As in Figure D.4 but for the Austin/Round Rock urban area.

U.S. Environmental Protection Agency Clean Air Markets Division, Clean Air Status and and Trends Network (CASTNET), Table OZONE_8HR_DMAX, last updated 2015-04-06. Available at www.epa.gov/castnet. Accessed 2015-04-09.

Interagency Monitoring of Protected Visual Environments (IMPROVE) network, Table EPA PM2.5 Mass FRM – Daily, Available at http://vista.cira.colostate.edu/improve/Data/IMPROVE/improve_data.htm. Accessed 2015-04-09.

All listed scripts will be supplied to TCEQ with the project’s Final Report.