Southeastern Modeling, Analysis and Planning Project

WRF Meteorological Modeling Performance Evaluation and Documentation

Prepared for:

Southeastern States Air Resource Managers, Inc.

526 Forest Pkwy Ste F,

Forest Park GA  30297-6140

Prepared by:

Atmospheric and Environmental Research, Inc.

131 Hartwell Ave,

Lexington, MA 02421

Version 2.0

2 December 2011

Table of Contents

1	Introduction	3

2	WRF model and Observational dataset descriptions	3

2.1	Model configuration and verification methodology	3

2.2	Modeling domains	5

2.2.1	Horizontal Grid	5

2.2.2	Vertical Layers	7

2.3	Data Nudging	8

2.3.1	Objective Analysis	9

2.3.2	Four-Dimensional Data Assimilation	9

2.3.3	Nudging Coefficients	9

2.4	Period of analysis	10

2.4.1	Production Run	10

2.4.2	Sensitivity Runs	10

2.5	Observational dataset	11

2.6	Meteorological variables and assessment measures	12

3	Evaluation techniques	12

3.1	Subjective	12

3.2	Objective	13

3.3	Spatial Averaging	14

4	Inspection of model configuration	15

5	Evaluation of Annual Production Run Meteorological Model Performance	16

5.1	Statistical analysis of WRF full-year production run	16

5.1.1	Task 2 A: Time series of spatial averages over all regions	16

5.1.2	Task 2 B: Monthly Bakergrams	28

5.1.3	Task 2 C: Bubble spatial plots of error statistics	40

5.1.4	Task 2 E: Time series at individual sites of forecast/observed values	47

5.1.5	Task 2 G: Scatter plots	63

5.1.6	Tasks 2 I and 5 C: Line plots of error statistics	85

5.1.7	Task 5 B 4: Tables of Annual Statistics	94

6	Evaluation of Sensitivity Runs Meteorological Model Performance	106

6.1	Statistical analysis of WRF sensitivity runs	106

6.1.1	Tasks 2 H 1 and 2 H 7 Statistical tables for FULL model domain	107

6.1.2	Tasks 2 H 1 and 2 H 7 Surface Statistical tables for SEMAP region	123

6.1.3	Task 2 H 2: Bakergrams	128

6.1.4	Task 2 H 4: Mean bias plots	140

6.1.5	Task 2 H 5: Wind speed plots	147

6.1.6	Task 2 H 6: Bubble spatial plots of error statistics	150

7	Meteorological modeling website and other technical aspects	155

7.1	Website development and documentation	155

7.2	Supplemental Dataset	155

8	Summary and conclusions	155

9	Appendix A	157

10	Appendix B	201

11	References	203

# Introduction

The purpose of this Contract is for meteorological model performance evaluation and documentation services in support of modeling analyses, control strategy assessments, and other air quality management needs.

Technical air quality analysis projects generally include compilation and analysis of existing ambient air quality monitoring data, collection of additional monitoring data as needed, assessment of air quality trends, preparation of emission inventories, development of emissions control scenarios, modeling of meteorology, emissions, and air quality, and completion of reports revealing the impact of emissions and the air quality benefits of varying emission control levels.  This meteorological modeling performance project will support this larger group of tasks.

SESARM intends to concurrently address, to the extent feasible, national ambient air quality standard (NAAQS) requirements and to evaluate progress towards long-term regional haze goals. Because similar pollutant emissions and atmospheric processes control chemical formation and transport for fine particles, ozone, and regional haze, similar technical analyses are necessary to evaluate air quality benefits of emissions controls.  SESARM will develop and operate a single integrated, one-atmosphere air quality modeling platform to provide the states with technical analyses that support the required air quality demonstrations.

Prior to this contract, an annual WRF model simulation was performed for 2007. The project supported by this Contract provides a detailed, quantitative and qualitative analysis of these simulated meteorological fields and their suitability for application to air quality modeling.

# WRF model and Observational dataset descriptions

## Model configuration and verification methodology

The WRF v. 3.1 mesoscale meteorology model is a fully compressible, non-hydrostatic model with an Eulerian mass dynamical core [Skamarock et al. 2008]. Two dynamical cores are available:  the Advanced Research WRF (ARW) core, developed and supported by NCAR, and the Nonhydrostatic Mesoscale Model (NMM) core, whose development is centered at NCEP’s Environmental Modeling Center (EMC) and support is provided by NCAR’s Development Testbed Center (DTC). The WRF-NMM is designed to be a real-time forecast model so physics schemes are preset. WRF-ARW (hereinafter WRF) was chosen because it includes multiple physics options for turbulence/diffusion, radiation (long and shortwave), land surface, surface layer, planetary boundary layer, cumulus, and microphysics.

Time integration is performed with a 3rd-order Runge-Kutta scheme. WRF has split time integration, which uses smaller time steps used for fast processes like sound waves or gravity waves. WRF was designed to conserve mass, momentum, entropy, and scalars using flux form prognostic equations.

WRF uses Eta (η) as a vertical coordinate, which is defined as the hydrostatic pressure difference from the layer to the top divided by the difference in the entire vertical domain so it is always between 1 (surface) and 0 (top). This vertical coordinate is terrain following hydrostatic pressure system.

The grid format is Arakawa- C with all variables in the center of the grid except for wind velocity, which is defined on the edges of the grid, and shared between adjacent grids.

The WRF core uses a 3rd-order Runge-Kutta split-explicit time integration scheme, high-order advection scheme, and is scalar conserving.  It contains complete Coriolis, curvature, and mapping terms.  Domain nesting is possible with one-way and two-way capabilities.  Full physics options are available to represent atmospheric radiation, surface and boundary layers, and cloud and precipitation processes.  The model contains gridded analysis and observational nudging four dimensional data assimilation (FDDA) capabilities that allow users to perform enhanced retrospective analyses.

The WRF modeling system is divided into two main components: the WRF Preprocessing System (WPS) and the WRF modeling core.  WPS contains three processors that define the WRF modeling domain, generate map, elevation/terrain, and land-use data, and generate horizontally interpolated input meteorological fields to the WRF grid.  The WRF modeling core interpolates the meteorological fields processed by WPS to the WRF vertical levels and generates initial and lateral boundary conditions.  The model solver integrates the atmospheric equations and interfaces with the physics parameterizations to generate forecasts of meteorological variables.

WRF uses physics sub-models to simulate land surface, surface layer, and boundary layer dynamics, along with cumulus convection, microphysics, and radiation. For each of these sub-models, WRF offers from 3 to 9 different parameterizations from which the modeler may select. To make these choices, the modeler must consider not only the individual merits of each parameterization but also how it interacts with the other physics sub-model options.  The following flow chart illustrated the interaction of the WRF physics sub-modules.

WRF version 3.1.1 (July 2009) was used for the sensitivity tests and the annual simulations. The main update to the model in this release was the addition of objective analysis (OBSGRID) and surface analysis nudging capabilities, an option preferred for generating meteorological data for input to air quality simulations

## Modeling domains

The domain used by the SEMAP has been expanded from the domain used for the 2002 State Implementation Plan modeling. This was done to coordinate with the other members of the WRF Work Group, which including LADCO, SESARM, and the State of Iowa.

### Horizontal Grid

The computational domain for this simulation consists of a coarse and fine grid nested domains.  The coarse domain consists of a Lambert conic conformal projection with specifications used in previous regional modeling studies, which is centered at 40° N and 97° W with true latitudes of 33 and 45° N.  The coarse grid structure consists of an array of 165 cells in the east-west direction and 129 cells in the north-south direction with a grid spacing of 36 kilometers (km).  This domain was designed maximize the usage of the Eta analysis region.

A nested domain is implemented to resolve finer scale meteorological features. The nested domain grid structure is a 250 by 250 array of grid cells with a grid spacing of 12 km.  This domain begins at node (66, 18) of the course domain.  The nested domain shares the northern, eastern, and southern boundaries of the EPA 12 km Eastern domain and western boundary of the Iowa/LADCO 12 km domain. A summary of the horizontal structure of the modeling domain is given in Table below.

| Domains | Outer (1) | Inner (2) |
| --- | --- | --- |
| Resolution (km) | 36 | 12 |
| Starting Location (i,j) | 1,1 | 66,18 |
| nx (E-W) | 165 | 250 |
| ny (N-S) | 129 | 250 |
| SW Coordinate (km) | -2952, -2304 | -612, -1692 |
| NE Coordinate (km) | 2952, 2304 | 2376, 1296 |

Using 12 km horizontal resolution is sufficiently high to simulate mesoscale features and is also in range to use both WRF microphysics and cumulus convection schemes.

### Vertical Layers

The vertical grid structure consists of 35 layers (34 layers) as shown below.

Each dot signifies one level. The height of the level is shown on the y-axis and the mass between layers is shown on the x-axis. The number of levels is more concentrated below 1 km.

This vertical structure was designed to provide increased resolution in the boundary layer and near the tropopause.  The second level is placed at ~20 meters so that the midpoint of the grid cells in the vertical direction is located at 10 meters and corresponds to the standard National Weather Service (NWS) anemometer height.  This avoids the use of a surface layer parameterization to vertically interpolate horizontal wind components to the standard NWS observational height. The top of the model is fixed at 50 mb.  A summary of the vertical structure of the modeling domain is given below.

| Level | Sigma | Height (m) | Pressure (mb) | Depth (m) |
| --- | --- | --- | --- | --- |
| 35 | 0.0000 | 18663 | 50 | 2034 |
| 34 | 0.0332 | 16629 | 82 | 1715 |
| 33 | 0.0682 | 14914 | 115 | 1515 |
| 32 | 0.1056 | 13399 | 150 | 1375 |
| 31 | 0.1465 | 12024 | 189 | 1255 |
| 30 | 0.1907 | 10769 | 231 | 1145 |
| 29 | 0.2378 | 9624 | 276 | 1045 |
| 28 | 0.2871 | 8579 | 323 | 955 |
| 27 | 0.3379 | 7624 | 371 | 870 |
| 26 | 0.3895 | 6754 | 420 | 790 |
| 25 | 0.4409 | 5964 | 469 | 715 |
| 24 | 0.4915 | 5249 | 517 | 645 |
| 23 | 0.5406 | 4604 | 564 | 580 |
| 22 | 0.5876 | 4024 | 608 | 520 |
| 21 | 0.6323 | 3504 | 651 | 465 |
| 20 | 0.6742 | 3039 | 690 | 415 |
| 19 | 0.7133 | 2624 | 728 | 370 |
| 18 | 0.7494 | 2254 | 762 | 330 |
| 17 | 0.7828 | 1924 | 794 | 293 |
| 16 | 0.8133 | 1631 | 823 | 259 |
| 15 | 0.8410 | 1372 | 849 | 228 |
| 14 | 0.8659 | 1144 | 873 | 200 |
| 13 | 0.8882 | 944 | 894 | 174 |
| 12 | 0.9079 | 770 | 913 | 150 |
| 11 | 0.9252 | 620 | 929 | 128 |
| 10 | 0.9401 | 492 | 943 | 108 |
| 9 | 0.9528 | 384 | 955 | 90 |
| 8 | 0.9635 | 294 | 965 | 74 |
| 7 | 0.9723 | 220 | 974 | 60 |
| 6 | 0.9796 | 160 | 981 | 48 |
| 5 | 0.9854 | 112 | 986 | 38 |
| 4 | 0.9900 | 74 | 991 | 30 |
| 3 | 0.9940 | 44 | 994 | 24 |
| 2 | 0.9974 | 20 | 998 | 20 |
| 1 | 1.0000 | 0 | 1000 | 0 |

## Data Nudging

To minimize the accumulation of model errors and retain as many mesoscale circulations as possible, the Objective Analysis and Four-Dimensional Data Assimilation (FDDA) techniques was used to include observations of the surface winds and upper-level meteorological information. The datasets used are high resolution (in time and space) and the technique has been applied in previous air quality modeling studies.  Gilliam et al. (2009) found big improvements in WRF results when OBSGRID, was used to lower error of analyses.

### Objective Analysis

The OBSGRID option, just introduced in WRF v. 3.1, is used for objective analysis. The goal of OBSGRID is to improve the first-guess gridded analysis by incorporating high-resolution observations with the low-resolution global analysis [Wang et al. 2009]. It also prepares the input files for surface nudging.

The Cressman style objective analysis was used, in which several successive scans nudge a “first-guess” field toward each neighboring observed value within a circular radius of influence from each grid point. A distance-weighted average of the difference between the “first guess” and the observations is added to the value of the “first guess” at each grid point. Once all grid points have been adjusted, the adjusted field is used as the “first guess” for another adjustment cycle with a smaller radius of influence.

### Four-Dimensional Data Assimilation

In contrast to objective analysis, which is used to improve initial and boundary conditions, FDDA incorporates observations during model integration [Stauffer et al. 1991]. The FDDA options chosen for the OTC WRF runs were analysis and surface nudging. The surface FDDA option creates a separate surface analysis that has a higher temporal resolution than full 3D analyses. The analysis nudging uses 3D gridded variables above the surface layer that are used throughout the WRF run to nudge model outputs toward the analysis, which is based on observations [Wang et al. 2009].

The input data was set at 3-hour intervals. In the boundary layer, only U and V winds were nudged. Nudging of temperature at the surface was not used because it can cause static instability if it is not consistent with temperature above the surface layer [Stauffer et al. 1991]. The water vapor mixing ratio is also not nudged in the PBL because supersaturation may occur when surface temperature is not nudged. In the free troposphere, nudging was used for U and V winds, temperature, and water vapor mixing ratio to help minimize large-scale errors.

### Nudging Coefficients

The strength of dynamical nudging depends on the magnitude of the nudging coefficients selected. The nudging coefficient for wind should not exceed that of the Coriolis parameter [Gupta et al. 1997]. The nudging coefficients used for the SEMAP WRF runs are listed below.

| Domain | Variable Nudged | Nudging Coefficient |
| --- | --- | --- |
| Outer Domain | U and V winds | 5.0 x 10-4 |
| Outer Domain | Temperature | 5.0 x 10-4 |
| Outer Domain | Water vapor mixing ratio | 1.0 x 10-5 |
| Outer Domain | Surface U and V winds | 5.0 x 10-4 |
| Inner Nest | U and V winds | 3.0 x 10-4 |
| Inner Nest | Temperature | 3.0 x 10-4 |
| Inner Nest | Water vapor mixing ratio | 1.0 x 10-5 |
| Inner Nest | Surface U and V winds | 3.0 x 10-4 |

The inner nest nudging coefficients are similar to those used in air quality retrospective studies. Gupta et al. (1997) tested various nudging coefficients for wind and found that 2.778 x 10-4 best replicated the energy spectrum. Mao et al. (2006) used 2.5 x 10-4 for temperature and wind and 1.0 x 10-5 for water vapor mixing ratio.  However, Gilliam and Pleim (2010) used a slightly higher nudging coefficient for temperature and wind (3.0 x 10-4) but the same for moisture (1.0 x 10-5).

The outer domain nudging coefficients were determined after testing at different levels. Using lower coefficients, the SEMAP found that the 3D analysis and surface nudging had very little impact on surface and upper air variables. As a result, higher settings were chosen for the outer domain to improve WRF performance

## Period of analysis

SEMAP provided AER with WRF model output for domains 1 and 2 from the year-long production run and nine sensitivity runs of the WRF model for selected periods during 2007.

### Production Run

Detailed model evaluation of the production run was performed for the 365-day period comprising the entire calendar year of 2007 for both domains. The full year of simulations was pieced together from 73 individual WRF runs of length five and a half days each. To avoid meteorological field contamination due to model spin-up, data from the first twelve hours of each model run were not analyzed. Thus, each five-day model segment contained data with forecast lengths of between 12 and 131 h.

### Sensitivity Runs

Detailed model comparison amongst the nine sensitivity runs was carried out for domain 2 for two evaluation periods of 20 days each, hereafter, the ‘Summer’ and ‘Winter’ periods. Respectively, these two sets of four individual WRF runs were valid for the calendar days 28 July 2007 through 16 August 2007, and 5 December 2007 through 24 December 2007. These dates were selected by the WRF Working Group based on known air quality exceedances throughout the Eastern US.

## Observational dataset

The WRF-MET meteorological evaluation software that was used in this study (refer to Section 3.2) was designed to ingest observations in prepBUFR format. A readily available and valuable source of observations in prepBUFR format was obtained from the UCAR web server (http://dss.ucar.edu/datasets/ds337.0/). While the observation types available in the prepBUFR files include the following platforms for 2007, version 2 of WRF-MET does not decode visibility, precipitable water, snow cover, or cloud elements:

| Air Temperature | Dew Point |
| --- | --- |
| Atmospheric Pressure Measurements | Temperature |
| Cloud Amount/Frequency | Geopotential Height |
| Cloud Base | Precipitable Water |
| Cloud Types | Precipitation Amount |
| Sea Level Pressure | Surface Winds |
| Sea Surface Temperature | Upper Level Winds |
| Snow | Visibility |
| Station Height |  |

The following is the general description from the UCAR web server that accompanies this dataset:

NCEP ADP Global Upper Air and Surface (PREPBUFR format) Weather Observations are composed of a global set of surface and upper air reports operationally collected by the National Centers for Environmental Prediction (NCEP). These include land surface, marine surface, radiosonde, pibal and aircraft reports from the Global Telecommunications System (GTS), profiler and US radar derived winds, SSM/I oceanic winds and TCW retrievals, and satellite wind data from the National Environmental Satellite Data and Information Service (NESDIS). The reports can include pressure, geopotential height, temperature, dew point temperature, wind direction and speed. Report time intervals range from hourly to 12 hourly. These data are the primary input to the Global Data Assimilation System (GDAS), which is used to create the NCEP Final Analyses (FNL).

WRF-MET was designed for the direct ingest of this dataset and so use of these data for the entire verification exercise minimized the time-consuming and error-prone task of aggregating and reformatting observations. This dataset spans the time period and geographical region of interest and thus the verification techniques maintained data integrity from a single source across all verification stratifications.

This dataset has undergone quality control procedures as part of preprocessing for NCEP’s Grid Point Statistical data assimilation system and, as a result, contains an estimation of the observational error. Below is a description from the UCAR web server of the quality control procedures:

The "PREPBUFR" processing is the final step in preparing the majority of conventional observational data for assimilation into the Global Forecast System and Global Data Assimilation System unified grid-point statistical interpolation analysis (GSI) (the "GFS" and "GDAS" networks). This step involves the execution of series of programs designed to assemble observations dumped from a number of on-line decoder databases, encode information about the observational error for each data type as well the background (first guess) interpolated to each data location, perform both rudimentary multi-platform quality control and more complex platform-specific quality control, and store the output in a monolithic BUFR file, known as PREPBUFR. The background guess information is used by certain quality control programs while the observation error is used by the analysis to weigh the observations. The structure of the BUFR file is such that each PREPBUFR processing step which changes a datum (either the observation itself, or its quality marker) records the change as an "event" with a program code and a reason code. Each time an event is stored, the previous events for the datum are "pushed down" in the stack. In this way, the PREPBUFR file contains a complete history of changes to the data throughout all of the PREPBUFR processing. The most recent changes are always at the top of the stack and are thus read first by any subsequent data decoder routine. It is expected that the data at the top of the stack are of the highest quality.

It should be noted that a software bug in WRF-MET inverted the stack. This resulted in a small number of observations of lower quality being used. Detailed evaluation by the UCAR software developers during their attempt to correct the problem determined that less than 10% of all observations were incorrectly selected and that temperature observations were the most affected. We believe that this is the source of some of the gross outliers, which are often unphysical, that are most readily seen in the scatter plots (Section 5.1.5).

## Meteorological variables and assessment measures

| Quantitative Assessment Metrics |  |  |
| --- | --- | --- |
| Wind Speed | RMSE | 2 m/s |
|  | Mean Bias | 0.5 m/s |
| Wind Direction | Gross Error | 30 deg |
|  | Mean Bias | 10 deg |
| Temperature | Gross Error | 2 K |
|  | Mean Bias | 0.5 K |
| Specific Humidity | Gross Error | 2 g/kg |
|  | Mean Bias | 1 g/kg |

The following six meteorological fields were objectively analyzed by WRF-MET: temperature (degrees Celsius), dew point temperature (degrees Celsius), specific humidity (g/kg), relative humidity (%), wind speed (m s-1), and wind direction (degrees). Not all tasks required the evaluation of all fields, however. The following Quantitative Assessment Metrics were considered as important thresholds during the analysis of the errors, including the choice of colors in plot legends:

# Evaluation techniques

## Subjective

A subset of the production and sensitivity run WRF output files, as well as input namelist files, were subjectively inspected to ensure that the WRF model was configured as intended, and also that the intended configuration is consistent with established combinations of model options. The following aspects of the model configuration were inspected:

- initialization data (horizontal grid spacing, frequency of boundary conditions)

- application of data assimilation

- time step

- placement and number of vertical levels

- one-way or two-way nested grid interaction

- strategy for piecing together coincident model runs

- physical parameterizations for cumulus convection, the planetary boundary layer scheme, explicit moisture, radiation and land-surface scheme.

## Objective

Version 2.0 of the WRF-MET meteorological modeling verification software (available from http://www.dtcenter.org/met/users/) was used to carry out the initial steps of the objective statistical analysis. A logical depiction of the components of the WRF-MET software package is shown below. Green indicates software and modules, and gray represents input and output files.

WRF-MET expects GRIB1 format model data, which was generated by processing all raw model runs received from SESARM in netCDF format through software named WRF-WPP. This is an NCEP-developed post processor that also converts the native model sigma level data to standard pressure levels. This will facilitate generation of statistics at levels typically available from radiosondes.

The WRF-MET software is composed of a number of individual processing steps. The input data – both point and gridded observations - first are converted into internal intermediate formats, and then raw statistics files are generated. These raw statistics are named “STAT” in the above flowchart and ultimately represent top-of-the-hour pairs of modeled and observed values. These take the form of both spatially-averaged modeled and observed values and, more fundamentally, matched pairs of true single-time modeled and observed values. These raw statistics have been provided to SESARM in zipped files on suitable media.

The STAT files were then aggregated and temporally averaged to produce summary statistics presented in the form of plots, charts, maps and tables. The aggregation and plotting steps were carried out through extensive use of shell and Perl scripts. The R analysis and display software was used to generate all graphics.

## Spatial Averaging

Statistics for a number of tasks have been spatially averaged into 27 states and Regional Planning Organization regions (RPOs): Alabama, Arkansas, District of Columbia, Florida, Georgia, Iowa, Illinois, Indiana, Kentucky, Lousiana, Maryland, Michigan, Mississippi, North Carolina, Ohio, Pennsylvania, South Carolina, Tennessee, Virgina, Wisconsin, West Virginia,MANEVU (ME, MA, VT, NH, NY, PA, NJ, MD, DE), CENRAPN (MN, IA, NE, KS, MO), CENRAPS (TX, LA, OK, AR), SEMAP [VISTAS] (VA, WV, KY, TN, NC, SC, GA, AL, MS, FL), MRPO (MI, WI, IN, IL, OH) and FULL (entire model domain)

*Some early plots and tables refer to VISTAS. Subsequently, the term SEMAP has been used.

Computation of statistics inside WRF-MET for different regions is accomplished using grid point masks based on state outlines as defined by the 2000 US Census. Because masking is grid point based, as opposed to observation location based, whether an observation is used in a specific masked region (for instance, a state) depends upon in which state the closest grid point resides. It is thus possible that observations can be used in different states depending on the domain in question. A small number of observations changed states between domains 1 and 2 in this study, but the overall effect on the statistics is negligible.

A second consequence of the grid point method is that statistics are unavailable for domain 1 for the District of Columbia. For domain 1 there are no grid points that lie inside that very small geographic area.

The following illustrates the geographical extent of the five RPOs for which statistics were computed. Note that the red WRAP RPO was not involved in this study:

# Inspection of model configuration

In our opinion, the WRF model configuration selected by SEMAP for the production run is acceptable for generation of meteorological fields to be input to air quality models. A reasonable mix of physics options and model configuration parameters was selected using an up-to-date release of the ARW-WRF model. Sufficient documentation in the form of log files and namelist files was provided by SESARM to be confident that the WRF model was run in the manner designed and intended.

While the choice in the production run to turn feedback off between nests allows for independent evaluation of forecasts from each of the two domains, ideally feedback would be active in order for the coarser outer domain fields to incorporate the meteorological fields from the innermost domain that have been influenced by the higher-resolution topography. It is noted that feedback between grids was active for all sensitivity runs. This fact contributes, in part, to the different error statistics between the sensitivity run that was chosen for the production run and the actual production run. The difference in statistics is also attributable to the different model initialization times of the run segments between the sensitivity and production runs.

The dependence on model length is considered a relatively minor factor in the overall quality of the forecast fields, though a degradation in the quality of the model 2-m dew point temperature field was noted (reported below in 5.1.1.2). This may be a result of the lack of nudging moisture in the PBL. Considered more important overall, however, is the quality of the individual initialization field for each run. It is hypothesized that any deleterious effects of the use of relatively long segments (132 h), as opposed to 30-h simulations, for instance, was generally negated by the use of nudging.

# Evaluation of Annual Production Run Meteorological Model Performance

## Statistical analysis of WRF full-year production run

Detailed analysis of each Production Run Task now follows.

### Task 2 A: Time series of spatial averages over all regions

This section discusses verification plots that show time series (of daily averages for annual and monthly plots, and hourly values for plots covering 5-day segments) of spatially averaged forecast and observed values of various surface fields (temperature, dew point, etc).  We concentrate here on domain-wide (domain 1 and 2) averages that highlight model performance over the entire model domain.  Statements concerning the model performance in this section apply to these domain-wide statistics.  The website contains additional plots for averages computed over various geographic regions.

#### Temperature

The annual observed and modeled temperature for the FULL region of Domains 1 and 2 shows that temperatures are generally well simulated.  The model temperatures track the observations well; however, they have a slight high bias during the cooler portions of the year, in January and February and again in December.  A slight negative bias is also found during a portion of the warm season from roughly June through early August.  The time series also suggests the model has some difficulty predicting periods of lower temperatures as it tends to not predict temperatures as low as the observed values.  Differences in the observed performance of the model between domain 1 and domain 2 were found to be minimal.

The two plots below demonstrate the different biases of the model prediction of air temperature for two different 5-day runs.  The first plot shows an approximate 1-1.5° C high bias in the model prediction for the period January 31 – February 4, 2007.  By contrast, a high bias of generally less than 0.5° C is found for the period July 5–9, 2007.  The transition from a high bias during the cool season to a negative bias during the warm season highlights the seasonality of the results from the WRF model.

#### Dew point temperature

The observed and modeled dew point temperature time series for domain 1 appears in the plot below.  It shows that there is a slight high bias in the dew point temperature throughout most of the year except during the autumn when the model simulation more closely matches the observations.

The following plots of monthly dew point temperature further detail the performance of the WRF model.  The first plot below shows good performance from the WRF model during the month of October.  The plot for the month of July shows that the time series is more disorganized and the model simulation does not track the trends in the observations particularly well.  The dew point temperatures in July are forecast about 1 C too warm on average.

A poor representation of the diurnal cycle appears to be contributing greatly to the observed high bias in the longer time series.  The plot below from February 25 – March 1, 2007 shows that the dew point is well simulated during the daytime hours when boundary layer mixing is strongest; however, the WRF simulation predicts the nighttime dew point temperature much too high – on the order of 1 C.

There is also evidence in the 5-day model simulations that the dew point temperature tends to develop a greater spread between the modeled value and the observed value at later forecast times.  The plot below shows an example of this tendency.  In the first 48 hours of the simulation, the modeled dew point temperature time series follows the observed time series relatively well, but diverges after this time resulting in a significant high bias of approximately 2C.

#### Relative humidity

The modeled relative humidity time series displays many of the same characteristics as the dew point temperature time series in that it carries a high bias relative to the observed values for much of the year excluding much of the autumn season.  The combination of air temperatures that are simulated near or below the observed values and dew point temperatures that are generally above the observed values is consistent with a heightened relative humidity profile, especially during the January through August time frame.  Comparing the two domains in the plots below, the modeled and observed relative humidity increases slightly for domain 2, but the trends are quite similar.

The plots below show 5-day model simulations of relative humidity during the summer and autumn to highlight the differences on a shorter time scale.  The top plot shows the relative humidity time series for June 30 – July 4, 2007.  The modeled relative humidity is slightly higher than the observed relative humidity with good performance during the daytime hours and poorer performance during the nighttime hours.  Similarly in the bottom plot, the model performance is better during the daytime in the autumn.  The modeled relative humidity, however, is predicted lower than the observations.

#### Specific humidity

The modeled and observed specific humidity time series is shown in the plot below.  The WRF model performs well at predicting the specific humidity especially during the cooler portions of the year, however, there is a slight high bias in the model simulation during the warmer months especially during June and July for domain 1.  The performance of the WRF model for domain 2 in the lower plot is significantly better during the warmer months.

#### Wind direction

The averaging procedure for wind direction differs from those of the other variables, since a simple arithmetic average of wind direction is not a meaningful quantity; instead, the time series show the wind direction of the averaged (in space and time, if applicable) wind vector.  Despite of the somewhat noisy appearance of these plots caused wind direction near south (+/- 180), some general observations are possible.  In the plot below it can be seen that the WRF model accurately captures the westerly wind tendency in domain 1 as indicated by the collection of data points between -180 and 0 in the lower portion of the plot which indicates winds with a westerly component.  The corresponding plot for domain 2 shows a greater tendency for a southerly component to the wind especially during the warmer months than domain 1.

#### Wind speed

The annual plot of wind speed for the observations and WRF model is shown below.   There is a consistent negative bias throughout the year in domain 1 and this is similar to the negative bias in wind speed found at many of the individual locations.  It appears that the negative bias is most pronounced during the early spring.  The lower plot shows the time series for domain 2 and it is apparent that the performance is better with significant improvement during the late winter and spring and excellent simulation of the wind speeds during the warmer months.

A closer examination of a shorter time scale reveals some important diurnal characteristics of the modeled wind speed.  First, there is a short period of time when the wind speed decreases in the early afternoon before resuming its increase to an evening maximum.  This pattern is observed in a majority of the individual 5-day runs throughout the year and results in the model having a negative bias during the entire year because the negative bias in the afternoon is so significant.  Also, in the warmer months there seems to be a high bias during the night as the modeled wind speeds are slightly higher than the observed wind speeds.

### Task 2 B: Monthly Bakergrams

The next section describes the variability of the daily mean absolute error and the daily mean bias for the production run through a series of Bakergrams.  The discussion focuses primarily on the “FULL” region (entire model domain) for Domains 1 and 2.

#### Temperature

The daily mean absolute errors (MAE) for the production run of the WRF model are greatest during the winter months and are approximately 2-3C for this period.  The errors decrease to roughly 1.5-2C on average during the summer months.   The daily mean bias in temperature shows a similar seasonal pattern to the mean absolute errors.  The temperatures have a high bias from the early autumn through about mid-winter.  The remaining portion of the year presents a negative bias. The mean absolute error variability is very similar for domain 2 except the mean errors are slightly smaller, perhaps 0.5C smaller on average; however, the mean bias over domain 2 increases slightly.  Additional Bakergrams below of the daily mean absolute errors and mean bias over Ohio are examples of greater variance over the individual states and smaller RPOs in these parameters.

#### Relative humidity

The relative humidity plots below show that the daily mean absolute errors are largest during the late winter and early spring with errors up to 15-20%.  The errors are smallest during the summer and early autumn time frame generally around 10%.  The relative humidity parameter in the WRF model has a high bias over most of the year in the daily mean except during the autumn when it has a slight negative bias.  The daily mean absolute errors and mean biases are slightly increased for domain 2, although they retain a similar seasonal pattern.  Comparing the daily mean bias Bakergrams from Florida and Ohio shows that there is a much more significant high bias over Ohio during the winter consistent with the overall pattern from the full domain.  In contrast, no obvious bias exists over Florida at any time during the year.

#### Wind Speed

The daily mean absolute error for wind speed in the WRF model is generally higher in the winter and spring than during the warmer months and this is most likely a result of higher average wind speeds that occur at these times of the year.  The mean absolute errors are generally under about 2 m s-1.  The mean bias is quite small (under 0.5 m s-1), with the only significant bias to appear on the plot being a negative bias during the spring months.  These statistics for domain 2 are very similar to domain 1.  Two additional plots below comparing wind speed bias over Ohio and West Virginia highlight the effect of elevated terrain on the WRF model results.  There is a consistent positive bias throughout the year over West Virginia (especially during the late summer and early autumn), while over Ohio the wind speed bias is generally negative.

### Task 2 C: Bubble spatial plots of error statistics

These plots show maps covering the entire modeling region (domain 1 and 2), with color-coded symbols at surface observation sites.  The color-coding corresponds to the value time-averaged error statistics (e.g., mean bias, MAE) at each observations site.  These plots help to identify spatial patterns of different error characteristics.

#### Temperature

The 2-meter / surface temperatures simulated by the WRF-MET model were generally accurate to within 3.0 °C of the observations for all U.S. stations within the model’s inner domain (12 km). The agreement between the simulated and observed temperatures was somewhat poorer in the cooler seasons than during the summer as the mean absolute error (MAE) for many locations was between 2 – 3 °C in winter, spring, and fall (see Figure below) but decreased to generally < 1.5 °C in all regions during the summer (see Figure below).  The MAEs in Southern Canada were notably higher during winter peaking at > 3.0 °C, but of the same magnitude as those in the U.S. during the other seasons.  Much of the temperature error during the winter and fall could be explained by warm biases of 1- 2 °C in the U.S., particularly in the Great Plains and Midwest, and 1– 3 °C in Southern Canada.  During the spring, biases of -1 to -2 °C were produced for many locations in the Midwest and Northeast.  As will be discussed in Section 5.1.4.1, some sites in the South and Southeast, particularly Tennessee and Alabama, showed a tendency towards a negative bias in temperature.  The spatial plots, however, did not show evidence of substantial widespread cool biases (< -1.0 °C) across that part of the U.S. during the warm months (see Figure for July mean bias below).

January Temperature MAE for Domain 2:

July Temperature MAE for Domain 2:

July Temperature Mean Bias for Domain 2:

The error characteristics in the portion of the outer domain (36km) that overlapped domain 2 were similar to those in domain 2 but of a slightly greater magnitude (~ 0.5 °C) presumably due to the coarser grid spacing.  Domain 1 covered the entire contiguous U.S. and MAEs were generally > 3.0 °C over the Rocky Mountains but < 2.5 °C along the immediate West Coast.

#### Relative humidity and dew point temperature

The WRF-MET 2-meter / surface relative humidity (RH) errors exhibited well-defined seasonal and spatial variability peaking in the northern half of domain 2 during late winter and early spring of 2007.  The larger late winter/early spring errors were primarily due to positive biases ranging from 20 - 40 % (see Figure below) and corresponding positive biases in dew point of 2 C to > 4 C indicated that the RH biases were due to an overprediction of absolute moisture content rather than an under prediction in temperature.  In addition the locations of the high positive RH biases were mostly over snow-covered areas suggesting that the model may not accurately simulate the moisture flux from snow (see Figure below).

February Relative Humidity Mean Bias for Domain 2:

National Snow Analysis for February 28, 2007 - While the area of snow cover varied in time this plot was representative of the areas of most persistent snow cover during February and early March of 2007. (Available from National Weather Service National Operational Hydrologic  Remote Sensing Center, http://www.nohrsc.noaa.gov/nsa/.)

The unusually mild early winter of 2006 -2007 (see statewide temperature rankings at http:// www.ncdc.noaa.gov) in the Midwest and Northeast caused a delay in the usual spread of snow cover in the region.    As temperatures cooled to slightly below normal in February the area of snow cover began to increase substantially.  It is interesting to note that the increase in the WRF-MET positive RH biases seemed to follow the increase in snow cover in the region, which provides further evidence that the model may have a tendency to overpredict the surface moisture flux over snow-covered land.

#### Wind speed

The MAEs for the WRF-MET 10- meter / surface wind speed were generally < 1.5 m s-1 throughout domain 2 with a few exceptions.   Along the coast of the northeastern U.S. and in southern Canada during the cooler months of January – April and November – December the MAEs generally ranged from 1.5 – 2.5 m s-1 with a few locations > 3.0 m s-1.  The errors seemed random with no consistent biases and were not dependent on the time of day.  For domain 1 the wind speed MAEs were similar to those of domain 2 in the eastern U.S, but were generally > 2.5 m s-1 over the Rocky Mountains.

#### Wind direction

During the winter months substantial directional errors in the WRF-MET 10-meter/surface winds were limited to the lee side of the Appalachian Mountains.   In this region the directional errors generally ranged from 40 – 60° (see Figure below).   With the juxtaposition of cold continental polar air and the warmer maritime air of the Gulf Stream, this region is an area of frequent cyclone development, and because of these dynamics and the complexity involved in simulating them larger wind errors are not surprising.

January Wind Direction Mean Absolute Error for Domain 2:

During the warmer months the large errors in the lee of the Appalachians persisted, and another area spanning the Gulf Coast states developed with errors exceeding 60° at many locations (see Figure below).  It is possible that the large directional errors in the Gulf Coast states were due in part to frequent convective activity that was unresolved at the model grid scale.  As with other variables, in domain 1 the Rocky Mountains caused substantial wind directional errors that in most locations exceeded 70 °.

July Wind Direction Mean Absolute Error for Domain 2:

### Task 2 E: Time series at individual sites of forecast/observed values

This task focuses on model evaluation at a limited number of individual METAR sites. The sites are identified in the following table.

| State | City | Observation Station | State | City | Observation Station |
| --- | --- | --- | --- | --- | --- |
| AL | BIRMINGHAM | KBHM | MO | ST. LOUIS | KSTL |
| AL | MOBILE/BATES | KMOB | MN | MINNEAPOLIS | KMSP |
| AR | LITTLE ROCK | KLIT | MS | JACKSON | KJAN |
| DC | WASHINGTON/NATL | KDCA | NC | CHARLOTTE | KCLT |
| FL | ORLANDO | KMCO | NE | NORTH PLATTE | KLBF |
| FL | TALLAHASSEE | KTLH | OH | CLEVELAND | KCLE |
| FL | MIAMI | KMIA | OK | OKLAHOMA CITY | KOKC |
| GA | ATLANTA | KATL | SC | COLUMBIA | KCAE |
| IA | DES MOINES | KDSM | SC | GREEN/SPARTANSBG | KGSP |
| IL | CHICAGO O'HARE | KORD | TN | CHATTANOOGA | KCHA |
| IL | PEORIA | KPIA | TN | KNOXVILLE | KTYS |
| IN | FORT WAYNE | KFWA | TN | MEMPHIS | KMEM |
| KS | DODGE CITY | KDDC | TN | NASHVILLE | KBNA |
| KY | COVINGTON | KCVG | TX | DALLAS/FT WORTH | KDFW |
| KY | LOUISVILLE | KSDF | VA | RICHMOND | KRIC |
| LA | SHREVEPORT | KSHV | WI | ASHWAUBENON | KGRB |
| MI | DETROIT/WAYNE | KDTW | WI | WAUSAU | KAUW |
| MI | HOUGHTON LAKE | KHTL | WV | CHARLESTON | KCRW |
| MO | KANSAS CITY/INTL | KMCI | WV | HUNTINGTON | KHTS |

#### Temperature

The temperatures produced by the WRF simulations were comparable to the temperature observations at most of the individual METAR locations.  The model temperatures track the observations relatively well; however, the variability of the model temperatures is generally lower than the observed values meaning the model simulations have difficulty replicating temperature extremes both high and low.

The WRF model performed the best at simulating temperatures at locations in the Great Lakes, Midwest, and central Plains regions.  At the sites in these regions, significant temperature biases were absent as the mean and variability are well represented.  For example, at KORD (Chicago, IL) the annual observed temperatures are well predicted by the model and there is no apparent bias at any period of time during the year.

At sites across the South and Southeast, especially during the warmer months, a negative temperature bias appears to be present in the WRF model.  This negative temperature bias is most apparent in the KBHM, KCHA, and KMEM annual observed and modeled temperature plots.  The KMEM (Memphis, TN) plot appears below.  Notice the consistent negative bias from late May through the middle of August in the modeled temperature relative to the observed temperature.  The results discussed in Section 5.1.3.1, however, indicate that errors of this magnitude are not typical of most stations in that region. During 2007, a major hydrological drought affected a large portion of the southeastern United States. The drought monitor map (at right, valid 14 August 2007) indicates the expanding moderate through exceptional drought during the summer of 2007. Failure of the WRF model to accurately initialize the surface and boundary layer conditions (e.g., dry soil) could have lead to or contributed further to this error.

As previously mentioned, the WRF-modeled temperatures tend to exhibit less variability than the observed temperatures.  The most obvious example of this is shown in the annual observed and modeled temperature plot for KMIA (Miami, FL) in the plot below.  The plot shows that the model has a poor representation of the periodic highs and lows.  This poor performance appears to be greatest during the summer and this is likely due to the inability of the model to accurately model the boundary layer moisture characteristics.  As will be discussed in a subsequent section, the WRF model grossly over predicts the dew point temperature at KMIA during June through October and this would contribute to a lower modeled diurnal range.

The 5-day model run from June 15-19, 2007 at KMIA shown below exemplifies the problem of modeling the diurnal cycle and summertime air mass convection. With the exception of the first daytime period, the high temperatures are close to the modeled values or under predicted and the low temperatures are over predicted for all days.

The observed diurnal temperature trace is sharply interrupted by the cooling effects of rainfall that are absent in the modeled fields. The related effect on the dew point temperature and relative humidity fields is also not captured by the model (not shown). Convective rainfall occurred on all days during this period except for the 19th. The correct placement and timing of convection is a particularly difficult task for meteorological models and this example does not necessarily suggest a chronic problem in the WRF model. A detailed analysis of precipitation was out of the scope of this project.

#### Dew point temperature

The WRF model results for the dew point variable generally indicate that it has a slight positive bias across a majority of the individual METAR sites, especially during the warm season.   As with temperatures, the best representations of dew point in the WRF simulations were found outside of the southeastern United States.  The plot below of the annual modeled and observed dew point at KORD (Chicago, IL) is an example of a particularly good dew point simulation.

The number of individual METAR sites that have positive dew point biases suggests that the WRF model potentially has an issue with the parameterization of moisture fluxes at the surface.  This bias is most noticeable across the Southeast, but also can be seen in the central Plains during the warm season.  The annual dew point plots below demonstrate that KMIA (Miami, FL) has a very strong positive dew point bias during the June through October time frame while KCAE also has a consistent positive bias during the late spring and summer.

A partial reason for the lack of skill in the WRF simulation of dew points during the warm season in the Southeast may be that the baseline model dew point temperature is too high.  In both plots below from KBHM (Birmingham, AL) – June 20–24, 2007 – and KCAE – June 25–29, 2007, the nocturnal rise in dew point is quantitatively similar for both the model results and observations; however, the model doesn’t simulate the drop in dew point associated with a well-mixed daytime boundary layer resulting in dew points that are generally too high.

#### Relative humidity

The trends observed in relative humidity for the WRF simulations closely resemble those of the dew point trends.  A number of the individual locations exhibit a positive relative humidity bias during the warm season especially over the southeastern portions of the United States.  A couple less common observations that differ from the dew point results are also found: a number of sites are associated with a positive bias during the winter or a transition to a neutral or negative bias during the autumn.   An example of each case is shown discussed further below.  It is also worth noting that the variability in the WRF relative humidity tends to be greater than the variability in the observed values.

The plot below shows the annual observed and modeled relative humidity for KAUW (Wausau, WI).  A strong positive bias is noted from January through the end of April and again starting in December.  Although it appears that a higher relative humidity as compared with the observations is seen throughout the year, the distinction is most pronounced during the cool season.  This effect could possibly be due to how the WRF handles snow cover in its simulation.  Another station that exhibits a significant positive bias throughout the year is KCLE (Cleveland, OH), which could be the result of its location along Lake Erie in which poor simulation of the moisture fluxes off of the lake could result in a persistently high relative humidity as compared with observations.

The 5-day run for KAUW during a period of relatively poor performance (Feb 5–9, 2007) shown in the plot below shows that there do not appear to be any problems simulating the diurnal cycle, but the overall relative humidity is forecast too high at all time periods.

The following plot from KBHM (Birmingham, AL) is a prime example of a station that has a positive relative humidity bias during the summer and transitions to a neutral or slightly negative bias during the autumn season.   Notice also the relatively large variability in the modeled relative humidity as compared with the observations especially during the winter and later in the fall.  It is important to keep in mind KBHM was under a significant drought during 2007, so this may have had an influence on these modeling results.

A 5-day model run depicting the modeled and observed relative humidity at KBHM for June 10–14, 2007 is shown in the plot below.  There is a noticeable positive bias in the relative humidity in the early morning time period and although the diurnal cycle seems to be well represented, the values remain fairly high because the dew points are modeled too close to the air temperatures resulting in over predicted relative humidity values.  This type of behavior is observed at a number of individual stations in the Southeast during the warm season.

#### Specific humidity

The specific humidity is generally modeled better than the relative humidity in the WRF model; however, it exhibits some of the same biases seen with the relative humidity when compared to the observations.  Many locations in the Southeast and the central Plains into the Midwest display a positive bias during the warmer months.  A plot of the specific humidity for KORD is shown in the plot below and represents an excellent fit between the model results and the observations.

The annual modeled and observed specific humidity plot for KDDC (Dodge City, KS) is shown in the below plot and is representative of a majority of the individual locations especially those in the central Plains and Southeast.  Notice how the specific humidity is well modeled in the early portion of the year, but from May to September it shows a positive bias and then from October to December the model results match the observations quite well.

#### Wind direction

The wind direction as modeled by the WRF simulations was good overall, but there were a few locations that had directional biases, one of which was likely related to topography.  At KCRW (Charleston, WV) shown below, the mean observed wind direction is generally westerly, between -30 and -150, but the model simulates too many wind directions that are southerly – near -180 or 180.  As this location is at a higher elevation, it is somewhat likely that terrain effects are contributing to the poor performance of the model.

The modeled wind direction at KORD (Chicago, IL) is particularly good, especially during the cool season.  The plot below shows that wind direction simulated by the model is very close to the observations from January through June and the model only experiences a slight decline in skill during the late summer and autumn and then recovers during the month of December.

#### Wind speed

The wind speeds depicted in simulations of the WRF model generally were under predicted by as much as 1–2 m s-1 relative to the observations of wind speeds at each of the individual METAR locations.  There are no particular regional trends observed for this variable, but it appears to be closely linked to local topography.   A typical example is shown below from KATL (Atlanta, GA).  Notice how the modeled wind speed is approximately 1 m s-1 below the observations and this is a consistent relationship year round.

A 5-day run plot below of the wind speed at KATL from August 19–23, 2007 shows in some detail that a possible reason the wind speeds are under predicted at KATL and a number of other locations is that the WRF model fails to capture the higher wind speeds with much accuracy.  Notice that over a 3-day period maximum observed wind speeds are near 5 m s-1, however, the model predicts a maximum wind speed roughly one half of this value near 2.5 m s-1.

The northern and central Plains as well as the Midwest tended to have the best representations of wind speeds in the model as these areas have relatively flat terrain and the presence of tall vegetation is generally less than in other regions.  As a result, wind speed observations at METAR sites in these locations are not greatly influenced by small-scale local effects.  Below is the plot from KPIA (Peoria, IL), which indicates that the model performed exceptionally at this location with regard to wind speed.

Although most sites conform to the overall 1–2 m s-1 negative bias in wind speed, a notable exception is KCRW (Charlestown, WV).  Figure 13 below indicates that the WRF model simulates wind speeds slightly above the observations, especially during the warm season.  Since KCRW is one of the locations at a higher elevation this may play a role in how the model simulates the wind speed at this location.

### Task 2 G: Scatter plots

#### Surface

The following scatter plots show the observed value relative to the modeled value of each variable.

##### Temperature

Surface temperatures simulated by the WRF model generally track the observations well and larger errors are often the result of bad observations.  In the monthly plot for January below, there is a fairly linear relationship; however, at the coldest temperatures the model simulates the temperature too warm relative to the observations.  The monthly plot for July shows no significant bias for the errors at the low end of the temperature scale, but the number of observational errors appears to increase with a greater number of values far to the left and right of the red 1:1 line.

##### Relative Humidity

The scatter plots for surface relative humidity show that the observed and modeled values are generally close to each other at low relative humidity.  The errors increase significantly above an observed relative humidity of approximately 30%.  The plot for July is similar except that the largest errors are reduced and there appears to be more of a bias towards higher modeled relative humidity values versus the observed values meaning a tendency for the value to fall above the red 1:1 line.  It is less obvious for the relative humidity data which values are related to errors in the observations and which are related to errors in the WRF model output.

##### Specific humidity

The surface specific humidity plots below show a good correlation between the observed and modeled values for the month of January.  As with the temperature data, there are a number of outlier observations especially to the right of the red 1:1 line.  In the month of July, the correlation between the observations and WRF values weakens and as with the relative humidity there appears to be a slight bias towards higher model values versus the observations.

##### Wind speed

The wind speed scatter plots for the months of January and July show that the WRF model performs relatively poorly at simulating wind speeds that are near zero as there are modeled values upwards of 10 m s-1 where the observations indicate calm winds.  Conversely, there is a tendency to underpredict wind speeds that are in excess of 10 m s-1, as a majority of the points fall to the right of the 1:1 line.

#### 850 mb and 700 mb

The following section will only show scatter plots at the 850-mb level as these are representative of the scatter plots at 700 mb.

##### Temperature

The observed and modeled temperatures are well correlated at the 850 and 700-mb levels.  There appears to be a slight tendency for more outliers left of the 1:1 line especially in January most likely indicating that observations are erroneously cold or the WRF simulation is producing temperatures that are too warm.

##### Specific Humidity

The 850-mb scatter plot for January indicates that the observations and modeled values are more closely related at low moisture levels, generally under 5 g kg-1.  The correlation between these data becomes weaker at high moisture levels indicated by the large number of outliers especially those to the right of the red 1:1 line suggesting observed values are erroneously high or the WRF simulation is producing values that are too low.  The July scatter plot shows that while the correlation appears to be slightly weaker where most of the data lies, there are fewer extreme outliers.

##### Wind Speed

The 850-mb scatter plots for wind speed show a stronger correlations between the observed wind speeds and the wind speeds simulated by the WRF model with perhaps a slight negative bias indicating that the model slightly underestimates the actual wind speed.  The correlation becomes slightly weaker in July as mean wind speeds decrease.  There is a larger number of significant outliers to the right of the 1:1 line in July suggesting that the WRF model may have a greater tendency to under predict wind speeds at this level in the warmer months.  The 700-mb wind speed scatter plots are similar except the correlation between the observed and modeled wind speeds improves because it is in most cases removed from the influence of the boundary layer.

#### 500 mb and 300 mb

Similarly to the previous section, the following section will mostly show scatter plots at the 500-mb level as these are very representative of the scatter plots at 300 mb.

##### Temperature

The scatter plots of temperature for both the 500-mb and 300-mb levels show that the observations and model data are well correlated with just a few significant outliers.  The correlation weakens a bit during July especially at locations with colder 500 mb temperatures at the low end of the plot.  The last plot shows the 300 mb temperature scatter plot which shows a slight positive bias at the coldest temperatures with observations slightly colder than the WRF simulation.

##### Specific Humidity

The specific humidity scatter plots for the 500-mb and 300-mb levels similar to the previous section show that the correlation between the observed and modeled data is better at low moisture values, in this case under about 1.5 g kg-1.  There does not appear to be any significant bias in these data as the points are evenly distributed to the left and right of the red 1:1 line.  As with the July scatter plot for 850 mb, the data is more spread out and the correlation appears a bit weaker.  At 300 mb, a significant positive bias in the specific humidity is noted for July with a majority of the points left of the 1:1 line indicating that the model is producing values that are higher than the observed values.

##### Wind Speed

At the 500-mb level, the observed and modeled wind speeds are well correlated with perhaps a slight negative bias in the WRF model output at wind speeds in excess of about 50 m s-1.  This pattern is similar for the month of July with observed wind speeds greater than the modeled wind speeds at the high end of the plot as well.

### Tasks 2 I and 5 C: Line plots of error statistics

The following section reports on the error statistics of the FULL region (the entire model domains) of both domains 1 and 2.  The analysis is conducted on 4 different pressure levels as well as the surface.  The pressure levels analyzed with an approximate height levels in parenthesis are: 300 mb (9500 m), 500 mb (5750 m), 700 mb (3000 m), and 850 mb (1500 m).  Additional plots for geographic sub-regions are available on the website.

#### 300 mb

The temperature time series shows that the WRF model has a slight, but consistent positive bias relative to the observations.  The errors for temperature at this level are quite small between 0.5 C and 1.0 C below the 2.0 C maximum error target.  The errors for the dew point temperature are quite large with a mean absolute error averaging about 6 C. The dew point temperature profile also shows that the modeled dew point transitions from a negative to neutral bias during the winter to a positive bias from late spring through summer.  The error statistics for relative humidity are consistent with those for the dew point temperature.  The positive bias in dew point contributes to a positive bias of 15-20% in relative humidity.  The specific humidity, wind speed, and wind direction are generally well simulated at the 300 mb level.  Correlations between the modeled and observed parameters are very high for temperature and wind speed, but indicate a weakened relationship for dew point and relative humidity.  Differences between domain 1 and domain 2 at the 300 mb level were found to be minimal.

Domain 1

Domain 2

#### 500 mb

The error statistics at 500 mb are very similar to those at 300 mb.  The temperature is well predicted by the WRF model with errors consistently below the 2 C error metric and the bias at this level is minimal.  The mean absolute errors for the dew point temperature are similarly high to the errors at 300 mb averaging near 6 C.  The root mean squared error (RMSE) is somewhat higher than at 300 mb suggesting that very large errors in the dew point temperature are possible at the 500-mb level.  The mean absolute error of the relative humidity field at the 500-mb level is slightly lower between 10 and 15% and the correlations between the WRF model simulation and the observations are significantly improved over those at the 300 mb.  Specific humidity, wind speed, and wind direction continue to be well simulated at this level as well.  Differences between domain 1 and domain 2 at the 500-mb level are minimal.

Domain 1

Domain 2

#### 700 mb

The error statistics for temperature at the 700-mb level show the parameter to be well simulated by the WRF model.  Absolute mean errors generally remain under 1 C and the model results are highly correlated with the observations.  The dew point temperature has a positive bias during the cool season with errors that are quite large, generally exceeding 4 C.  The observed bias also appears to be more significant for domain 1 than domain 2.  The relative humidity has a mean absolute error of near 10% at this level, but the bias is neutral.  At the 700-mb level, the specific humidity has nearly a 1 g kg-1 mean absolute error during the summer months.  The modeled wind speed remains highly correlated with the observations at this level although the bias is slightly negative and the RMSE is close to the maximum RMSE target of 2 m s-1.  The wind direction errors are larger than at higher pressure levels, but have a mean absolute error of less than 20.  The wind parameters are generally modeled better in domain 2 than in domain 1.

Domain 1

Domain 2

#### 850 mb

The 850-mb temperature is well simulated by the WRF model and remains highly correlated with the observations.  The 850-mb dew point temperature maintains a slight positive bias in domain 1 that does not appear in domain 2.  The mean absolute errors and RMSE are moderately reduced as compared with those at the 700-mb level.  The correlation between the modeled dew point and the observations also improves slightly.  The relative humidity at this level has small bias and mean absolute errors that are near 10%.  Although the bias is small, the mean absolute errors for specific humidity during the summer months approach 2 g kg-1.  The correlations for relative humidity and specific humidity are much higher during the same period for domain 1 than domain 2.  The WRF model wind speed remains highly correlated with the observations at the 850-mb level although the bias is slightly negative and the RMSE is near the maximum RMSE target of 2 m s-1.   Wind direction errors are near 20 in domain 1, but decrease somewhat for domain 2.  The wind parameters are, similar to the 700-mb level, modeled better in domain 2 than in domain 1.

Domain 1

Domain 2

#### Surface

The temperature errors increase significantly at the surface as compared with pressure levels above the surface.  Although the bias remains relatively small, the mean absolute errors are close to 2 C, which is very near the maximum error target.  The modeled temperatures have slightly better error statistics for domain 2 than domain 1.  The dew point temperatures simulated by the WRF model have a positive bias during the winter months, but a more nearly neutral bias the rest of the year.  The mean absolute errors are highest during the cool season with values approaching 3 C, larger than the target threshold of 2 C.  The biases found in the relative humidity field are similar to the dew point temperature – larger in the winter months and more neutral during the remainder of the year.  The correlations for the relative humidity parameters fall significantly for domain 2.  The mean absolute errors for specific humidity are largest during the summer and slightly larger for domain 2.  The bias in the specific humidity field at the surface is neutral.   The surface wind speed mean absolute errors are near 2 m s-1, but the bias is minimal.  The modeled wind direction at the surface has the largest error of any of the levels that have been examined.  There is a mean absolute error of roughly 40 that far exceeds the maximum target given for gross error in wind direction.

Domain 1

Domain 2

#### Monthly plots (surface)

Monthly plots from January and July in domain 1 are shown below to highlight the seasonality of the WRF model performance at the surface.  The January plot shows that temperatures at the surface are more strongly correlated with the observations than in July; however, the mean absolute error remains relatively large near 2 C.  The dew point temperature shows a similar pattern except that it runs a more consistent positive bias especially during the month of January.  The mean absolute errors in relative humidity are larger in the month of January and, as with dew point temperature, there is a slight positive bias for this period.  The correlation coefficient between the relative humidity modeled by WRF and the observations are depressed especially in January, most likely a result of the larger absolute error and observed variability in dew point temperature errors.  The mean absolute errors in the specific humidity are slightly larger in July, but this is likely due to higher moisture availability during the warm season.  The mean bias data for specific humidity shows that generally the WRF over predicts the specific humidity in July.  Wind speeds, consistent with earlier results, have mean absolute errors of under 2 m s-1, with very small bias during both January and July.  Wind direction errors are quite large around 40 at the surface, as the model is likely not simulating the frictional effects near the surface particularly well.

### Task 5 B 4: Tables of Annual Statistics

This section presents summary tables of annual statistics for domain 2 modeled fields for each region. These tables are available on the project web site in Task 5 B 4. Brief discussion is provided for the regions FULL and SEMAP. For the metrics that are compared to the target ranges in Section 2.6, green indicates that the modeled error lies within the target range, while red indicates that is lies outside the target range. This simple color coding is only designed to highlight model performance and should not be construed as a simple pass/fail judgment. The reader should be reminded that the values contained in Section 2.6 are target values from the historical record. Note that rounding may introduce small disagreements between values in different columns.

#### FULL

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature (degrees C) | 18082480 | 12.7 | 12.6 | 0.2 | 1.9 | 2.5 | 0.92 | 1.001 |
| Dew Point Temperature (degrees C) | 16272421 | 7.4 | 6.8 | 0.7 | 2.2 | 3.0 | 0.89 | 1.002 |
| Relative Humidity (%) | - 16272365 | 73.2 | 69.8 | 3.4 | 11.4 | 14.6 | 0.59 | 1.049 |
| Specific Humidity (g/kg) | 17015610 | 8.0 | 7.9 | 0.2 | 1.0 | 1.3 | 0.90 | 1.028 |
| Wind Speed (m/s) | 17648777 | 3.4 | 3.5 | -0.2 | 1.4 | 1.8 | 0.68 | 0.994 |
| Wind Direction (degrees) | 17648777 |  |  | -4.2 | 34.6 |  |  |  |

##### Temperature

Annual MAEs for temperature lie within the target value and daily values do not exhibit any unexpected characteristics. Daily MAE values are largest, often upwards of 2.5 degrees Celsius, for a number of days during the cold months of November through April, with summertime values at times below 1.5 degree Celsius. Some of the larger daily errors are clustered together in time, often in five-day segments, which strongly indicates that individual model runs suffered from poor initialization. Annual values of mean bias are small. Daily values during the summer almost exclusively are within 1 degree Celsius, while the most extreme values occurred during the three other seasons. The spring and early summer from March through July produced the vast majority of days with a negative temperature bias. Days with the largest positive bias occurred mostly from October through April. An especially poor period with large (positive) forecast errors occurred in early April.

##### Dew Point Temperature

The annual mean bias of 0.7 degrees indicates that, on average, modeled dew point temperatures were higher than observed. Significantly contributing to this domain-wide bias is the widespread over-prediction of dew point temperatures in the northern parts of the modeling domain during the colder months. This problem persists from December through April. The MAEs are notably larger during the winter months, again predominantly in the northern part of the modeling domain. Problems with snow cover in the model may be a contributing factor.

##### Relative Humidity

Annual mean bias values also indicate modeled fields with slightly higher levels of relative humidity than observed. The occurrence of days with over-predicted relative humidity is concentrated strongly in December and from January through May and, again, in the northern parts of the modeling domain. March, by far, has the most days with the largest mean bias values. The only months with daily negative mean bias values of magnitude greater than 5% are October and November. The overall annual MAE of 11.4% is generally representative of individual daily MAE values. As might be expected, there is a minimum during the summer months, while maximum winter gross errors exceed 15% on a handful of days.

##### Specific Humidity

A small positive mean bias, well within the target range, is seen in the annual specific humidity field, which agrees with the characteristics of the dew point temperature and relative humidity. A positive bias in the Northeastern states is present during March and April. The magnitude, persistence and large-scale nature of this bias, especially during the cold season when absolute values of the atmospheric moisture are at a minimum, again suggests a link to how the model handles snowcover. MAE values overall are very close to the target value and are largest south of 40 oN latitude during the months of August and September. The lack of a widespread and persistent mean bias signature at this time, indicative of errors all of the same sign, suggests that the larger MAE values may be largely due to the fact that atmospheric moisture content is close to its maximum and thus subject to the largest error.

##### Wind Speed

The annual mean bias lies within the target range, and both it and the MAE are quite small and show no significant temporal variability during the course of the year. MAE errors overall are slightly higher in the northern parts of the modeling domain in the winter and spring when overall wind speeds are highest and the passage of synoptic weather systems increases the variability of the wind field. The annual RMSE is slightly less than the target of 2 m/s.

##### Wind Direction

(Note that, by convention, wind direction indicates the direction from which the wind is blowing. Here we denote, for instance, zero degrees as a North wind, 90 degrees as an East wind and -90 degrees as a West wind.) While the mean bias is well inside the target range, the MAE is slightly larger than the target value of 30 degrees. Thus, while there are substantial deviations from the observed wind, they do not exhibit a substantial bias in direction. The wind direction field is strongly affected by local topography, and, while there are no expansive and persistent regions of significant mean bias, individual stations for any one day can exhibit large biases. These individual episodes of large bias are likely tied to poor modeling of synoptic features and can be sufficiently persistent to affect monthly averages, especially in mountainous or otherwise elevated terrain. Indeed, such areas suffer from the largest and most persistent MAE values. This is most noticeable in the Appalachian mountains throughout the year. During the summer months with lighter winds, individual sites throughout the modeling domain can suffer at times from large MAE values.

#### SEMAP

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 3250685 | 17.9 | 17.6 | 0.3 | 1.8 | 2.3 | 0.81 | 1.001 |
| Dew Point Temperature | 3104743 | 10.9 | 10.6 | 0.4 | 2.2 | 2.8 | 0.78 | 1.001 |
| Relative Humidity | 3104716 | 68.9 | 67.5 | 1.5 | 11.2 | 14.2 | 0.47 | 1.021 |
| Specific Humidity | 3107904 | 9.6 | 9.5 | 0.2 | 1.2 | 1.5 | 0.76 | 1.019 |
| Wind Speed | 3139180 | 2.5 | 2.6 | -0.0 | 1.3 | 1.6 | 0.52 | 1.115 |
| Wind Direction | 3139180 |  |  | -1.5 | 42.8 |  |  |  |

##### Temperature

Annual MAEs for temperature lie within the target value and daily values do not exhibit any unexpected characteristics. Daily MAE values are largest, often upwards of 2.5 degrees Celsius, for a number of days during the cold months of October through April, with summertime values mostly below 2 degrees Celsius. Some of the larger daily errors are clustered together in time, often in five-day segments, strongly indicating that individual model runs suffered from poor initialization. As for FULL, an especially bad forecast segment occurred in early April. Annual values of mean bias are small and, again, do not indicate a problem. Daily values during the summer almost exclusively are within 1 degree Celsius, while the most extreme values occurred during the three other seasons. During these cooler seasons, individual days exhibit the largest mean biases, sometimes in excess of 2.5 degrees Celsius.

##### Dew Point Temperature

The annual mean bias of 0.4 degrees indicates that, on average, modeled dew point temperatures were higher than observed. Of note are the episodes of poorly predicted dew point temperatures centered over North Carolina during November. Detailed analysis is beyond the scope of this project, but during this month there were a number of consecutive days that exhibited large negative mean bias values. The largest MAEs are found in the mountainous or otherwise elevated regions.

##### Relative Humidity

Annual mean bias values also indicate modeled fields with slightly higher levels of relative humidity than observed. The occurrence of days with under-predicted relative humidity is concentrated strongly in November, as noted above for dew point temperature. During the rest of the year, there is a predominance of positive daily mean bias of up to 10-15%. The annual MAE of 11.2% is generally representative of individual daily MAE values. As might be expected, there is a minimum during the summer months, while maximum winter gross errors exceed 15% on a handful of days. These are focused somewhat during the month of November.

##### Specific Humidity

A small positive mean bias, well within the target range, is seen in the annual specific humidity field, which agrees with the characteristics of the dew point temperature and relative humidity. A positive bias, with a corresponding increase in MAE, in some individual locations that are mostly located in mountainous terrain or plateaus, develops during the summer, peaks in August, then diminishes during the fall. Mean bias values in the fall and early winter are then near zero or slightly negative. It is hypothesized that the positive bias that peaks in August may reflect inadequate model representation of topography, with model locations being positioned at lower elevations than in reality and subjected to higher values of PBL moisture in an otherwise horizontally-uniform moisture environment.

##### Wind Speed

The annual mean bias and MAE both lie within the target range and show no important temporal variability during the course of the year. The annual RMSE is acceptably less than the target of 2 m/s.

##### Wind Direction

The mean bias for SEMAP lies within the target range, while, as for FULL, the MAE is slightly larger than the target of 30 degrees. As for FULL, deviations of the wind from observed while large do not exhibit a substantial bias in direction. This meteorological field is strongly affected by local topography, and, while there are no expansive and persistent regions of significant mean bias (since the signs of the individual errors often cancel), regions of mountainous topography do suffer from the largest and most persistent MAE values. Along the spine of the Appalachian mountains, it is commonplace throughout the year to exceed the target MAE value. During the summer months with lighter winds, individual sites throughout the modeling domain can suffer at times from large MAE values.

#### CENRAPN

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 3251073 | 9.4 | 9.2 | 0.2 | 2.0 | 2.4 | 0.82 | 1.001 |
| Dew Point Temperature | 3246133 | 4.2 | 3.4 | 0.8 | 2.3 | 2.8 | 0.74 | 1.003 |
| Relative Humidity | 3246133 | 74.3 | 70.4 | 3.9 | 11.2 | 13.8 | 0.52 | 1.054 |
| Specific Humidity | 3246299 | 6.9 | 6.7 | 0.2 | 0.9 | 1.1 | 0.74 | 1.052 |
| Wind Speed | 3232104 | 3.8 | 4.0 | -0.2 | 1.3 | 1.6 | 0.63 | 0.999 |
| Wind Direction | 3232104 |  |  | -5.7 | 31.0 |  |  |  |

#### CENRAPS

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 3034465 | 18.4 | 18.0 | 0.5 | 1.8 | 2.3 | 0.74 | 1.002 |
| Dew Point Temperature | 3014970 | 11.4 | 11.7 | -0.3 | 2.1 | 2.7 | 0.75 | 0.999 |
| Relative Humidity | 3014970 | 68.5 | 70.5 | -2.0 | 10.8 | 13.6 | 0.50 | 0.967 |
| Specific Humidity | 3015096 | 10.1 | 10.3 | -0.2 | 1.2 | 1.5 | 0.70 | 0.975 |
| Wind Speed | 2969315 | 3.1 | 3.3 | -0.2 | 1.3 | 1.6 | 0.60 | 1.013 |
| Wind Direction | 2969315 |  |  | -1.0 | 35.9 |  |  |  |

#### MANEVU

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 2002709 | 9.8 | 9.9 | -0.2 | 1.8 | 2.3 | 0.82 | 0.999 |
| Dew Point Temperature | 1914305 | 5.2 | 3.5 | 1.7 | 2.6 | 3.2 | 0.66 | 1.006 |
| Relative Humidity | 1914304 | 77.4 | 68.0 | 9.4 | 13.4 | 16.4 | 0.44 | 1.157 |
| Specific Humidity | 1923324 | 6.8 | 6.3 | 0.4 | 0.9 | 1.2 | 0.64 | 1.132 |
| Wind Speed | 1895332 | 3.0 | 3.1 | -0.1 | 1.4 | 1.7 | 0.49 | 1.049 |
| Wind Direction | 1895332 |  |  | -4.3 | 41.0 |  |  |  |

#### MRPO

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 2919819 | 9.9 | 9.8 | 0.1 | 1.9 | 2.5 | 0.77 | 1.000 |
| Dew Point Temperature | 2750615 | 5.1 | 4.0 | 1.1 | 2.3 | 3.0 | 0.67 | 1.004 |
| Relative Humidity | 2750615 | 75.6 | 69.8 | 5.8 | 11.8 | 14.5 | 0.48 | 1.085 |
| Specific Humidity | 2751158 | 6.9 | 6.6 | 0.3 | 0.9 | 1.1 | 0.69 | 1.074 |
| Wind Speed | 2898290 | 3.4 | 3.5 | -0.1 | 1.3 | 1.7 | 0.53 | 1.063 |
| Wind Direction | 2898290 |  |  | -9.3 | 37.2 |  |  |  |

#### AL

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 267984 | 18.5 | 18.1 | 0.4 | 1.9 | 2.2 | 0.54 | 1.002 |
| Dew Point Temperature | 259275 | 11.5 | 11.0 | 0.6 | 2.2 | 2.6 | 0.52 | 1.002 |
| Relative Humidity | 259269 | 68.4 | 66.8 | 1.5 | 11.1 | 13.4 | 0.28 | 1.029 |
| Specific Humidity | 259281 | 9.9 | 9.5 | 0.4 | 1.2 | 1.5 | NA | 1.039 |
| Wind Speed | 257290 | 2.3 | 2.4 | -0.1 | 1.2 | 1.5 | 0.29 | 1.303 |
| Wind Direction | 257290 |  |  | 4.8 | 47.1 |  |  |  |

#### AR

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 329163 | 16.6 | 16.3 | 0.3 | 1.8 | 2.2 | 0.57 | 1.001 |
| Dew Point Temperature | 328968 | 10.0 | 9.9 | 0.1 | 1.9 | 2.3 | 0.53 | 1.000 |
| Relative Humidity | 328968 | 69.8 | 69.1 | 0.7 | 10.6 | 12.7 | 0.33 | 1.003 |
| Specific Humidity | 328984 | 9.3 | 9.2 | 0.1 | 1.0 | 1.2 | NA | 1.006 |
| Wind Speed | 316425 | 2.7 | 2.7 | -0.0 | 1.2 | 1.5 | 0.40 | 1.164 |
| Wind Direction | 316425 |  |  | -5.4 | 42.3 |  |  |  |

#### DC

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 12369 | 14.5 | 14.8 | -0.3 | 1.8 | 1.9 | 0.00 | 0.999 |
| Dew Point Temperature | 12369 | 6.4 | 6.5 | -0.2 | 2.3 | 2.3 | 0.00 | 1.000 |
| Relative Humidity | 12369 | 63.7 | 60.4 | 3.3 | 11.9 | 11.9 | 0.00 | 1.063 |
| Specific Humidity | 12369 | 7.2 | 7.4 | -0.2 | 1.0 | 1.0 | NA | 1.015 |
| Wind Speed | 12342 | 2.5 | 3.8 | -1.3 | 1.6 | 1.6 | 0.00 | 0.591 |
| Wind Direction | 12342 |  |  | 27.6 | 32.2 |  |  |  |

#### FL

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | - Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 677614 | 22.4 | 22.1 | 0.3 | 1.6 | 2.0 | 0.64 | 1.001 |
| Dew Point Temperature | 582110 | 16.4 | 16.1 | 0.3 | 1.9 | 2.4 | 0.48 | 1.001 |
| Relative Humidity | 582110 | 72.6 | 71.5 | 1.1 | 9.7 | 11.9 | 0.36 | 1.015 |
| Specific Humidity | 582161 | 12.4 | 12.3 | 0.1 | 1.3 | 1.7 | 0.51 | 1.009 |
| Wind Speed | 657579 | 2.6 | 3.2 | -0.5 | 1.4 | 1.7 | 0.41 | 0.918 |
| Wind Direction | 657579 |  |  | 0.2 | 41.7 |  |  |  |

#### GA

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 403666 | 18.7 | 18.2 | 0.4 | 1.9 | 2.4 | 0.57 | 1.002 |
| Dew Point Temperature | 399664 | 11.3 | 11.2 | 0.1 | 2.4 | 3.0 | 0.51 | 1.000 |
| Relative Humidity | 399653 | 67.1 | 67.6 | -0.5 | 11.6 | 14.4 | 0.29 | 0.995 |
| Specific Humidity | 399751 | 9.8 | 9.7 | 0.1 | 1.3 | 1.7 | 0.65 | 1.001 |
| Wind Speed | 388828 | 2.4 | 2.2 | 0.2 | 1.2 | 1.5 | 0.34 | 1.589 |
| Wind Direction | 388828 |  |  | -4.5 | 46.9 |  |  |  |

#### IA

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 791648 | 10.3 | 10.1 | 0.2 | 1.9 | 2.2 | 0.65 | 1.001 |
| Dew Point Temperature | 791118 | 5.0 | 4.7 | 0.4 | 2.2 | 2.6 | 0.50 | 1.001 |
| Relative Humidity | 791118 | 74.5 | 71.8 | 2.8 | 10.6 | 12.5 | 0.34 | 1.034 |
| Specific Humidity | 791118 | 7.2 | 7.1 | 0.1 | 0.9 | 1.1 | NA | 1.031 |
| Wind Speed | 789918 | 3.8 | 4.0 | -0.2 | 1.3 | 1.6 | 0.46 | 1.022 |
| Wind Direction | 789918 |  |  | -4.8 | 30.9 |  |  |  |

#### IL

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 812881 | 12.5 | 12.1 | 0.4 | 1.9 | 2.6 | 0.63 | 1.001 |
| Dew Point Temperature | 810870 | 6.5 | 5.6 | 0.9 | 2.5 | 3.4 | 0.45 | 1.003 |
| Relative Humidity | 810870 | 71.9 | 68.0 | 3.8 | 11.8 | 14.3 | 0.32 | 1.056 |
| Specific Humidity | 810884 | 7.5 | 7.2 | 0.3 | 1.0 | 1.4 | 0.61 | 1.061 |
| Wind Speed | 808417 | 3.2 | 3.6 | -0.3 | 1.3 | 1.8 | 0.41 | 0.997 |
| Wind Direction | 808417 |  |  | -7.7 | 40.4 |  |  |  |

#### IN

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 226668 | 11.9 | 11.9 | 0.0 | 1.8 | 2.1 | 0.58 | 1.000 |
| Dew Point Temperature | 211546 | 6.6 | 5.6 | 1.0 | 2.2 | 2.5 | 0.53 | 1.004 |
| Relative Humidity | 211546 | 74.2 | 68.3 | 5.9 | 11.7 | 13.5 | 0.35 | 1.089 |
| Specific Humidity | 211562 | 7.5 | 7.1 | 0.3 | 0.9 | 1.1 | NA | 1.077 |
| Wind Speed | 225658 | 3.2 | 3.7 | -0.5 | 1.3 | 1.6 | 0.36 | 1.083 |
| Wind Direction | 225658 |  |  | -9.8 | 36.3 |  |  |  |

#### KY

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 185936 | 14.6 | 14.6 | -0.0 | 1.9 | 2.2 | 0.49 | 1.000 |
| Dew Point Temperature | 185352 | 8.4 | 7.4 | 0.9 | 2.3 | 2.7 | 0.45 | 1.003 |
| Relative Humidity | 185352 | 70.9 | 65.3 | 5.6 | 12.6 | 14.8 | 0.27 | 1.090 |
| Specific Humidity | 185355 | 8.3 | 7.9 | 0.4 | 1.1 | 1.3 | NA | 1.069 |
| Wind Speed | 178798 | 2.5 | 2.6 | -0.1 | 1.2 | 1.5 | 0.30 | 1.404 |
| Wind Direction | 178798 |  |  | -19.9 | 44.1 |  |  |  |

#### LA

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 340415 | 20.1 | 19.8 | 0.3 | 1.8 | 2.2 | 0.51 | 1.001 |
| Dew Point Temperature | 325728 | 14.7 | 14.5 | 0.2 | 1.8 | 2.3 | 0.43 | 1.001 |
| Relative Humidity | 325728 | 75.0 | 74.1 | 0.8 | 10.3 | 12.6 | 0.26 | 1.011 |
| Specific Humidity | 325736 | 11.6 | 11.5 | 0.1 | 1.2 | 1.5 | NA | 1.010 |
| Wind Speed | 326209 | 2.6 | 2.6 | 0.1 | 1.2 | 1.5 | 0.39 | 1.355 |
| Wind Direction | 326209 |  |  | 0.1 | 41.9 |  |  |  |

#### MD

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 244761 | 13.7 | 13.8 | -0.0 | 1.9 | 2.3 | 0.53 | 1.000 |
| Dew Point Temperature | 227744 | 7.5 | 7.1 | 0.4 | 2.1 | 2.5 | 0.34 | 1.001 |
| Relative Humidity | 227743 | 69.8 | 67.4 | 2.4 | 11.6 | 13.8 | 0.27 | 1.046 |
| Specific Humidity | 227748 | 7.7 | 7.7 | 0.0 | 1.0 | 1.2 | NA | 1.031 |
| Wind Speed | 232396 | 2.9 | 2.9 | -0.0 | 1.3 | 1.6 | 0.32 | 1.322 |
| Wind Direction | 232396 |  |  | -13.1 | 42.1 |  |  |  |

#### MI

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 838372 | 8.2 | 8.4 | -0.1 | 1.9 | 2.3 | 0.68 | 1.000 |
| Dew Point Temperature | 719683 | 4.2 | 2.9 | 1.3 | 2.1 | 2.6 | 0.63 | 1.005 |
| Relative Humidity | 719683 | 77.9 | 70.7 | 7.2 | 11.7 | 13.9 | 0.42 | 1.112 |
| Specific Humidity | 720172 | 6.3 | 6.0 | 0.4 | 0.8 | 0.9 | NA | 1.099 |
| Wind Speed | 834088 | 3.7 | 3.6 | 0.1 | 1.4 | 1.7 | 0.44 | 1.164 |
| Wind Direction | 834088 |  |  | -13.1 | 37.7 |  |  |  |

#### MS

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 189575 | 19.1 | 18.6 | 0.5 | 1.8 | 2.2 | 0.54 | 1.002 |
| Dew Point Temperature | 189113 | 12.7 | 12.3 | 0.4 | 2.0 | 2.4 | 0.53 | 1.001 |
| Relative Humidity | 189113 | 70.7 | 69.9 | 0.8 | 10.5 | 12.5 | 0.31 | 1.011 |
| Specific Humidity | 189124 | 10.5 | 10.2 | 0.3 | 1.1 | 1.4 | NA | 1.024 |
| Wind Speed | 182594 | 2.4 | 2.5 | -0.1 | 1.2 | 1.5 | 0.38 | 1.366 |
| Wind Direction | 182594 |  |  | -2.4 | 43.7 |  |  |  |

#### NC

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 603286 | 16.8 | 16.4 | 0.3 | 1.9 | 2.3 | 0.62 | 1.001 |
| Dew Point Temperature | 594875 | 9.2 | 8.8 | 0.4 | 2.3 | 2.9 | 0.58 | 1.001 |
| Relative Humidity | 594875 | 65.8 | 64.7 | 1.0 | 11.5 | 13.9 | 0.37 | 1.024 |
| Specific Humidity | 594968 | 8.8 | 8.5 | 0.3 | 1.1 | 1.4 | 0.68 | 1.029 |
| Wind Speed | 583869 | 2.5 | 2.3 | 0.2 | 1.3 | 1.6 | 0.40 | 1.518 |
| Wind Direction | 583869 |  |  | -1.2 | 45.5 |  |  |  |

#### OH

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 339806 | 11.4 | 11.3 | 0.1 | 1.8 | 2.1 | 0.59 | 1.000 |
| Dew Point Temperature | 335165 | 6.0 | 5.2 | 0.8 | 2.2 | 2.6 | 0.38 | 1.003 |
| Relative Humidity | 335165 | 74.1 | 69.0 | 5.1 | 12.1 | 14.2 | 0.30 | 1.076 |
| Specific Humidity | 335177 | 7.2 | 6.9 | 0.2 | 0.9 | 1.1 | NA | 1.066 |
| Wind Speed | 333355 | 3.0 | 3.4 | -0.4 | 1.3 | 1.5 | 0.43 | 1.038 |
| Wind Direction | 333355 |  |  | -10.1 | 37.8 |  |  |  |

#### PA

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 371524 | 10.5 | 10.7 | -0.2 | 1.8 | 2.1 | 0.72 | 0.999 |
| Dew Point Temperature | 365735 | 6.1 | 4.0 | 2.1 | 2.8 | 3.6 | 0.39 | 1.008 |
| Relative Humidity | 365735 | 77.9 | 67.3 | 10.6 | 14.1 | 17.3 | 0.33 | 1.181 |
| Specific Humidity | 365743 | 7.2 | 6.5 | 0.7 | 1.0 | 1.3 | NA | 1.162 |
| Wind Speed | 353747 | 2.8 | 2.9 | -0.2 | 1.3 | 1.6 | 0.41 | 1.133 |
| Wind Direction | 353747 |  |  | -4.4 | 42.2 |  |  |  |

#### SC

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 177570 | 18.2 | 17.9 | 0.3 | 1.7 | 2.1 | 0.44 | 1.001 |
| Dew Point Temperature | 168137 | 10.3 | 10.1 | 0.2 | 2.2 | 2.6 | 0.62 | 1.001 |
| Relative Humidity | 168128 | 65.3 | 64.5 | 0.8 | 10.7 | 12.7 | 0.35 | 1.019 |
| Specific Humidity | 168145 | 9.3 | 9.1 | 0.2 | 1.1 | 1.3 | NA | 1.018 |
| Wind Speed | 170346 | 2.6 | 2.9 | -0.3 | 1.2 | 1.5 | 0.33 | 1.148 |
| Wind Direction | 170346 |  |  | -0.2 | 41.2 |  |  |  |

#### TN

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 167722 | 16.3 | 16.1 | 0.1 | 1.8 | 2.2 | 0.64 | 1.001 |
| Dew Point Temperature | 167612 | 9.2 | 8.6 | 0.6 | 2.1 | 2.5 | 0.52 | 1.002 |
| Relative Humidity | 167612 | 67.7 | 64.4 | 3.3 | 11.4 | 13.6 | 0.35 | 1.052 |
| Specific Humidity | 167631 | 8.7 | 8.4 | 0.3 | 1.1 | 1.3 | NA | 1.043 |
| Wind Speed | 160097 | 2.4 | 2.4 | -0.0 | 1.3 | 1.5 | 0.34 | 1.440 |
| Wind Direction | 160097 |  |  | -15.0 | 46.4 |  |  |  |

#### VA

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 403095 | 14.7 | 14.7 | -0.0 | 2.0 | 2.4 | 0.62 | 1.000 |
| Dew Point Temperature | 384814 | 7.4 | 7.5 | -0.1 | 2.2 | 2.8 | 0.54 | 1.000 |
| Relative Humidity | 384813 | 66.5 | 65.9 | 0.5 | 12.0 | 14.6 | 0.31 | 1.008 |
| Specific Humidity | 387698 | 7.9 | 8.0 | -0.1 | 1.1 | 1.3 | 0.75 | 0.987 |
| Wind Speed | 391915 | 2.6 | 2.5 | 0.1 | 1.3 | 1.6 | 0.43 | 1.289 |
| Wind Direction | 391915 |  |  | -1.9 | 47.0 |  |  |  |

#### WI

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 702092 | 7.3 | 7.5 | -0.1 | 1.9 | 2.3 | 0.62 | 1.000 |
| Dew Point Temperature | 672962 | 3.2 | 2.0 | 1.2 | 2.3 | 2.7 | 0.54 | 1.004 |
| Relative Humidity | 672962 | 78.1 | 71.0 | 7.1 | 12.0 | 14.2 | 0.33 | 1.114 |
| Specific Humidity | 672975 | 6.2 | 6.0 | 0.2 | 0.8 | 0.9 | NA | 1.097 |
| Wind Speed | 696772 | 3.6 | 3.3 | 0.3 | 1.2 | 1.6 | 0.42 | 1.277 |
| Wind Direction | 696772 |  |  | -2.3 | 36.0 |  |  |  |

#### WV

| Field | Number of Obs | Model Mean | Obs Mean | Mean Bias | Mean Absolute Error | Root-Mean-Squared Error | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Temperature | 174258 | 12.1 | 12.2 | -0.1 | 2.0 | 2.4 | 0.54 | 1.000 |
| Dew Point Temperature | 174172 | 7.0 | 5.7 | 1.3 | 2.4 | 2.9 | 0.41 | 1.005 |
| Relative Humidity | 174172 | 75.1 | 68.4 | 6.7 | 13.6 | 16.1 | 0.26 | 1.116 |
| Specific Humidity | 174172 | 7.8 | 7.3 | 0.5 | 1.1 | 1.3 | NA | 1.100 |
| Wind Speed | 167864 | 2.7 | 2.0 | 0.6 | 1.5 | 1.8 | 0.26 | 2.436 |
| Wind Direction | 167864 |  |  | -28.9 | 53.5 |  |  |  |

Disgregarding wind direction, there is excellent model performance in the tables above relative to the target metrics. All variables for each region have error values (mean bias or MAE/RMSE) less than or equal to the target metrics, except for the one isolated case of Washington DC’s wind speed bias. Note that the sample size for this region is between one and two orders of magnitude smaller than for other regions and oftentimes there is only one available observing site.

Across most regions, wind direction MAE are larger than the target of 30 degrees, though most often this statistic is marginally outside the target range. The mean bias, on the other hand, does not indicate a chronic problem with the modeled wind vectors, thus the large MAE values may be associated in general with poor model representation of topography, including coastlines and perhaps the occurrence and timing of sea breezes.

# Evaluation of Sensitivity Runs Meteorological Model Performance

## Statistical analysis of WRF sensitivity runs

The model configurations for each sensitivity run, as retrieved from raw WRF netcdf files, are listed in the following table. Note that sensitivity run 7 was not available for the summer period.

| CENTER |  | Sensitivity run number/name | MOIST PHYSICS | LONG WAVE | SHORT WAVE | LAND SURFACE | SOIL LAYERS | PBL | SURFACE LAYER | CUMULUS |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| IA | 1 - px-acm2_wsm5 |  | WSM5(4) | RRTM(1) | Dudhia(1) | PX(7) | 2 | ACM2(7) | PX(7) | KF (1) |
| IA | 2 - px-acm2_wsm6 |  | WSM6(6) | RRTM(1) | Dudhia(1) | PX(7) | 2 | ACM2(7) | PX(7) | KF (1) |
| IA | 3 - px_acm2_morr_ rrtmg |  | Morrison (10) | RRTMG(4) | RRTMG(4) | PX(7) | 2 | ACM2(7) | PX(7) | KF (1) |
| IA | 4 - px-acm2_morr_ rrtmg_ipxwf |  | Morrison (10) | RRTMG(4) | RRTMG(4) | PX(7) IPXWRF | 2 | ACM2(7) | PX(7) | KF (1) |
| NY/ UMD | 5 - myj.wsm5 |  | WSM5(4) | RRTM(1) | Dudhia(1) | Noah(2) | 4 | MYJ(2) | MO(2) | KF (1) |
| NY/ UMD | 6 - myj.wsm6 |  | WSM6(6) | RRTM(1) | Dudhia(1) | Noah(2) | 4 | MYJ(2) | MO(2) | KF (1) |
| NY/ UMD | 7 - ysu.wsm5 |  | WSM5(4) | RRTM(1) | Dudhia(1) | Noah(2) | 4 | YSU(1) | MO(1) | KF (1) |
| NY/ UMD | 8 - ysu.wsm6 |  | WSM6(6) | RRTM(1) | Dudhia(1) | Noah(2) | 4 | YSU(1) | MO(1) | KF (1) |
| NY/ UMD | 9 - OTC |  | WSM6(6) | RRTM(1) | Dudhia(1) | PX(7) | 2 | Blackadar (11) | PX(7) | KF (1) |

Based on experience, clearly identifying a single model configuration that minimizes model error for all meteorological fields for large geographical areas and for long periods of time is frequently a futile exercise. Model errors for a specific choice of model parameters can only be minimized over individual runs for short periods of time in limited geographical areas. Results of the objective statistical analysis for the nine sensitivity runs presented to AER bear out the belief that there are simply no clearly identifiable ‘best’ model configurations that minimize errors in all fields.

AER does not disagree with the method that identified sensitivity run 3 (px_acm2_morr_rrtmg) as the ‘best’ one, and therefore the one that should be used as the Production Run. It is important to remember that, in general, the best means to minimize model error is to use a state-of-the-art model with accepted combinations of recent model physics’ schemes, since some are not compatible with others either by software design or because of scientific limitations. Acceptance is conveyed by the model and physics algorithm developers and peer-reviewed literature. All configurations above meet these basic requirements, except the modified Blackadar scheme. This scheme from the OTC run is not available as a standard PBL option in WRF and therefore lacks both a long history of use in conjunction with the other options and has not undergone full vetting by the WRF Developers’ Committee.

### Tasks 2 H 1 and 2 H 7 Statistical tables for FULL model domain

This section describes the performance of sensitivity runs of the WRF model as characterized by tables of their error statistics.  The discussion here focuses on statistics computed over the entire model domain (“FULL”) for model domain 2, at four pressure levels and at the surface.  These analyses are performed for the summer and winter episodes as described above.  The pressure levels analyzed with approximate height levels in parentheses are: 300 mb (9500 m), 500 mb (5750 m), 700 mb (3000 m), and 850 mb (1500 m).

#### 300 mb

The temperature fields at 300 mb for all of the sensitivity runs including the production run are very similar although the bias and standard errors are smallest for the production run itself.   The WRF temperatures are highly correlated with the observations.  The statistics are quite similar between the summer and winter episode.  The sensitivity runs have a similar bias to the production run in the relative humidity field at 300 mb at roughly a 10% positive bias.  The mean absolute error for the production run is slightly larger than the other sensitivity runs, upwards of over 2% larger.  The relative humidity field at this level is, however, not well correlated with the observed relative humidity with a correlation coefficient of approximately 0.6.  The mean absolute errors are also slightly larger for the winter episode.

Temperature

Summer

Winter

Relative humidity

Summer

Winter

For the wind field statistics, the production run and the other sensitivity runs display a slight negative bias in their wind speeds relative to observed values.  The mean absolute errors for the wind speeds average slightly less than 1.5 m s-1 for the summer episode, but are slightly larger for the winter episode at just under 2.0 m s-1.  The wind speeds from the production run have the least error in either season.  The WRF wind speeds are highly correlated with the observations at 0.97 and 0.98 for the summer and winter, respectively.   For the wind direction, the production and sensitivity runs have very small biases and the mean absolute errors range from 4-5 degrees in the winter episode to 8-9 degrees in the summer episode.

Wind speed

Summer

Winter

Wind Direction

Summer

Winter

#### 500 mb

The statistics from the WRF sensitivity runs at 500 mb for temperature are quite similar to those at the 300-mb level.  The temperature biases are smaller than at 500 mb and range from 0.1-0.2 C in the winter episode to 0.3-0.4 C in the summer episode.  The production run has the smallest mean absolute errors of all of the runs.  The correlation coefficients for all runs remain very high at 0.98 or above.  The relative humidity fields show significant improvement over those at the 300-mb level with mean bias reduced to 2% and 6% above the observed value for summer and winter, respectively.  The mean absolute errors and the correlation coefficients are better for the relative humidity than at 300 mb over both the summer and winter episodes.

Temperature

Summer

Winter

Relative Humidity

Summer

Winter

The wind speeds at 500 mb are strongly correlated with the observations during both the summer and winter sensitivity runs.  The mean absolute error for both periods is slightly reduced as compared with 300 mb, but this may be more due to the lower average wind speed at the 500-mb level.  The mean absolute error in the wind direction increases slightly at 500 mb, but of all the sensitivity runs the production run has the lowest errors at 9.8° and 5.4 m s-1, respectively, for the summer and winter episodes, respectively.

Wind Speed

Summer

Winter

Wind Direction

Summer

Winter

#### 700 mb

At 700 mb, the temperature field shows a slight positive bias of 0.1-0.2 C, but the mean absolute errors remain small especially for the production run.  The mean absolute errors in temperature are slightly larger during the winter episode, but the correlation coefficients indicate a very good relationship with the observations during both periods.  In the relative humidity field, there is a slight positive bias during the winter episode; however, this is significantly smaller than either at 300 mb or 500 mb.  A negative bias appears for the summer episode at nearly 3% lower than the observed relative humidity on average.  Mean absolute errors are greater than those at 500 mb for the summer episode at over 13%, but are reduced for the winter period.  Correlation coefficients are similar in this respect with the values for the summer episode especially poor at around 0.65.

Temperature

Summer

Winter

Relative Humidity

Summer

Winter

For the wind speeds at 700 mb, the biases remain less than 1 m s-1 during the winter episode.  The mean absolute errors are also relatively small at 700 mb with the production run displaying the smallest errors of all the sensitivity runs.  The modeled wind speeds at this level are still well correlated with the observations for both the summer and winter episodes.  The wind direction errors at 700 mb are increased relative to those at 500 mb.

Wind Speed

Summer

Winter

Wind Direction

Summer

Winter

#### 850 mb

The temperatures at 850 mb have mean absolute errors near 0.5 C for both the summer and winter episode with the production run observed to have the smallest errors of the sensitivity runs.  The mean bias is very small during the summer period, but has a slight positive bias during the winter period.  Temperatures remain very well correlated with the observations at the 850-mb level.  The relative humidity at this level now displays a consistent negative bias for all of the sensitivity runs during both the summer and winter episodes.  Mean absolute errors run just over 11% for both episodes.  The sensitivity runs are poorly correlated with the observations during the summer period averaging near 0.6; however, this does improve during the winter up to just above 0.8.

Temperature

Summer

Winter

Relative Humidity

Summer

Winter

As with the other pressure levels, the wind speed continues to show a consistent negative bias of between 0.5 and 1 m s-1.   The mean absolute errors remain small with the production run having the smallest error during the winter episode.  The wind speeds remain well correlated with the observed wind speeds; however, the correlation coefficients are not as high as at the other pressure levels.  The wind direction errors are larger lower in the atmosphere.  The production run has the smallest errors, but these are still above 11 for the winter episode and above 15 for the summer episode with larger values found for each of the other sensitivity runs.

Wind Speed

Summer

Winter

Wind Direction

Summer

Winter

#### Surface

At the surface, temperatures display a slight, but consistent negative bias during the summer episode with a less obvious bias during the winter period although the production run has a positive bias of 0.6C.  The mean absolute errors increase significantly at the surface relative to those at 850 mb to around 1.5C in the summer episode and 2C in the winter episode.  The majority of the sensitivity runs including the production run have similar performance in this regard.  The temperatures are more strongly correlated in the winter episode than during the summer.  The relative humidity fields generally have a positive bias for both periods, but somewhat larger for the winter episode.  A number of the sensitivity runs perform more strongly than the production run during the winter with mean absolute errors 1-2% lower.  The WRF relative humidity fields are poorly correlated with the observations at the surface with the majority of runs falling under a 0.6 correlation coefficient.  An additional variable, specific humidity, is also analyzed at the surface.  There is a slight, but consistent positive bias in the specific humidity at the surface during the winter period, but the bias varies between the different sensitivity runs for the summer.  The mean error is smaller during the winter, but this is likely a result of lower moisture content in the atmosphere at this time of year.  As with the relative humidity, the specific humidity modeled by WRF sensitivity runs is more strongly correlated with the observations during the winter than summer.

Temperature

Summer

Winter

Relative Humidity

Summer

Winter

Specific Humidity

Summer

Winter

The wind speeds are biased negative for the production run in both the winter and summer episodes, but the remaining sensitivity runs are consistently positive although the absolute value of the bias is relatively small in both cases.  The mean absolute errors are between 1.2 and 1.4 ms-1.  The correlation coefficients are generally low indicating that the relationship between the WRF-modeled and observed winds is not particularly strong.  The wind direction data shows the largest deviations between the sensitivity runs and the actual observations with mean absolute errors exceeding 30 in the winter episode and 40 in the summer episode.

Wind Speed

Summer

Winter

Wind Direction

Summer

Winter

### Tasks 2 H 1 and 2 H 7 Surface Statistical tables for SEMAP region

The following tables summarize the domain 2 Summer and Winter statistics at the surface for each sensitivity run for SEMAP. The discussion is limited to noteworthy aspects of the statistics, with a focus on those metrics that can be compared to the target Metrics from Section 2.6.

#### Temperature

Summer Temperature Celsius

| Run | - Number of Obs | Model Mean | Observed Mean | Mean Bias | Mean Absolute Error | Root Mean Square Deviation | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| production | 176141 | 27.5 | 27.6 | -0.1 | 1.5 | 2.0 | 0.68 | 1.000 |
| 1-px-acm2_wsm5 | 176141 | 27.4 | 27.6 | -0.2 | 1.5 | 2.0 | 0.69 | 0.999 |
| 2-px-acm2_wsm6 | 176141 | 27.4 | 27.6 | -0.2 | 1.5 | 2.0 | 0.69 | 0.999 |
| 3-px_acm2_morr_rrtmg | 176141 | 27.5 | 27.6 | -0.1 | 1.5 | 1.9 | 0.69 | 1.000 |
| 4-px-acm2_morr_rrtmg_ipxwrf | 176141 | 27.8 | 27.6 | 0.2 | 1.6 | 2.1 | 0.67 | 1.001 |
| 5-myj.wsm5 | 176141 | 27.5 | 27.6 | -0.1 | 1.5 | 2.0 | 0.71 | 1.000 |
| 6-myj.wsm6 | 176141 | 27.5 | 27.6 | -0.1 | 1.5 | 2.0 | 0.71 | 1.000 |
| 7-ysu.wsm5 | 0 | NA | NA | NA | NA | NA | NA | NA |
| 8-ysu.wsm6 | 176141 | 27.6 | 27.6 | 0.0 | 1.6 | 2.1 | 0.70 | 1.000 |
| 9-OTC | 176141 | 27.2 | 27.6 | -0.4 | 1.5 | 1.9 | 0.71 | 0.999 |

Winter Temperature Celsius

| Run | Number of Obs | Model Mean | Observed Mean | Mean Bias | Mean Absolute Error | Root Mean Square Deviation | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| production | 199175 | 11.4 | 11.0 | 0.4 | 2.1 | 2.7 | 0.89 | 1.001 |
| 1-px-acm2_wsm5 | 199175 | 11.0 | 11.0 | 0.0 | 2.1 | 2.7 | 0.89 | 1.000 |
| 2-px-acm2_wsm6 | 199175 | 11.0 | 11.0 | 0.0 | 2.1 | 2.7 | 0.89 | 1.000 |
| 3-px_acm2_morr_rrtmg | 199175 | 11.3 | 11.0 | 0.3 | 2.1 | 2.6 | 0.89 | 1.001 |
| 4-px-acm2_morr_rrtmg_ipxwrf | 199175 | 11.5 | 11.0 | 0.5 | 2.2 | 2.8 | 0.89 | 1.002 |
| 5-myj.wsm5 | 199175 | 10.6 | 11.0 | -0.4 | 1.8 | 2.3 | 0.90 | 0.999 |
| 6-myj.wsm6 | 199175 | 10.6 | 11.0 | -0.4 | 1.8 | 2.4 | 0.90 | 0.999 |
| 7-ysu.wsm5 | 199175 | 10.5 | 11.0 | -0.5 | 1.8 | 2.3 | 0.91 | 0.998 |
| 8-ysu.wsm6 | 199175 | 10.5 | 11.0 | -0.5 | 1.8 | 2.3 | 0.91 | 0.998 |
| 9-OTC | 199175 | 11.2 | 11.0 | 0.2 | 1.9 | 2.4 | 0.91 | 1.001 |

Model performance was generally very good for all sensitivity runs for temperature. Summer correlation values were notably lower than for winter. It is anticipated that correlation values will be lower during periods of more uniform temperature, such as during the summer, with short-term periods of cooling driven largely by less predictable diurnal convection. Mean bias values fell within target values, as did MAE values, except for the winter period for runs that utilized the px-acm2 parameterizations. These runs had gross errors of slightly larger than the target of 2 degrees Celsius.

#### Relative Humidity

Summer RH %

| Run | Number of Obs | Model Mean | Observed Mean | Mean Bias | Mean Absolute Error | Root Mean Square Deviation | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| production | 167793 | 72.4 | 69.3 | 3.1 | 9.7 | 12.3 | 0.43 | 1.049 |
| 1-px-acm2_wsm5 | 167793 | 72.4 | 69.3 | 3.1 | 10.0 | 12.6 | 0.45 | 1.048 |
| 2-px-acm2_wsm6 | 167793 | 72.4 | 69.3 | 3.1 | 10.0 | 12.6 | 0.45 | 1.048 |
| 3-px_acm2_morr_rrtmg | 167793 | 72.4 | 69.3 | 3.1 | 10.0 | 12.6 | 0.44 | 1.048 |
| 4-px-acm2_morr_rrtmg_ipxwrf | 167793 | 73.4 | 69.3 | 4.1 | 10.3 | 12.9 | 0.38 | 1.070 |
| 5-myj.wsm5 | 167793 | 70.2 | 69.3 | 0.9 | 9.5 | 12.1 | 0.48 | 1.015 |
| 6-myj.wsm6 | 167793 | 70.1 | 69.3 | 0.8 | 9.5 | 12.1 | 0.48 | 1.015 |
| 7-ysu.wsm5 | 0 | NA | NA | NA | NA | NA | NA | NA |
| 8-ysu.wsm6 | - 167793 | 64.9 | 69.3 | -4.4 | 9.4 | 12.2 | 0.51 | 0.935 |
| 9-OTC | 167793 | 73.6 | 69.3 | 4.3 | 10.5 | 13.3 | 0.45 | 1.069 |

Winter RH %

| - Run | Number of Obs | Model Mean | Observed Mean | Mean Bias | Mean Absolute Error | Root Mean Square Deviation | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| production | 188866 | 74.6 | 75.5 | -0.9 | 12.1 | 15.2 | 0.46 | 0.980 |
| 1-px-acm2_wsm5 | 188866 | 77.2 | 75.5 | 1.7 | 12.4 | 15.5 | 0.44 | 1.019 |
| 2-px-acm2_wsm6 | 188866 | 77.1 | 75.5 | 1.7 | 12.4 | 15.5 | 0.44 | 1.019 |
| 3-px_acm2_morr_rrtmg | 188866 | 76.0 | 75.5 | 0.5 | 11.8 | 14.9 | 0.46 | 1.000 |
| 4-px-acm2_morr_rrtmg_ipxwrf | 188866 | 77.3 | 75.5 | 1.9 | 12.0 | 15.1 | 0.41 | 1.019 |
| 5-myj.wsm5 | 188866 | 79.3 | 75.5 | 3.8 | 9.6 | 12.2 | 0.49 | 1.056 |
| 6-myj.wsm6 | 188866 | 79.5 | 75.5 | 4.0 | 9.8 | 12.4 | 0.48 | 1.060 |
| 7-ysu.wsm5 | 188866 | 76 | 75.5 | 0.5 | 8.9 | 11.5 | 0.57 | 1.010 |
| 9-ysu.wsm6 | 188866 | 76.2 | 75.5 | 0.7 | 9.0 | 11.6 | 0.56 | 1.013 |
| 9-OTC | 188866 | 76.2 | 75.5 | 0.8 | 11.6 | 14.6 | 0.50 | 1.005 |

Modeled relative humidity forecasts typically exhibited a positive bias of up to 4%, as indicated also by the multiplicative bias scores. It should be kept in mind that relative humidity is dependent not only on the absolute amount of atmospheric moisture, but also the temperature. See the specific humidity tables below for evaluation solely of the amount of moisture. Use of the MYJ planetary boundary layer scheme improved the modeled RH fields in the summer, with the best scores across all statistics. In the winter, on the other hand, this scheme’s large positive mean bias values, but relatively small MAE errors, indicate that the sign of the forecast errors did not cancel and RH was generally overpredicted.  The YSU PBL scheme had the best error statistics in the winter.

#### Specific Humidity

Summer SPFH g/kg

| Run | Number of Obs | Model Mean | Observed Mean | Mean Bias | Mean Absolute Error | Root Mean Square Deviation | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| production | 167951 | 16.1 | 15.9 | 0.3 | - 1.7 | 2.2 | 0.56 | - 1.017 |
| 1-px-acm2_wsm5 | 167951 | 15.9 | 15.9 | 0.1 | 1.8 | 2.2 | 0.59 | 1.005 |
| 2-px-acm2_wsm6 | 167951 | 15.9 | 15.9 | 0.1 | 1.8 | 2.2 | 0.59 | 1.005 |
| 3-px_acm2_morr_rrtmg | 167951 | 16.1 | 15.9 | 0.2 | 1.8 | 2.3 | 0.57 | 1.013 |
| 4-px-acm2_morr_rrtmg_ipxwrf | 167951 | 16.8 | 15.9 | 0.9 | 2.0 | 2.5 | 0.50 | 1.057 |
| 5-myj.wsm5 | 167951 | 15.5 | 15.9 | -0.3 | 1.8 | 2.3 | 0.61 | 0.980 |
| 6-myj.wsm6 | 167951 | 15.5 | 15.9 | -0.3 | 1.8 | 2.3 | 0.61 | 0.979 |
| 7-ysu.wsm5 | 0 | NA | NA | NA | NA | NA | NA | NA |
| 8-ysu.wsm6 | 167951 | 14.4 | 15.9 | -1.4 | 2.0 | 2.5 | 0.62 | 0.909 |
| 9-OTC | 167951 | 16.1 | 15.9 | 0.3 | 1.9 | 2.4 | 0.57 | 1.016 |

Winter SPFH g/kg

| Run | Number of Obs | Model Mean | Observed Mean | Mean Bias | Mean Absolute Error | Root Mean Square Deviation | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| production | 189058 | 6.7 | 6.8 | -0.1 | 1.0 | 1.2 | 0.83 | 0.983 |
| 1-px-acm2_wsm5 | 189058 | 6.8 | 6.8 | 0.0 | 0.9 | 1.2 | 0.84 | 0.997 |
| 2-px-acm2_wsm6 | 189058 | 6.7 | 6.8 | 0.0 | 0.9 | 1.2 | 0.84 | 0.997 |
| 3-px_acm2_morr_rrtmg | 189058 | 6.8 | 6.8 | 0.0 | 0.9 | 1.2 | 0.84 | 0.999 |
| 4-px-acm2_morr_rrtmg_ipxwrf | 189058 | 7.0 | 6.8 | 0.2 | 0.9 | 1.2 | 0.83 | 1.027 |
| 5-myj.wsm5 | 189058 | 6.9 | 6.8 | 0.1 | 0.8 | 1.1 | 0.85 | 1.023 |
| 6-myj.wsm6 | 189058 | 6.9 | 6.8 | 0.1 | 0.8 | 1.1 | 0.85 | 1.023 |
| 7-ysu.wsm5 | 189058 | 6.6 | 6.8 | -0.2 | 0.8 | 1.1 | 0.85 | 0.970 |
| 8-ysu.wsm6 | 189058 | 6.6 | 6.8 | -0.2 | 0.8 | 1.1 | 0.85 | 0.970 |
| 9-OTC | 189058 | 6.9 | 6.8 | 0.1 | 0.9 | 1.2 | 0.84 | 1.005 |

All sensitivity runs met the target values for mean bias and mean absolute error (with one exception). Use of the YSU PBL scheme appears to result in the lowest amount of atmospheric moisture, perhaps related to deficiencies in this scheme’s amount of vertical mixing. As expected, error statistics for the winter are smaller, due largely to the significantly smaller amount of atmospheric moisture during this time period. It is suggested, therefore, that statistics for the summer period represent a more rigorous evaluation. Of note is the degradation of the model performance (e.g., increased positive mean bias) through introduction of the IPX-WRF (runs number 4). The IPXWRF software is used to initialize soil moisture in the PX land surface model.

#### Wind Speed

Summer WSPD m/s

| Run | Number of Obs | Model Mean | Observed Mean | Mean Bias | Mean Absolute Error | Root Mean Square Deviation | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| production | 168497 | 1.9 | 1.8 | 0.1 | - 1.3 | 1.6 | 0.35 | 1.319 |
| 1-px-acm2_wsm5 | 168497 | 2.2 | 1.8 | 0.3 | 1.2 | 1.5 | 0.40 | 1.462 |
| 2-px-acm2_wsm6 | 168497 | 2.2 | 1.8 | 0.3 | 1.2 | 1.5 | 0.40 | 1.462 |
| 3-px_acm2_morr_rrtmg | 168497 | 2.2 | 1.8 | 0.3 | 1.2 | 1.5 | 0.40 | 1.468 |
| 4-px-acm2_morr_rrtmg_ipxwrf | 168497 | 2.2 | 1.8 | 0.3 | 1.2 | 1.6 | 0.41 | 1.475 |
| 5-myj.wsm5 | 168497 | 2.3 | 1.8 | 0.5 | 1.2 | 1.5 | 0.42 | 1.496 |
| 6-myj.wsm6 | 168497 | 2.3 | 1.8 | 0.5 | 1.2 | 1.5 | 0.42 | 1.496 |
| 7-ysu.wsm5 | 0 | NA | NA | NA | NA | NA | NA | NA |
| 8-ysu.wsm6 | 168497 | 2.1 | 1.8 | 0.2 | 1.2 | 1.5 | 0.41 | 1.395 |
| 9-OTC | 168497 | 1.6 | 1.8 | -0.3 | 1.2 | 1.6 | 0.40 | 1.015 |

Winter WSPD m/s

| Run | Number of Obs | Model Mean | Observed Mean | Mean Bias | Mean Absolute Error | Root Mean Square Deviation | Correlation Coefficient | Multiplicative Bias |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| production | 194801 | 2.6 | 2.5 | 0.0 | 1.2 | 1.5 | 0.57 | 1.085 |
| 1-px-acm2_wsm5 | 194801 | 2.8 | 2.5 | 0.3 | 1.2 | 1.5 | 0.62 | 1.194 |
| 2-px-acm2_wsm6 | 194801 | 2.8 | 2.5 | 0.3 | 1.2 | 1.5 | 0.62 | 1.195 |
| 3-px_acm2_morr_rrtmg | 194801 | 2.9 | 2.5 | 0.3 | 1.2 | 1.5 | 0.61 | 1.207 |
| 4-px-acm2_morr_rrtmg_ipxwrf | 194801 | 2.9 | 2.5 | 0.3 | 1.2 | 1.5 | 0.61 | 1.209 |
| 5-myj.wsm5 | 194801 | 3.0 | 2.5 | 0.5 | 1.2 | 1.5 | 0.62 | 1.257 |
| 6-myj.wsm6 | 194801 | 3.0 | 2.5 | 0.5 | 1.2 | 1.5 | 0.61 | 1.255 |
| 7-ysu.wsm5 | 194801 | 2.9 | 2.5 | 0.4 | 1.3 | 1.6 | 0.60 | 1.241 |
| 8-ysu.wsm6 | 194801 | 2.9 | 2.5 | 0.4 | 1.3 | 1.6 | 0.60 | 1.241 |
| 9-OTC | 194801 | 2.5 | 2.5 | 0.0 | 1.2 | 1.5 | 0.60 | 1.034 |

All sensitivity runs for each time period met the target mean bias and RMSE ranges for wind speed.  MAE and RMSE differences amongst the runs were unimportant. Overall mean bias errors were small and positive, likely influenced by the models’ well-known inability to reproduce coincident observations of calm air.  Interestingly, the production run had the smallest overall mean bias, especially when compared against the sensitivity run that used same model configuration. As mentioned above, one factor contributing to this difference in error statistics is the different start times for each 5.5-day run segment. Overall, the correlation coefficient scores were significantly better during the winter, likely a result of the predominance of more robust synoptic-scale flow.

#### Wind Direction

Summer WDIR degrees

| Run | Number of Obs | Model Mean | Observed Mean | Mean Bias | Mean Absolute Error |
| --- | --- | --- | --- | --- | --- |
| production | 168497 | -128.3 | -132.4 | 4.1 | 54.7 |
| 1-px-acm2_wsm5 | 168497 | -132.1 | -132.4 | 0.3 | 52.6 |
| 2-px-acm2_wsm6 | 168497 | -131.8 | -132.4 | 0.6 | 52.6 |
| 3-px_acm2_morr_rrtmg | 168497 | -133.6 | -132.4 | -1.2 | 52.5 |
| 4-px-acm2_morr_rrtmg_ipxwrf | 168497 | -137.7 | -132.4 | -5.3 | 52.3 |
| 5-myj.wsm5 | 168497 | -132.1 | -132.4 | 0.3 | 51.7 |
| 6-myj.wsm6 | 168497 | -132.2 | -132.4 | 0.2 | 51.7 |
| 7-ysu.wsm5 | 0 | NA | NA | NA | NA |
| 8-ysu.wsm6 | 168497 | -133.6 | -132.4 | -1.2 | 52.0 |
| 9-OTC | 168497 | -130.7 | -132.4 | 1.7 | 52.1 |

Winter WDIR degrees

| Run | Number of Obs | Model Mean | Observed Mean | Mean Bias | Mean Absolute Error |
| --- | --- | --- | --- | --- | --- |
| production | 194801 | -29.0 | -48.0 | 19.0 | 40.9 |
| 1-px-acm2_wsm5 | 194801 | -13.0 | -48.0 | 35.0 | 38.2 |
| 2-px-acm2_wsm6 | 194801 | -14.1 | -48.0 | 33.9 | 38.3 |
| 3-px_acm2_morr_rrtmg | 194801 | -26.5 | -48.0 | 21.5 | 38.4 |
| 4-px-acm2_morr_rrtmg_ipxwrf | 194801 | -31.7 | -48.0 | 16.4 | 38.5 |
| 5-myj.wsm5 | 194801 | -23.6 | -48.0 | 24.4 | 38.2 |
| 6-myj.wsm6 | 194801 | -22.8 | -48.0 | 25.2 | 38.2 |
| 7-ysu.wsm5 | 194801 | - -71.8 | -48.0 | -23.8 | 38.4 |
| 8-ysu.wsm6 | 194801 | -71.7 | -48.0 | -23.7 | 38.4 |
| 9-OTC | 194801 | -77.6 | -48.0 | -29.6 | 38.2 |

The production run, and closely-related sensitivity run number three, exhibit two of the smallest mean bias values during the winter when none of the bias values lie within the target of less than 10 degrees. For the summer, given that the observations for METAR sites are provided only to the nearest 5 degrees, we consider the variability in the mean bias values to be unimportant. For both time periods, MAE errors lie outside the target value of 30 degrees. The inter-run variability is very small and there is no clear “best” choice based solely on that metric.

### Task 2 H 2: Bakergrams

The following section describes the variability of the daily mean absolute error and the daily mean bias for the sensitivity runs through a series of Bakergrams.  The discussion will focus primarily on the FULL region for domain 2 only.

#### Temperature

The mean absolute errors of the temperature during summer are indistinguishable for the different sensitivity runs, all showing an MAE between 1.5C and 2C.  All the sensitivity runs for this period except for the px-acm2_morr_rrtmg_ixpwf demonstrate a consistent negative bias for temperature in their 20-day simulation.  The px-acm2_morr_rrtmg_ixpwf has a slight positive bias near the end of the run.  For the winter period, the mean absolute errors are somewhat larger overall, ranging from 1.5C to 3.0C although there is slightly greater disparity between the individual runs.  The OTC run exhibits the lowest mean absolute errors.  The sensitivity runs are split in terms of the mean bias in temperature during winter: about half the members show a positive bias, and the other half shows a negative bias.

Summer

Winter

#### Relative Humidity

The relative humidity fields in the summer sensitivity runs show that the mean absolute error averages less than 15% for all of the runs.  There is some variability among the individual members, but no significant outliers.  The mean bias plot for the summer episode indicates that there is a neutral to slight positive bias for all of the runs except for the ysu.wsm6 run which has a few daily values that represent a negative bias.  Overall, the winter daily values of the mean absolute errors are slightly larger than during summer, upwards of 15-20% in the first 4 sensitivity runs.  The mean bias is fairly uniform, however, with all of the runs showing a consistent 5-10% positive bias in the relative humidity.

Summer

Winter

#### Wind Speed

The summer episode of the sensitivity runs shows that the absolute mean error of the wind speed is very uniform among the individual members.  An error of 1-1.5 m s-1 is given for each of the 8 members with wind speed data at all 20 days of the sensitivity run.  The Bakergram for the mean bias during the summer period does not show any significant bias for any of the individual runs.  The winter sensitivity runs have slightly larger mean absolute errors, but as with the production run this is likely only due to higher average wind speeds during the winter.  A few of the daily values for 4 of the sensitivity runs during this period exhibit a slight positive bias, but otherwise the bias appears to be quite small.

Summer

Winter

### Task 2 H 4: Mean bias plots

This section describes the mean bias for each of the meteorological variables simulated by the WRF model for a summer and winter episode each consisting of 20 consecutive days.  The production run, px_acm2_morr_rrtmg, is the red curve in the following plots.  The discussion here focuses on the whole domain (“FULL”) - plots for geographic sub-regions are available from the website.

#### Temperature

The sensitivity runs are generally in line with the production run for the summer episode with a slightly negative mean temperature bias indicating the simulations were slightly too cold.  The px-acm2_morr_rrtmg_ipxwrf run contoured in green was the single outlier, but consistently the best run during this episode with a mean temperature bias near zero except near the end of the 20-day period.  For the winter run the sensitivity runs appear to be split into two groups with half of the sensitivity runs, including the production run, slightly too warm and the other half slightly too cold.  This difference is especially apparent during the 12-20 December period.

#### Dew point temperature

The sensitivity runs for the summer episode shows that most runs (including the production run) have a dew point temperature bias near zero.  The px-acm2_morr_rrtmg_ipxwrf run has a 1C warm bias and the ysu.wsm6 run a 1-1.5C cold bias.  The winter episode generally displays a positive dew point temperature bias especially from the production run.  About half of the sensitivity runs display a smaller bias than the production run.  The positive bias during the winter episode tends to be most consistent over the upper Midwest and Ohio Valley possibly due to the presence of snow cover that may affect the ability of the WRF to simulate dew points. A sample plot from Illinois is included to demonstrate the consistent nature of the positive bias.

#### Relative Humidity

The sensitivity runs for relative humidity during the summer episode indicate all of the runs except for one significant outlier track closely to the production run and have a positive mean bias of approximately 2-3%.  The ysu.wsm6 sensitivity run consistently shows a negative mean bias of roughly the same 2-3%.  In the winter episode, all of the sensitivity runs including the production run generally have a positive bias of about 5%.  The myj.wsm6 run has the largest bias of the sensitivity runs reaching upwards of 10% during the 16-20 December period when it diverges from the majority of the runs.

#### Specific Humidity

The summer episode sensitivity runs for specific humidity look qualitatively very similar to the dew point temperature sensitivity runs.  The mean specific humidity value is much lower in winter due to lower absolute moisture levels and this is contributing to a perceived lack of a significant bias during the winter episode. However, it appears that as with the dew point temperature the specific humidity has the most significant and consistent positive bias farther to the north over the Upper Midwest and Ohio Valley states.  As with the dew point temperature for the summer episode, the px-acm2_morr_rrtmg_ipxwrf run has a positive bias of about 0.5 g kg-1 and the ysu.wsm6 has about a 1 g kg-1 negative bias.  The majority of the sensitivity runs, however, track closely to the production run.

#### Wind Speed

The production and sensitivity runs for the summer and winter episodes are very similar to one another and exhibit only very slight positive biases.  The OTC sensitivity run is the single outlier especially during the winter episode when it runs about a 0.5 m s-1 negative bias relative to the observed wind speed values.  Overall, for both episodes the mean bias is small.

### Task 2 H 5: Wind speed plots

The following section shows time series of spatial averages for the surface wind speed from observations (in black) and simulated by the WRF model for 5-day periods for the summer and winter episodes of the sensitivity runs.  The production run, px_acm2_morr_rrtmg, is the red curve in the following plots.

The following plot shows the individual 5-day time series for the summer episode of the WRF sensitivity runs.  The production run and most of the sensitivity runs have an under representation of the variability in the observed wind speeds with winds that are simulated too low in the daytime hours and too high during the night.  The myj.wsm5 and myj.wsm6 sensitivity runs perform well at simulating the peak wind speeds that occur in the daytime hours although perhaps they show a slight positive bias.  The OTC run accurately simulates the wind speed minima that occur during the nighttime and early morning hours, but at other times is the poorest of the sensitivity runs when compared with the observed wind speeds as it predicts the lowest wind speed maxima.

The sensitivity plots for one of the 5-day periods from the summer episode is shown below to highlight one of the regional differences.  The plot for Florida is similar to the plot for the entire domain 2, with an under representation of the variability in the observed wind speeds. However, for this the production run and a number of the other sensitivity runs appear to be much better at simulating the wind speed minima (they are within about 0.5 m s-1 of the observed wind speed minimum).

The winter episode shows some considerable differences compared to the summer episode.  The observed wind speeds in the winter episode generally have greater variability than during the summer.  The production run performs well at simulating the maximum wind speeds, but still appears to overestimate the wind speed during the nighttime and early morning hours when there is a stable boundary layer and the wind speeds are reduced.  The ysu.wsm6 has the largest errors of all runs during much of the winter episode and performs the worst during the periods where the boundary layer becomes stable and the observed wind speeds decrease significantly.  The OTC sensitivity run consistently has the lowest wind speeds of all the sensitivity runs, similar to what is found for the summer episode.

Winter episode plots for separate 5-day periods for Indiana and West Virginia are show below again to highlight some of the regional differences.  The production run in the Indiana plot generally tracks the observations well and does not appear to show a systematic bias.  A couple of the sensitivity runs (among them myj.wsm5, myj.wsm6, and ysu.wsm6) overpredict the wind speed during maxima in the observed wind speed.  In the West Virginia plot there is a tendency for all of the sensitivity runs, including the production run, to overestimate the wind speed.  The myj.wsm5 and myj.wsm6 runs had especially strong wind speeds relative to the observed wind speeds.  The behavior of the WRF simulation over West Virginia is likely attributable to the higher terrain across much of the state (as was also found for the year-long statistics of the production run).

### Task 2 H 6: Bubble spatial plots of error statistics

This section discusses the results of the sensitivity runs in terms of the spatial “bubble” plots of bias and MAE. These plots are analogous to those for the production run in Task 2 C.

#### Temperature

The 2-meter / surface temperature mean absolute errors (MAE) of domain 2 were generally less than 2.5 ⁰C for both winter and summer for all sensitivity runs. However, there were some differences in the biases between the runs.  For winter the OTC, px_acm2_morr_rrtmg_ipxwrf, and px_acm2_morr_rrtmg runs exhibited warm biases > 1.0 C over the Great Plains, southern Canada and many locations along East Coast.   The px_acm2_morr_rrtmg run, shown below, was the best of the three as the other two featured more locations with larger biases > 2.0 C.

Winter Temperature Mean Bias for Domain 2 for px_acm2_morr_rrtmg

The runs myj_wsm5, myj_wsm6, ysu_wsm5, and ysu_wsm6 featured a cold winter bias of < -2.0 C over Wisconsin and eastern Minnesota as shown below for myj_wsm6.  The cold bias was only present on 4 of the 19 days of the winter runs and interestingly was present for those same days, December 13, 16, 17, 19, in all the sensitivity runs.  The fact that the cold bias only shows up as a seasonal bias in these four runs was that much larger positive biases (> 2.5 C) on other days for the other runs negated this cold bias.   There were no substantial winter temperature biases for the px_acm2_wsm5 and px_acm2_wsm6 runs.

Winter Temperature Mean Bias for Domain 2 for myj_wsm6

During summer only the px_acm2_morr_rrtmg_ipxwrf run exhibited notable biases.  For this run there was a general 0.5 warm bias in the northern half of domain 2 and with an embedded cool bias of < - 2.0 C immediately north of the Great Lakes.

#### Relative humidity and dew point

The WRF-MET 2-meter / surface relative humidity (RH) MAEs were generally < 10 % during the summer and < 15 % in winter for all runs.  During winter all runs exhibited a moist bias of 10 – 15 % from the upper Midwest to New England.   The 2-meter dew point errors exhibited greater inter- run variability. During winter runs OTC, px_acm2_morr_rrtmg_ipxwrf, px_acm2_morr_rrtmg featured a moist dew point bias of > 1.5 C over much of the northern half of domain 2, while the other runs featured a moist bias < 1.5 C.  During summer the biases in the dew point temperature did not seem to have any discernable pattern in all but two runs.  For px_acm2_morr_rrtmg_ipxwrf, there was a notable moist bias > 1.5 C over much of the domain and for ysu_wsm6 there was dry bias of < -2.0 C in many locations as shown below.  In neither case was the bias dependent on the time of the day, but the cool bias in the latter run seemed to be more persistent.

Summer Nighttime Dew point Mean Bias for Domain 2 for px_acm2_morr_rrtmg_ipxwrf

#### Wind speed

During winter the MAEs for the WRF-MET 10- meter / surface wind speed were generally < 1.5 m s-1 for all runs over much of domain 2. The exceptions were along the northeastern coast of the U.S. and in southern Canada where the MAEs were > 1.5 m s-1 with some locations > 2.0 m s-1.  There did not seem to be any notable differences in the wind errors between the sensitivity runs and biases seemed to be < 0.5 ms-1 at most locations.

During summer the 10-meter wind MAEs were generally < 1.5 ms-1 with biases < 0.5 ms-1 for all runs.  There was some inter-run variability in summer as runs OTC, ysu-wsm5 and ysu_wsm6 exhibited MAEs of 1.5 – 2.0 m s-1 at many locations along coast of the northeastern U.S. and in southern Canada

#### Wind direction

The wind direction errors exhibited the same characteristics as reported in Task 2 C.  There were no distinguishable differences between the sensitivity runs.

# Meteorological modeling website and other technical aspects

## Website development and documentation

The project web site (http://sesarm.aer.com) serves as the primary means for distributing and evaluating the substantial meteorological modeling results/summaries and communicating the corresponding documentation to SESARM. This web site satisfies the requirements for 'Task 5 Technical Meteorological Modeling Web Site' of contract S-2009-09-01. All content on this web site has been approved by the SESARM Project Officer prior to posting. The website presents objective evaluation results as itemized by Amendment 2 of the project Contract, signed 4 August 2011.

## Supplemental Dataset

All WRF-MET point_stat output files (referred to earlier in this document as ‘STAT’ files) have been provided on an external drive to SESARM. These data represent the raw pairs of modeled and observed values, including spatially-averaged values, that are valid at the top of each hour. Data are available for the full calendar year for the production run, and for each hour during each sensitivity period for the sensitivity runs. Data are available for domains 1 and 2 for the production run, and domain 2 for each sensitivity run.

# Summary and conclusions

Do the model results fit our conceptual understanding?

Meteorological fields generated during this modeling effort agree with our conceptual understanding of the physical processes of the atmosphere, and, as such, are representative of the observed weather conditions. The WRF-ARW model, when configured in the manner followed by SESARM, ensures that the physical processes of the atmosphere, such as heat and moisture fluxes, interact according to the laws of motion in a manner that is reproducible and verifiable using observations, subject to known, acceptable, model limitations.

Are diurnal features adequately captured?

The WRF model adequately captures the diurnal cycle of temperature, moisture and wind. During periods when the surface flow is forced more strongly by synoptic weather systems, such as during the winter, forecast errors for the wind speeds are minimized. During spring and summer when strong insolation increases the depth of the mixed layer, the model tends to underpredict surface wind speeds during the afternoon. Conversely, the model maintains mixing that is too strong during the overnight hours.

Do the wind fields agree with the observations?

Overall surface wind fields are well represented in the model and provide valuable estimation of the flow fields in the troposphere. Domain-wide wind speed forecasts averaged over full days and for longer timer periods did not exhibit a prolonged or substantial speed bias and wind speed RMSE errors were generally near or slightly exceeded the critical threshold of 2 m s-1 only for the coarse domain. Values of correlation coefficient are higher than the critical threshold of approximately 0.60. These values do not suggest a systematic problem with the model configuration or performance. Errors values for wind fields above the PBL and removed from the effects of orography were noticeably smaller.

Wind direction forecasts did not exhibit substantial biases, however, mean absolute errors for both domains were at or slightly above the critical threshold of 30 degrees. In combination, these two sets of errors suggest that the direction errors were somewhat random in nature, which is not unexpected when compared with other modeling studies.

Do the temperature and moisture fields generally match the observations?

Overall the temperature and moisture fields generally match the observations. Mean biases in temperature were typically within the critical threshold of 0.5 oC, with domain 2 values slightly less than those for domain 1. Mean absolute error values during the wintertime were approximately 2 oC resulting primarily from warm biases. Summertime values were somewhat less. Correlation coefficient values were significantly higher than the critical threshold.

Both mean bias and mean absolute errors for specific humidity were well within acceptable thresholds, even during the summer when levels of moisture in the atmosphere are significantly higher than in the cold winter months. Bias values for all months and both domains were small and positive, with similar characteristics seen in the dew point temperature. A positive bias of moisture may be related to the presence of snow cover in the northern US during the winter and spring, Though a causal relationship is not possible to ascertain, the positive bias of moisture was most pronounced when snow cover was present.

Do the meteorological fields produce acceptable air quality results?

AER recommends the future use of these WRF runs for the following reasons: 1) the fields were generated from the latest version of a state-of-the-art meteorological model, 2) proper configuration of the model was followed, 3) error values are consistent with those in the literature, and 4) there is an absence of obvious, profound, systematic modeling errors. Despite the overall recommendation, it is suggested that future users of subsets of these raw meteorological fields and subsequent air quality fields closely inspect the quality of the individual WRF runs, since the quality of any one run can vary considerably. As well, patterns that seem to emerge from some of the analyses (e.g., time series at METAR sites) are sometimes not representative of broader geographical regions (viz. corresponding bubble spatial plots). The user should make an assessment for their own needs based on the time and region of interest. The individual poor forecasts are often attributed to errors in the initial conditions of the model and may ultimately be linked to failures in the observational network after substantial scientific investigation. Typically these poor model forecasts are limited in space and time.

# Appendix A

This appendix lists the location details of all Continental US METAR locations whose data are potentially in the prepBUFR observation dataset that was used in this evaluation study. These are observation locations that are generally available over the GTS communication channel that is the standard transmission channel for NCEP. We know of no source that monitors on a site-by-site basis what is available at any observation time. Observations come and go from this set for a number of reasons, including, but not limited to, communications failures, equipment failures, typographical errors and various attempts at quality control. All observations in the prepBUFR files were ingested and used by WRF-MET. There are no known observations that are never used within CONUS and, while a very small number of observations may change states depending on which modeling domain is used, they are never used in multiple states at once for each domain.

This study used Version 2 of WRF-MET, which does not have the capability to assign station IDs to the latitude/longitude pairs associated with each observation.

CD  STATION         ICAO  IATA  SYNOP   LAT     LONG   ELEV (m)

AL ALABASTER        KEET  EET          33 11N  086 47W  173

AL ALBERTVILLE MUNI K8A0  8A0          34 14N  086 15W  315

AL ALEXANDER/RUSSEL KALX  ALX          32 55N  085 58W  209

AL ANDALUSIA/OPP AR K79J  79J          31 19N  086 24W   94

AL ANNISTON         KANB  ANB          33 35N  085 51W  183

AL AUBURN UNIV. (AM KAUB  AUB          32 36N  085 30W  198

AL AUBURN OPELIKA A KAUO  AUO          32 37N  085 25W  236

AL BIRMINGHAM       KBHM  BHM   72228  33 34N  086 45W  197

AL OZARK/CAIRNS AAF KOZR  OZR          31 17N  085 43W   93

AL CENTREVILLE WSM  K0A8  0A8   72229  32 53N  087 15W  140

AL COURTLAND        K9A4  9A4          34 40N  087 21W  179

AL CULLMAN/FOLSOM F K3A1  3A1          34 16N  086 51W  294

AL DECATUR          KDCU  DCU          34 39N  086 57W  176

AL DOTHAN           KDHN  DHN          31 19N  085 27W  113

AL EUFAULA          KEUF  EUF          31 57N  085 08W   87

AL EVERGREEN        KGZH  GZH          31 25N  087 03W   78

AL FAIRHOPE         KCQF  CQF          30 28N  087 53W   28

AL FLORALA MUNI     K0J4  0J4          31 03N  086 19W   96

AL FT PAYNE/ISBELL  K4A9  4A9          34 28N  085 43W  267

AL FT RUCKER/SHELL  KSXS  SXS          31 22N  085 51W  122

AL FT RUCKER/HANCHE KHEY  HEY          31 21N  085 39W   97

AL GADSDEN MUNI     KGAD  GAD          33 58N  086 04W  173

AL GREENVILLE       KPRN  PRN          31 51N  086 37W  138

AL GULF SHORES      KJKA  JKA          30 17N  087 40W    5

AL HALEYVILLE       K1M4  1M4          34 17N  087 36W  284

AL HUNTSVILLE       KHSV  HSV   72323  34 39N  086 47W  191

AL HUNTSVILLE/MADIS KMDQ  MDQ          34 52N  086 33W  230

AL JASPER/WALKER    KJFX  JFX          33 54N  087 19W  148

AL LOWE AHP/FT. RUC KLOR  LOR          31 21N  085 45W   74

AL MAXWELL AFB/MGM  KMXF  MXF          32 22N  086 22W   51

AL MOBILE/BROOKLEY  KBFM  BFM          30 37N  088 04W   20

AL MOBILE/BATES     KMOB  MOB   72223  30 41N  088 15W   63

AL MONTGOMERY       KMGM  MGM   72226  32 18N  086 24W   63

AL MUSCLE SHOAL     KMSL  MSL          34 45N  087 36W  164

AL PHI OIL PLAT     KRAM  RAM          29 03N  088 05W   10

AL REDSTONE ARSENAL KHUA  HUA   74001  34 41N  086 41W  209

AL SCOTTSBORO       K4A6  4A6          34 41N  086 00W  198

AL TROY             KTOI  TOI          31 51N  086 01W  118

AL TUSCALOOSA       KTCL  TCL          33 13N  087 37W   49

AL VIOSCA KNOLL768  KVOA  VOA          29 14N  087 47W   53

AR ARKADELPHIA      KM89  M89          34 06N  093 04W   56

AR BATESVILLE       KBVX  BVX          35 43N  091 39W  141

AR BENTONVILLE      KH00  H00          36 21N  094 13W  395

AR BENTONVILLE      KVBT  VBT          36 21N  094 13W  395

AR BENTONVILLE (NW) KXNA  XNA          36 17N  094 18W  392

AR BLYTHEVILLE      KHKA  HKA          35 56N  089 50W   78

AR BLYTHEVILLE      KBYH  BYH          35 58N  089 57W   78

AR CAMDEN           KCDH  CDH          33 37N  092 46W   35

AR CLINTON MUNI     KCCA  CCA          35 36N  092 27W  157

AR CORNING          K4M9  4M9          36 24N  090 39W   90

AR DE QUEEN         KDEQ  DEQ          34 03N  094 24W  106

AR EL DORADO        KELD  ELD          33 13N  092 49W   81

AR FAYETTEVILLE     KFYV  FYV          36 01N  094 10W  379

AR FLIPPIN (AWOS)   KFLP  FLP          36 17N  092 35W  219

AR FORT SMITH       KFSM  FSM   72344  35 20N  094 22W  140

AR HARRISON         KHRO  HRO          36 16N  093 09W  417

AR HOT SPRINGS      KHOT  HOT          34 29N  093 06W  162

AR JONESBORO        KJBR  JBR          35 50N  090 39W   79

AR LITTLE ROCK      KLIT  LIT          34 44N  092 14W   79

AR LITTLE ROCK AFB  KLRF  LRF          34 55N  092 09W   95

AR MENA INTERMTN    KMEZ  MEZ          34 33N  094 12W  329

AR W MEMPHIS MUNI   KAWM  AWM          35 08N  090 14W   65

AR MONTICELLO       KLLQ  LLQ          33 38N  091 45W   83

AR MOUNT IDA        KMWT  MWT          34 33N  093 35W  214

AR MOUNTAIN HOME    KBPK  BPK          36 22N  092 28W  280

AR N. LITTLE ROCK   KLZK  LZK   72340  34 50N  092 16W  173

AR NEWPORT          KM19  M19          35 38N  091 11W   73

AR PINE BLUFF/GRIDR KPBF  PBF          34 11N  091 56W   62

AR ROGERS           KROG  ROG          36 22N  094 05W  415

AR RUSSELLVILLE     KRUE  RUE          35 15N  093 06W  115

AR SEARCY           KSRC  SRC          35 15N  091 44W   80

AR SILOAM SPRING    KSLG  SLG          36 12N  094 28W  300

AR SPRINGDALE MUNI  KASG  ASG          36 10N  094 07W  412

AR STUTTGART        KSGT  SGT          34 36N  091 34W   68

AR TEXARKANA        KTXK  TXK          33 27N  093 59W  111

AR WALNUT RIDGE     KARG  ARG          36 07N  090 55W   83

AZ PHOENIX/SKY HRBR KPHX  PHX   72278  33 26N  112 03W  336

AZ TUCSON           KTUS  TUS          32 08N  110 57W  776

AZ BUCKEYE          KBXK  BXK          33 25N  112 41W  315

AZ BULLHEAD CITY    KIFP  IFP          35 09N  114 34W  212

AZ CASA GRANDA      KCGZ  CGZ          32 57N  111 46W  446

AZ CHANDLER/MESA    KIWA  IWA          33 18N  111 39W  412

AZ CHANDLER         KCHD  CHD          33 16N  111 49W  379

AZ DAVIS/TUCSON     KDMA  DMA          32 10N  110 52W  824

AZ DOUGLAS BISBEE   KDUG  DUG          31 28N  109 35W 1265

AZ FLAGSTAFF        KFLG  FLG   72375  35 08N  111 40W 2134

AZ FORT HUACHUCA    KFHU  FHU   72273  31 35N  110 21W 1438

AZ GILA BEND (AAF)  KGBN  GBN   74724  32 52N  112 43W  262

AZ GLENDALE         KGEU  GEU          33 32N  112 18W  325

AZ LUKE AFB/GLENDAL KLUF  LUF          33 31N  112 22W  332

AZ GOODYEAR MUNICIP KGYR  GYR          33 25N  112 22W  295

AZ GRAND CANYON     KGCN  GCN   72378  35 57N  112 09W 2016

AZ GRAND CANYON/VAL K40G  40G          35 38N  112 09W 1830

AZ KINGMAN          KIGM  IGM   72370  35 15N  113 56W 1032

AZ LAKE HAVASU CITY KHII  HII          34 34N  114 22W  238

AZ MESA/FALCON FLD  KFFZ  FFZ          33 28N  111 43W  424

AZ NOGALES          KOLS  OLS          31 25N  110 51W 1184

AZ PAGE             KPGA  PGA   72371  36 55N  111 27W 1307

AZ PAYSON           KPAN  PAN          34 15N  111 20W 1572

AZ PHOENIX/DEER VLY KDVT  DVT          33 41N  112 04W  443

AZ PIONEER AIRFIELD KALK  ALK          31 36N  110 26W 1453

AZ PRESCOTT         KPRC  PRC          34 39N  112 25W 1524

AZ SAFFORD          KSAD  SAD          32 51N  109 38W  962

AZ SCOTTSDALE       KSDL  SDL          33 37N  111 55W  447

AZ SEDONA           KSEZ  SEZ          34 51N  111 47W 1473

AZ SHOW LOW MUNICIP KSOW  SOW          34 16N  110 00W 1954

AZ ST. JOHNS        KSJN  SJN          34 31N  109 23W 1745

AZ TUCSON/RYAN FLD  KRYN  RYN          32 09N  111 10W  737

AZ WILLIAMS CLARK   KCMR  CMR          35 18N  112 12W 2036

AZ WINDOW ROCK      KRQE  RQE          35 39N  109 04W 2055

AZ WINSLOW          KINW  INW   72374  35 02N  110 43W 1490

AZ YUMA INTL        KYUM  YUM   72280  32 38N  114 35W   63

AZ YUMA MCAS        KNYL  NYL   69960  32 38N  114 37W   65

CA ALAMEDA NAS      KNGZ  NGZ   74506  37 46N  122 19W    4

CA ALTURAS          KAAT  AAT          41 29N  120 34W 1331

CA APPLE VALLEY     KAPV  APV          34 34N  117 11W  934

CA ARCATA/EUREKA    KACV  ACV          40 59N  124 06W   67

CA AUBURN MUNI      KAUN  AUN          38 57N  121 05W  467

CA AVALON           KAVX  AVX   72292  33 24N  118 25W  482

CA BAKERSFIELD      KBFL  BFL   72384  35 26N  119 03W  151

CA BEALE AFB/MARYSV KBAB  BAB          39 07N  121 25W   34

CA BEAUMONT         KBUO  BUO          33 55N  116 58W  692

CA BIG BEAR CITY    KL35  L35          34 16N  116 51W 2059

CA BICYCLE LAKE     KBYS  BYS   74611  35 17N  116 37W  716

CA BISHOP           KBIH  BIH   72480  37 22N  118 21W 1253

CA BLYTHE           KBLH  BLH          33 37N  114 43W  119

CA BURBANK          KBUR  BUR   72288  34 12N  118 22W  217

CA BURNEY           KBNY  BNY          40 52N  121 40W  957

CA CALEXICO INTL    KCXL  CXL          32 40N  115 30W    1

CA CAMARILLO        KCMA  CMA          34 13N  119 04W   23

CA CAMP PENDLETON   KNFG  NFG          33 17N  117 20W   24

CA CAMPO            KCZZ  CZZ          32 38N  116 28W  807

CA CARLSBAD         KCRQ  CRQ          33 08N  117 17W   99

CA CHICO MUNICIPAL  KCIC  CIC   72497  39 47N  121 50W   73

CA CHINA LAKE (NAF) KNID  NID   74612  35 40N  117 40W  696

CA CHINO            KCNO  CNO          33 59N  117 37W  207

CA COLUMBIA         KO22  O22          38 02N  120 25W  646

CA CONCORD          KCCR  CCR          38 00N  122 03W   11

CA CORONA MUNI      KAJO  AJO          33 54N  117 36W  163

CA CRESCENT CITY    KCEC  CEC          41 46N  124 13W   17

CA DAGGETT          KDAG  DAG          34 51N  116 47W  587

CA DAVIS            KEDU  EDU          38 32N  121 47W   21

CA EDWARDS AFB      KEDW  EDW   72381  34 53N  117 52W  702

CA EDWARDS N-AUX    K9L2  9L2          34 58N  117 52W  701

CA EL CENTRO NAF    KNJK  NJK   72281  32 49N  115 40W  -13

CA EL MONTE         KEMT  EMT   74704  34 04N  118 01W   90

CA EMIGRANT GAP     KBLU  BLU          39 17N  120 42W 1609

CA EUREKA/MURRAY    KEKA  EKA   72594  40 48N  124 10W   18

CA FRESNO           KFAT  FAT   72389  36 47N  119 43W  104

CA FRESNO CHANDLER  KFCH  FCH          36 43N  119 49W   85

CA FULLERTON        KFUL  FUL          33 52N  117 59W   35

CA HALF MOON BAY    KHAF  HAF          37 31N  122 30W   21

CA HANFORD          KHJO  HJO          36 19N  119 37W   74

CA HANFORD          KO18  O18          36 19N  119 38W   74

CA HAWTHORNE        KHHR  HHR          33 55N  118 20W   18

CA HAYWARD          KHWD  HWD          37 40N  122 07W   21

CA HOLLISTER MUNI   KCVH  CVH          36 54N  121 25W   70

CA HUNTER LIGGET    KHGT  HGT   69002  36 00N  121 13W  310

CA IMPERIAL         KIPL  IPL          32 50N  115 35W  -17

CA IMPERIAL BEACH   KNRS  NRS          32 34N  117 07W    7

CA INYOKERN         KIYK  IYK          35 40N  117 49W  749

CA LA VERNE/BRACKET KPOC  POC          34 06N  117 46W  308

CA LANCASTER/FOX    KWJF  WJF          34 43N  118 13W  715

CA LEMOORE NAS/REEV KNLC  NLC   74702  36 19N  119 57W   72

CA LINCOLN          KLHM  LHM          38 55N  121 21W   37

CA LIVERMORE        KLVK  LVK          37 42N  121 49W  117

CA LOMPOC           KLPC  LPC   74606  34 40N  120 28W   27

CA LONG BEACH       KLGB  LGB   72297  33 49N  118 09W   10

CA LOS ALAMITOS AAF KSLI  SLI          33 46N  118 02W   11

CA LOS ANGELES      KLAX  LAX   72295  33 56N  118 23W   46

CA LOS ANGELES      KCQT  CQT          34 01N  118 17W   56

CA LA / WHITEMAN    KWHP               34 15N  118 25W  306

CA MADERA           KMAE  MAE          36 59N  120 07W   77

CA MAMMOTH/JUNE LAK KMMH  MMH          37 37N  118 49W 2173

CA MARYSVILLE       KMYV  MYV          39 06N  121 34W   21

CA MATHER FIELD     KMHR  MHR          38 32N  121 17W   29

CA MCCLELLAN AFB    KMCC  MCC          38 40N  121 24W   23

CA MERCED           KMCE  MCE          37 17N  120 30W   47

CA MERCED/CASTLE AF KMER  MER   72481  37 23N  120 34W   57

CA MIRAMAR NAS/SAN  KNKX  NKX   72293  32 52N  117 08W  146

CA MODESTO          KMOD  MOD          37 37N  120 57W   29

CA MOFFETT NAS/MTN  KNUQ  NUQ   74509  37 24N  122 03W   12

CA MOJAVE           KMHV  MHV          35 04N  118 09W  849

CA MONTAGUE/SISKIYO KSIY  SIY          41 46N  122 28W  807

CA MONTEREY         KMRY  MRY          36 35N  121 51W   66

CA MOUNT SHASTA     KMHS  MHS   72595  41 19N  122 19W 1078

CA MOUNT WILSON     KMWS  MWS   72289  34 13N  118 04W 1739

CA NAPA             KAPC  APC          38 12N  122 17W   13

CA NEEDLES          KEED  EED   72380  34 46N  114 37W  302

CA NEWHALL          K3A6  3A6          34 22N  118 34W  427

CA NORTH ISLAND NAS KNZY  NZY          32 42N  117 13W    8

CA NOVATO/GNOSS FLD KDVO  DVO          38 09N  122 33W    1

CA OAKLAND          KOAK  OAK   72493  37 42N  122 13W   26

CA OCEANSIDE        KOKB  OKB          33 13N  117 21W    8

CA OCEANSIDE/RED-B  KNXF  NXF          33 17N  117 27W   27

CA ONTARIO          KONT  ONT          34 03N  117 35W  274

CA OROVILLE         KOVE  OVE          39 30N  121 37W   58

CA OXNARD           KOXR  OXR          34 12N  119 12W   20

CA PALM SPRINGS     KPSP  PSP          33 49N  116 30W  135

CA PALMDALE         KPMD  PMD   72382  34 38N  118 05W  780

CA PALO ALTO        KPAO  PAO          37 28N  122 07W    2

CA PASO ROBLES      KPRB  PRB          35 40N  120 38W  245

CA PETALUMA         KO69  O69          38 15N  122 36W   28

CA PLACERVILLE      KPVF  PVF          38 43N  120 45W  789

CA POINT MUGU NAS   KNTD  NTD   72391  34 07N  119 07W    4

CA PORTERVILLE      KPTV  PTV          36 01N  119 04W  135

CA PT. PIEDRAS BLAN K87Q  87Q   72390  35 40N  121 17W   27

CA RED BLUFF        KRBL  RBL   72591  40 09N  122 15W  104

CA REDDING          KRDD  RDD   72592  40 31N  122 18W  155

CA RIVERSIDE        KRAL  RAL          33 57N  117 27W  252

CA RIVERSIDE/MARCH  KRIV  RIV   72286  33 52N  117 16W  469

CA RAMONA           KRNM  RNM          33 02N  116 55W  427

CA SACRAMENTO       KSAC  SAC          38 30N  121 30W   11

CA SACRAMENTO/METRO KSMF  SMF          38 42N  121 36W    6

CA SALINAS          KSNS  SNS          36 40N  121 36W   30

CA SAN BERNARDINO   KSBD  SBD          34 06N  117 14W  353

CA SAN CARLOS AIRPO KSQL  SQL          37 31N  122 15W    1

CA SAN CLEMENTE IS. KNUC  NUC          33 01N  118 34W   55

CA SAN DIEGO        KSAN  SAN   72290  32 44N  117 11W   12

CA SAN DIEGO/MNTGMY KMYF  MYF          32 49N  117 08W  136

CA SAN DIEGO/BROWN  KSDM  SDM          32 35N  117 00W  159

CA SAN DIEGO/SANTEE KSEE  SEE          32 49N  116 58W  117

CA SAN FRANCISCO    KSFO  SFO   72494  37 37N  122 22W    3

CA SAN JOSE         KSJC  SJC   74505  37 22N  121 55W   24

CA SAN JOSE/REID    KRHV  RHV          37 19N  121 49W   41

CA SAN LUIS OBISPO  KSBP  SBP          35 14N  120 38W   59

CA SAN NICOLAS ISLA KNSI  NSI   72291  33 13N  119 27W  154

CA SANDBERG         KSDB  SDB   72383  34 45N  118 43W 1377

CA COSTA MESA       KSNA  SNA          33 40N  117 52W   16

CA SANTA BARBARA    KSBA  SBA          34 26N  119 51W    3

CA SANTA MARIA      KSMX  SMX   72394  34 54N  120 27W   74

CA SANTA MONICA     KSMO  SMO          34 01N  118 27W   57

CA SANTA ROSA       KSTS  STS          38 30N  122 49W   39

CA SANTA YNEZ       KIZA  IZA          34 36N  120 04W  205

CA SHELTER COVE     KO87  O87          40 01N  124 04W   21

CA SOUTH LAKE TAHOE KTVL  TVL          38 54N  120 00W 1924

CA STOCKTON         KSCK  SCK   72492  37 53N  121 13W   10

CA SUSANVILLE MUNI  KSVE  SVE   72584  40 22N  120 34W 1263

CA THERMAL/PALM SPG KTRM  TRM          33 38N  116 10W  -35

CA TORRANCE MUNICIP KTOA  TOA          33 47N  118 19W   31

CA TRAVIS AFB/FAIRF KSUU  SUU   74516  38 16N  121 55W   19

CA TRUCKEE TAHOE    KTRK  TRK          39 19N  120 07W 1798

CA TEHACHAPI        KTSP  TSP          35 08N  118 26W 1220

CA TWENTYNINE PALMS KNXP  NXP   69015  34 17N  116 10W  626

CA UKIAH            KUKI  UKI          39 07N  123 12W  187

CA VACAVILLE        KVCB  VCB          38 23N  121 57W   33

CA VAN NUYS         KVNY  VNY          34 13N  118 29W  245

CA VANDENBERG AFB   KVBG  VBG   72393  34 44N  120 35W  112

CA VANDENBERG RANGE KXVW  XVW          34 43N  120 34W  100

CA VICTORVILLE      KVCV  VCV          34 35N  117 23W  876

CA VISALIA MUNI     KVIS  VIS          36 19N  119 24W   89

CA WATSONVILLE      KWVI  WVI          36 56N  121 47W   43

CA WEAVERVILLE      KO54  O54          40 45N  122 55W  717

CO DENVER (DIA)     KDEN  DEN   72565  39 50N  104 39W 1640

CO AKRON            KAKO  AKO          40 10N  103 13W 1421

CO ALAMOSA          KALS  ALS   72462  37 26N  105 52W 2299

CO ASPEN            KASE  ASE          39 14N  106 52W 2354

CO BERTHOUD PASS    K0CO  0CO          39 48N  105 46W 4113

CO BROOMFIELD/JEFFC KBJC  BJC          39 55N  105 07W 1724

CO BUCKLEY ANGB/DEN KBKF  BKF          39 43N  104 45W 1726

CO BURLINGTON       KITR  ITR          39 15N  102 17W 1279

CO CANON CITY       K1V6  1V6          38 26N  105 06W 1658

CO COLORADO SPRINGS KCOS  COS   72466  38 49N  104 43W 1856

CO CO SPNGS MEADOW  KFLY  FLY          38 57N  104 34W 2096

CO CORTEZ           KCEZ  CEZ          37 18N  108 38W 1797

CO CRAIG            KCAG  CAG   72570  40 30N  107 31W 1887

CO DENVER/ARAPAHOE  KAPA  APA          39 34N  104 51W 1775

CO DENVER F. RANGE  KFTG  FTG          39 47N  104 33W 1675

CO DURANGO          KDRO  DRO          37 09N  107 46W 2035

CO EAGLE CO. REGION KEGE  EGE          39 38N  106 55W 1993

CO FORT CARSON      KFCS  FCS   72468  38 40N  104 46W 1789

CO FT COLLINS/LOVEL KFNL  FNL          40 27N  105 01W 1529

CO GRAND JUNCTION   KGJT  GJT   72476  39 07N  108 31W 1475

CO GREELEY/WELD     KGXY  GXY          40 25N  104 37W 1420

CO GUNNISON         KGUC  GUC          38 31N  106 55W 2339

CO HAYDEN/YAMPA     KHDN  HDN   72571  40 28N  107 13W 2012

CO LA JUNTA         KLHX  LHX          38 03N  103 31W 1277

CO LAMAR            KLAA  LAA   72463  38 04N  102 41W 1119

CO LEADVILLE        KLXV  LXV   72467  39 14N  106 19W 3028

CO LIMON            KLIC  LIC          39 16N  103 40W 1630

CO MEEKER           KEEO  EEO          40 03N  107 53W 1930

CO MONTROSE         KMTJ  MTJ          38 30N  107 54W 1750

CO NUCLA/HOPKINS F  KAIB  AIB          38 14N  108 34W 1811

CO PUEBLO           KPUB  PUB   72464  38 17N  104 30W 1420

CO RIFLE            KRIL  RIL          39 32N  107 44W 1678

CO SAGUACHE MUNI    K04V  04V          38 06N  106 10W 2385

CO SPRINGFIELD      KSPD  SPD          37 17N  102 37W 1335

CO STEAMBOAT SPRING KSBS  SBS          40 31N  106 52W 2096

CO MT WERNER SBS    K3MW  3MW          40 27N  106 45W 3241

CO TELLURIDE REGION KTEX  TEX          37 57N  107 54W 2769

CO TRINIDAD/ANIMAS  KTAD  TAD          37 16N  104 19W 1756

CO USAF ACADEMY/COS KAFF  AFF   74531  38 58N  104 49W 2003

CO YUMA             KYMA               40 06N  102 43W 1261

CO FLAGLER          KFLA               39 17N  103 04W 1507

CO IDALIA           KIDL               39 42N  102 17W 1209

CO KIRK             K72C               39 36N  102 34W 1280

CO STERLING         KSTK               40 37N  103 16W 1231

CO STRATTON         KSTR               39 18N  102 36W 1345

CO RED CLIFF PASS   KCCU               39 28N  106 09W 3680

CO RANGELY          K4V0               40 06N  108 46W 1608

CO BUENA VISTA      KAEJ               38 49N  106 07W 2422

CO DELTA/BLAKE FLD  KAJZ               38 47N  108 04W 1583

CO SALIDA           KANK               38 32N  106 03W 2293

CO BOULDER MUNI     KBDU               40 02N  105 14W 1612

CO ELLICOTT-B AFA   KABH               38 45N  104 18W 1840

CO ERIE MUNI        KEIK               40 01N  105 03W 1564

CO FORT MORGAN      KFMM               40 20N  103 48W 1393

CO GRANBY/GRAND CO  KGNB               40 05N  105 55W 2500

CO HOLYOKE          KHEQ               40 34N  102 16W 1137

CO LONGMONT/VBRAND  KLMO               40 10N  105 10W 1541

CO PAGOSA SPRINGS   KPSO               37 17N  107 03W 2334

CO WOLF CREEK PASS  KCPW               37 27N  106 48W 3584

CO CORONA PASS      KCRV               40 03N  105 35W 3538

CO MONUMENT/ELBERT  KMNH               39 13N  104 39W 2152

CO MONARCH PASS     KMYP               38 29N  106 19W 3667

CO LA VETA PASS     KVTP               37 30N  105 10W 3114

CO WALDEN           K33V               40 45N  106 16W 2484

CO KREMMLING AWOS   K20V               40 03N  106 22W 2259

CO WILKERSON PASS   K4BM               39 03N  105 31W 3438

CO SUNLIGHT         K5SM               39 26N  107 23W 3232

CO COTTONWOOD PASS  K7BM               38 47N  106 13W 2995

CO WRAY             K2V5               40 06N  102 14W 1118

CT BRIDGEPORT       KBDR  BDR   72504  41 10N  073 08W    7

CT CHESTER          KSNC  SNC          41 23N  072 30W  127

CT DANBURY          KDXR  DXR          41 22N  073 29W  139

CT GROTON/NEW LONDO KGON  GON          41 20N  072 03W    3

CT HARTFORD/BRAINAR KHFD  HFD          41 44N  072 39W    4

CT MERIDEN          KMMK  MMK          41 31N  072 50W   31

CT NEW HAVEN/TWEED  KHVN  HVN          41 16N  072 52W    4

CT OXFORD/WATERBURY KOXC  OXC          41 28N  073 08W  222

CT WILLIMANTIC      KIJD  IJD          41 45N  072 11W   75

CT WINDSOR LOCKS    KBDL  BDL   72508  41 56N  072 41W   60

DE DOVER AFB        KDOV  DOV          39 07N  075 28W    9

DE GEORGETOWN       KGED  GED          38 41N  075 22W   19

DE WILMINGTON       KILG  ILG          39 40N  075 36W   28

FL APALACHICOLA     KAAF  AAF   72220  29 44N  085 02W    5

FL BARTOW MUNICIPAL KBOW  BOW          27 56N  081 46W   39

FL BOCA RATON       KBCT  BCT          26 23N  080 06W    4

FL BONIFAY TRI-CTY  K1J0  1J0          30 51N  085 36W   26

FL BROOKSVILLE      KBKV  BKV          28 28N  082 27W   20

FL CAPE CANAVERAL   KXMR  XMR   74794  28 28N  080 32W    3

FL CECIL FIELD NAS  KNZC  NZC          30 13N  081 52W   25

FL CRESTVIEW        KCEW  CEW          30 47N  086 31W   55

FL CROSS CITY       KCTY  CTY   72212  29 38N  083 06W   13

FL CRYSTAL RIVER    KCGC  CGC          28 52N  082 34W    3

FL DAYTONA BEACH    KDAB  DAB          29 11N  081 04W    9

FL DESTIN           KDTS  DTS          30 24N  086 28W    5

FL DUKE FLD/EGLIN   KEGI  EGI   74778  30 38N  086 31W   59

FL EGLIN AFB/VALPAR KVPS  VPS   72221  30 28N  086 31W   26

FL EVERGLADES CITY  KEGC  EGC          25 51N  081 23W    2

FL FERNANDINA BEACH KFHB  FHB          30 37N  081 28W    5

FL FLAMINGO/MONROE  KFLM  FLM          25 08N  080 55W    2

FL FT LAUDERD/HOLLY KFLL  FLL          26 04N  080 09W    3

FL FT LAUDERD/EXEC  KFXE  FXE          26 12N  080 11W    6

FL FORT MYERS       KRSW  RSW          26 32N  081 45W   10

FL FORT MYERS       KFMY  FMY          26 35N  081 52W    4

FL FORT PIERCE      KFPR  FPR          27 30N  080 23W   13

FL GAINESVILLE      KGNV  GNV          29 42N  082 17W   45

FL HOLLYWOOD        KHWO  HWO          26 00N  080 14W    2

FL HOMESTEAD AFB    KHST  HST          25 28N  080 22W    2

FL HURLBURT FIELD   KHRT  HRT   74777  30 25N  086 40W   12

FL INVERNESS        KX40  X40          28 48N  082 19W   15

FL JACKSONVIL/CRAIG KCRG  CRG          30 20N  081 31W   13

FL JACKSONVILLE NAS KNIP  NIP          30 13N  081 40W    7

FL JACKSONVILLE     KJAX  JAX   72206  30 30N  081 41W   10

FL JACKSONVIL/CECIL KVQQ  VQQ          30 13N  081 53W   25

FL JACKSONVIL/NAVAL KNEN  NEN          30 21N  081 53W   30

FL KEYSTONE HEIGHTS K42J  42J          29 51N  082 03W   60

FL KEY WEST NAS     KNQX  NQX          24 34N  081 40W    2

FL KEY WEST         KEYW  EYW   72201  24 33N  081 45W    5

FL KISSIMMEE/ORLAND KISM  ISM          28 17N  081 26W   25

FL LAKELAND REGIONA KLAL  LAL          27 58N  082 01W   43

FL LEESBURG         KLEE  LEE          28 49N  081 49W   23

FL MACDILL AFB/TAMP KMCF  MCF   74788  27 51N  082 31W    4

FL MARATHON         KMTH  MTH          24 44N  081 03W    2

FL MARIANNA         KMAI  MAI          30 50N  085 11W   32

FL MAYPORT NAS      KNRB  NRB          30 23N  081 25W    5

FL MELBOURNE        KMLB  MLB   72204  28 06N  080 39W   11

FL MERRITT ISLAND   KCOI  COI          28 20N  080 41W    2

FL MIAMI/TAMIAMI    KTMB  TMB          25 39N  080 26W    2

FL MIAMI            KMIA  MIA          25 48N  080 17W    3

FL MIAMI/OPA LOCKA  KOPF  OPF          25 55N  080 17W   16

FL MIAMI BEACH      KMBF  MBF          25 46N  080 08W    1

FL MILTON/WHITING S KNDZ  NDZ          30 42N  087 01W   54

FL NAPLES MUNICIPAL KAPF  APF          26 09N  081 46W    3

FL NASA SHUTTLE FCL KTTS  TTS          28 37N  080 43W    3

FL NEW PORT RICHEY  KRRF  RRF          28 11N  082 38W   12

FL NEW SMYRNA BEACH KEVB  EVB          29 03N  080 57W    3

FL OCALA MUNI       KOCF  OCF          29 10N  082 13W   27

FL OCEAN REEF       KOCR  OCR          25 18N  080 16W    2

FL OKEECHOBEE CTY   KOBE  OBE          27 16N  080 51W   11

FL ORLANDO          KMCO  MCO   72205  28 25N  081 20W   29

FL ORLANDO          KORL  ORL          28 33N  081 20W   37

FL ORMOND BEACH VOR KOMN  OMN          29 18N  081 07W    9

FL PALM COAST       KXFL  XFL          29 28N  081 12W   10

FL PANAMA CITY      KECP  ECP          30 21N  085 48W   21

FL PATRICK AFB/COCO KCOF  COF   74795  28 13N  080 35W    3

FL PENSACOLA        KPNS  PNS   72222  30 29N  087 11W   38

FL PENSACOLA NAS    KNPA  NPA          30 21N  087 19W    9

FL PERRY FOLEY      K40J  40J          30 04N  083 34W   13

FL PLANT CITY MUNI  KPCM  PCM          28 00N  082 10W   47

FL POMPANO BEACH    KPMP  PMP          26 15N  080 07W    6

FL PORT RICHEY      KX41  X41          28 21N  082 37W   12

FL PUNTA GORDA      KPGD  PGD          26 55N  082 00W    7

FL QUINCY           K2J9  2J9          30 36N  084 33W   69

FL SANFORD/ORLANDO  KSFB  SFB          28 46N  081 13W   17

FL SARASOTA/BRADENT KSRQ  SRQ          27 24N  082 34W   18

FL ST AUGUSTINE     KSGJ  SGJ   99441  29 58N  081 19W    3

FL ST PETERSBURG    KPIE  PIE          27 55N  082 41W    4

FL ST PETERS/ALBERT KSPG  SPG          27 46N  082 38W    2

FL STUART/WITHAM    KSUA  SUA          27 11N  080 13W    6

FL TALLAHASSEE      KTLH  TLH   72214  30 24N  084 21W   19

FL TAMPA            KTPA  TPA   72211  27 58N  082 32W   11

FL TAMPA/O KNIGHT   KTPF  TPF          27 55N  082 27W    3

FL TAMPA/VANDENBURG KVDF  VDF          28 01N  082 21W    7

FL TITUSVILLE       KTIX  TIX          28 30N  080 48W   11

FL TYNDALL AFB      KPAM  PAM   74775  30 04N  085 34W    5

FL VENICE           KVNC  VNC          27 04N  082 26W    6

FL VERO BEACH       KVRB  VRB          27 39N  080 25W   10

FL THE VILLAGES     KVVG  VVG          28 57N  081 58W   27

FL WEST PALM BEACH  KPBI  PBI   72203  26 41N  080 06W    6

FL WHITING FLD NAS  KNSE  NSE          30 43N  087 01W   61

FL WINTER HAVEN     KGIF  GIF          28 04N  081 45W   43

FL HURLBERT         KQHY               30 24N  086 39W    8

GA ALBANY           KABY  ABY   72216  31 32N  084 12W   58

GA AMERICUS         KACJ  ACJ          32 07N  084 11W  143

GA ALMA             KAMG  AMG          31 32N  082 30W   59

GA ATHENS           KAHN  AHN   72311  33 57N  083 20W  244

GA ATLANTA          KATL  ATL   72219  33 38N  084 27W  296

GA ATLANTA/FULTON   KFTY  FTY          33 47N  084 31W  263

GA ATLANTA/PAULDING KPUJ  PUJ          33 55N  084 56W  393

GA AUGUSTA/DANIEL   KDNL  DNL          33 28N  082 02W  130

GA AUGUSTA/BUSH     KAGS  AGS   72218  33 22N  081 58W   44

GA BAINBRIDGE       KBGE  BGE          30 58N  084 38W   43

GA BLAIRSVILLE      KDZJ  DZJ          34 51N  084 00W  582

GA BLAKELY EARLY C  KBIJ  BIJ          31 24N  084 54W   65

GA BRUNSWICK        KSSI  SSI          31 09N  081 23W    6

GA BRUNSWICK/GLYNCO KBQK  BQK          31 15N  081 28W    8

GA CANTON/CHEROKEE  K47A  47A          34 19N  084 25W  372

GA CARROLLTON/GRAY  KCTJ  CTJ          33 38N  085 09W  354

GA CARTERSVILLE     KVPC  VPC          34 08N  084 51W  222

GA CLAXTON/EVANS CO KCWV  CWV          32 12N  081 52W   33

GA COLUMBUS         KCSG  CSG          32 31N  084 57W  135

GA COVINGTON        K9A1  9A1          33 38N  083 51W  247

GA DALTON           KDNN  DNN          34 43N  084 52W  216

GA DOBBINS AFB/MARI KMGE  MGE   72227  33 55N  084 31W  326

GA DOUGLAS MUNI     KDQH  DQH          31 29N  082 52W   78

GA DUBLIN           KDBN  DBN          32 34N  082 59W   94

GA ELBERTON PATZ F  K27A  27A          34 06N  082 49W  184

GA FT BENNING/COLUM KLSF  LSF   72225  32 19N  085 00W   71

GA FT STEWART/WRIGH KLHW  LHW   72209  31 52N  081 34W   14

GA GAINESVILLE      KGVL  GVL          34 16N  083 50W  386

GA GREENSBORO       K3J7  3J7          33 36N  083 08W  206

GA HOMERVILLE       KHOE  HOE          31 03N  082 47W   57

GA JESUP/WAYNE CTY  KJES  JES          31 33N  081 53W   33

GA LA GRANGE        KLGC  LGC          33 00N  085 04W  211

GA LAWRENCEVILLE    KLZU  LZU          33 59N  083 58W  323

GA MACON            KMCN  MCN   72217  32 41N  083 39W  109

GA MARIETTA MCCOLUM KRYY  RYY          34 01N  084 36W  317

GA MCDUFFIE/THOMSON KHQU  HQU          33 32N  082 31W  152

GA MILLEDGEVILLE    KMLJ  MLJ          33 09N  083 14W  117

GA MOODY AFB/VALDOS KVAD  VAD   74781  30 58N  083 12W   71

GA MOULTRIE MUNI    KMGR  MGR          31 05N  083 48W   90

GA MOULTRIE/SPENCE  KMUL  MUL          31 08N  083 42W   89

GA NEWNAN           KCCO  CCO          33 19N  084 46W  296

GA PEACHTREE/DEKALB KPDK  PDK          33 53N  084 18W  302

GA PEACHTREE CITY   KFFC  FFC   72215  33 21N  084 34W  262

GA ROME             KRMG  RMG   72320  34 21N  085 10W  193

GA SAVANNAH         KSAV  SAV   72207  32 07N  081 12W   14

GA SAVANNAH/HUNTER  KSVN  SVN   74780  32 01N  081 09W   13

GA STATESBORO       KTBR  TBR          32 29N  081 44W   57

GA SWAINSBORO       KSBO  SBO          32 37N  082 22W  100

GA SYLVANIA         KJYL  JYL          32 39N  081 36W   57

GA TIFTON           KTMA  TMA          31 26N  083 29W  109

GA THOMASTON UPSON  KOPN  OPN          32 57N  084 16W  243

GA THOMASVILLE      KTVI  TVI          30 54N  083 53W   80

GA VALDOSTA REGIONA KVLD  VLD          30 46N  083 16W   62

GA VIDALIA MUNI     KVDI  VDI          32 12N  082 22W   84

GA WARNER ROBINS AF KWRB  WRB          32 37N  083 35W   90

GA WASHINGTON       KIIY  IIY          33 47N  082 49W  197

GA WAYCROSS/WARE CO KAYS  AYS   72213  31 15N  082 24W   46

GA WINDER/BARROW    KWDR  WDR          33 59N  083 40W  287

IA ALGONA           KAXA  AXA          43 04N  094 16W  372

IA AMES             KAMW  AMW          41 59N  093 37W  279

IA ANKENY           KIKV  IKV          41 41N  093 34W  275

IA ATLANTIC         KAIO  AIO          41 23N  095 02W  360

IA AUDUBON          KADU  ADU          41 42N  094 55W  392

IA BOONE MUNI       KBNW  BNW          42 02N  093 50W  354

IA BURLINGTON       KBRL  BRL          40 46N  091 08W  211

IA CARROLL          KCIN  CIN          42 02N  094 46W  375

IA CEDAR RAPIDS     KCID  CID   72545  41 53N  091 43W  264

IA CENTERVILLE MUNI KTVK  TVK          40 41N  092 54W  314

IA CHARITON         KCNC  CNC          41 01N  093 22W  320

IA CHARLES CITY     KCCY  CCY          43 04N  092 37W  343

IA CHEROKEE         KCKP  CKP          42 44N  095 33W  374

IA CLARINDA         KICL  ICL          40 43N  095 01W  303

IA CLARION          KCAV  CAV          42 45N  093 46W  354

IA CLINTON MUNI     KCWI  CWI          41 49N  090 19W  216

IA COUNCIL BLUFFS   KCBF  CBF          41 16N  095 46W  382

IA CRESTON          KCSQ  CSQ          41 01N  094 22W  394

IA DAVENPORT/QUAD C KDVN  DVN   74455  41 37N  090 35W  230

IA DECORAH          KDEH  DEH          43 16N  091 43W  353

IA DENISON          KDNS  DNS          41 58N  095 22W  388

IA DES MOINES       KDSM  DSM   72546  41 32N  093 40W  295

IA DUBUQUE          KDBQ  DBQ   72547  42 24N  090 42W  326

IA ESTHERVILLE      KEST  EST          43 24N  094 45W  401

IA FAIRFIELD        KFFL  FFL          41 02N  091 58W  244

IA FOREST CITY      KFXY  FXY          43 14N  093 37W  375

IA FORT DODGE       KFOD  FOD   72549  42 32N  094 10W  355

IA FORT MADISON     KFSW  FSW          40 40N  091 19W  221

IA GRINNELL REG     KGGI  GGI          41 43N  092 44W  307

IA HARLAN MUNI      KHNR  HNR          41 35N  095 20W  375

IA INDEPENDENCE     KIIB  IIB          42 27N  091 57W  298

IA IOWA CITY        KIOW  IOW          41 38N  091 33W  198

IA IOWA FALLS MUNI  KIFA  IFA          42 28N  093 16W  347

IA KEOKUK MUNI      KEOK  EOK          40 28N  091 25W  205

IA KNOXVILLE        KOXV  OXV          41 17N  093 07W  283

IA LAMONI           KLWD  LWD          40 38N  093 54W  346

IA LE MARS          KLRJ  LRJ          42 46N  096 12W  365

IA MARSHALLTOWN     KMIW  MIW          42 07N  092 55W  296

IA MASON CITY       KMCW  MCW          43 09N  093 20W  369

IA MONTICELLO MUNI  KMXO  MXO          42 13N  091 10W  259

IA MT PLEASANT      KMPZ  MPZ          40 57N  091 31W  224

IA MUSCATINE        KMUT  MUT          41 22N  091 09W  167

IA NEWTON MUNI      KTNU  TNU          41 40N  093 01W  290

IA OELWEN           KOLZ  OLZ          42 40N  091 58W  328

IA ORANGE CITY      KORC  ORC          42 58N  096 04W  431

IA OSCEOLA MUNI     KI75  I75          41 03N  093 41W  339

IA OSKALOOSA MUNI   KOOA  OOA          41 14N  092 30W  256

IA OTTUMWA          KOTM  OTM          41 06N  092 27W  256

IA PELLA            KPEA  PEA          41 24N  092 57W  270

IA PERRY MUNI       KPRO  PRO          41 50N  094 10W  308

IA RED OAK          KRDK  RDK          41 01N  095 16W  318

IA SHELDON          KSHL  SHL          43 13N  095 49W  432

IA SHENANDOAH MUNI  KSDA  SDA          40 45N  095 25W  296

IA SIOUX CITY       KSUX  SUX   72557  42 23N  096 23W  338

IA SPENCER          KSPW  SPW   72650  43 10N  095 13W  403

IA STORM LAKE       KSLB  SLB          42 36N  095 13W  454

IA VINTON           KVTI  VTI          42 13N  092 01W  255

IA WASHINGTON       KAWG  AWG          41 16N  091 40W  230

IA WATERLOO         KALO  ALO   72548  42 33N  092 24W  264

IA WEBSTER CITY     KEBS  EBS          42 25N  093 52W  342

IA ADAIR I80        XADA               41 29N  094 43W  450

IA ALGONA US18      XALG               43 05N  094 23W  337

IA ALTON IA10       XATN               42 49N  096 05W  430

IA ALTOONA I80/US65 XALT               41 40N  093 31W  262

IA AMES I35         XAME               42 02N  093 34W  313

IA ANKENY I35       XANK               41 46N  093 34W  264

IA AVOCA I80        XAVO               41 29N  095 17W  377

IA BURLINGTON US34  XBUR               40 49N  091 05W  218

IA CARROLL US30     XCAR               42 05N  094 53W  389

IA CEDAR RAPIDS 380 XCDR               41 49N  091 40W  236

IA CEDAR RAPID US30 XCID               41 56N  091 41W  246

IA CENTERVILLE IA2  XCEN               40 44N  093 00W  295

IA COUNCIL BLUFF 80 XCOU               41 14N  095 52W  268

IA CRESTON US34     XCRE               41 04N  094 18W  388

IA DAVENPORT 80/280 XDAV               41 36N  090 41W  206

IA DECORAH IA9      XDEC               43 14N  091 41W  310

IA DES MOINES I35   XDSM               41 32N  093 46W  224

IA DES MOINES I235  XDES               41 35N  093 37W  281

IA DE SOTO 80/US169 XDST               41 32N  094 06W  291

IA DE WITT US30/61  XDEW               41 50N  090 34W  240

IA DUBUQUE US20     XDUB               42 29N  090 44W  250

IA FORT DODGE US20  XFOD               42 26N  094 11W  286

IA GRINNELL I80     XGRI               41 41N  092 44W  281

IA IOWA CITY US218  XIAC               41 38N  091 35W  254

IA IOWA CITY I80    XIOW               41 41N  091 35W  250

IA JEFFERSON IA4    XJEF               42 03N  094 23W  310

IA LEON I35/IA2     XLEO               40 44N  093 50W  349

IA MANCHESTER US20  XMAN               42 26N  091 26W  309

IA MAQUOKETA 61/64  XMAQ               42 04N  090 41W  251

IA MARSHALTOWN US30 XMAR               42 01N  092 58W  291

IA MASON CITY I35   XMCW               43 02N  093 20W  376

IA MISSOURI VAL I29 XMIS               41 32N  095 55W  309

IA MT PLEASANT U218 XMOU               40 53N  091 33W  133

IA NEW HAMPTON US18 XNEW               43 02N  092 28W  327

IA ONAWA I29        XONA               41 52N  096 01W  303

IA OSCEOLA I35      XOSC               41 02N  093 47W  334

IA OTTUMWA US63     XOTT               41 01N  092 25W  196

IA PELLA IA163      XPEL               41 23N  092 52W  271

IA RED OAK US34/71  XRED               40 59N  094 59W  290

IA SIDNEY I29/IA2   XSID               40 41N  095 47W  287

IA SIGOURNEY IA92   XSIG               41 20N  092 19W  202

IA SIOUX CITY I29   XSIO               42 29N  096 23W  364

IA SPENCER US18     XSPE               43 08N  095 05W  452

IA STORM LAKE 71/3  XSTO               42 44N  095 09W  405

IA TIPTON I80       XTIP               41 38N  091 08W  236

IA URBANA I380      XURB               42 19N  091 59W  302

IA WATERLOO US20    XWAT               42 27N  092 19W  269

IA WILLIAMS I35     XWIL               42 32N  093 34W  325

IA WILLAIMSBURG I80 XWBG               41 41N  092 01W  260

IA HANLONTOWN I35   XHAN               43 23N  093 20W  333

IA STEAMBOAT R US20 XSBI               42 27N  093 03W  348

IA IDA GROVE US59   XIGI               42 21N  095 29W  376

IA CEDAR RAPIDS U30 XCRI               41 56N  091 33W  232

IA BEDFORD          XBFI               40 41N  094 43W  357

IA COLFAX           XCFI               41 41N  093 16W  247

IA EDDYVILLE        XDYI               41 09N  092 39W  244

IA SIBLEY           XSYI               43 26N  095 43W  480

IA TAMA             XTMI               41 58N  092 18W  250

IA CANTRIL          XCTI               40 40N  092 04W  211

IA DENISON          XDNI               41 55N  095 20W  426

IA PLAINFIELD       XPFI               42 50N  092 32W  294

IA QUAD CITIES      XQCI               41 31N  090 31W  177

IA STATE FAIR       Q03I               41 36N  093 34W  333

IA ADAIR            QADI               41 31N  094 35W  417

IA ADEL             QAEI               41 37N  094 01W  292

IA AFTON            QAFI               41 02N  094 11W  387

IA ALGONA           QAGI               43 04N  094 14W  341

IA ANKENY           QAKI               41 44N  093 37W  310

IA ALBIA            QALI               41 02N  092 49W  312

IA AMES             QAMI               42 03N  093 38W  309

IA ANITA            QATI               41 27N  094 46W  387

IA AUDUBON          QAUI               41 43N  094 55W  426

IA BEDFORD          QBDI               40 40N  094 43W  350

IA BELMOND          QBKI               42 51N  093 36W  372

IA BLOOMFIELD       QBMI               40 45N  092 25W  260

IA BOONE            QBOI               42 02N  093 47W  353

IA BROOKLYN         QBRI               41 44N  092 27W  280

IA BUSSEY           QBSI               41 13N  092 53W  265

IA BAXTER           QBXI               41 50N  093 09W  304

IA BLANK PARK ZOO   QBZI               41 32N  093 37W  294

IA CARROLL          QCAI               42 04N  094 52W  423

IA COON RAPIDS      QCBI               41 53N  094 41W  386

IA CHARLES CITY     QCCI               43 04N  092 40W  304

IA CORYDON          QCDI               40 45N  093 19W  331

IA CENTERVILLE      QCEI               40 44N  092 52W  299

IA CLARION          QCGI               42 44N  093 44W  377

IA CHARITON         QCHI               41 01N  093 19W  332

IA CLEAR LAKE       QCKI               43 08N  093 22W  368

IA CRESCO           QCQI               43 23N  092 06W  390

IA CORNING          QCNI               40 59N  094 44W  390

IA COLO             QCOI               42 01N  093 19W  315

IA CRESTON          QCRI               41 04N  094 22W  417

IA INWOOD           QCSI               43 19N  096 26W  449

IA DSM CHRISTIAN    QDCI               41 38N  093 48W  279

IA SHELDON          QDNI               43 11N  095 50W  433

IA RIVER WOODS      QDRI               41 34N  093 35W  304

IA EAGLE GROVE      QEGI               42 40N  093 55W  339

IA FARNHAMVILLE     QFAI               42 20N  094 25W  374

IA FOREST CITY      QFCI               43 16N  093 39W  384

IA FORT DODGE       QFDI               42 31N  094 11W  353

IA FONTANELLE       QFOI               41 17N  094 34W  423

IA GLIDDEN          QGLI               42 04N  094 44W  387

IA GARNER           QGNI               43 05N  093 37W  373

IA GRIMES           QGRI               41 41N  093 48W  310

IA HULL             QHII               43 11N  096 08W  436

IA HAMPTON          QHPI               42 45N  093 11W  328

IA HUMBOLDT         QHUI               42 43N  094 14W  379

IA ICA              QIAI               41 35N  093 44W  285

IA IOWA FALLS       QIFI               42 32N  093 16W  376

IA INDIANOLA        QINI               41 22N  093 33W  310

IA JORDAN CREEK     QJCI               41 34N  093 46W  283

IA JEFFERSON        QJEI               42 00N  094 23W  338

IA SOUTH HAMILTON   QJWI               42 18N  093 39W  340

IA KCCI             QKCI               41 35N  093 38W  274

IA KNOXVILLE        QKNI               41 19N  093 07W  301

IA KANAWHA          QKWI               42 56N  093 47W  369

IA LEON             QLEI               40 45N  093 44W  365

IA LATIMER          QLMI               42 45N  093 22W  398

IA LAMONI           QLOI               40 38N  093 56W  359

IA LUVERNE          QLUI               42 55N  094 05W  360

IA ROCK RAPIDS      QLYI               43 25N  096 10W  410

IA MARSHALLTOWN     QMAI               42 01N  092 55W  314

IA MADRID           QMDI               41 53N  093 49W  310

IA LAKE MILLS       QMII               43 25N  093 32W  393

IA MALLARD          QMLI               42 56N  094 41W  387

IA MONTEZUMA        QMNI               41 35N  092 31W  309

IA MOUNT AYR        QMOI               40 43N  094 14W  396

IA MANSON-NW        QMSI               42 30N  094 22W  368

IA MURRAY           QMUI               41 02N  093 57W  365

IA MASON CITY       QMWI               43 08N  093 14W  357

IA MASON CITY       QMYI               43 09N  093 10W  335

IA NEWTON           QNEI               41 42N  093 02W  305

IA THOMPSON         QNII               43 22N  093 42W  384

IA NORTHWOOD        QNKI               43 27N  093 12W  371

IA NORA SPRINGS     QNSI               43 09N  093 00W  332

IA NEVADA           QNVI               42 01N  093 26W  313

IA OSAGE            QOAI               43 17N  092 49W  353

IA OSKALOOSA        QOCI               41 18N  092 39W  245

IA OGDEN            QOGI               42 02N  094 02W  332

IA OSCEOLA          QOSI               41 02N  093 46W  353

IA PANORA           QPAI               41 42N  094 22W  356

IA PELLA            QPEI               41 25N  092 54W  289

IA NORTH POLK       QPKI               41 47N  093 43W  286

IA PERRY            QPYI               41 50N  094 05W  312

IA MASON CITY       QRMI               43 08N  093 06W  328

IA ROCKWELL CITY    QROI               42 23N  094 38W  384

IA ROCKFORD         QRRI               43 03N  092 56W  316

IA ROCKWELL         QRSI               42 59N  093 06W  351

IA RUSSELL          QRUM               44 19N  095 57W  466

IA RICEVILLE        QRVI               43 21N  092 30W  382

IA ST ANSGAR        QSAI               43 22N  092 55W  358

IA SIBLEY           QSBI               43 24N  095 45W  462

IA STUART           QSTI               41 30N  094 19W  367

IA SULLY            QSUI               41 34N  092 50W  289

IA TITONKA          QTKI               43 14N  094 02W  348

IA MESKWAKI SS TAMA QTQI               41 59N  092 39W  256

IA UNION            QUNI               42 14N  093 04W  301

IA VENTURA          QVTI               43 08N  093 29W  386

IA WALL LAKE        QWAI               42 16N  095 06W  426

IA WEBSTER CITY     QWBI               42 28N  093 49W  345

IA WOODWARD         QWGI               41 52N  093 55W  291

IA WINTERSET        QWII               41 20N  094 01W  356

ID BOISE            KBOI  BOI   72681  43 34N  116 14W  875

ID BURLEY           KBYI  BYI          42 33N  113 46W 1264

ID BONNERS FERRY    K65S  65S          48 44N  116 18W  713

ID CALDWELL         KEUL  EUL          43 37N  116 37W  740

ID CHALLIS          KLLJ  LLJ          44 31N  114 13W 1536

ID COEUR D'ALENE    KCOE  COE          47 46N  116 49W  707

ID DRIGGS REED MEM  KDIJ  DIJ          43 45N  111 06W 1899

ID GRANGEVILLE      KGIC  GIC   72687  45 57N  116 07W 1009

ID HAILEY/FRIEDMAN  KSUN  SUN          43 30N  114 17W 1620

ID IDAHO FALLS      KIDA  IDA          43 31N  112 04W 1453

ID JEROME COUNTY    KJER  JER          42 43N  114 27W 1222

ID LEWISTON         KLWS  LWS   72783  46 22N  117 01W  438

ID LOWELL/ELK CITY  KP69  P69          46 09N  115 36W  480

ID MALAD CITY       KMLD  MLD          42 10N  112 17W 1373

ID MALTA (AWRS)     K77M  77M          42 19N  113 19W 1375

ID MCCALL           KMYL  MYL          44 53N  116 06W 1533

ID MOUNTAIN HOME    KMUO  MUO          43 02N  115 52W  913

ID MULLAN PASS      KMLP  MLP          47 27N  115 40W 1837

ID NAMPA            KMAN  MAN          43 35N  116 31W  774

ID POCATELLO        KPIH  PIH   72578  42 55N  112 34W 1359

ID REXBURG          KRXE  RXE          43 50N  111 48W 1480

ID SALMON/LEMHI     KSMN  SMN   72686  45 07N  113 52W 1233

ID SODA SPRINGS     KU78  U78          42 38N  111 34W 1780

ID SPENCER          KS14  S14          44 21N  112 10W 1800

ID STANLEY          KSNT  SNT          44 10N  114 56W 1980

ID TWIN FALLS       KTWF  TWF          42 29N  114 29W 1266

ID WALL/SANDPOINT   KSZT  SZT          48 18N  116 34W  648

IL ALTON/ST LOUIS R KALN  ALN          38 53N  090 02W  166

IL BLOOMINGTON/NORM KBMI  BMI          40 28N  088 55W  267

IL CAHOKIA/ST LOUIS KCPS  CPS          38 34N  090 09W  126

IL CAIRO            KCIR  CIR          37 04N  089 13W   98

IL CARBONDALE/MURPH KMDH  MDH          37 47N  089 15W  130

IL CARMI MUNI       KCUL               38 05N  088 07W  118

IL CENTRALIA        KENL  ENL          38 31N  089 06W  163

IL CHAMPAIGN/URBANA KCMI  CMI          40 02N  088 16W  228

IL CHICAGO          KMDW  MDW   72534  41 47N  087 45W  188

IL CHICAGO O'HARE   KORD  ORD   72530  41 59N  087 56W  200

IL CHICAGO/AURORA   KARR  ARR   74465  41 46N  088 29W  215

IL CHICAGO/DUPAGE   KDPA  DPA          41 55N  088 15W  231

IL CHICAGO/MEIGS    KCGX  CGX          41 52N  087 35W  181

IL CHICAGO/LANSING  KIGQ  IGQ          41 32N  087 32W  188

IL DANVILLE         KDNV  DNV          40 12N  087 36W  212

IL DECATUR          KDEC  DEC          39 50N  088 51W  207

IL DE KALB          KDKB  DKB          41 56N  088 42W  279

IL EFFINGHAM        K1H2  1H2          39 04N  088 32W  179

IL FAIRFIELD        KFWC  FWC          38 23N  088 25W  133

IL FREEPORT         KFEP  FEP          42 15N  089 35W  262

IL FLORA            KFOA  FOA          38 40N  088 27W  144

IL GALESBURG        KGBG  GBG          40 56N  090 26W  233

IL HARRISBURG       KHSB  HSB          37 49N  088 33W  121

IL JACKSONVILLE     KIJX  IJX          39 46N  090 14W  190

IL JOLIET           KJOT  JOT          41 31N  088 10W  177

IL KANKAKEE         KIKK  IKK          41 04N  087 51W  192

IL LACON            KC75  C75          41 01N  089 23W  173

IL LAWRENCEVILLE    KLWV  LWV          38 46N  087 36W  131

IL LINCOLN          KAAA  AAA          40 10N  089 20W  182

IL LITCHFIELD MUNI  K3LF  3LF          39 10N  089 40W  210

IL MACOMB MUNI      KMQB  MQB          40 31N  090 39W  215

IL MARION REGIONAL  KMWA  MWA          37 45N  089 01W  144

IL MARSEILLES       KMMO  MMO   74460  41 22N  088 40W  225

IL MATTOON/CHARLEST KMTO  MTO          39 28N  088 16W  214

IL METROPOLIS       KM30  M30          37 11N  088 45W  117

IL MOLINE/QUAD CITY KMLI  MLI   72544  41 27N  090 31W  184

IL MORRIS-WASHBURN  KC09  C09          41 26N  088 25W  178

IL MOUNT CARMEL     KAJG  AJG          38 36N  087 44W  131

IL MOUNT VERNON     KMVN  MVN          38 19N  088 52W  146

IL OLNEY-NOBLE      KOLY  OLY          38 43N  088 11W  147

IL PALWAUKEE        KPWK  PWK          42 07N  087 54W  203

IL PEORIA           KPIA  PIA   72532  40 40N  089 41W  205

IL PARIS            KPRG  PRG          39 42N  087 40W  199

IL PERU             KVYS  VYS          41 21N  089 09W  199

IL PITTSFIELD       KPPQ  PPQ          39 38N  090 47W  216

IL PONTIAC          KPNT  PNT          40 55N  088 38W  201

IL QUINCY MUNI/BALD KUIN  UIN          39 57N  091 12W  234

IL RANTOUL          KTIP  TIP          40 18N  088 09W  225

IL ROBINSON MUNI    KRSV  RSV          39 01N  087 39W  141

IL ROCKFORD         KRFD  RFD   72543  42 12N  089 06W  221

IL ROCHELLE/KORITZ  KRPJ  RPJ          41 53N  089 05W  238

IL ROMEOVILLE/CHI   KLOT  LOT          41 37N  088 06W  205

IL SALEM/LECKRONE   KSLO  SLO          38 39N  088 58W  175

IL SAVANNA/TRITOWN  KSFY  SFY          42 03N  090 07W  188

IL SCOTT AFB/BELLEV KBLV  BLV          38 32N  089 50W  138

IL SPARTA           KSAR  SAR          38 09N  089 42W  164

IL SPRINGFIELD      KSPI  SPI   72439  39 51N  089 41W  181

IL STERLING ROCKFAL KSQI  SQI          41 45N  089 40W  197

IL TAYLORVILLE      KTAZ  TAZ          39 32N  089 20W  190

IL WAUKEGAN         KUGN  UGN          42 25N  087 52W  222

IN ANDERSON MUNICIP KAID  AID          40 07N  085 37W  280

IN AUBURN DEKALB C  KGWB  GWB          41 18N  085 04W  269

IN BLOOMINGTON      KBMG  BMG          39 09N  086 37W  257

IN COLUMBUS/BALKALA KBAK  BAK          39 16N  085 54W  200

IN ELKHART MUNICIPA KEKM  EKM          41 43N  086 00W  237

IN EVANSVILLE       KEVV  EVV   72432  38 03N  087 31W  117

IN FORT WAYNE       KFWA  FWA   72533  40 59N  085 11W  248

IN GARY REGIONAL    KGYY  GYY          41 37N  087 25W  180

IN GOSHEN           KGSH  GSH          41 32N  085 47W  252

IN GRISSOM AFB/PERU KGUS  GUS          40 38N  086 09W  247

IN HUNTINGBURG      KHNB  HNB          38 15N  086 57W  161

IN INDIANAPOLIS     KEYE  EYE          39 50N  086 18W  248

IN INDIANAPOLIS     KIND  IND   72438  39 43N  086 18W  241

IN INDIANAPOLIS EXC KTYQ  TYQ          40 02N  086 15W  281

IN KNOX STARKE CTY  KOXI  OXI          41 20N  086 40W  209

IN KOKOMO           KOKK  OKK          40 32N  086 04W  253

IN LAFAYETTE        KLAF  LAF          40 25N  086 56W  182

IN MARION MUNI      KMZZ  MZZ          40 29N  085 41W  262

IN MUNCIE           KMIE  MIE          40 14N  085 24W  285

IN RENSSELAER       KRZL  RZL          40 57N  087 11W  213

IN ROCHESTER        KRCR  RCR          41 04N  086 11W  241

IN SHELBYVILLE      KGEZ  GEZ          39 35N  085 48W  245

IN SOUTH BEND       KSBN  SBN   72535  41 42N  086 19W  237

IN TERRE HAUTE      KHUF  HUF   72437  39 27N  087 20W  179

IN VALPARAISO       KVPZ  VPZ          41 27N  087 00W  231

IN WARSAW MUNI      KASW  ASW          41 16N  085 50W  259

KS BURLINGTON       KUKL  UKL          38 18N  095 44W  358

KS CHANUTE          KCNU  CNU          37 40N  095 29W  297

KS COFFEYVILLE      KCFV  CFV          37 05N  095 34W  225

KS CONCORDIA        KCNK  CNK   72458  39 33N  097 39W  447

KS DODGE CITY       KDDC  DDC   72451  37 46N  099 58W  789

KS ELKHART          KEHA  EHA   72460  37 00N  101 54W 1099

KS EMPORIA          KEMP  EMP          38 20N  096 12W  367

KS FORT LEAVENWORTH KFLV  FLV          39 22N  094 55W  235

KS FT RILEY/MARSHAL KFRI  FRI   72455  39 02N  096 46W  325

KS GARDEN CITY      KGCK  GCK          37 55N  100 43W  877

KS GOODLAND         KGLD  GLD   72465  39 22N  101 42W 1113

KS GREAT BEND       KGBD  GBD          38 21N  098 52W  575

KS HAYS MUNI        KHYS  HYS          38 51N  099 16W  609

KS HILL CITY        KHLC  HLC          39 22N  099 50W  669

KS HUTCHINSON       KHUT  HUT          38 04N  097 52W  467

KS INDEPENDENCE     KIDP  IDP          37 10N  095 47W  251

KS JENNINGS         KJN8  JN8          39 41N  100 18W  762

KS LAWRENCE         KLWC  LWC          39 01N  095 13W  253

KS LEOTI/HOARD MEML K3K7  3K7          38 27N  101 21W 1007

KS LIBERAL          KLBL  LBL          37 02N  100 58W  879

KS MANHATTAN        KMHK  MHK          39 08N  096 41W  317

KS MCCONNELL AFB    KIAB  IAB          37 37N  097 16W  418

KS MEDICINE LODGE   KP28  P28   72452  37 17N  098 33W  469

KS NEW ALMELO       KAL8  AL8          39 36N  100 07W  734

KS NEWTON           KEWK  EWK          38 02N  097 16W  467

KS OLATHE/INDUSTRIA KIXD  IXD          38 49N  094 53W  342

KS OLATHE/EXECUTIVE KOJC  OJC          38 51N  094 44W  326

KS PARSONS          KPPF  PPF          37 20N  095 30W  274

KS PITTSBURG        KPTS  PTS          37 27N  094 44W  290

KS PRATT INDUST     KPTT  PTT          37 42N  098 45W  595

KS RUSSELL          KRSL  RSL          38 52N  098 49W  567

KS SALINA           KSLN  SLN          38 47N  097 39W  385

KS TOPEKA/FORBES    KFOE  FOE          38 56N  095 39W  320

KS TOPEKA           KTOP  TOP   72456  39 04N  095 38W  268

KS WAKEENEY         KWK6  WK6          39 02N  099 53W  752

KS WICHITA          KICT  ICT   72450  37 39N  097 26W  407

KS WICHITA/JABARA   KAAO  AAO          37 45N  097 13W  434

KS WINFIELD/ARKANSA KWLD  WLD          37 10N  097 02W  350

KS OBERLIN          KOIN  OIN          39 50N  100 32W  824

KS SYRACUSE         K3K3  3K3          38 00N  101 45W 1013

KS NORTON           KNRN  NRN          39 51N  099 54W  727

KS TRIBUNE          KTRB  TRB          38 27N  101 45W 1103

KS COLBY            KCBK  CBK          39 26N  101 03W  971

KS GRINNELL         KGNL  GNL          39 08N  100 37W  887

KS HOXIE            KHOX  HOX          39 22N  100 26W  833

KS PHILLIPSBURG     KPHG  PHG          39 44N  099 19W  582

KS HERNDON          KHD7  HD7          39 55N  100 50W  814

KS LOGAN            KLG5  LG5          39 40N  099 34W  597

KS ST FRANCIS       KSYF  SYF          39 46N  101 48W 1025

KS ABILENE          KK78  K78          38 54N  097 14W  352

KS ELLSWORTH        K9K7  9K7          38 45N  098 14W  492

KS EUREKA           K13K  13K          37 51N  096 18W  369

KS FORT SCOTT       KFSK  FSK          37 48N  094 46W  280

KS HUGOTON          KHQG  HQG          37 10N  101 22W  956

KS JOHNSON          K2K3  2K3          37 35N  101 44W 1013

KS KINGMAN          K9K8  9K8          37 40N  098 07W  490

KS MCPHERSON        KMPR  MPR          38 21N  097 41W  457

KS OAKLEY           KOEL  OEL          39 07N  100 49W  929

KS SCOTT CITY       KTQK  TQK          38 28N  100 53W  904

KS SMITH CENTER     KK82  K82          39 46N  098 48W  549

KS ULYSSES          KULS  ULS          37 36N  101 22W  936

KS WELLINGTON       KEGT  EGT          37 19N  097 23W  390

KY BOWLING GREE     KBWG  BWG          36 58N  086 25W  168

KY COVINGTON        KCVG  CVG   72421  39 03N  084 40W  269

KY DANVILLE         KDVK  DVK          37 35N  084 46W  312

KY HOPKINSVILLE     KHOP  HOP   74671  36 40N  087 30W  174

KY FORT KNOX/GODMAN KFTK  FTK   72424  37 53N  085 58W  230

KY FRANKFORT        KFFT  FFT          38 11N  084 54W  236

KY GLASGOW          KGLW  GLW          37 02N  085 57W  218

KY HENDERSON CITY   KEHR  EHR          37 49N  087 40W  117

KY LEXINGTON        KLEX  LEX   72422  38 02N  084 36W  300

KY LONDON           KLOZ  LOZ   72329  37 05N  084 04W  362

KY LOUISVILLE       KSDF  SDF   72423  38 11N  085 44W  146

KY LOUISVILLE/BOWMN KLOU  LOU          38 13N  085 40W  164

KY LOUISVILLE/WFO   KLMK  LMK          38 14N  085 40W  166

KY MIDDLESBORO-BELL K1A6  1A6          36 37N  083 44W  352

KY MONTICELLO/WAYNE KEKQ  EKQ          36 51N  084 51W  294

KY MURRAY KYLE-OAK  KCEY  CEY          36 40N  088 22W  176

KY NOCTOR/JACKSON   KJKL  JKL          37 36N  083 19W  416

KY OWENSBORO/DAVIES KOWB  OWB          37 43N  087 10W  124

KY PADUCAH          KPAH  PAH   72435  37 04N  088 46W  119

KY RICHMOND/MADISON KI39  I39          37 38N  084 20W  306

KY SOMERSET         KSME  SME          37 03N  084 37W  283

LA ABBEVILLE CC MEM K0R3  0R3          29 59N  092 05W    5

LA ALEXANDRIA       KAEX  AEX   74754  31 20N  092 34W   27

LA ALEXANDRIA       KESF  ESF          31 24N  092 17W   28

LA AMELIA/LAKE PALO K7R3  7R3          29 41N  091 05W    2

LA AUDUBON PARK     KAUD  AUD          29 56N  090 08W    2

LA BARKSDALE AFB    KBAD  BAD          32 30N  093 40W   51

LA BASTROP MOREHOUS KBQP  BQP          32 45N  091 53W   51

LA BATON ROUGE      KBTR  BTR          30 32N  091 09W   21

LA BOGALUSA CARR F  KBXA  BXA          30 49N  089 52W   36

LA BOOTHVILLE       KBVE  BVE   72232  29 19N  089 24W    1

LA CAMERON HELIPORT K7R5  7R5          29 46N  093 17W    1

LA CHENAULT/LK CHAS KCWF  CWF          30 13N  093 09W    4

LA DERIDDER/FSS     KDRI  DRI          30 50N  093 20W   63

LA EUGENE IS 330    K3B6  3B6          28 14N  091 41W    1

LA FORT POLK FR     KPOE  POE   72239  31 03N  093 12W  101

LA PEASON RIDG/POLK KAQV  AQV          31 24N  093 18W  111

LA FULLERTON/POLK   KBKB  BKB          31 00N  092 58W   94

LA SELF STRIP/POLK  KDNK  DNK          31 07N  092 58W  114

LA FOURCHON (SAWRS) K9F2  9F2          29 06N  090 12W    2

LA PORT FOURCHON    KXPY  XPY          29 07N  090 12W    2

LA GALLIANO         KGAO  GAO          29 27N  090 16W    1

LA GALLIANO HELIPRT K2GL  2GL          29 25N  090 18W    3

LA GRAND ISLE (SAWR KAXO  AXO   99429  29 15N  089 58W    2

LA GREEN CANYON 184 KXCN  XCN          27 46N  091 31W    1

LA GREEN CANYON 52  K28K  28K          27 53N  091 30W    1

LA HAMMOND          KHDC  HDC          30 31N  090 25W   13

LA HIGH ISLAND 334A KH02  H02          28 06N  093 24W   30

LA HIGH IS. 264C    KH08  H08          28 28N  093 44W   30

LA OFFSHORE         KH78  H78          28 11N  088 29W   43

LA HOUMA TERREBONNE KHUM  HUM          29 34N  090 40W    3

LA INTRACOASTAL CIT K7R4  7R4          29 46N  092 07W    5

LA LAFAYETTE        KLFT  LFT          30 12N  092 00W   11

LA LAKE CHARLES     KLCH  LCH   72240  30 08N  093 13W    4

LA MINDEN           KMNE  MNE          32 39N  093 18W   85

LA MONROE           KMLU  MLU          32 31N  092 02W   29

LA NATCHITOCHES     KIER  IER          31 44N  093 06W   37

LA NEW IBERIA       KARA  ARA          30 02N  091 53W   18

LA NEW ORLEANS/LAKE KNEW  NEW          30 03N  090 02W    3

LA NEW ORLEANS NAS  KNBG  NBG          29 49N  090 01W    1

LA NEW ORLEANS/INTL KMSY  MSY   72231  30 00N  090 15W    5

LA OAKDALE ALLEN P  KACP  ACP          30 45N  092 41W   33

LA RUSTON REGIONAL  KRSN  RSN          32 31N  092 35W   95

LA PATTERSON MEMORI KPTN  PTN          29 43N  091 19W    3

LA SALT POINT       KP92  P92          29 34N  091 32W    1

LA SHIP SHOAL 207   KGSM  GSM          28 32N  090 59W    1

LA SHREVEPORT/DWNTN KDTN  DTN          32 33N  093 45W   53

LA SHREVEPORT       KSHV  SHV   72248  32 27N  093 50W   83

LA SLIDELL 22       KASD  ASD          30 21N  089 49W    9

LA S MARSH ISL 268  KSRN  SRN          29 07N  091 52W    1

LA S MARSH ISLAND   K7R8  7R8          28 18N  091 58W    1

LA SOUTH TIMBALIER  KS58  S58          28 31N  090 34W    1

LA SULPHUR          KUXL  UXL          30 08N  093 23W    4

LA TALLULAH/VICKSBU KTVR  TVR          32 21N  091 02W   26

LA VENICE HELIPORT  K1B7  1B7          29 21N  089 26W    1

LA E. CAMERON 346   KE12  E12          28 04N  092 42W    1

LA W. CAMERON 560   KW60  W60          28 09N  093 21W    1

LA PHI OIL PLATF    KQT8  QT8          27 32N  092 26W    1

LA PHI OIL PLATF    KQT9  QT9          27 47N  090 38W    1

LA PHI OIL PLATF    K4CO  4CO          29 47N  093 11W    1

LA PHI OIL PLATF    K4C0  4C0          29 47N  093 11W    1

LA GARDEN BANKS172  KGHB  GHB          27 50N  091 59W   30

LA MISS CANYAN 807  KCYD  CYD          28 10N  089 13W    1

LA INDEPEDENCE 920  KIPN  IPN          28 05N  087 59W   41

LA MAGNOLIA OILP    KGBK  GBK          27 12N  092 12W   59

LA SHIP SHOAL178    KSPR  SPR          28 36N  091 12W   31

LA MISS CANYON 311A KMDJ  MDJ          28 39N  089 48W   31

LA E CAMERON278OILP KEHC  EHC          28 26N  092 53W   31

LA SABINE 13B  OILP KVBS  VBS          29 29N  093 38W   31

LA S MARSH268  OILP KSCF  SCF          29 07N  091 52W    8

LA VERMILLION26OILP KVNP  VNP          29 28N  092 22W   28

LA EUGENE I 215OILP KEIR  EIR          28 38N  091 29W   28

LA ATLANTIS    OILP KATP  ATP          27 12N  090 02W   62

LA E CAMERON 47JP   KCMB  CMB          29 26N  092 59W   29

LA GREEN CANYON 338 KGRY  GRY          27 38N  090 27W   58

LA HIGH IS. 179A    KXIH  XIH          29 11N  094 31W   75

LA MAIN PASS 289C   KVKY  VKY          29 15N  088 26W   32

LA MAIN PASS 140B   KMIS  MIS          29 18N  088 51W   32

LA S TIMBALIER 301B KSTZ  STZ          28 10N  090 40W   25

LA VERMILION 331    KVQT  VQT          28 16N  092 16W   25

LA MISS CANYON 474  KIKT  IKT          28 31N  088 17W   25

MA BEDFORD          KBED  BED   74490  42 28N  071 18W   50

MA BEVERLY          KBVY  BVY          42 35N  070 55W   28

MA BOSTON           KBOS  BOS   72509  42 22N  071 01W    6

MA CHATHAM          KCHH  CHH   74494  41 40N  069 58W   16

MA CHATHAM          KCQX  CQX          41 41N  070 00W   19

MA CHICOPEE/WESTOVE KCEF  CEF   74491  42 12N  072 31W   75

MA CHICOPEE/WESTOVE             72491  42 12N  072 31W   75

MA EAST MILTON      KMQE  MQE          42 13N  071 07W  199

MA FALMOUTH/OTIS AB KFMH  FMH          41 38N  070 31W   40

MA FITCHBURG        KFIT  FIT          42 33N  071 45W  103

MA HYANNIS          KHYA  HYA          41 40N  070 16W   22

MA LAWRENCE         KLWM  LWM          42 43N  071 08W   54

MA MARSHFIELD       KGHG  GHG          42 06N  070 40W    4

MA MARTHAS VINEYARD KMVY  MVY          41 24N  070 37W   16

MA NANTUCKET        KACK  ACK   72506  41 15N  070 04W   16

MA NEW BEDFORD      KEWB  EWB          41 41N  070 57W   23

MA NORTH ADAMS      KAQW  AQW          42 42N  073 10W  199

MA NORWOOD          KOWD  OWD          42 11N  071 10W   19

MA ORANGE           KORE  ORE          42 34N  072 17W  164

MA PITTSFIELD       KPSF  PSF          42 26N  073 17W  355

MA PLYMOUTH         KPYM  PYM          41 55N  070 44W   43

MA PROVINCETOWN     KPVC  PVC          42 04N  070 13W    2

MA SOUTH WEYMOUTH   KNZW  NZW          42 08N  070 55W   49

MA TAUNTON          KTAN  TAN          41 53N  071 01W    7

MA WESTFIELD        KBAF  BAF          42 10N  072 43W   83

MA WORCESTER        KORH  ORH          42 16N  071 52W  304

MD ABERDEEN/PHILLIP KAPG  APG   74002  39 28N  076 10W   18

MD ANNAPOLIS/NAVAL  KNAK               38 59N  076 29W    2

MD ANDREWS AFB      KADW  ADW   74594  38 49N  076 52W   86

MD BALTO/WASH INTL  KBWI  BWI   72406  39 10N  076 41W   59

MD BALTIMORE/MARTIN KMTN  MTN          39 19N  076 25W    7

MD BALTIMORE/DWNTWN KDMH  DMH          39 17N  076 37W    5

MD CAMBRIDGE-DORCH  KCGE  CGE          38 32N  076 02W    6

MD CAMP DAVID       KRSP               39 39N  077 28W  561

MD CARROLL CNTY     KDMW  DMW          39 36N  077 00W  241

MD COLLEGE PARK     KCGS  CGS          38 59N  076 55W   15

MD CUMBERLAND REG   KCBE  CBE          39 37N  078 46W  236

MD EASTON/NEWNAM    KESN  ESN          38 48N  076 04W   22

MD FORT MEADE       KFME  FME          39 05N  076 46W   46

MD FORT RITCHIE     KJWX  JWX          39 43N  077 25W   84

MD FREDERICK        KFDK  FDK          39 25N  077 22W   92

MD GARRETT CO       K2G4  2G4          39 35N  079 20W  894

MD GAITHERSBURG     KGAI  GAI          39 10N  077 10W  165

MD HAGERSTOWN       KHGR  HGR          39 42N  077 44W  224

MD OCEAN CITY       KOXB  OXB          38 19N  075 07W    3

MD PATUXENT RIVER   KNHK  NHK   72404  38 16N  076 24W   12

MD RIDGELY          KRJD  RJD          38 58N  075 52W   20

MD SALISBURY        KSBY  SBY          38 20N  075 30W   19

MD STEVENSVILLE     KW29  W29          38 59N  076 20W   15

MD ST MARYS (DUKE)  K2W6  2W6          38 19N  076 33W   44

MD WEBSTER NAVAL    KNUI  NUI          38 09N  076 26W    7

ME AUBURN/LEWISTON  KLEW  LEW          44 02N  070 16W   88

ME AUGUSTA          KAUG  AUG          44 19N  069 48W  109

ME BANGOR           KBGR  BGR   72607  44 48N  068 49W   57

ME BAR HARBOR       KBHB  BHB          44 27N  068 22W   26

ME BRUNSWICK NAS    KNHZ  NHZ   74392  43 52N  069 55W   23

ME CARIBOU          KCAR  CAR   72712  46 52N  068 01W  191

ME CLAYTON LAKE     K40B  40B          46 37N  069 31W  314

ME EASTPORT         KEPM  EPM   72608  44 55N  067 01W   14

ME FRENCHVILLE      KFVE  FVE          47 17N  068 18W  301

ME FRYEBURG         KIZG  IZG          43 59N  070 57W  135

ME GREENVILLE       KGNR  GNR   72619  45 28N  069 33W  427

ME HOULTON          KHUL  HUL          46 07N  067 48W  150

ME MILLINOCKET      KMLT  MLT          45 39N  068 42W  123

ME OLD TOWN/DEWITT  KOLD  OLD          44 57N  068 40W   39

ME PORTLAND         KPWM  PWM   72606  43 39N  070 18W   15

ME PRESQUE ISLE     KPQI  PQI   72713  46 40N  068 02W  146

ME ROCKLAND/KNOX    KRKD  RKD          44 04N  069 05W   17

ME SANFORD MUNI     KSFM  SFM          43 23N  070 43W   74

ME WATERVILLE       KWVL  WVL          44 31N  069 40W  101

ME WISCASSET        KIWI  IWI          43 58N  069 43W   20

MI ADRIAN           KADG  ADG          41 52N  084 05W  241

MI ALMA             KAMN  AMN          43 19N  084 41W  230

MI ALPENA           KAPN  APN   72639  45 04N  083 34W  210

MI ANN ARBOR        KARB  ARB          42 13N  083 44W  251

MI BAD AXE          KBAX  BAX          43 47N  082 59W  234

MI BATTLE CREEK     KBTL  BTL          42 19N  085 14W  290

MI BELLAIRE         KACB  ACB          44 59N  085 12W  190

MI BEAVER ISLAND    KSJX  SJX          45 42N  085 34W  204

MI BENTON HARBOR    KBEH  BEH          42 08N  086 25W  191

MI BIG RAPIDS       KRQB  RQB          43 43N  085 30W  302

MI CADILLAC/WEXFORD KCAD  CAD          44 16N  085 25W  398

MI CARO/TUSCOLA     KCFS  CFS          43 28N  083 27W  214

MI CHARLEVOIX       KCVX  CVX          45 18N  085 16W  204

MI CHARLOTTE        KFPK  FPK          42 34N  084 49W  272

MI CHEBOYGAN        KSLH  SLH          45 39N  084 31W  195

MI CHIPPEWA INTL    KCIU  CIU          46 15N  084 28W  244

MI COLDWATER        KOEB  OEB          41 55N  085 02W  292

MI COPPER HARBOR    KP59  P59          47 28N  087 53W  190

MI DETROIT/CITY AIR KDET  DET          42 24N  083 01W  190

MI DETROIT/WILLOW   KYIP  YIP          42 14N  083 32W  215

MI DETROIT/WAYNE    KDTW  DTW   72537  42 14N  083 20W  195

MI DETROIT/GROSSE I KONZ  ONZ          42 06N  083 09W  180

MI DRUMMOND IS      KDRM  DRM          46 00N  083 45W  204

MI ESCANABA         KESC  ESC   72648  45 45N  087 01W  187

MI FLINT            KFNT  FNT   72637  42 58N  083 45W  233

MI FRANKFORT        KFKS  FKS          44 38N  086 12W  193

MI FREMONT          KFFX  FFX          43 26N  086 00W  236

MI GAYLORD          KGLR  GLR          45 01N  084 41W  407

MI GRAND RAPIDS     KGRR  GRR   72635  42 53N  085 31W  237

MI GRAYLING AF      KGOV  GOV          44 41N  084 44W  353

MI GWINN/SAWYER     KSAW  SAW          46 21N  087 24W  372

MI HANCOCK          KCMX  CMX   72744  47 10N  088 29W  326

MI HARBOR SPRINGS   KMGN  MGN          45 26N  084 55W  209

MI HILLSDALE        KJYM  JYM          41 55N  084 35W  360

MI HOLLAND          KBIV  BIV          42 45N  086 06W  208

MI HOUGHTON LAKE    KHTL  HTL   72638  44 21N  084 40W  351

MI HOWELL           KOZW  OZW          42 37N  083 58W  293

MI IONIA            KY70  Y70          42 56N  085 04W  250

MI IRON MOUNTAIN    KIMT  IMT          45 49N  088 07W  349

MI IRONWOOD         KIWD  IWD          46 31N  090 07W  375

MI JACKSON/REYNOLDS KJXN  JXN          42 16N  084 28W  305

MI KALAMAZOO        KAZO  AZO          42 14N  085 33W  272

MI LAMBERTVILLE     KDUH  DUH          41 44N  083 39W  204

MI LANSING          KLAN  LAN   72539  42 47N  084 35W  264

MI LUDINGTON/MASON  KLDM  LDM          43 58N  086 24W  197

MI MACKINAC ISLAND  KMCD  MCD          45 51N  084 38W  226

MI MANISTEE         KMBL  MBL          44 16N  086 15W  189

MI MANISTIQUE       KISQ  ISQ          45 58N  086 10W  209

MI MANISTIQUE       KP75  P75          45 57N  086 14W  178

MI MARSHALL         KRMY  RMY          42 14N  084 57W  287

MI MASON            KTEW  TEW          42 34N  084 25W  280

MI MENOMINEE        KMNM  MNM          45 07N  087 37W  191

MI MIDLAND BARSTOW  KIKW  IKW          43 40N  084 16W  194

MI MONROE           KTTF  TTF          41 56N  083 25W  188

MI MOUNT PLEASANT   KMOP  MOP          43 37N  084 44W  230

MI MUNISING LAKESH  KP53  P53          46 25N  086 39W  187

MI MUSKEGON         KMKG  MKG   72636  43 10N  086 14W  191

MI NEWBERRY         KERY  ERY          46 18N  085 27W  265

MI OSCODA/WURTSMITH KOSC  OSC          44 27N  083 24W  193

MI OWOSSO           KRNP  RNP          43 00N  084 08W  224

MI PELLSTON         KPLN  PLN          45 34N  084 48W  217

MI PONTIAC          KPTK  PTK          42 40N  083 25W  299

MI PORT HOPE        KP58  P58          44 01N  082 48W  179

MI PORT HURON       KPHN  PHN          42 55N  082 31W  198

MI ROGERS CITY      KPZQ  PZQ          45 24N  083 49W  204

MI SAGINAW          KMBS  MBS          43 32N  084 05W  202

MI SAGINAW/BROWNE   KHYX  HYX          43 26N  083 52W  183

MI SALEM (LANSING)  KSVM  SVM          42 25N  083 36W  290

MI SAULT STE MARIE  KSSM  SSM          46 28N  084 22W  221

MI SAULT STE MARIE  KANJ  ANJ   72734  46 28N  084 22W  218

MI SELFRIDGE ANGB   KMTC  MTC          42 37N  082 49W  177

MI SOUTH HAVEN      KLWA  LWA          42 21N  086 15W  203

MI STURGIS/KIRSCH   KIRS  IRS          41 48N  085 26W  282

MI THREE RIVERS     KHAI  HAI          41 58N  085 36W  251

MI TRAVERSE CIT     KTVC  TVC          44 44N  085 34W  190

MI TROY/OAKLAND     KVLL  VLL          42 33N  083 11W  222

MN AITKIN NDB       KAIT  AIT          46 32N  093 40W  367

MN ALBERT LEA       KAEL  AEL          43 40N  093 22W  383

MN ALEXANDRIA       KAXN  AXN          45 52N  095 24W  433

MN APPLETON MUNICIP KAQP  AQP          45 13N  096 00W  311

MN AUSTIN MUNI      KAUM  AUM          43 40N  092 55W  375

MN BAUDETTE         KBDE  BDE          48 44N  094 37W  329

MN BEMIDJI          KBJI  BJI   72755  47 30N  094 55W  420

MN BENSON MUNI      KBBB  BBB          45 19N  095 39W  317

MN BIGFORK          KFOZ  FOZ          47 46N  093 39W  411

MN BRAINERD         KBRD  BRD          46 24N  094 08W  372

MN BUFFALO MUNI     KCFE  CFE          45 10N  093 51W  295

MN CAMBRIDGE MUNI   KCBG  CBG          45 34N  093 16W  287

MN CANBY/MYERS FLD  KCNB  CNB          44 44N  096 16W  364

MN CLOQUET          KCOQ  COQ          46 42N  092 30W  390

MN COOK MUNI AIRPOR KCQM  CQM          47 49N  092 41W  404

MN CRANE LAKE       KCDD  CDD          48 16N  092 29W  341

MN CROOKSTON MUNI   KCKN  CKN          47 51N  096 37W  273

MN DETROIT LAKES    KDTL  DTL          46 49N  095 52W  426

MN DODGE CENTER     KTOB  TOB          44 01N  092 49W  398

MN DULUTH (SKY HARB KDYT  DYT          46 43N  092 02W  186

MN DULUTH           KDLH  DLH   72745  46 51N  092 12W  435

MN ELBOW LAKE MUNI  KY63  Y63          45 59N  096 00W  367

MN ELY MUNI         KELO  ELO          47 49N  091 49W  443

MN EVELETH MUNI     KEVM  EVM          47 23N  092 30W  421

MN FAIRMONT MUNI    KFRM  FRM          43 38N  094 25W  354

MN FARIBAULT MUNI   KFBL  FBL          44 19N  093 19W  322

MN FERGUS FALLS     KFFM  FFM   72656  46 16N  096 09W  361

MN FLAG ISLAND      KFGN  FGN          49 19N  094 52W  328

MN FOSSTON          KFSE  FSE          47 35N  095 46W  389

MN GLENCOE          KGYL  GYL          44 45N  094 05W  302

MN GLENWOOD         KGHW  GHW          45 38N  095 19W  425

MN GRAND MARAIS     KGNA               47 45N  090 21W  185

MN GRAND MARAIS     KGRM  GRM          47 50N  090 23W  505

MN GRAND MARAIS/COO KCKC               47 50N  090 22W  548

MN GRAND RAPIDS     KGPZ  GPZ          47 13N  093 31W  413

MN GRANITE FALLS    KGDB  GDB          44 45N  095 33W  319

MN HALLOCK          KHCO  HCO          48 45N  096 56W  250

MN HIBBING          KHIB  HIB          47 23N  092 50W  412

MN HINCKLEY         K04W  04W          46 01N  092 54W  311

MN HUTCHINSON       KHCD  HCD          44 52N  094 22W  323

MN INTERNTNL FALLS  KINL  INL   72747  48 34N  093 24W  360

MN JACKSON MUNI     KMJQ  MJQ          43 38N  094 58W  441

MN LAKE ELMO        K21D  21D          45 00N  092 51W  285

MN LITCHFIELD       KLJF  LJF          45 06N  094 30W  348

MN LITTLE FALLS     KLXL  LXL          45 57N  094 20W  342

MN LONG PRAIRIE     K14Y  14Y          45 54N  094 52W  406

MN LONGVILLE        KXVG  XVG          46 59N  094 12W  407

MN LUVERNE          KLYV  LYV          43 37N  096 13W  436

MN MADISON          KDXX  DXX          44 59N  096 10W  330

MN MANKATO          KMKT  MKT          44 13N  093 55W  311

MN MARSHALL/RYAN    KMML  MML          44 27N  095 49W  359

MN MAPLE LAKE       KMGG  MGG          45 14N  093 59W  313

MN MCGREGOR/IVERSON KHZX  HZX          46 37N  093 19W  374

MN MINNEAPOLIS      KMSP  MSP   72658  44 53N  093 14W  265

MN MINNEAPLS/BLAINE KANE  ANE          45 08N  093 13W  278

MN MNPLS/FLYING CLD KFCM  FCM          44 50N  093 28W  280

MN MNPLS/CRYSTAL    KMIC  MIC          45 04N  093 21W  265

MN MNPLS/AIRLAKE    KLVN  LVN          44 38N  093 14W  293

MN MONTEVIDEO       KMVE  MVE          44 58N  095 43W  315

MN MOORHEAD MUNI    KJKJ  JKJ          46 50N  096 40W  280

MN MOOSE LAKE       KMZH  MZH          46 25N  092 48W  328

MN MORA             KJMR  JMR          45 53N  093 16W  309

MN MORRIS MUNI      KMOX  MOX          45 34N  095 58W  344

MN CANBY/MYERS FLD  K27D  27D          44 44N  096 16W  363

MN NEW ULM MUNI     KULM  ULM          44 19N  094 30W  308

MN OLIVIA           KOVL  OVL          44 47N  095 02W  328

MN ORR              KORB  ORB          48 01N  092 52W  397

MN ORTONVILLE       KVVV  VVV          45 18N  096 25W  335

MN OWATONNA         KOWA  OWA          44 07N  093 15W  350

MN PARK RAPIDS      KPKD  PKD          46 54N  095 04W  442

MN PAYNESVILLE      KPEX  PEX          45 22N  094 45W  360

MN PINE RIVER REGIO KPWC  PWC          46 43N  094 22W  395

MN PIPESTONE        KPQN  PQN          43 58N  096 19W  529

MN PRESTON          KFKA  FKA          43 41N  092 11W  389

MN PRINCETON MUNI   KPNM  PNM          45 33N  093 36W  298

MN RED WING         KRGK  RGK          44 35N  092 29W  239

MN REDWOOD FALL     KRWF  RWF          44 33N  095 05W  311

MN ROCHESTER        KRST  RST   72644  43 54N  092 30W  403

MN ST MARY HOSPITAL K9MN  9MN          44 01N  092 29W  355

MN ROSEAU MUNI      KROX  ROX          48 51N  095 42W  323

MN RUSH CITY        KROS  ROS          45 42N  092 57W  281

MN SAUK CENTRE      KD39  D39          45 42N  094 56W  380

MN SILVER BAY       KBFW  BFW          47 12N  091 24W  331

MN SLAYTON          KDVP  DVP          43 59N  095 47W  495

MN SOUTH ST PAUL    KSGS  SGS          44 51N  093 02W  250

MN ST. CLOUD        KSTC  STC   72655  45 33N  094 03W  314

MN ST. JAMES        KJYG  JYG          43 59N  094 33W  325

MN ST. PAUL         KSTP  STP          44 56N  093 03W  219

MN STANTON          KSYN  SYN          44 29N  093 01W  280

MN STAPLES          KSAZ  SAZ          46 22N  094 48W  392

MN THIEF RIVER      KTVF  TVF          48 04N  096 10W  340

MN TOFTE (RAMOS)    KP61  P61          47 34N  090 49W  241

MN TRACY            KTKC  TKC          44 15N  095 37W  408

MN TWO HARBORS      KTWM  TWM          47 02N  091 45W  328

MN WADENA MUNI      KADC  ADC          46 27N  095 13W  418

MN WARROAD          KRRT  RRT   72756  48 56N  095 20W  327

MN WASECA           KACQ  ACQ          44 04N  093 33W  343

MN WASKISH MUNI     KVWU  VWU          48 09N  094 31W  360

MN WHEATON NDB      KETH  ETH          45 47N  096 33W  313

MN WILLMAR/RICE     KILL  ILL          45 07N  095 04W  344

MN WILLMAR MUNI     KBDH  BDH          45 07N  095 08W  247

MN WINDOM MUNI AIRP KMWM  MWM          43 54N  095 06W  430

MN WINONA MUNI      KONA  ONA          44 04N  091 42W  200

MN WORTHINGTON      KOTG  OTG          43 38N  095 34W  480

MN AUSTIN           QASM               43 40N  092 57W  365

MN ALBERT LEA       QBLM               43 38N  093 23W  383

MN PRESTON          QFCM               43 40N  092 00W  318

MN ROCHESTER        QFMM               44 01N  092 28W  304

MN GRAND MEADOW     QGMM               43 43N  092 30W  403

MN HILLS            QHIM               43 31N  096 22W  342

MN ROCHESTER        QJAM               44 04N  092 29W  317

MN ROCHESTER        QKGM               44 02N  092 28W  319

MN LEROY            QLEM               43 31N  092 31W  393

MN MADISON          QMAM               45 01N  096 11W  340

MN WORTHINGTON      QMOM               43 38N  095 38W  483

MN LUVERNE          QMSM               43 40N  096 13W  443

MN LAMBERTON        QRCM               44 14N  095 16W  349

MN WEST CONCORD     QTMM               44 08N  092 54W  387

MN ROCHESTER        QWLM               43 59N  092 26W  312

MO BOONVILLE        KVER  VER          38 57N  092 41W  218

MO BRANSON          KBBG  BBG          36 32N  093 12W  397

MO CAMDENTON        KH21  H21          37 58N  092 41W  324

MO CAPE GIRARDEAU   KCGI  CGI          37 14N  089 35W  102

MO CHILLICOTHE      KCDJ  CDJ          39 49N  093 35W  234

MO CLINTON          KGLY  GLY          38 21N  093 41W  251

MO COLUMBIA         KCOU  COU   72445  38 49N  092 13W  271

MO FARMINGTON       KFAM  FAM          37 46N  090 25W  288

MO FORT LEONARD WOO KTBN  TBN          37 43N  092 07W  353

MO GRANDVIEW        KGVW  GVW          38 50N  094 34W  337

MO JEFFERSON CITY   KJEF  JEF          38 36N  092 09W  168

MO JOPLIN           KJLN  JLN          37 09N  094 30W  299

MO KAISER MEM       KAIZ  AIZ          38 06N  092 32W  265

MO KANSAS CITY/INTL KMCI  MCI   72446  39 18N  094 44W  320

MO KANSAS CITY/DNTN KMKC  MKC          39 07N  094 36W  227

MO KIRKSVILLE       KIRK  IRK          40 06N  092 33W  293

MO LEES SUMMIT MUNI KLXT  LXT          38 58N  094 22W  306

MO MALDEN           KMAW  MAW          36 36N  090 00W   90

MO MONETT           KHFJ  HFJ          36 54N  094 01W  401

MO POPULAR BLUFF    KPOF  POF          36 46N  090 19W   99

MO SEDALIA          KDMO  DMO          38 43N  093 10W  274

MO SPICKARD         KP35  P35   72540  40 15N  093 43W  270

MO SPRINGFIELD      KSGF  SGF   72440  37 14N  093 23W  390

MO ST. CHARLES      KSET  SET          38 56N  090 26W  134

MO ST. JOSEPH       KSTJ  STJ   72449  39 46N  094 55W  247

MO ST. LOUIS/SPIRIT KSUS  SUS          38 39N  090 39W  141

MO ST. LOUIS        KSTL  STL   72434  38 45N  090 22W  171

MO VICHY/ROLLA      KVIH  VIH          38 08N  091 46W  333

MO WARRENSBURG SKYH KRCM  RCM          38 47N  093 48W  244

MO WEST PLAINS      KUNO  UNO   72348  36 53N  091 54W  372

MO WHITEMAN AFB     KSZL  SZL          38 43N  093 32W  265

MS BAY ST LOUIS     KHSA  HSA          30 22N  089 27W    7

MS BILOXI/KEESLER   KBIX  BIX          30 25N  088 55W   10

MS BROOKHAVEN       K1R7  1R7          31 36N  090 25W  149

MS CLARKSDALE       KCKM  CKM          34 18N  090 31W   53

MS COLUMBUS AFB     KCBM  CBM          33 38N  088 27W   67

MS CORINTH/R TURNER KCRX  CRX          34 55N  088 36W  130

MS GOLDEN/COLUMBUS  KGTR  GTR          33 27N  088 34W   80

MS GREENVILLE       KGLH  GLH          33 30N  090 59W   45

MS GREENWOOD        KGWO  GWO          33 30N  090 05W   44

MS GULFPORT         KGPT  GPT          30 25N  089 05W   14

MS HATTIESBURG      KHBG  HBG          31 16N  089 15W   44

MS JACKSON/HAWKINS  KHKS  HKS          32 20N  090 13W  108

MS JACKSON          KJAN  JAN   72235  32 19N  090 05W   91

MS MCCOMB           KMCB  MCB          31 11N  090 28W  124

MS MERIDIAN NAS     KNMM  NMM          32 32N  088 34W   97

MS MERIDIAN/KEY FLD KMEI  MEI   72234  32 20N  088 45W   89

MS NATCHEZ/HARDY    KHEZ  HEZ          31 37N  091 17W   83

MS OLIVE BRANCH     KOLV  OLV          34 59N  089 47W  122

MS OXFORD           KUOX  UOX          34 23N  089 32W  137

MS PASCAGOULA       KPQL  PQL          30 28N  088 32W    6

MS PASCAGOULA       KPGL  PGL          30 24N  088 29W    3

MS PINE BELT RGNL   KPIB  PIB          31 28N  089 19W   91

MS RAYMOND          KM16  M16          32 18N  090 25W   76

MS STARKVILLE       KSTF  STF          33 26N  088 51W  102

MS TUNICA           KUTA  UTA          34 41N  090 21W   59

MS TUPELO           KTUP  TUP   72332  34 16N  088 46W  105

MS INNOVATOR OILP   KMYT  MYT          28 13N  089 37W   23

MS WEST DELTA 27A   KDLP  DLP          29 07N  089 33W   36

MT BAKER            KBHK               46 21N  104 15W  902

MT ENNIS BIG SKY    KEKS  EKS          45 16N  111 39W 1641

MT BILLINGS         KBIL  BIL   72677  45 48N  108 33W 1091

MT BOZEMAN          KBZN  BZN          45 47N  111 10W 1361

MT BROWNING         K8S0  8S0          48 36N  113 07W 1419

MT BUTTE            KBTM  BTM          45 58N  112 30W 1688

MT COLSTRIP         KM46  M46          45 51N  106 43W 1044

MT CUT BANK         KCTB  CTB          48 36N  112 22W 1175

MT DILLON           KDLN  DLN          45 15N  112 33W 1591

MT DRUMMOND         K3DU  3DU          46 40N  113 09W 1202

MT EKALAKA          K97M  97M          45 53N  104 32W 1068

MT GLASGOW          KGGW  GGW   72768  48 13N  106 37W  694

MT GLENDIVE         KGDV  GDV          47 07N  104 47W  749

MT GREAT FALLS      KGTF  GTF   72775  47 28N  111 23W 1119

MT HAMILTON/RAVALLI KHMM  HMM          46 15N  114 09W 1110

MT HARLOWTON        K3HT  3HT          46 25N  109 49W 1268

MT HAVRE            KHVR  HVR   72777  48 33N  109 46W  787

MT HELENA           KHLN  HLN   72772  46 36N  111 58W 1182

MT JORDAN           KJDN  JDN          47 20N  106 57W  801

MT KALISPELL        KGPI  GPI          48 19N  114 15W  908

MT LAUREL           K6S8  6S8          45 42N  108 46W 1072

MT LEWISTOWN        KLWT  LWT          47 02N  109 28W 1270

MT LIVINGSTON       KLVM  LVM          45 42N  110 26W 1418

MT MALMSTROM AFB    KGFA  GFA          47 30N  111 10W 1075

MT MALTA            KM75  M75          48 22N  107 55W  687

MT MILES CITY       KMLS  MLS   74230  46 26N  105 53W  801

MT MISSOULA         KMSO  MSO   72773  46 55N  114 06W  975

MT PLAINS           KS34  S34          47 28N  114 54W  752

MT ROUNDUP          KRPX  RPX          46 28N  108 33W 1065

MT SCOBEY           K9S2  9S2          48 48N  105 26W  742

MT SIDNEY RICHLAND  KSDY  SDY          47 42N  104 12W  605

MT THOMPSON FALLS   K3TH  3TH          47 36N  115 22W  725

MT TWIN BRIDGES     K7S1  7S1          45 32N  112 18W 1456

MT WEST YELLOWSTONE KWEY  WEY   72676  44 38N  111 05W 2031

MT WOLF POINT       KOLF  OLF          48 06N  105 35W  604

MT WEST YELLOWSTONE KWYS  WYS          44 40N  111 07W 2025

NC AHOSKIE/TRI COUN KASJ  ASJ          36 17N  077 10W   21

NC ALBEMARLE STANLY KVUJ  VUJ          35 25N  080 09W  186

NC ANDREWS          KRHP  RHP          35 11N  083 51W  518

NC WADESBORO/ANSON  KAFP  AFP          35 01N  080 05W   92

NC ASHEBORO MUNI    KHBI  HBI          35 39N  079 54W  205

NC ASHEVILLE        KAVL  AVL   72315  35 26N  082 32W  670

NC BEAUFORT         KMRH  MRH          34 44N  076 39W    3

NC BOGUE/SWANSBORO  KNJM  NJM          34 40N  077 01W    7

NC BOONE WATAUGA CT KTNB  TNB          36 12N  081 39W  911

NC BURLINGTON       KBUY  BUY          36 03N  079 28W  180

NC CAPE HATTERAS    KHAT  HAT   72302  35 16N  075 32W    3

NC CHAPEL HILL      KIGX  IGX          35 56N  079 04W  155

NC CHARLOTTE        KCLT  CLT   72314  35 13N  080 57W  220

NC CHERRY POINT     KNKT  NKT   72309  34 53N  076 52W    9

NC CLINTON          KCTZ  CTZ          34 59N  078 22W   45

NC CONCORD REG ARPT KJQF  JQF          35 23N  080 43W  210

NC CURRITUCK        KONX  ONX          36 24N  076 01W    5

NC DAVIDSON CTY AP  KEXX  EXX          35 47N  080 18W  223

NC EDENTON          KEDE  EDE          36 01N  076 34W    6

NC ELIZABETH CITY   KECG  ECG          36 16N  076 11W   11

NC ELIZABETHTOWN    KEYF  EYF          34 36N  078 35W   41

NC ERWIN/HARNETT CO KHRJ  HRJ          35 23N  078 44W   60

NC FAYETTEVILLE     KFAY  FAY          34 59N  078 53W   55

NC FORT BRAGG/SIMMO KFBG  FBG   74693  35 07N  078 55W   74

NC GASTONIA         KAKH  AKH          35 12N  081 09W  243

NC GOLDSBORO/S. J.  KGSB  GSB          35 19N  077 58W   33

NC GOLDSBORO/WAYNE  KGWW  GWW          35 28N  077 58W   41

NC GREENSBORO       KGSO  GSO   72317  36 06N  079 57W  275

NC HATTERAS/MITCHEL KHSE  HSE          35 14N  075 37W    3

NC HENDERSON/OXFORD KHNZ  HNZ          36 22N  078 32W  161

NC HICKORY          KHKY  HKY          35 45N  081 23W  354

NC HOFFMAN/MACKALL  KHFF  HFF          35 01N  079 30W  115

NC JACKSONVILLE     KOAJ  OAJ          34 49N  077 37W   29

NC JEFFERSON        KGEV  GEV          36 26N  081 25W  969

NC KENANSVILLE/DUPL KDPL  DPL          35 00N  077 58W   42

NC KILL DEVIL HILLS KFFA  FFA          36 01N  075 40W    4

NC KINSTON/STALLING KISO  ISO          35 19N  077 37W   29

NC LINCOLNTON       KIPJ  IPJ          35 29N  081 10W  267

NC LOUISBURG/FRANKL KLHZ  LHZ          36 01N  078 20W  113

NC LUMBERTON        KLBT  LBT          34 36N  079 04W   37

NC MACON/FRANKLIN   K1A5  1A5          35 13N  083 25W  616

NC MANTEO/DARE CO   KMQI  MQI          35 55N  075 42W    4

NC MAXTON           KMEB  MEB          34 47N  079 22W   67

NC MONROE           KEQY  EQY          35 01N  080 37W  212

NC MORGANTON/LENOIR KMRN  MRN          35 49N  081 37W  387

NC MOUNT AIRY/SURRY KMWK  MWK          36 28N  080 33W  380

NC NEW BERN         KEWN  EWN          35 04N  077 03W    3

NC NEW RIVER MCAS   KNCA  NCA          34 43N  077 27W    8

NC PINEY ISLAND     KNBT  NBT          35 01N  076 28W    5

NC PITT GREENVILLE  KPGV  PGV          35 37N  077 24W    8

NC POPE AFB         KPOB  POB   72303  35 10N  079 01W   66

NC RALEIGH/DURHAM   KRDU  RDU   72306  35 52N  078 47W  130

NC REIDSVILLE       KSIF  SIF          36 26N  079 51W  211

NC ROANOKE RAPIDS   KRZZ  RZZ          36 26N  077 43W   75

NC ROANOKE RAPIDS   KIXA  IXA          36 20N  077 38W   44

NC ROCKINGHAM       KRCZ  RCZ          34 53N  079 46W  109

NC ROCKY MOUNT      KRWI  RWI          35 51N  077 54W   47

NC ROXBORO PERSON   KTDF  TDF          36 17N  078 59W  186

NC RUTHERFORDTON    KFQD  FQD          35 25N  081 56W  329

NC SALISBURY ROWAN  KRUQ  RUQ          35 39N  080 31W  236

NC SANFORD          KTTA  TTA          35 35N  079 06W   75

NC SHELBY MUNI      KEHO  EHO          35 15N  081 36W  258

NC SOUTHERN PINES   KSOP  SOP          35 13N  079 24W  141

NC SMITHFIELD JOHNS KJNX  JNX          35 32N  078 23W   50

NC STATESVILLE      KSVH  SVH          35 45N  080 57W  294

NC SOUTHPORT        KSUT  SUT          33 56N  078 05W    8

NC TARBORO          KETC  ETC          35 56N  077 33W   17

NC WASHINGTON       KOCW  OCW          35 34N  077 03W   12

NC WHITEVILLE       KCPC  CPC          34 17N  078 43W   29

NC WILKESBORO       KUKF  UKF          36 13N  081 06W  396

NC WILMINGTON       KILM  ILM          34 16N  077 54W    9

NC WINSTON SALEM    KINT  INT   72319  36 08N  080 13W  291

ND BEACH            K20U  20U          46 56N  103 59W  840

ND BISMARCK         KBIS  BIS   72764  46 46N  100 45W  505

ND BOWMAN MUNI      KBPP  BPP          46 11N  103 26W  902

ND CANDO            K9D7  9D7          48 29N  099 14W  452

ND CARRINGTON       K46D  46D          47 27N  099 09W  490

ND CAVALIER         K2C8  2C8          48 47N  097 38W  272

ND COOPERSTOWN      KS32  S32          47 25N  098 06W  684

ND CROSBY           KD50  D50          48 56N  103 18W  595

ND DEVILS LAKE      KDVL  DVL   72757  48 07N  098 55W  443

ND DEVILS LAKE      KP11  P11   72758  48 07N  098 55W  443

ND DICKINSON        KDIK  DIK          46 48N  102 48W  788

ND FARGO            KFAR  FAR   72753  46 56N  096 49W  277

ND GARRISON         KN60  N60          47 39N  101 26W  582

ND GRAFTON          KGAF  GAF          48 24N  097 22W  252

ND GRAND FORKS AFB  KRDR  RDR          47 58N  097 24W  278

ND GRAND FORKS      KGFK  GFK          47 57N  097 11W  190

ND GWINNER          KGWR  GWR          46 13N  097 39W  386

ND HARVEY           K5H4  5H4          47 47N  099 56W  490

ND HAZEN            KHZE  HZE          47 17N  101 35W  553

ND HETTINGER        KHEI  HEI          46 01N  102 39W  828

ND JAMESTOWN        KJMS  JMS          46 56N  098 40W  455

ND LANGDON/ROBERTSN KD55  D55          48 45N  098 24W  491

ND LIDGERWOOD (RAMO KP67  P67          46 06N  097 09W  351

ND LINTON           K7L2  7L2          46 13N  100 15W  543

ND MANDAN           KY19  Y19          46 46N  100 54W  593

ND MINOT AFB        KMIB  MIB          48 25N  101 20W  508

ND MINOT            KMOT  MOT          48 15N  101 16W  519

ND OAKES            K2D5  2D5          46 10N  098 05W  407

ND ROLLA            K06D  06D          48 53N  099 37W  556

ND RUGBY            KRUG  RUG          48 23N  100 01W  472

ND STANLEY          K08D  08D          48 18N  102 24W  684

ND TIOGA            KD60  D60          48 23N  102 54W  693

ND VALLEY CITY      KBAC  BAC          46 56N  098 01W  684

ND WALHALLA         K96D  96D          48 56N  097 54W  290

ND WATFORD CITY     KS25  S25          47 48N  103 15W  644

ND WILLISTON        KISN  ISN   72767  48 10N  103 38W  579

ND WAHPETON         KBWP  BWP          46 15N  096 37W  295

NE AINSWORTH MUNICI KANW  ANW          42 34N  100 00W  789

NE ALBION MUNI      KBVN  BVN          41 44N  098 03W  551

NE ALLIANCE         KAIA  AIA          42 03N  102 48W 1196

NE AURORA           KAUH  AUH          40 53N  098 00W  550

NE BEATRICE MUNICIP KBIE  BIE          40 17N  096 45W  403

NE BLAIR MUNI       KBTA  BTA          41 25N  096 07W  396

NE BROKEN BOW MUNI  KBBW  BBW          41 26N  099 38W  776

NE CHADRON          KCDR  CDR          42 50N  103 06W 1010

NE COLUMBUS MUNI    KOLU  OLU          41 27N  097 19W  440

NE FALLS CITY/BRENN KFNB  FNB          40 04N  095 35W  300

NE FREMONT MUNI ARP KFET  FET          41 27N  096 31W  367

NE GRAND ISLAND     KGRI  GRI   72552  40 58N  098 19W  561

NE HASTINGS         KHSI  HSI          40 36N  098 26W  591

NE HEBRON MUNI      KHJH  HJH          40 09N  097 35W  447

NE HOLDREGE/BREWSTR KHDE  HDE          40 27N  099 19W  705

NE IMPERIAL MUNICIP KIML  IML          40 31N  101 37W  998

NE KEARNEY MUNI     KEAR  EAR          40 43N  099 00W  649

NE KIMBALL MUNI     KIBM  IBM          41 11N  103 41W 1501

NE LEXINGTON        KLXN  LXN          40 47N  099 46W  734

NE LINCOLN          KLNK  LNK   72551  40 51N  096 46W  364

NE MCCOOK           KMCK  MCK          40 12N  100 35W  782

NE NEBRASKA CITY    KAFK  AFK          40 36N  095 51W  354

NE NORFOLK          KOFK  OFK   72556  41 59N  097 26W  470

NE NORTH OMAHA      KOVN  OVN   72553  41 22N  096 01W  406

NE NORTH PLATTE     KLBF  LBF   72562  41 07N  100 40W  847

NE O'NEILL          KONL  ONL          42 28N  098 40W  619

NE OFFUTT AFB/BELLE KOFF  OFF   72554  41 07N  095 55W  319

NE OGALLALA         KOGA  OGA          41 07N  101 46W  999

NE OMAHA/EPPLEY     KOMA  OMA   72550  41 19N  095 54W  312

NE OMAHA/MILLARD    KMLE  MLE          41 12N  096 07W  320

NE ORD/SHARP FIELD  KODX  ODX          41 37N  098 57W  631

NE PLATTSMOUTH MUNI KPMV  PMV          40 57N  095 55W  367

NE SCOTTSBLUFF      KBFF  BFF   72566  41 52N  103 35W 1203

NE SIDNEY           KSNY  SNY   72561  41 06N  102 59W 1307

NE TEKAMAH          KTQE  TQE          41 46N  096 11W  312

NE THEDFORD/THOMAS  KTIF  TIF          41 58N  100 34W  892

NE VALENTINE        KVTN  VTN   72567  42 52N  100 33W  788

NE WAHOO            KAHQ  AHQ          41 14N  096 36W  374

NE WAYNE MUNI       KLCG  LCG          42 15N  096 59W  436

NE YORK             KJYR  JYR          40 54N  097 37W  509

NE BENKELMAN-JONES  K42V  42V          40 03N  101 32W  953

NE CHAMPION         KCHM  CHM          40 23N  101 43W 1029

NH BERLIN           KBML  BML   72616  44 35N  071 11W  345

NH CONCORD          KCON  CON   72605  43 12N  071 30W  103

NH JAFFREY          KAFN  AFN          42 48N  072 00W  313

NH KEENE/DILLANT    KEEN  EEN          42 53N  072 16W  149

NH LACONIA MUNI     KLCI  LCI          43 34N  071 25W  166

NH LEBANON          KLEB  LEB   72611  43 38N  072 18W  171

NH MANCHESTER       KMHT  MHT          42 56N  071 26W   81

NH MOUNT WASHINGTON KMWN  MWN   72613  44 16N  071 17W 1910

NH NASHUA/BOIRE FLD KASH  ASH          42 47N  071 31W   61

NH PLYMOUTH         K1P1  1P1          43 47N  071 45W  154

NH PORTSMOUTH/PEASE KPSM  PSM          43 04N  070 49W   31

NH ROCHESTER        KDAW  DAW          43 17N  070 55W  100

NH WHITEFIELD       KHIE  HIE          44 22N  071 33W  318

NJ ANDOVER          K12N  12N          41 01N  074 44W  177

NJ ATLANTIC CITY    KACY  ACY   72407  39 28N  074 35W   23

NJ BELMAR/FARMDALE  KBLM  BLM          40 10N  074 07W   48

NJ CALDWELL         KCDW  CDW          40 53N  074 17W   64

NJ LAKEHURST NAS    KNEL  NEL   72409  40 01N  074 20W   31

NJ LINDEN           KLDJ  LDJ          40 37N  074 15W    7

NJ MILLVILLE        KMIV  MIV          39 22N  075 05W   23

NJ MORRISTOWN MUNI  KMMU  MMU          40 47N  074 25W   57

NJ MOUNT HOLLY      KVAY  VAY          39 56N  074 50W   15

NJ NEWARK           KEWR  EWR   72502  40 41N  074 10W    7

NJ SOMERVILLE       KSMQ  SMQ          40 37N  074 40W   30

NJ SUSSEX           KFWN  FWN          41 12N  074 38W  133

NJ TETERBORO        KTEB  TEB          40 52N  074 03W    7

NJ TOMS RIVER       KMJX  MJX          39 56N  074 18W   25

NJ TRENTON          KTTN  TTN          40 17N  074 49W   59

NJ WILDWOOD         KWWD  WWD          39 01N  074 55W    7

NJ WOODBINE MUNI    KOBI  OBI          39 13N  074 48W   13

NJ WRIGHTST/MCGUIRE KWRI  WRI          40 01N  074 35W   41

NM GALLUP           KGUP  GUP          35 31N  108 48W 1971

NM ALAMOGORDO WHITE KALM  ALM   72269  32 49N  105 58W 1279

NM ALBUQUERQUE      KABQ  ABQ   72365  35 03N  106 37W 1618

NM ANGEL FIRE       KAXX  AXX          36 25N  105 17W 2548

NM ARTESIA          KATS  ATS          32 51N  104 28W 1081

NM BELEN            KE80  E80          34 39N  106 50W 1582

NM CANNON AFB/CLOVI KCVS  CVS          34 22N  103 19W 1309

NM CARLSBAD         KCNM  CNM          32 20N  104 15W  985

NM CHAMA (AWRS)     KE33  E33          36 53N  106 34W 2393

NM CLAYTON          KCAO  CAO   72360  36 27N  103 09W 1514

NM CLINES CORNERS   KCQC  CQC          35 00N  105 40W 2160

NM CLOVIS MUNI      KCVN  CVN          34 25N  103 04W 1284

NM CORONA/LINCOLN   K4CR  4CR          34 06N  105 40W 1981

NM DEMING           KDMN  DMN          32 16N  107 43W 1311

NM DOUBLE EAGLE II  KAEG  AEG          35 08N  106 48W 1779

NM FARMINGTON       KFMN  FMN          36 45N  108 14W 1685

NM GRANTS           KGNT  GNT          35 10N  107 54W 1987

NM HOBBS/LEA CO.    KHOB  HOB          32 40N  103 13W 1115

NM HOLLOMAN AFB     KHMN  HMN   74732  32 51N  106 05W 1248

NM LAS CRUCES INTL  KLRU  LRU          32 16N  106 55W 1358

NM LAS VEGAS        KLVS  LVS          35 39N  105 08W 2091

NM LOS ALAMOS       KLAM  LAM          35 52N  106 16W 2179

NM MORIARTY         K4MY  4MY          34 59N  106 00W 1890

NM MORIARTY         K0E0  0E0          34 59N  106 00W 1890

NM RATON            KRTN  RTN          36 44N  104 30W 1936

NM ROSWELL          KROW  ROW   72268  33 18N  104 30W 1112

NM SIERRA B/RUIDOSO KSRR  SRR          33 28N  105 31W 2076

NM SANTA FE         KSAF  SAF          35 37N  106 06W 1930

NM SANTA TERESA     K5T6  5T6          31 53N  106 42W 1253

NM SILVER CITY      KSVC  SVC   72272  32 38N  108 09W 1659

NM SOCORRO          KONM  ONM   72362  34 01N  106 54W 1410

NM TAOS MUNI APT    KSKX  SKX          36 27N  105 40W 2161

NM TORREON/CUBA     K4SL  4SL          35 47N  107 10W 2106

NM TRUTH OR CONSEQ. KTCS  TCS   72271  33 14N  107 16W 1469

NM TUCUMCARI        KTCC  TCC          35 11N  103 36W 1235

NM WHITE SANDS TEST K2C2  2C2   74734  32 22N  106 28W 1244

NM MCGREGOR RANGE   KQMG               32 03N  106 09W 1283

NV AUSTIN           KU31  U31          39 30N  117 04W 2014

NV BATTLE MOUNTAIN  KB23  B23          40 36N  116 52W 1381

NV BOULDER CITY     KBVU  BVU          35 57N  114 52W  671

NV CARSON CITY      KCXP  CXP          39 11N  119 43W 1435

NV DESERT R/MERCURY KDRA  DRA   72387  36 37N  116 02W 1006

NV ELKO             KEKO  EKO          40 50N  115 47W 1547

NV ELY              KELY  ELY   72486  39 18N  114 51W 1906

NV EUREKA           KP68  P68          39 36N  116 00W 1809

NV FALLON NAS       KNFL  NFL          39 25N  118 42W 1199

NV INDIAN SPRINGS   KINS  INS          36 35N  115 40W  955

NV IND SPRNG RANGE  KL63  L63   69017  36 31N  115 34W  972

NV LAS VEGAS        KLAS  LAS   72386  36 05N  115 09W  636

NV LAS VEGAS/NELLIS KLSV  LSV          36 13N  115 01W  570

NV VEGAS/HENDERSON  KHND  HND          35 59N  115 08W  749

NV LOVELOCK         KLOL  LOL   72580  40 04N  118 34W 1189

NV NORTH LAS VEGAS  KVGT  VGT          36 12N  115 12W  671

NV RENO             KRNO  RNO   72488  39 29N  119 46W 1342

NV TONOPAH          KTPH  TPH   72485  38 03N  117 05W 1652

NV TONOPAH RANGE74  KBJN  BJN   72282  37 37N  116 16W 1756

NV WELLS            K9BB  9BB          41 06N  114 58W 1723

NV WILDHORSE RES/EL KAWH  AWH          41 40N  115 46W 1902

NV WINNEMUCCA       KWMC  WMC   72583  40 54N  117 48W 1310

NY ALBANY           KALB  ALB   72518  42 45N  073 48W   92

NY BINGHAMTON       KBGM  BGM   72515  42 13N  075 59W  490

NY BUFFALO/CHEEKTOW KBUF  BUF   72528  42 56N  078 44W  211

NY CANANDAIGUA      KD38  D38          42 54N  077 19W  248

NY DANSVILLE        KDSV  DSV   72523  42 34N  077 43W  198

NY DUNKIRK          KDKK  DKK   99425  42 30N  079 17W  202

NY EAST HAMPTON     KHTO  HTO          40 57N  072 15W   17

NY ELMIRA           KELM  ELM          42 09N  076 54W  302

NY ENDICOTT/TRI-CIT KCZG  CZG          42 05N  076 06W  254

NY FARMINGDALE      KFRG  FRG          40 44N  073 25W   21

NY FORT DRUM/WHEELE KGTB  GTB   74370  44 02N  075 43W  207

NY FULTON           KFZY  FZY          43 21N  076 23W  138

NY GLENS FALLS      KGFL  GFL          43 20N  073 37W  103

NY HAMILTON MUNICIP KVGC  VGC          42 51N  075 34W  347

NY HORNELL          K4G6  4G6          42 23N  077 41W  372

NY HUDSON/COLUMBIA  K1B1  1B1          42 17N  073 43W   60

NY ISLIP            KISP  ISP          40 48N  073 06W   43

NY ITHACA/TOMPKINS  KITH  ITH          42 28N  076 27W  335

NY JAMESTOWN        KJHW  JHW          42 08N  079 16W  525

NY MASSENA          KMSS  MSS          44 56N  074 51W   66

NY MONTAUK          KMTP  MTP          41 04N  071 55W    2

NY MONTGOMERY       KMGJ  MGJ          41 31N  074 16W  108

NY MONTICELLO       KMSV  MSV          41 42N  074 47W  428

NY NEWBURGH/STEWART KSWF  SWF          41 30N  074 05W  150

NY NIAGARA FALLS    KIAG  IAG          43 07N  078 56W  182

NY NORWICH          KOIC  OIC          42 34N  075 31W  312

NY NYC/JFK ARPT     KJFK  JFK   74486  40 38N  073 46W    9

NY NYC/LA GUARDIA   KLGA  LGA   72503  40 47N  073 53W   11

NY NYC/CENTRAL PARK KNYC  NYC          40 47N  073 58W   11

NY OGDENSBURG INTL  KOGS  OGS          44 40N  075 28W   91

NY OLEAN            KOLE  OLE          42 14N  078 22W  651

NY PENN YAN         KPEO  PEO          42 39N  077 03W  256

NY PLATTSBURGH      KPLB  PLB          44 41N  073 32W  106

NY PLATTSBURGH AFB  KPBG  PBG          44 39N  073 28W   72

NY POTSDAM          KPTD  PTD          44 41N  074 57W  144

NY POUGHKEEPSIE     KPOU  POU          41 38N  073 53W   46

NY ROCHESTER        KROC  ROC   72529  43 07N  077 41W  178

NY ROME/GRIFFISS AF KRME  RME          43 13N  075 24W  154

NY SARANAC LAKE     KSLK  SLK          44 24N  074 12W  498

NY SARATOGA SPRINGS K5B2  5B2          43 03N  073 52W  132

NY SCHENECTADY AIRP KSCH  SCH          42 51N  073 55W  115

NY SHIRLEY          KHWV  HWV          40 49N  072 52W   21

NY SIDNEY           KN23  N23          42 18N  075 25W  313

NY SYRACUSE         KSYR  SYR   72519  43 07N  076 06W  127

NY UTICA            KUCA  UCA          43 09N  075 23W  228

NY WATERTOWN        KART  ART          43 59N  076 02W  100

NY WELLSVILLE       KELZ  ELZ          42 06N  077 59W  646

NY WESTHAMPTON BEAC KFOK  FOK          40 51N  072 37W   33

NY WHITE PLAINS     KHPN  HPN          41 04N  073 42W  121

NY WILLIAMSON/SODUS KSDC  SDC          43 14N  077 07W  129

OH AKRON (FULTON)   KAKR  AKR          41 02N  081 28W  363

OH AKRON            KCAK  CAK   72521  40 55N  081 27W  377

OH ASHTABULA        KHZY  HZY          41 47N  080 42W  276

OH ATHENS           KUNI  UNI          39 13N  082 14W  193

OH BELLEFONTAINE    KEDJ  EDJ          40 22N  083 49W  342

OH CINCINNAT/LUNKEN KLUK  LUK          39 06N  084 25W  155

OH CLEVELAND        KBKL  BKL          41 32N  081 40W  184

OH CLEVELAND        KCLE  CLE   72524  41 25N  081 51W  233

OH CLEVLND/CUYAHOGA KCGF  CGF          41 34N  081 28W  268

OH COLUMBUS         KCMH  CMH   72428  40 00N  082 53W  247

OH COLUMBUS/ST UNIV KOSU  OSU          40 05N  083 05W  280

OH COLUMBUS/BOLTON  KTZR  TZR          39 54N  083 08W  276

OH DAYTON           KDAY  DAY   72429  39 54N  084 13W  305

OH DAYTON           KMGY  MGY          39 36N  084 14W  289

OH DEFIANCE         KDFI  DFI          41 20N  084 26W  219

OH DELAWARE         KDLZ  DLZ          40 17N  083 07W  289

OH FINDLAY          KFDY  FDY          41 01N  083 40W  247

OH FLAG CITY        KFBC  FBC          41 01N  083 40W  247

OH HAMILTON         KHAO  HAO          39 22N  084 31W  188

OH LANCASTER        KLHQ  LHQ          39 45N  082 40W  260

OH LIMA             KAOH  AOH          40 42N  084 01W  296

OH LORAIN/ELYRIA    KLPR  LPR          41 21N  082 11W  241

OH MANSFIELD        KMFD  MFD          40 49N  082 31W  395

OH MARION           KMNN  MNN          40 37N  083 04W  301

OH MARYSVILLE       KMRT  MRT          40 13N  083 21W  312

OH MIDDLETOWN/HOOK  KMWO  MWO          39 32N  084 24W  198

OH MT VERNON/KNOX   K4I3  4I3          40 20N  082 32W  363

OH NEW PHILADELPHIA KPHD  PHD          40 28N  081 25W  272

OH NEWARK           KVTA  VTA          40 01N  082 28W  268

OH RAVENA/PORTAGE   KPOV  POV          41 13N  081 15W  365

OH RICKENBACKER ANG KLCK  LCK          39 49N  082 55W  227

OH SPRINGFIELD MUNI KSGH  SGH          39 50N  083 50W  321

OH TOLEDO           KTDZ  TDZ          41 34N  083 29W  189

OH TOLEDO           KTOL  TOL   72536  41 35N  083 48W  210

OH VERSAILLES       KVES  VES          40 12N  084 32W  307

OH WILLOUGHBY       KLNN  LNN          41 40N  081 22W  191

OH WILMINGTON       KILN  ILN   72426  39 26N  083 48W  322

OH WOOSTER          KBJJ  BJJ          40 52N  081 53W  343

OH WRIGHT PATERSON  KFFO  FFO   74570  39 49N  084 02W  251

OH YOUNGSTOWN       KYNG  YNG   72525  41 15N  080 40W  360

OH ZANESVILLE       KZZV  ZZV          39 57N  081 54W  268

OK ADA              KADH  ADH          34 48N  096 40W  308

OK ARDMORE          KADM  ADM          34 17N  097 01W  232

OK ARDMORE/EXEC     K1F0  1F0          34 09N  097 07W  257

OK ALTUS AFB        KLTS  LTS   72352  34 40N  099 16W  420

OK ALTUS/QUARTZ MTN KAXS  AXS          34 42N  099 20W  437

OK ALVA             KAVK  AVK          36 46N  098 40W  449

OK ATOKA            KAQR  AQR          34 24N  096 09W  189

OK BARTLESVILLE     KBVO  BVO          36 46N  096 01W  210

OK BLACKWELL        KBKN  BKN          36 45N  097 21W  314

OK CLAREMORE        KGCM  GCM          36 18N  095 29W  221

OK CHANDLER         KCQB  CQB          35 43N  096 49W  300

OK CHICKASHA        KCHK  CHK          35 06N  097 58W  351

OK CLINTON          KCSM  CSM          35 21N  099 12W  588

OK CLINTON          KCLK  CLK          35 32N  098 56W  492

OK CUSHING          KCUH  CUH          35 57N  096 46W  279

OK DUNCAN           KDUC  DUC          34 28N  097 58W  339

OK DURANT           KDUA  DUA          33 57N  096 24W  213

OK EL RENO MUNI     KRQO  RQO          35 28N  098 00W  432

OK ENID/VANCE AFB   KEND  END          36 19N  097 55W  398

OK ENID/WOODRING    KWDG  WDG          36 22N  097 46W  356

OK FORT SILL        KFSI  FSI   72355  34 39N  098 24W  362

OK FREDERICK/ALTUS  KFDR  FDR          34 21N  098 59W  386

OK GAGE             KGAG  GAG          36 18N  099 46W  668

OK GROVE            KGMJ  GMJ          36 36N  094 44W  254

OK GUTHRIE          KGOK  GOK          35 51N  097 24W  324

OK GUYMON           KGUY  GUY          36 41N  101 30W  948

OK HOBART           KHBR  HBR          34 59N  099 03W  473

OK IDABEL           K4O4  4O4          33 55N  094 52W  144

OK LAWTON           KLAW  LAW          34 33N  098 25W  337

OK MCALESTER        KMLC  MLC          34 53N  095 47W  232

OK MUSKOGEE         KMKO  MKO          35 39N  095 22W  185

OK NORMAN/WESTHEIME KOUN  OUN   72357  35 15N  097 28W  357

OK OKLAHOMA CITY    KOKC  OKC   72353  35 23N  097 36W  390

OK OKLA CITY/WILEY  KPWA  PWA          35 32N  097 39W  397

OK OKLA CITY/PAGE   KRCE  RCE          35 29N  097 49W  413

OK OKMULGEE         KOKM  OKM          35 40N  095 57W  220

OK PAULS VALLEY     KPVJ  PVJ          34 43N  097 13W  295

OK PONCA CITY       KPNC  PNC          36 44N  097 06W  308

OK POTEAU (R KERR)  KRKR  RKR          35 01N  094 37W  138

OK SALLISAW         KJSV  JSV          35 26N  094 48W  161

OK SEMINOLE         KSRE  SRE          35 16N  096 41W  312

OK SHAWNEE          KSNL  SNL          35 21N  096 57W  327

OK STIGLER          KGZL  GZL          35 17N  095 06W  183

OK STILLWATER       KSWO  SWO          36 10N  097 05W  290

OK TAHLEQUAH        KTQH  TQH          35 56N  095 00W  266

OK TINKER AFB       KTIK  TIK   72354  35 25N  097 22W  394

OK TULSA            KTUL  TUL   72356  36 12N  095 53W  207

OK TULSA            KRVS  RVS          36 03N  095 59W  200

OK WATONGA          KJWG  JWG          35 52N  098 25W  472

OK WEATHERFORD      KOJA  OJA          35 33N  098 40W  490

OK WOODWARD         KWWR  WWR          36 27N  099 31W  667

OR PORTLAND         KPDX  PDX   72698  45 35N  122 36W    7

OR HILLSBORO/PORTLD KHIO  HIO          45 33N  122 57W   68

OR ASTORIA          KAST  AST   72791  46 09N  123 53W    3

OR AURORA           KUAO  UAO          45 15N  122 46W   59

OR BAKER            KBKE  BKE          44 51N  117 49W 1024

OR BEND             KBDN  BDN          44 06N  121 12W 1055

OR BROOKINGS        KBOK  BOK   72598  42 04N  124 17W  140

OR BURNS            KBNO  BNO   72683  43 36N  118 57W 1264

OR CAPE BLANCO (CGS K92S  92S   72599  42 50N  124 34W   57

OR CASCADE LOCKS ST KCZK  CZK          45 40N  121 52W   46

OR CLATSOP SPIT     K3CL  3CL          46 14N  124 01W    9

OR CORVALLIS MUNI A KCVO  CVO          44 30N  123 16W   75

OR EUGENE           KEUG  EUG   72693  44 08N  123 13W  114

OR FLORENCE MUNI    K6S2  6S2          43 59N  124 07W   16

OR GOLD BEACH       K4S1  4S1          42 25N  124 25W    5

OR HERMISTON        KHRI  HRI          45 50N  119 16W  195

OR JOHN DAY STATE   KGCD  GCD          44 23N  118 58W 1127

OR KLAMATH FALLS    KLMT  LMT          42 09N  121 43W 1246

OR LA GRANDE        KLGD  LGD          45 16N  118 00W  827

OR LAKEVIEW         KLKV  LKV          42 10N  120 24W 1441

OR MCMINNVILLE      KMMV  MMV          45 12N  123 08W   48

OR MEACHAM          KMEH  MEH          45 31N  118 25W 1136

OR MEDFORD          KMFR  MFR   72597  42 23N  122 52W  396

OR NEWPORT          KJNW  JNW          44 34N  124 04W   48

OR NEWPORT          KONP  ONP   72695  44 34N  124 02W   48

OR NORTH BEND       KOTH  OTH   72691  43 25N  124 15W    4

OR ONTARIO          KONO  ONO          44 01N  117 01W  667

OR PENDLETON        KPDT  PDT   72688  45 42N  118 50W  462

OR REDMOND          KRDM  RDM          44 15N  121 09W  938

OR ROME             KP88  P88          42 53N  117 39W 1162

OR ROME             KREO  REO          42 35N  117 52W 1235

OR ROOSTER ROCK     KRRK  RRK          45 33N  122 58W   50

OR ROSEBURG         KRBG  RBG   72690  43 14N  123 21W  154

OR SALEM            KSLE  SLE   72694  44 54N  123 00W   59

OR SCAPPOOSE        KSPB  SPB          45 46N  122 52W   14

OR SEA LION CAVES   K2SL  2SL          44 07N  124 08W  121

OR SEXTON SUMMIT    KSXT  SXT          42 36N  123 22W 1170

OR THE DALLES       KDLS  DLS          45 37N  121 10W   71

OR TILLAMOOK        KTMK  TMK          45 25N  123 49W   11

OR TROUTDALE        KTTD  TTD          45 33N  122 25W   14

PA ALLENTOWN        KABE  ABE   72517  40 39N  075 27W  114

PA ALLENTOWN QUEEN  KXLL  XLL          40 34N  075 29W  122

PA ALTOONA          KAOO  AOO          40 18N  078 19W  455

PA AVOCA/WILKES B.  KAVP  AVP   72513  41 20N  075 43W  291

PA BEAVER FALLS ARP KBVI  BVI          40 46N  080 24W  382

PA BRADFORD         KBFD  BFD          41 48N  078 38W  647

PA BUTLER CO.       KBTP  BTP          40 46N  079 57W  380

PA CLEARFIELD       KFIG  FIG          41 03N  078 25W  462

PA COATESVILLE      KMQS  MQS          39 59N  075 52W  201

PA DOYLESTOWN       KDYL  DYL          40 20N  075 07W  117

PA DU BOIS          KDUJ  DUJ          41 10N  078 54W  554

PA ERIE             KERI  ERI   72526  42 05N  080 11W  222

PA FOUNTAIN DALE    KRYT               39 44N  077 26W  274

PA FRANKLIN         KFKL  FKL          41 22N  079 52W  469

PA HARRISBURG       KCXY  CXY          40 13N  076 51W  106

PA INDIANA/STEWART  KIDI  IDI          40 37N  079 05W  429

PA JOHNSTOWN        KJST  JST          40 19N  078 50W  694

PA LANCASTER        KLNS  LNS          40 07N  076 18W  125

PA LATROBE/WESTMORL KLBE  LBE          40 16N  079 24W  361

PA MEADVILLE        KGKJ  GKJ          41 38N  080 13W  428

PA MIDDLETOWN       KMDT  MDT          40 12N  076 46W   95

PA MT. POCONO       KMPO  MPO          41 08N  075 23W  577

PA MUIR AAF/INDIANT KMUI  MUI          40 25N  076 34W  149

PA NEW CASTLE MUNI  KUCP  UCP          41 02N  080 25W  327

PA PHILADELPHIA     KPHL  PHL   72408  39 52N  075 14W   18

PA PHILADELPHIA/NE  KPNE  PNE          40 05N  075 01W   28

PA PHILLY WINGS F   KLOM  LOM          40 08N  075 16W   92

PA PHILIPSBURG/MID  KPSB  PSB   72512  40 52N  078 04W  594

PA PITTSBURGH/ALLEG KAGC  AGC          40 21N  079 55W  389

PA PITTSBURGH       KPIT  PIT   72520  40 30N  080 16W  357

PA POTTSTOWN        KPTW  PTW          40 14N  075 33W   92

PA QUAKERTOWN ARP   KUKT  UKT          40 26N  075 23W  160

PA READING          KRDG  RDG          40 22N  075 58W  109

PA SELINSGROVE      KSEG  SEG          40 49N  076 52W  134

PA STATE COLLEGE    KUNV  UNV          40 51N  077 50W  378

PA WASHINGTON       KAFJ  AFJ          40 07N  080 16W  361

PA WILLIAMSPORT     KIPT  IPT   72514  41 15N  076 55W  164

PA YORK             KTHV  THV          39 55N  076 53W  144

RI BLOCK ISLAND     KBID  BID          41 10N  071 34W   33

RI N. KINGSTON/QUON KOQU  OQU          41 36N  071 25W    6

RI NEWPORT          KUUU  UUU          41 32N  071 17W   51

RI PAWTUCKET        KSFZ  SFZ          41 55N  071 30W  134

RI PROVIDENCE/GREEN KPVD  PVD   72507  41 43N  071 26W   16

RI WESTERLY         KWST  WST          41 21N  071 48W   21

SC AIKEN            KAIK  AIK          33 39N  081 41W  161

SC ANDERSON         KAND  AND          34 30N  082 43W  239

SC BARNWELL         KBNL  BNL          33 15N  081 23W   75

SC BEAUFORT MCAS    KNBC  NBC          32 28N  080 43W   12

SC BEAUFORT CNTY    KARW  ARW          32 25N  080 38W    4

SC BERKELEY MONCKS  KMKS  MKS          33 11N  080 02W   23

SC CAMDEN WOODWARD  KCDN  CDN          34 17N  080 34W   92

SC CHARLESTON       KCHS  CHS   72208  32 54N  080 02W   13

SC CHARLESTON/EXEC  KJZI  JZI          32 42N  080 00W    6

SC CHERAW           KCQW  CQW          34 43N  079 57W   73

SC CHESTER CATAWBA  KDCM  DCM          34 47N  081 12W  200

SC CLEMSON          KCEU  CEU          34 40N  082 53W  268

SC COLUMBIA         KCAE  CAE   72310  33 56N  081 07W   70

SC COLUMBIA-OWENS   KCUB  CUB          33 58N  081 00W   56

SC CONWAY-HORRY CTY KHYW  HYW          33 50N  079 07W   11

SC DARLINGTON CNTY  KUDG  UDG          34 27N  079 53W   59

SC FLORENCE         KFLO  FLO          34 11N  079 44W   44

SC GEORGETOWN       KGGE  GGE          33 19N  079 19W   12

SC GREENWOOD        KGRD  GRD          34 15N  082 09W  185

SC GREER/SPARTANSBG KGSP  GSP   72312  34 54N  082 13W  287

SC GREENVILLE       KGMU  GMU          34 51N  082 21W  315

SC GREENVILLE       KGYH  GYH          34 45N  082 23W  292

SC HARTSVILLE       KHVS  HVS          34 24N  080 07W  111

SC HILTON HEAD      KHXD  HXD          32 13N  080 42W    6

SC KINGSTREE        KCKI  CKI          33 43N  079 51W   21

SC LANCASTER        KLKR  LKR          34 43N  080 51W  149

SC LAURENS          KLUX  LUX          34 30N  081 57W  213

SC MANNING          KMNI  MNI          33 35N  080 13W   32

SC MARION           KMAO  MAO          34 11N  079 20W   28

SC MARLBORO CNTY    KBBP  BBP          34 37N  079 44W   45

SC MCENTIRE ANG BAS KMMT  MMT          33 55N  080 47W   77

SC MOUNT PLEASANT   KLRO  LRO          32 54N  079 47W    4

SC MYRTLE BEACH     KMYR  MYR   74791  33 40N  078 55W    8

SC NORTH MYRTLE BEA KCRE  CRE          33 49N  078 43W   10

SC NEWBERRY         KEOE  EOE          34 19N  081 38W  174

SC ORANGEBURG       KOGB  OGB          33 28N  080 51W   59

SC PICKENS          KLQK  LQK          34 49N  082 42W  309

SC ROCK HILL        KUZA  UZA          34 59N  081 03W  202

SC SHAW AFB/SUMTER  KSSC  SSC   74790  33 58N  080 28W   74

SC SPARTANBURG VOR  KSPA  SPA          35 02N  081 56W  303

SC SUMMERVILLE      KDYB  DYB          33 04N  080 17W   18

SC SUMTER           KSMS  SMS          34 00N  080 22W   56

SC WALTERBORO       KRBW  RBW          32 55N  080 38W   31

SC WINNSBORO        KFDW  FDW          34 19N  081 07W  176

SD ABERDEEN         KABR  ABR   72659  45 27N  098 25W  397

SD BELLE FOURCHE    KEFC  EFC          44 43N  103 52W  982

SD BROOKINGS        KBKX  BKX          44 17N  096 49W  502

SD BUFFALO          K2WX  2WX          45 36N  103 33W  915

SD CHAMBERLAIN      K9V9  9V9   72653  43 46N  099 19W  519

SD CUSTER           KCUT  CUT          43 44N  103 37W 1725

SD ELLSWORTH AFB    KRCA  RCA          44 08N  103 05W  999

SD FAITH            KD07  D07          45 02N  102 01W  784

SD HURON            KHON  HON   72654  44 23N  098 14W  390

SD LEMMON           KY22  Y22   72669  45 55N  102 10W  781

SD MADISON          KMDS  MDS          44 01N  097 05W  524

SD MITCHELL         KMHE  MHE          43 46N  098 01W  397

SD MOBRIDGE         KMBG  MBG   72668  45 33N  100 25W  510

SD PHILIP           KPHP  PHP          44 03N  101 36W  672

SD PIERRE           KPIR  PIR          44 23N  100 17W  526

SD PINE RIDGE       KIEN  IEN          43 02N  102 31W  998

SD RAPID CITY/WFO   KUNR  UNR   72662  44 04N  103 12W 1027

SD RAPID CITY       KRAP  RAP          44 03N  103 03W  965

SD SIOUX FALLS      KFSD  FSD   72651  43 35N  096 45W  436

SD SISSETON         K8D3  8D3          45 40N  096 59W  354

SD SPEARFISH/CLYDE  KSPF  SPF          44 28N  103 46W 1188

SD WATERTOWN        KATY  ATY          44 54N  097 09W  532

SD WINNER           KICR  ICR          43 23N  099 51W  621

SD YANKTON          KYKN  YKN          42 55N  097 22W  398

SD BELLE FOURCHE    KBEL  BEL          44 48N  104 03W  971

SD JCT SD44 US385   KPAC  PAC          44 06N  103 31W 1494

SD ELLSWORTH I90    KEL0  EL0          44 07N  103 05W  939

SD WASTA I90        KWTA               44 05N  102 29W  810

SD SHERIDAN LAKE RD KSL0  SL0          44 02N  103 26W 1478

SD EDGEMONT US18    KED0  ED0          43 17N  103 53W 1136

SD SD-WY BORDR US85 KNE0  NE0          44 11N  104 03W 1940

SD GLAD VLY SD20    KGLV               45 24N  101 47W  753

SD BATESLAND US18   KBAT  BAT          43 08N  102 06W 1042

SD LEMMON US12      KLEM  LEM          45 56N  102 11W  783

SD ANDOVER          KADR  ADR          45 26N  097 56W  453

SD BELVIDERE        KBVD  BVD          43 54N  101 08W  728

SD BRANDT           KBRA  BRA          44 41N  096 51W  564

SD CACTUS FLATS     KCAC  CAC          43 50N  101 46W  751

SD KADOKA           KKAD  KAD          43 52N  101 50W  765

SD FREDERICK        KFED  FED          45 56N  098 34W  427

SD HARROLD          KHRD  HRD          44 32N  099 42W  547

SD HERRIED          KHER  HER          45 51N  100 06W  513

SD RELIANCE         KREL  REL          43 52N  099 34W  542

SD SUMMIT           KSUM  SUM          45 20N  097 05W  607

SD TOLSTOY          KTLS  TLS          45 13N  099 40W  596

SD VICTOR           KVIC  VIC          45 54N  096 52W  329

SD VIVIAN           KVIV  VIV          44 14N  100 22W  588

SD ABERDEEN         QABS               45 29N  098 28W  395

SD ALCESTER         QALS               43 01N  096 38W  342

SD BALTIC           QBAS               43 46N  096 44W  342

SD BRANDON          QBRS               43 36N  096 35W  342

SD FT THOMPSON      QCES               44 04N  099 26W  440

SD CLARK            QCLS               44 53N  097 44W  342

SD REDFIELD         QDPS               44 53N  098 31W  397

SD LEAD             QDSS               44 21N  103 47W 1560

SD ELKTON           QEKS               44 14N  096 29W  342

SD PARKSTON         QESS               43 24N  097 59W  426

SD GETTYSBURG       QETS               45 01N  099 58W  628

SD GARRETSON        QGAS               43 43N  096 30W  342

SD CHAMBERLAIN      QHAS               43 48N  099 20W  447

SD BRITTON          QHHS               45 47N  097 45W  414

SD HOWARD           QHOS               44 01N  097 32W  342

SD WINNER           QHSS               43 23N  099 52W  622

SD HURON            QHUS               44 21N  098 14W  388

SD KADOKA           QKAS               43 50N  101 31W  749

SD BROOKINGS        QKGS               44 17N  096 47W  495

SD MILLER           QLES               44 31N  098 59W  484

SD CLEAR LAKE       QLKS               44 45N  096 41W  342

SD COLTON           QLTS               43 44N  096 55W  478

SD MADISON          QMMS               44 01N  097 06W  510

SD MILBANK          QMIS               45 12N  096 38W  354

SD MISSION          QMSS               43 18N  100 40W  784

SD MOBRIDGE         QOBS               45 32N  100 26W  511

SD PIERRE           QPIS               44 22N  100 21W  493

SD ELK POINT        QPJS               42 41N  096 41W  342

SD RAPID CITY       QRAS               44 04N  103 13W  998

SD RAMONA           QRMS               44 07N  097 13W  342

SD FSD PH           QSAS               43 32N  096 43W  440

SD FSD PAV          QSBS               43 32N  096 44W  449

SD EUREKA           QSDS               45 46N  099 37W  576

SD FLANDREAU        QSNS               44 03N  096 36W  478

SD MITCHELL         QTMS               43 43N  098 02W  395

SD MARTY            QTYS               43 00N  098 09W  441

SD VERMILLION       QVMS               42 47N  096 56W  373

SD WALL             QWAS               43 59N  102 14W  854

SD WEBSTER          QWES               45 20N  097 31W  342

SD WATERTOWN        QWTS               44 55N  097 06W  544

SD YANKTON          QYAS               42 53N  097 23W  382

TN BOLIVAR/WHITEHST KM08  M08          35 13N  089 03W  152

TN BRISTOL          KTRI  TRI          36 29N  082 24W  474

TN CHATTANOOGA      KCHA  CHA   72324  35 02N  085 12W  210

TN CLARKSVILLE      KCKV  CKV          36 37N  087 25W  165

TN COLUMBIA         KMRC  MRC          35 33N  087 11W  208

TN COVINGTON MUNI   KM04  M04          35 35N  089 35W   85

TN CROSSVILLE       KCSV  CSV          35 57N  085 05W  570

TN DICKSON          KM02  M02          36 08N  087 26W  272

TN DYERSBURG        KDYR  DYR          36 00N  089 24W  103

TN ELIZABETHTON     K0A9  0A9          36 22N  082 10W  486

TN FAYETTEVILLE     KFYM  FYM          35 04N  086 34W  300

TN FRANKLIN WILKINS KM52  M52          35 39N  088 23W  157

TN GALLATIN         KM33  M33          36 23N  086 25W  178

TN HUNTINGDON       KHZD  HZD          36 05N  088 28W  151

TN JACKSON          KMKL  MKL          35 36N  088 55W  128

TN KNOXVILLE        KTYS  TYS   72326  35 49N  083 59W  302

TN LAWRENCEBURG     K2M2  2M2          35 14N  087 15W  286

TN LEBANON          KM54  M54          36 11N  086 19W  180

TN LEWISBURG        KLUG  LUG          35 30N  086 48W  219

TN LIVINGSTON       K8A3  8A3          36 25N  085 19W  419

TN MCMINNVILLE      KRNC  RNC          35 42N  085 51W  315

TN MEMPHIS          KMEM  MEM   72334  35 04N  089 59W   86

TN MILLINGTON/88D   KNQA  NQA          35 21N  089 52W   86

TN MURFREESBORO     KMBT  MBT          35 53N  086 23W  188

TN NASHVILLE        KBNA  BNA   72327  36 07N  086 41W  210

TN NASHV/JC TUNE    KJWN  JWN          36 11N  086 53W  151

TN OAK RIDGE        KOQT  OQT          36 01N  084 14W  277

TN PARIS HENRY CTY  KPHT  PHT          36 20N  088 23W  177

TN PORTLAND         K1M5  1M5          36 36N  086 29W  250

TN ROCKWOOD         KRKW  RKW          35 55N  084 41W  508

TN SAVANNAH HARDIN  KSNH  SNH          35 10N  088 13W  144

TN SHELBYVILLE      KSYI  SYI          35 34N  086 27W  245

TN SMYRNA           KMQY  MQY          36 01N  086 31W  166

TN SPARTA           KSRB  SRB          36 03N  085 32W  313

TN TULLAHOMA        KTHA  THA          35 23N  086 15W  331

TN UNION CITY       KUCY  UCY          36 23N  088 59W  104

TN WINCHESTER       KBGF  BGF          35 11N  086 04W  299

TX DALLAS/FT WORTH  KDFW  DFW   72259  32 54N  097 01W  174

TX ABILENE          KABI  ABI   72266  32 25N  099 41W  548

TX ALICE            KALI  ALI          27 44N  098 01W   53

TX ALPINE-CASPARIS  KE38  E38          30 23N  103 41W 1376

TX AMARILLO         KAMA  AMA   72363  35 13N  101 42W 1093

TX ANGLETON         KLBX  LBX          29 07N  095 28W    6

TX ARLINGTON        KGKY  GKY          32 40N  097 06W  192

TX AUSTIN           KAUS  AUS   72254  30 11N  097 41W  166

TX AUSTIN EXEC      KEDC  EDC          30 24N  097 34W  189

TX AUSTIN/MABRY     KATT  ATT          30 19N  097 46W  201

TX BAY CITY         KBYY  BYY          28 58N  095 52W   14

TX BEAUMONT/PORT AR KBPT  BPT   72241  29 57N  094 02W    5

TX BEAUMONT         KBMT  BMT          30 04N  094 13W   10

TX BEEVILLE MUNI    KBEA  BEA          28 22N  097 47W   82

TX BIG SPRING       KBPG  BPG          32 13N  101 31W  784

TX BORGER           KBGD  BGD          35 42N  101 24W  927

TX BOWIE            K0F2  0F2          33 36N  097 47W  336

TX BRADY            KBBD  BBD          31 11N  099 19W  557

TX BRECKENRIDGE     KBKD  BKD          32 43N  098 53W  392

TX BRENHAM          K11R  11R          30 13N  096 22W   94

TX BRIDGEPORT       KXBP               33 11N  097 50W  260

TX BROWNSVILLE      KBRO  BRO   72250  25 55N  097 25W    7

TX BROWNWOOD        KBWD  BWD          31 48N  098 57W  423

TX BURNET           KBMQ  BMQ          30 44N  098 14W  389

TX CALDWELL         KRWV  RWV          30 31N  096 42W  119

TX CANADIAN         KHHF  HHF          35 54N  100 24W  712

TX DIMMIT           KFTN  FTN          28 13N  100 01W  236

TX CASTROVILLE MUNI KCVB  CVB          29 21N  098 51W  235

TX CHILDRESS        KCDS  CDS          34 26N  100 17W  594

TX CLARKSVILLE      KLBR  LBR          33 36N  095 04W  134

TX CLEBURNE         KCPT  CPT          32 21N  097 26W  260

TX CLEVELAND MUNI   K6R3  6R3          30 21N  095 00W   46

TX COLEMAN MUNI     KCOM  COM          31 50N  099 24W  517

TX COLLEGE STATION  KCLL  CLL          30 35N  096 22W   96

TX COMANCHE         KMKN  MKN          31 55N  098 36W  423

TX CONROE           KCXO  CXO          30 21N  095 25W   75

TX CORPUS CHRISTI   KCRP  CRP   72251  27 46N  097 30W   14

TX C. CHRISTI NAS   KNGP  NGP          27 41N  097 16W    6

TX CORSICANA        KCRS  CRS          32 02N  096 24W  133

TX COTULLA          KCOT  COT          28 27N  099 13W  140

TX CROCKETT         KDKR  DKR          31 18N  095 24W  106

TX DALHART          KDHT  DHT          36 01N  102 33W 1217

TX DALLAS/LOVE FLD  KDAL  DAL   72258  32 51N  096 51W  158

TX DALLAS/REDBIRD   KRBD  RBD          32 41N  096 52W  203

TX DALLAS NAS/HENSL KNBE  NBE          32 43N  096 58W  150

TX DALLAS/ADDISON   KADS  ADS          32 58N  096 49W  196

TX DECATUR          KLUD  LUD          33 15N  097 35W  319

TX DEL RIO          KDRT  DRT   72261  29 22N  100 55W  313

TX DENTON           KDTO  DTO          33 12N  097 12W  196

TX DRYDEN           K6R6  6R6          30 03N  102 13W  701

TX DUMAS/MOORE CTY  KDUX  DUX          35 51N  102 01W 1129

TX DYESS AFB/ABILEN KDYS  DYS   69019  32 25N  099 50W  545

TX EDINBURG INTL    KEBG  EBG          26 27N  098 08W   24

TX EL PASO          KELP  ELP   72270  31 49N  106 23W 1197

TX FALFURRIAS       KBKS  BKS          27 12N  098 07W   34

TX FORT BLISS       KBIF  BIF          31 51N  106 23W 1203

TX FORT STOCKTON    KFST  FST          30 55N  102 55W  918

TX FORT WORTH       KFTW  FTW          32 50N  097 22W  214

TX FORT WORTH/88D   KFWS  FWS          32 34N  097 18W  208

TX FORT WORTH NAS   KNFW  NFW          32 47N  097 26W  198

TX FREDERICKSBURG   KT82  T82          30 15N  098 55W  517

TX FT WORTH/ALLIANC KAFW  AFW          32 58N  097 19W  233

TX FT HOOD/KILLEEN  KHLR  HLR   72257  31 08N  097 43W  283

TX GALVESTON        KGLS  GLS   72242  29 16N  094 52W    6

TX GAINESVILLE      KGLE  GLE          33 39N  097 12W  256

TX UVALDE/GARNER F  KUVA  UVA          29 13N  099 45W  287

TX GATESVILLE       KGOP  GOP          31 25N  097 48W  276

TX GEORGETOWN       KGTU  GTU          30 41N  097 41W  240

TX GIDDINGS-LEE     KGYB  GYB          30 10N  096 59W  148

TX GILMER MUNI      KJXI  JXI          32 42N  094 57W  127

TX GRAHAM MUNI      KRPH  RPH          33 07N  098 33W  342

TX GRANBURY MUNI    KGDJ  GDJ          32 27N  097 49W  237

TX GRAND PRAIRIE    KGPM  GPM          32 42N  097 03W  180

TX GREENVILLE/MAJOR KGVT  GVT          33 04N  096 04W  163

TX GUADALUPE PASS   KGDP  GDP   72262  31 50N  104 49W 1692

TX HAMILTON MUNI    KMNZ  MNZ          31 40N  098 09W  396

TX HARLINGEN        KHRL  HRL          26 14N  097 39W   10

TX HEARNE MUNI      KLHB  LHB          30 52N  096 37W   87

TX HEBBRONVILLE     KHBV  HBV          27 21N  098 44W  202

TX HENDERSON RUSK C KRFI  RFI          32 09N  094 51W  135

TX HEREFORD MUNI    KHRX  HRX          34 52N  102 20W 1154

TX HILLSBORO        KINJ  INJ          32 05N  097 06W  209

TX HONDO            KHDO  HDO          29 22N  099 10W  282

TX HORSESHOE BAY    KDZB  DZB          30 32N  098 22W  334

TX HOUSTON/DW HOOKS KDWH  DWH          30 04N  095 33W   46

TX HOUSTON/HOBBY    KHOU  HOU          29 38N  095 17W   36

TX HOUSTON          KLVJ  LVJ          29 31N  095 15W   12

TX HOUSTON EXEC     KTME  TME          29 48N  095 54W   51

TX HOUSTON/INTNL    KIAH  IAH   72243  30 00N  095 22W   36

TX HOUSTON/ELLINGTO KEFD  EFD          29 36N  095 10W   10

TX HOUSTON/SUGAR LA KSGR  SGR          29 37N  095 39W    2

TX HOUSTON/UNIV     KMCJ  MCJ          29 43N  095 24W   69

TX HOUSTON/SOUTHWST KAXH  AXH          29 31N  095 29W   21

TX INGLESIDE/TP MCC KTFP  TFP          27 55N  097 13W    6

TX HUNTSVILLE       KUTS  UTS          30 45N  095 35W  104

TX JACKSONVILLE     KJSO  JSO          31 52N  095 13W  206

TX JASPER           KJAS  JAS          30 53N  094 02W   65

TX JUNCTION         KJCT  JCT   74740  30 30N  099 46W  522

TX KELLY AFB        KSKF  SKF          29 22N  098 34W  210

TX GRAY/FT HOOD     KGRK  GRK          31 04N  097 50W  309

TX KERRVILLE        KERV  ERV          29 59N  099 05W  510

TX KICKAPOO         KCWC  CWC          33 51N  098 29W  305

TX KILLEEN MUNI     KILE  ILE          31 04N  097 40W  258

TX KINGSVILLE NAS   KNQI  NQI          27 30N  097 49W   15

TX LA GRANGE        K3T5  3T5          29 54N  096 57W   99

TX LAGO VISTA       KRYW  RYW          30 30N  097 58W  376

TX LAMPASAS         KLZZ  LZZ          31 06N  098 12W  371

TX LANCASTER        KLNC  LNC          32 35N  096 43W  153

TX LAREDO           KLRD  LRD   72252  27 33N  099 28W  155

TX LAUGHLIN AFB     KDLF  DLF          29 22N  100 46W  330

TX LLANO            KAQO  AQO          30 47N  098 40W  336

TX LONGVIEW         KGGG  GGG   72247  32 23N  094 43W  107

TX LUBBOCK          KLBB  LBB   72267  33 40N  101 49W  993

TX LUFKIN           KLFK  LFK          31 14N  094 45W   88

TX MARFA            KMRF  MRF   72264  30 22N  104 01W 1481

TX MARSHALL         KASL  ASL          32 31N  094 18W  109

TX MC GREGOR        KPWG  PWG          31 28N  097 19W  180

TX MCALLEN          KMFE  MFE          26 11N  098 15W   38

TX MCKINNEY         KTKI  TKI          33 11N  096 35W  179

TX MESQUITE         KHQZ  HQZ          32 45N  096 32W  136

TX MIDLAND          KMAF  MAF   72265  31 57N  102 12W  874

TX MIDLAND AIRPARK  KMDD  MDD          32 02N  102 06W  854

TX MIDLOTHIAN/WAX   KJWY  JWY          32 27N  096 55W  217

TX MINEOLA/QUITMAN  KJDD  JDD          32 45N  095 30W  132

TX MINERAL WELLS    KMWL  MWL          32 47N  098 04W  284

TX MOUNT PLEASANT   KOSA  OSA          33 06N  094 58W  111

TX NACOGDOCHES      KOCH  OCH          31 34N  094 43W  108

TX NEW BRAUNFELS    KBAZ  BAZ          29 43N  098 03W  195

TX ODESSA           KODO  ODO          31 55N  102 24W  902

TX ORANGE GROVE     KNOG  NOG          27 54N  098 03W   78

TX ORANGE           KORG  ORG          30 04N  093 48W    4

TX OZONA            KOZA  OZA          30 44N  101 12W  726

TX PALACIOS         KPSX  PSX          28 43N  096 15W    5

TX PALESTINE MUNI   KPSN  PSN          31 47N  095 42W  129

TX PAMPA            KPPA  PPA          35 37N  101 00W  989

TX PAMPA M VISTA R  KBPC  BPC          35 53N  101 02W  845

TX PARIS/COX FIELD  KPRX  PRX          33 37N  095 27W  167

TX PECOS CITY       KPEQ  PEQ          31 23N  103 31W  797

TX PERRYTON         KPYX  PYX          36 25N  100 45W  889

TX PHILLIPS OIL     K25T  25T          26 56N  094 41W   10

TX PLAINVIEW        KPVW  PVW          34 10N  101 43W 1028

TX PLEASANTON MUNI  KPEZ  PEZ          28 57N  098 31W  132

TX PORT ARANSAS     KRAS  RAS          27 49N  097 05W    2

TX PORT ISABEL      KPIL  PIL          26 10N  097 20W    5

TX PORT LAVACA      KPKV               28 39N  096 41W    9

TX RANDOLPH AFB     KRND  RND          29 31N  098 16W  232

TX REESE AFB/LUBBOC KREE  REE          33 36N  102 02W 1017

TX ROBSTOWN/NUECES  KRBO  RBO          27 47N  097 41W   24

TX ROCKPORT         KRKP  RKP          28 05N  097 03W    6

TX ROCKSPRINGS      KECU  ECU          29 57N  100 10W  725

TX SABINE PASS      KRPE  RPE   99426  29 41N  093 57W    3

TX SAN ANGELO       KSJT  SJT   72263  31 22N  100 30W  576

TX S ANTONIO/STINSN KSSF  SSF          29 20N  098 28W  176

TX SAN ANTONIO      KSAT  SAT   72253  29 32N  098 28W  243

TX S ANTONIO-BOERNE K5C1  5C1          29 43N  098 42W  423

TX SAN MARCOS       KHYI  HYI          29 53N  097 52W  182

TX SAN MARCOS       KT98  T98          29 53N  097 52W  182

TX SANDERSON (RAMOS KP07  P07   74730  30 10N  102 25W  865

TX SEMINOLE         KGNC  GNC          32 41N  102 39W 1010

TX SHERMAN/DENISON  KGYI  GYI          33 43N  096 40W  228

TX SNYDER/WINSTON   KSNK  SNK          32 42N  100 57W  741

TX SONORA           KSOA  SOA          30 35N  100 39W  652

TX SOUTH PADRE IS   KSPL  SPL          26 04N  097 09W    4

TX STEPHENVILLE     KSEP  SEP   72260  32 13N  098 10W  402

TX SULPHUR SPRINGS  KSLR  SLR          33 10N  095 37W  149

TX SWEETWATER       KSWW  SWW          32 28N  100 28W  727

TX TEMPLE/MILLER    KTPL  TPL          31 08N  097 24W  208

TX TERRELL          KTRL  TRL          32 43N  096 16W  145

TX TYLER            KTYR  TYR          32 22N  095 24W  165

TX VERNON WILBARGER KF05  F05          34 14N  099 17W  386

TX VICTORIA         KVCT  VCT   72255  28 52N  096 56W   31

TX WACO             KACT  ACT   72256  31 37N  097 14W  151

TX WACO/TSTI        KCNW  CNW          31 38N  097 04W  156

TX WHARTON          KARM  ARM          29 15N  096 09W   31

TX WICHITA FALL     KSPS  SPS   72351  33 59N  098 30W  308

TX WINK             KINK  INK          31 47N  103 12W  855

TX WESLACO MIDVALLY KT65  T65          26 11N  097 58W   21

TX ZAPATA           KAPY  APY          26 58N  099 15W  129

TX BRAZOS 133B OILP KBBF  BBF          27 50N  096 01W   36

TX GALVESTON424 OIL KGVX  GVX          28 35N  094 59W   39

TX H ISLAND376 OILP KHQI  HQI          27 58N  093 40W   26

TX BRAZOS 451 OILP  KBQX  BQX          28 30N  095 43W   34

TX MUSTANG IS31 OIL KMIU  MIU          27 17N  096 44W   36

TX MUSTANG A85A OIL KMZG  MZG          27 44N  096 11W   29

TX W CAMERON 368A   KCRH  CRH          28 55N  093 18W   34

TX E BREAKS BOOMVNG KVAF  VAF          27 21N  094 38W   46

TX ALAMINOS CANYON  KHHV  HHV          26 56N  094 41W   25

TX E BREAKS 165     KEMK  EMK          27 49N  094 19W   26

TX GUNNISON 668     KGUL  GUL          27 18N  093 32W   30

TX N PADRE 975      KOPM  OPM          26 50N  096 56W   25

TX FRIONA 2NE       XFAS               34 39N  102 41W 1222

TX DIMMITT 2NE      XDMS               34 34N  102 18W 1181

TX TULIA 2NE        XTIS               34 33N  101 44W 1060

TX SILVERTON 7E     XSVS               34 27N  101 11W  976

TX MULESHOE 2S      XMUS               34 12N  102 45W 1160

TX AMHERST 1NE      XAMH               34 01N  102 24W 1112

TX PLAINVIEW 1S     XPVS               34 11N  101 42W 1024

TX FLOYDADA 2NE     XFLS               34 00N  101 20W  969

TX ROARING S. 3N    XRRS               33 56N  100 51W  797

TX MORTON 1NE       XMNS               33 44N  102 44W 1144

TX LEVELLAND 4S     XLDS               33 32N  102 22W 1066

TX LUBBOCK 12W      XREE               33 36N  102 03W 1020

TX RALLS 1SE        XRLS               33 40N  101 23W  944

TX PLAINS 3N        XPPS               33 14N  102 50W 1131

TX BROWNFIELD 2S    XBWS               33 09N  102 16W 1010

TX TAHOKA 3NE       XTAS               33 12N  101 47W  946

TX POST 1NE         XPTS               33 12N  101 22W  792

TX SEMINOLE 2N      XSMS               32 44N  102 38W 1010

TX SLATON 2NE       XSLS               33 27N  101 37W  934

TX ABERNATHY 5NE    XARS               33 53N  101 45W 1016

TX OLTON 6S         XONS               34 06N  102 07W 1087

TX O'DONNELL 1N     XOES               32 59N  101 50W  931

TX SEAGRAVES 1SW    XSGV               32 56N  102 34W 1024

TX LAMESA 2SE       XLES               32 42N  101 56W  901

TX HART 3N          XHRS               34 25N  102 06W 1126

TX GAIL 2SE         XGGS               32 45N  101 25W  778

TX WHITE R LAKE 6NW XWVS               33 32N  101 10W  824

TX GRAHAM 5SW       XGHS               33 05N  101 31W  869

TX SUNDOWN 8SW      XSDS               33 23N  102 37W 1105

TX ANTON 6S         XAOS               33 44N  102 11W 1038

TX FLUVANNA 3W      XFVS               32 54N  101 12W  825

TX SPUR 1W          XSPR               33 29N  100 53W  697

TX LUBBOCK 3W       XLBW               33 35N  101 54W  985

TX GUTHRIE 10W      XPFS               33 34N  100 29W  609

TX CLARENDON        XCES               34 55N  100 56W  864

TX PADUCA 10SW      XPAD               33 53N  100 24W  616

TX SNYDER 3E        XSYS               32 43N  100 52W  741

TX MEMPHIS 1NE      XMES               34 44N  100 32W  627

TX JAYTON 1S        XJTS               33 14N  100 34W  613

TX PAMPA 2E         XPMS               35 32N  100 56W  974

TX HEREFORD 2NW     XHES               34 50N  102 25W 1182

TX ASPERMONT 3NE    XASR               33 10N  100 12W  530

TX GOODLETT 3W      XGDS               34 21N  099 56W  501

TX MCLEAN 1E        XMCS               35 14N  100 34W  873

TX DENVER CITY 7W   XDVS               32 59N  102 56W 1113

TX LAKE ALAN HENRY  XAHS               33 05N  101 03W  705

TX WOLFFORTH 6SW    XWOS               33 25N  102 03W 1008

TX ANDREWS 2E       XANS               32 19N  102 31W  966

TX TURKEY 2WSW      XTUR               34 23N  100 56W  747

TX CHILDRESS 2NNE   XCXS               34 27N  100 12W  593

NM TATUM NM 2SW     XTAT               33 14N  103 21W 1225

NM DORA 2SW         XDR1               33 55N  103 21W 1317

NM NORTHFIELD 1S    XNOR               34 16N  100 36W  637

TX SAN ANGELO 7NW   XASU               31 33N  100 31W  597

TX ST LAWRENCE 5SW  XST1               31 39N  101 36W  821

TX HASKELL 1NW      XHA1               33 10N  099 45W  488

TX WALL 1E          XWA1               31 22N  100 17W  572

TX SEYMOUR 3NW      XSE1               33 38N  099 17W  397

TX KNOX CITY 3NW    XKN1               33 27N  099 52W  448

TX PINE SPRNGS GMNP XGU1               31 53N  104 49W 1698

TX ODELL 4ENE       XOD1               34 22N  099 21W  418

TX COYANOSA 2N      XCO1               31 17N  103 04W  783

TX QUITAQUE 3NNW    XQU1               34 25N  101 04W  808

UT SALT LAKE CITY   KSLC  SLC   72572  40 47N  111 58W 1286

UT BLANDING         K4BL  4BL   72472  37 37N  109 28W 1840

UT BRYCE CANYON     KBCE  BCE          37 42N  112 09W 2312

UT CEDAR CITY       KCDC  CDC          37 42N  113 06W 1714

UT DELTA            KU24  U24   72479  39 20N  112 35W 1414

UT DUGWAY PRVG GROU KDPG  DPG   74003  40 12N  112 55W 1326

UT GREEN RIVER RANG KU28  U28   72477  39 00N  110 10W 1241

UT HANKSVILLE       K4HV  4HV   72473  38 22N  110 43W 1314

UT HEBER/RUSS MCDON K36U  36U          40 29N  111 26W 1718

UT HILL AFB/OGDEN   KHIF  HIF          41 07N  111 58W 1459

UT LOGAN            KLGU  LGU          41 47N  111 51W 1355

UT MILFORD          KMLF  MLF   72475  38 25N  113 01W 1534

UT MOAB             KCNY  CNY          38 46N  109 45W 1390

UT OGDEN            KOGD  OGD   72575  41 12N  112 01W 1353

UT PRICE            KPUC  PUC   72470  39 37N  110 45W 2091

UT PROVO MUNI       KPVU  PVU          40 13N  111 43W 1369

UT ROOSEVELT        KU67  U67   74420  40 17N  109 58W 1556

UT SAINT GEORGE     KDXZ  DXZ          37 05N  113 36W  896

UT SALT LAKE MUNI   KU42  U42          40 37N  112 00W 1402

UT TOOELE           KT62  T62          40 19N  112 17W 1628

UT VERNAL           KVEL  VEL          40 27N  109 31W 1604

UT WENDOVER (AUT)   KENV  ENV   72581  40 43N  114 01W 1292

VA ROANOKE          KROA  ROA   72411  37 19N  079 58W  362

VA ABINGDON         KVJI  VJI          36 40N  082 01W  631

VA BLACKSBURG/VA T. KBCB  BCB          37 13N  080 25W  650

VA CHARLOTTESVILLE  KCHO  CHO          38 08N  078 27W  192

VA CHESAPEAKE       KCPK  CPK   99402  36 40N  076 19W    6

VA CULPEPER         KCJR  CJR          38 32N  077 52W   97

VA DANVILLE         KDAN  DAN          36 34N  079 20W  175

VA DUBLIN/NEW RIV   KPSK  PSK          37 07N  080 40W  642

VA EMPORIA          KEMV  EMV          36 41N  077 29W   39

VA FARMVILLE        KFVX  FVX          37 21N  078 25W  125

VA FENTRESS NAVAL   KNFE  NFE          36 42N  076 08W    5

VA FORT BELVOIR/DAV KDAA  DAA          38 43N  077 10W   21

VA FRANKLIN/J B ROS KFKN  FKN          36 42N  076 54W   12

VA FREDERICKSB/SHAN KEZF  EZF          38 16N  077 27W   26

VA FT EUSTIS/FELKER KFAF  FAF          37 07N  076 37W    4

VA HILLSVILLE/GALAX KHLX  HLX          36 46N  080 49W  834

VA HOT SPRINGS/INGA KHSP  HSP          37 57N  079 49W 1156

VA JONESVILLE       K0VG  0VG          36 39N  083 13W  430

VA LANGLEY AFB/HAMP KLFI  LFI   74598  37 04N  076 22W    3

VA LEESBURG/GODFREY KJYO  JYO          39 04N  077 34W  119

VA LOUISA           KLKU  LKU          38 01N  077 58W  150

VA LURAY            KLUA  LUA          38 40N  078 30W  275

VA LYNCHBURG        KLYH  LYH   72410  37 19N  079 12W  295

VA MANASSAS MUNI    KHEF  HEF          38 43N  077 31W   59

VA MARION / WYTHEVI KMKJ  MKJ          36 53N  081 20W  780

VA MARTINSVILLE     KMTV  MTV          36 38N  080 01W  287

VA MELFA/ACCOMACK   KMFV  MFV          37 38N  075 46W   15

VA NEWPORT NEWS     KPHF  PHF          37 08N  076 30W   12

VA NORFOLK NAS/CHAM KNGU  NGU          36 55N  076 16W    5

VA NORFOLK          KORF  ORF   72308  36 54N  076 12W   14

VA NORFOLK/WAKEFLD  KAKQ  AKQ          36 59N  077 00W   34

VA NORFOLK/HAMPTON  KPVG  PVG          36 47N  076 27W    7

VA OCEANA NAS/SOUCE KNTU  NTU          36 49N  076 01W    7

VA ORANGE           KOMH  OMH          38 15N  078 03W  142

VA PETERSBURG       KPTB  PTB          37 10N  077 31W   59

VA QUANTICO MCAF    KNYG  NYG          38 30N  077 17W    4

VA RICHMOND/HANOVER KOFP  OFP          37 42N  077 26W   62

VA RICHMOND/CHESTER KFCI  FCI          37 24N  077 31W   72

VA RICHMOND         KRIC  RIC   72401  37 31N  077 19W   50

VA SOUTH HILL/MECKL KAVC  AVC          36 41N  078 02W  135

VA STAFFORD REGNL   KRMN  RMN          38 24N  077 27W   65

VA STAUNTON/SHENAND KSHD  SHD          38 16N  078 54W  366

VA SUFFOLK          KSFQ  SFQ          36 41N  076 36W   22

VA RICHLANDS        KJFZ  JFZ          37 04N  081 48W  809

VA TAPPAHANNOCK     KXSA  XSA          37 52N  076 54W   42

VA WALLOPS ISLAND   KWAL  WAL   72402  37 56N  075 28W   14

VA WARRENTON        KHWY  HWY          38 35N  077 43W  103

VA WASH DC/DULLES   KIAD  IAD   72403  38 56N  077 27W   93

VA WEST POINT       KFYJ  FYJ          37 31N  076 46W    7

VA WILLIAMSBURG     KJGG  JGG          37 14N  076 43W   15

VA WINCHESTER RGNL  KOKV  OKV          39 08N  078 09W  222

VA WISE/LONESOME PI KLNP  LNP          36 58N  082 31W  817

VT BENNINGTON       KDDH  DDH   72516  42 53N  073 15W  244

VT BURLINGTON       KBTV  BTV   72617  44 28N  073 09W  105

VT HIGHGATE         KFSO  FSO          44 56N  073 06W   70

VT LYNDONVILLE      KCDA  CDA          44 34N  072 01W  363

VT MORRISVILLE      KMVL  MVL          44 30N  072 37W  234

VT MONTPELIER/BARRE KMPV  MPV          44 12N  072 34W  343

VT NEWPORT STATE    KEFK  EFK   72612  44 53N  072 14W  284

VT RUTLAND STATE    KRUT  RUT          43 31N  072 57W  240

VT SPRINGFIELD      KVSF  VSF          43 21N  072 31W  175

VT ST. JOHNSBURY    K1V4  1V4   72614  44 25N  072 01W  210

WA ARLINGTON MUNI   KAWO  AWO          48 10N  122 10W   42

WA BELLINGHAM       KBLI  BLI          48 48N  122 32W   50

WA BREMERTON NTNL   KPWT  PWT          47 30N  122 45W  147

WA BURLINGTON/MT V  KBVS  BVS   74200  48 28N  122 25W   43

WA CHEHALIS CENTRAL KCLS  CLS          46 41N  122 59W   54

WA COLVILLE MUNICIP KCQV  CQV          48 32N  117 52W  572

WA DEER PARK        KDEW  DEW   72787  47 58N  117 26W  674

WA EASTSOUND/ORCAS  KORS  ORS          48 42N  122 55W    9

WA ELLENSBURG       KELN  ELN          47 02N  120 32W  519

WA EPHRATA          KEPH  EPH          47 18N  119 31W  383

WA EVERETT          KPAE  PAE          47 55N  122 17W  180

WA FAIRCHILD AFB    KSKA  SKA          47 37N  117 39W  750

WA FORT LEWIS/GRAY  KGRF  GRF   74207  47 04N  122 34W   92

WA FRIDAY HARBOR    KFHR  FHR          48 31N  123 02W   32

WA HANFORD          KHMS  HMS   72784  46 34N  119 35W  223

WA HOQUIAM          KHQM  HQM          46 58N  123 56W    7

WA KELSO LONGVEIW   KKLS  KLS          46 07N  122 54W    5

WA MOSES LAKE       KMWH  MWH          47 12N  119 19W  362

WA OAK HARBOR AIRPA KOKH  OKH   72796  48 15N  122 40W   59

WA OLYMPIA          KOLM  OLM   72792  46 58N  122 54W   58

WA OMAK             KOMK  OMK   72789  48 28N  119 31W  395

WA PASCO            KPSC  PSC          46 16N  119 07W  121

WA PORT ANGELES     KCLM  CLM          48 07N  123 30W   85

WA PULLMAN/MOSCOW   KPUW  PUW          46 45N  117 07W  773

WA PUYALLUP/THUN    KPLU  PLU          47 06N  122 17W  164

WA QUILLAYUTE       KUIL  UIL   72797  47 56N  124 33W   54

WA RENTON           KRNT  RNT          47 30N  122 13W   21

WA SEATTLE/BOEING   KBFI  BFI          47 33N  122 19W    4

WA SEATTLE/METRO    KSEA  SEA   72793  47 27N  122 19W  136

WA SHELTON          KSHN  SHN          47 14N  123 08W   82

WA SPOKANE/FELTS    KSFF  SFF          47 41N  117 19W  609

WA SPOKANE/METRO    KGEG  GEG   72785  47 37N  117 32W  735

WA STAMPEDE PASS    KSMP  SMP          47 17N  121 20W 1208

WA TACOMA           KTIW  TIW          47 16N  122 35W   89

WA TACOMA/MC CHORD  KTCM  TCM   74206  47 07N  122 28W   98

WA TOLEDO WINLOCK M KTDO  TDO          46 28N  122 47W  113

WA VANCOUVER        KVUO  VUO          45 37N  122 39W    8

WA WALLA WALLA      KALW  ALW   72788  46 06N  118 17W  363

WA WENATCHEE        KEAT  EAT          47 24N  120 12W  377

WA WHIDBEY IS. NAS  KNUW  NUW   69023  48 21N  122 39W   14

WA YAKIMA           KYKM  YKM   72781  46 34N  120 32W  324

WI ANTIGO/LANGLADE  KAIG               45 10N  089 07W  464

WI APPLETON/OUTAGAM KATW  ATW          44 15N  088 31W  280

WI ASHWAUBENON      KGRB  GRB   72645  44 29N  088 08W  208

WI ASHLAND          KASX  ASX          46 33N  090 55W  251

WI BLACK RIVER FALL KBCK  BCK          44 15N  090 51W  255

WI BOSCOBEL         KOVS  OVS          43 09N  090 41W  202

WI BURLINGTON       KBUU  BUU          42 41N  088 18W  237

WI CLINTONVILLE     KCLI  CLI          44 36N  088 43W  250

WI CUMBERLAND       KUBE  UBE          45 30N  091 59W  379

WI DELLS/BARABOO    KDLL  DLL          43 31N  089 46W  299

WI EAGLE RIVER      KEGV  EGV          45 55N  089 16W  500

WI EAU CLAIRE       KEAU  EAU          44 52N  091 28W  276

WI FOND DU LAC      KFLD  FLD          43 46N  088 29W  240

WI HAYWARD          KHYR  HYR          46 01N  091 27W  370

WI JANESVILLE/ROCK  KJVL  JVL          42 37N  089 01W  246

WI JUNEAU           KUNU  UNU          43 25N  088 42W  285

WI KENOSHA          KENW  ENW          42 36N  087 56W  219

WI LA CROSSE        KLSE  LSE   72643  43 53N  091 15W  199

WI LADYSMITH/RUSK   KRCX  RCX          45 30N  091 00W  377

WI LAND O LAKES     KLNL  LNL          46 09N  089 13W  520

WI LONE ROCK        KLNR  LNR          43 13N  090 11W  217

WI MADISON          KMSN  MSN   72641  43 08N  089 21W  261

WI MANITOWAC MUNI   KMTW  MTW          44 07N  087 40W  198

WI MARSHFIELD       KMFI  MFI          44 38N  090 11W  379

WI MEDFORD          KMDZ  MDZ          45 06N  090 17W  448

WI MENOMONIE MUNI   KLUM  LUM          44 54N  091 52W  273

WI MERRILL          KRRL  RRL          45 12N  089 43W  401

WI MIDDLETON        KC29  C29          43 07N  089 32W  283

WI MILWAUKEE        KMKE  MKE          42 57N  087 54W  206

WI MILWAUKEE/LAWREN KMWC  MWC          43 07N  088 01W  227

WI MINERAL POINT    KMRJ  MRJ          42 53N  090 13W  359

WI MINOCQUA/WOODRUF KARV  ARV          45 55N  089 43W  496

WI MONROE           KEFT  EFT          42 36N  089 35W  331

WI MOSINEE/CENTRAL  KCWA  CWA          44 46N  089 40W  389

WI NEW RICHMOND MUN KRNH  RNH          45 09N  092 32W  304

WI OSCEOLA          KOEO  OEO          45 18N  092 41W  275

WI OSHKOSH          KOSH  OSH          43 58N  088 33W  248

WI PHILLIPS/PRICE   KPBH  PBH          45 42N  090 24W  449

WI PLATTEVILLE      KPVB  PVB          42 41N  090 27W  313

WI PRAIRIE DU CHIEN KPDC  PDC          43 01N  091 07W  201

WI RACINE           KRAC  RAC          42 46N  087 49W  203

WI RHINELANDER      KRHI  RHI          45 38N  089 29W  485

WI RICE LAKE        KRPD  RPD          45 25N  091 46W  338

WI SHAWANO MUNI     KEZS  EZS          44 47N  088 34W  248

WI SHEBOYGAN        KSBM  SBM   99433  43 47N  087 51W  232

WI SIREN            KRZN  RZN          45 49N  092 22W  301

WI SPARTA/MCCOY AAF KCMY  CMY          43 58N  090 44W  256

WI STEVENS POINT    KSTE  STE          44 32N  089 31W  338

WI STURGEON BAY     KSUE  SUE          44 51N  087 25W  221

WI SUPERIOR         KSUW  SUW          46 41N  092 06W  206

WI TOMAHAWK REGNL   KTKV  TKV          45 28N  089 48W  453

WI VIROQUA MUNI     KY51  Y51          43 35N  090 54W  394

WI VOLK/CAMP DOUGLA KVOK  VOK          43 55N  090 16W  277

WI WATERTOWN        KRYV  RYV          43 10N  088 43W  254

WI WAUKESHA CNTY    KUES  UES          43 02N  088 14W  278

WI WAUPACA          KPCZ  PCZ          44 20N  089 01W  252

WI WAUSAU           KAUW  AUW   72646  44 56N  089 37W  362

WI WAUTOMA          KY50  Y50          44 02N  089 18W  262

WI WEST BEND        KETB  ETB          43 25N  088 08W  270

WI WISCONSIN RAPIDS KISW  ISW          44 22N  089 50W  307

WV BECKLEY          KBKW  BKW   72412  37 48N  081 07W  764

WV BLUEFIELD        KBLF  BLF          37 18N  081 12W  873

WV BUCKHANNON       KW22  W22          39 00N  080 16W  499

WV CHARLESTON       KCRW  CRW   72414  38 23N  081 35W  309

WV CLARKSBURG       KCKB  CKB          39 18N  080 13W  360

WV ELKINS           KEKN  EKN   72417  38 53N  079 51W  603

WV HUNTINGTON       KHTS  HTS   72425  38 22N  082 33W  254

WV LEWISBURG/GREENB KLWB  LWB          37 52N  080 24W  702

WV MARTINSBURG      KMRB  MRB          39 24N  077 59W  164

WV MORGANTOWN       KMGW  MGW          39 39N  079 55W  378

WV PARKERSBURG      KPKB  PKB   72427  39 21N  081 25W  262

WV PT PLEASANT      K3I2  3I2          38 55N  082 06W  196

WV PETERSBURG       KW99  W99          39 00N  079 09W  293

WV PINEVILLE        KI16  I16          37 36N  081 34W  544

WV SUTTON/BRAXTON   K48I  48I          38 41N  080 39W  397

WV WHEELING         KHLG  HLG          40 10N  080 39W  372

WV WHITE SULPHUR SP KSSU  SSU          37 46N  080 19W  549

WY BIG PINEY        KBPI  BPI   72671  42 35N  110 06W 2117

WY BUFFALO          KBYG  BYG          44 23N  106 43W 1497

WY CASPER           KCPR  CPR   72569  42 54N  106 28W 1621

WY CHEYENNE         KCYS  CYS   72564  41 09N  104 49W 1868

WY CODY             KCOD  COD   72670  44 31N  109 01W 1553

WY DOUGLAS          KDGW  DGW   72568  42 48N  105 23W 1499

WY ELK MOUNTAIN     KEHY  EHY          41 44N  106 28W 2225

WY EVANSTON         KEVW  EVW          41 16N  111 02W 2177

WY GILLETTE         KGCC  GCC   72665  44 20N  105 33W 1320

WY GREYBULL         KGEY  GEY          44 31N  108 05W 1191

WY HULETT MUNI      KW43  W43          44 40N  104 34W 1300

WY JACKSON          KJAC  JAC          43 37N  110 44W 1961

WY LANDER           KLND  LND   72576  42 49N  108 44W 1694

WY LARAMIE          KLAR  LAR          41 19N  105 40W 2216

WY NEWCASTLE MONDEL KECS  ECS          43 53N  104 19W 1273

WY PINEDALE R WENZ  KPNA  PNA          42 48N  109 48W 2160

WY RAWLINS MUNICIPA KRWL  RWL          41 47N  107 12W 2077

WY RIVERTON         KRIW  RIW   72672  43 04N  108 28W 1697

WY ROCK SPRINGS     KRKS  RKS   72574  41 36N  109 04W 2060

WY SHERIDAN         KSHR  SHR   72666  44 46N  106 58W 1202

WY TORRINGTON       KTOR  TOR          42 04N  104 09W 1277

WY WORLAND          KWRL  WRL          43 58N  107 57W 1271

WY YELLOWSTONE LAKE KP60  P60          44 33N  110 25W 2388

WY BORDEAUX         KBRX  BRX          41 57N  104 57W 1524

WY CONTINENTAL DIV  KCTD  CTD          41 43N  107 47W 2146

WY VEDAUWOO         KVDW  VDW          41 09N  105 24W 2542

WY PUMPKIN VINE     KPUM  PUM          41 03N  105 28W 2420

WY ARLINGTON        KARL               41 36N  106 13W 2179

WY BITTERCREEK      KBIT  BIT          41 39N  108 35W 2160

WY CEMETERY SEPARA  KCMS  CMS          41 32N  109 28W 1949

WY FIRST DIVIDE     KFIR  FIR          41 18N  110 46W 2294

WY I-25 DIVIDE      KIDV  IDV          43 56N  106 39W 1531

WY 20 MILE HILL     KTMH  TMH          43 07N  106 20W 1743

WY DEAD HORSE       KDHS  DHS          44 13N  106 06W 1241

WY INYAN KARA       KIKA  IKA          44 18N  104 38W 1319

WY BELLE FOURCHE    KBFU  BFU          43 56N  105 27W 1422

WY WHITAKER         KWTR  WTR          41 25N  104 52W 1882

WY PATHFINDER HILL  KPAT               42 34N  106 51W 1912

WY BEAVER RIM       KBVR  BVR          42 35N  108 17W 2072

WY SIBLEY PEAK      KSIB  SIB          42 27N  105 02W 1500

WY PINEY CREEK      KPIN  PIN          44 34N  106 49W 1392

WY HILAND           KHLD  HLD          43 06N  107 19W 1846

WY DEER CREEK       KDRC  DRC          42 50N  105 52W 1543

WY SKYLINE          KSKL  SKL          41 07N  106 34W 2451

WY RIM / PINEDALE   KRIM  RIM          43 06N  110 19W 2294

WY SAGE / JUNCTION  KSGE  SGE          41 47N  110 30W 2354

WY GUNBARREL        KGUN  GUN          41 26N  104 21W 1695

WY CHIEF JOSEPH     KCHJ  CHJ          44 45N  109 22W 2512

WY SHUTE CREEK      KSHC  SHC          41 56N  109 58W 2043

WY MAMMOTH/YELLOWST KMMM  MMM          44 59N  110 42W 1902

WY TOWER FALLS      KTOW  TOW          44 54N  110 23W 1800

WY MEETEETSE RIM    KMTR               44 16N  108 52W 1803

DC WASHINGTON/NATL  KDCA  DCA   72405  38 51N  077 02W   18

# Appendix B

This appendix lists the location details of all Continental US upper air locations whose data are potentially in the prepBUFR observation dataset that was used in this evaluation study.

CD  STATION         ICAO  IATA  SYNOP   LAT     LONG   ELEV

AL BRMNGHM/ALABASTR KBMX  BMX   72230  33 10N  086 46W  197

AR N. LITTLE ROCK   KLZK  LZK   72340  34 50N  092 16W  173

AZ FLAGSTAFF/BELLEM KFGZ  FGZ   72376  35 14N  111 49W 2192

AZ PHOENIX/WFO      KPSR  PSR   74626  33 26N  112 01W  342

AZ TUCSON/WFO       KTWC  TWC   72274  32 14N  110 57W  752

CA EDWARDS AFB      KEDW  EDW   72381  34 53N  117 52W  702

CA MIRAMAR NAS/SAN  KNKX  NKX   72293  32 52N  117 08W  146

CA OAKLAND          KOAK  OAK   72493  37 42N  122 13W   26

CA VANDENBERG AFB   KVBG  VBG   72393  34 44N  120 35W  112

CO DENVER/STAPLETON KDNR  DNR   72469  39 45N  104 52W 1608

CO GRAND JUNCTION   KGJT  GJT   72476  39 07N  108 31W 1475

FL CAPE CANAVERAL   KXMR  XMR   74794  28 28N  080 32W    3

FL JACKSONVILLE     KJAX  JAX   72206  30 30N  081 41W   10

FL KEY WEST         KKEY  KEY          24 33N  081 47W    5

FL MIAMI/WFO        KMFL  MFL   72202  25 45N  080 22W    5

FL TALLAHASSEE      KTLH  TLH   72214  30 24N  084 21W   19

FL TAMPA BAY/RUSKIN KTBW  TBW   72210  27 42N  082 24W   12

GA PEACHTREE CITY   KFFC  FFC   72215  33 21N  084 34W  262

IA DAVENPORT/QUAD C KDVN  DVN   74455  41 37N  090 35W  230

ID BOISE            KBOI  BOI   72681  43 34N  116 14W  875

IL LINCOLN          KILX  ILX   74560  40 09N  089 20W  177

KS DODGE CITY       KDDC  DDC   72451  37 46N  099 58W  789

KS TOPEKA           KTOP  TOP   72456  39 04N  095 38W  268

LA LAKE CHARLES     KLCH  LCH   72240  30 08N  093 13W    4

LA SHREVEPORT       KSHV  SHV   72248  32 27N  093 50W   83

LA SLIDELL/88D      KLIX  LIX   72233  30 20N  089 50W    7

MA CHATHAM          KCHH  CHH   74494  41 40N  069 58W   16

ME CARIBOU          KCAR  CAR   72712  46 52N  068 01W  191

ME GRAY/PORTLAND    KGYX  GYX   74389  43 53N  070 15W  125

MI DETROIT/WHITE LK KDTX  DTX   72632  42 42N  083 28W  327

MI GAYLORD/ALPENA   KAPX  APX   72634  44 54N  084 43W  446

MN CHANHASSEN       KMPX  MPX   72649  44 51N  093 34W  288

MN INTERNTNL FALLS  KINL  INL   72747  48 34N  093 24W  360

MO SPRINGFIELD      KSGF  SGF   72440  37 14N  093 23W  390

MS JACKSON          KJAN  JAN   72235  32 19N  090 05W   91

MT GLASGOW          KGGW  GGW   72768  48 13N  106 37W  694

MT GREAT FALLS/88D  KTFX  TFX   72776  47 28N  111 23W 1132

NC GREENSBORO       KGSO  GSO   72317  36 06N  079 57W  275

NC MOREHEAD/NEWPORT KMHX  MHX   72305  34 46N  076 52W    9

ND BISMARCK         KBIS  BIS   72764  46 46N  100 45W  505

NE NORTH PLATTE     KLBF  LBF   72562  41 07N  100 40W  847

NE OMAHA/VALLEY/88D KOAX        72558  41 19N  096 22W  350

NM ALBUQUERQUE      KABQ  ABQ   72365  35 03N  106 37W 1618

NM SANTA TERESA/88D KEPZ  EPZ   72364  31 52N  106 42W 1251

NV ELKO/WFO         KLKN  LKN   72582  40 52N  115 43W 1608

NV LAS VEGAS/WFO    KVEF  VEF   72388  36 03N  115 11W  694

NV RENO/WFO         KREV  REV   72489  39 34N  119 48W 1516

NY ALBANY           KALB  ALB   72518  42 45N  073 48W   92

NY BROOKHAVEN/UPTON KOKX  OKX   72501  40 52N  072 52W   26

NY BUFFALO/CHEEKTOW KBUF  BUF   72528  42 56N  078 44W  211

OH WILMINGTON       KILN  ILN   72426  39 26N  083 48W  322

OK NORMAN/WESTHEIME KOUN  OUN   72357  35 15N  097 28W  357

OR MEDFORD          KMFR  MFR   72597  42 23N  122 52W  396

OR SALEM            KSLE  SLE   72694  44 54N  123 00W   59

PA PITTSBURGH       KPIT  PIT   72520  40 30N  080 16W  357

SC CHARLESTON       KCHS  CHS   72208  32 54N  080 02W   13

SD ABERDEEN         KABR  ABR   72659  45 27N  098 25W  397

SD RAPID CITY/WFO   KUNR  UNR   72662  44 04N  103 12W 1027

TN NASHVILLE        KBNA  BNA   72327  36 07N  086 41W  210

TX AMARILLO         KAMA  AMA   72363  35 13N  101 42W 1093

TX BROWNSVILLE      KBRO  BRO   72250  25 55N  097 25W    7

TX CORPUS CHRISTI   KCRP  CRP   72251  27 46N  097 30W   14

TX DEL RIO          KDRT  DRT   72261  29 22N  100 55W  313

TX FORT WORTH       KFWD  FWD   72249  32 49N  097 17W  196

TX MIDLAND          KMAF  MAF   72265  31 57N  102 12W  874

UT SALT LAKE CITY   KSLC  SLC   72572  40 47N  111 58W 1286

VA ROANOKE/BLACKSBG KRNK  RNK   72318  37 12N  080 24W  648

VA WALLOPS ISLAND   KWAL  WAL   72402  37 56N  075 28W   14

VA WASH DC/DULLES   KIAD  IAD   72403  38 56N  077 27W   93

WA QUILLAYUTE       KUIL  UIL   72797  47 56N  124 33W   54

WA SPOKANE          KOTX  OTX   72786  47 41N  117 38W  727

WI ASHWAUBENON      KGRB  GRB   72645  44 29N  088 08W  208

WY RIVERTON         KRIW  RIW   72672  43 04N  108 28W 1697

# References

Gilliam, R., and J. Pleim, 2010. Performance assessment of the Plein-Xiu LSM, Pleim surface-layer and ACM PBL Physics in version 3.0 of WRF-ARW (accepted by Journal of Applied Meteorology and Climate)

_____, and T. Otte, 2009. Multiscale meteorological modeling for air quality applications, EPA Atmospheric and Modeling Analysis Division Peer Review Conference, January 27–29, Research Triangle Park, NC

Gupta, S., R. McNider, M. Trainer, R. Zamora, K. Knupp, and M. Singh, 1997. Nocturnal wind structure and plume growth rates due to inertial oscillations, J. App. Meteor., 36, 1050–1063

SEMAP, 2010: Protocol for WRF Meteorological Modeling in Support of Regional SIP Air Quality Modeling in the SEMAP, draft 20 August 2010.

Skamarock, W., J. Klemp, J. Dudhia, D. Gill, D. Barker, M. Duda, X.-Y. Huang, W. Wang, and  J. Powers, July 2008. A description of the Advanced Research WRF Version 3, NCAR Tech. Note, NCAR TN-475+STR, 113 pp.

Stauffer, D., N. Seaman, and F. Binkowski, 1991. Use of four-dimensional data assimilation in a limited-area mesoscale model. Part II: Effects of data assimilation within the planetary boundary layer. Mon. Wea. Rev., 119, 734–754

Wang, W., C. Bruyère, M. Duda, J. Dudhia, D. Gill, H.-C. Lin, J. Michalakes, S. Rizvi, and X. Zhang, 2009. Weather Research and Forecasting ARW Users Manual (NCAR) 310 pp.